IEEE P802.11
Wireless LANs

Lucent/Harris TGb proposal comparison matrix input

Date:	           June, 25, 1998

Author: 	Jan Boer / Carl Andren

Input for the comparison matrix provided by Lucent and Harris for their compromise proposal.

General description:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Modulation Technique |  | 16-CCK complementary code keying 16 chip spreading |  |
| Data Rate(s) |  | 1,2,5 and 10 Mbit/s |  |
| Sensitivity |  | -91 dBm @ 5Mbit/s -88 dBm @ 10Mbit/s |  |
| Reference submissions |  | Harris MBOK doc 70254, 70867, 71447, 80467B, 80477B, 97/124 Lucent PPM doc 98/10r1 98/11 98/99 98/100 Combined doc 98/246 new doc 98/xxxx draft text |  |

Receiver structure:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Receiver structure description |  | Matched filter Decoding simple with Hadamar transforms simple DFE possible to increase performance |  |
| RF/IF complexity relative to current low rate PHYs. |  | Same as low rate PHYs |  |
| Baseband processing complexity. relative to current low rate PHYs. (Gate Count, MIPS) |  | twice low rate PHYs for moderate complexity receiver, with MF (not required) complexity trade off for performance |  |
| Equaliser Complexity and performance impact (if applicable). |  | 40K gates. Will improve delay spread from 100 ns to 300 ns. This is roughly double the gate count |  |
| Antenna Diversity and performance impact. |  | Same possibilities as low rate PHY with long PLCP header. |  |

Multipath and Noise performance:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # PER vs. multipath RMS delay spread (no noise). Delay spread @ 10% PER for 64 and 1000 byte packets. |  | 10 Mbit/s RAKE 64 byte: 90 ns 1K byte: 65 ns RAKE-ISI Equaliser 64 byte: 144 ns 1K byte: 87 ns RAKE-ICI/ISI Equaliser 64 byte: 333 ns 1 kbyte: 226 ns 5 Mbit/s RAKE 64 byte: 235 ns 1 kbyte: 180 ns RAKE-ISI Equaliser 64 byte: 450 ns 1 kbyte: 380 ns RAKE-ICI/ISI Equaliser This configuration is not needed |  |
| PER vs. thermal noise w/ multipath @ 10% PER. Eb/No @ 20% PER for 64 and 1000 byte packets. |  | 10 Mbit/s RAKE 64 byte: 15.2 dB 1 kbyte: 17.5 dB RAKE-ISI Equaliser 64 byte: 15 dB 1 kbyte: 17.5 dB RAKE-ICI/ISI Equaliser 64 byte: 15.5 dB 1 kbyte: 17.7 dB 5 Mbit/s RAKE 64 byte: 15 dB 1 kbyte: 17 dB RAKE-ISI Equaliser 64 byte: 16 dB 1 kbyte: 18 dB RAKE-ICI/ISI Equaliser Not needed |  |
| PER vs. thermal noise (no multipath). Eb/No @ 10% PER for 64 and 1000 byte packets. |  | 10 Mbit/s RAKE 64 byte: 5.5 dB 1 kbyte: 7 dB RAKE-ISI Equaliser 64 byte: 5.5 dB 1 kbyte: 7 dB RAKE-ICI/ISI Equaliser 64 byte: 5.5 dB 1 kbyte: 7 dB 5 Mbit/s RAKE 64 byte: 5.5 dB 1 kbyte: 7 dB RAKE-ISI Equaliser 64 byte: 5.5 dB 1 kbyte: 7 dB RAKE-ICI/ISI Equaliser 64 byte: 5.5 dB 1 kbyte: 7 dB |  |

Carrier and Data frequency accuracy:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Required Carrier frequency accuracy. |  | 25 PPM = low rate PHYs |  |
| Degradation at worst case carrier frequency offset. |  | Negligible with carrier tracking Similar to low rate PHYs Easy carrier tracking, but non coherent processing OK |  |
| Data clock frequency accuracy. |  | 25 PPM |  |
| Degradation at worst case data clock frequency offset. |  | CMF gives optimal timing Tracking circuits should compensate |  |

Overhead related parameters:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Preamble length |  | Long preamble + header = 192 microseconds Short preamble + header = 75 microseconds |  |
| Does the preamble length include receive antenna diversity? Yes or no. |  | Long preamble, same as low rate PHY: yes Short preamble: yes 30 Microseconds (1.5 slottime) reserved for diversity |  |
| Does the preamble length include equaliser training? Yes or no. |  | Long preamble: yes Short preamble: yes (24 microseconds reserved) |  |
| Slot time. |  | = low rate PHY 20 microseconds |  |
| CCA mechanism description. |  | = low rate PHY |  |
| Co-Channel signal detection time. |  | Energy detect time = current PHY 15 microseconds |  |
| RX/TX turnaround time. |  | = low rate PHY 5 microseconds. |  |
| SIFS. |  | = low rate PHY 10 microseconds |  |

Spectral efficiency, Cell density related parameters:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Channelization scheme |  | = low rate PHY |  |
| Cell planing scheme |  | = low rate PHY 3 independent channels |  |
| Adjacent channel interference rejection. |  | 32-35 dB |  |
| Co-channel interference rejection. |  | 6dB |  |
| S/J where CW interference gives 10% PER. |  | 8 dB at 10 Mbit/s, 5 dB with 5 Mbit/s |  |
| Other interference immunity tests. |  | GFSK immunity is the same as CW |  |
| Co-Channel signal detection time. |  | = low rate PHY |  |
| Total number of channels in 2.4GHz band. |  | = low rate PHY, 3 colocated channels for FCC or ETSI Total tuneable channels: FCC: 11 ETSI: 13 MTP: 1 |  |
| Aggregate throughput. |  | Dependent on cell topology. e.g. three channels in one cell gives 3 * throughput or 30 Mbit/s |  |

# Misc. critical performance factors:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Phase noise sensitivity |  | comparable to low rate PHY (QPSK) |  |
| RF PA backoff |  | 4-5dB |  |
| DC power consumption |  | Comparable to low rate PHY PCMCIA form factor and spec. TX < 300mA @ 3V RX < 250A @ 3V |  |

Interoperability:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Interoperability / Co-existence strategy with current low rate PHYs |  | Long Preamble: interoperable and coexistent Optional short preamble: low rate PHY is coexistent with transmitter using short preamble and high rate receiver recognises both long and short preamble : interoperable Interoperable with FH using FH header |  |
| Is the proposal Interoperable at the data level? |  | Yes |  |
| Is the proposal Interoperable at the antenna level? |  | yes |  |
| Performance penalty due to Interoperability / Coexistence. |  | Long preamble: 192 micro PHY overhead Short preamble: overhead reduces with factor 3 |  |

General Information:

|  |  | Lucent/Harris |  |
| --- | --- | --- | --- |
| # Has the submission of the required IEEE letter covering IP been made? Yes or No |  | yes |  |
| Applicable patent numbers |  | none |  |
| Point of contact |  | Lucent: Bruce Tuch Lucent Technologies PO Box 755 3430 AT Nieuwegein, The Netherlands tel: +31 30 6097527 fax: +31 30 6097556 Al Petrick Harris Semiconductor PO Box 883 Melbourne, FL 32905 tel: 407-729-4944 fax: 407-724-7886 |  |