# Technical Requirements

A summary of key technical requirements for the RF front ends for all three specifications appears below:

| Parameter | IEEE 802.11 FHSS | HRFWG SWAP CA | Bluetooth |
| --- | --- | --- | --- |
| Operating Frequency | 2.400 - 2.4835 GHz | 2.400 - 2.4835 GHz | 2.400 - 2.4835 GHz |
| Spread Spectrum Method | FHSS | FHSS | FHSS |
| Data Rate | 1 Mbps 2 Mbps | 1 Mbps 2 Mbps | 1 Mbps 2 Mbps (future) |
| Modulation Method | 2-FSK 4-FSK (optional) | 2-FSK 4-FSK (required) | 2-FSK TBD |
| Modulation Index (h) | 0.32 0.16 |  | 0.32 TBD |
| Effective Filter BW | Gaussian (BT = 0.5) |  | ????? |
| Hop Rate | 2.5 Hz | 50 Hz | 1600 Hz (max) |
| Channel Switch Time | 224 microsec | 300 microsec | 220 microsec |
| Rx/Tx Turnaround Time | 19 microsec | 25 microsec | 220 microsec |
| Antenna Diversity | Optional |  | Not Required |
| Tx RF Power | <1W (US) 100 mW (Europe & Japan) | 100 mW (North America) | 1 mW |
| Rx Sensitivity | -80 dBm @ 1 Mbps -75 dBm @ 2 Mbps | -76 dBm @ 1 Mbps | -70 dBm @ 1 Mbps |
| Tx Channel Freq. Tol. | +/- 60 kHz |  | ? |
| Tx Spectrum Shape | ¦N-M¦ = 2 -20 dBm or -40 dBc ¦N-M¦ > 3 -40 dBm or -60 dBc |  | ? |

Radio Requirements for IEEE802.11, HRFWG, and Bluetooth