IEEE P802.11
Wireless LANs

Micrilor TGb proposal comparison matrix

Date:	June 29, 1998

Author: 	John Cafarella

General description:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Modulation Technique |  |  | # 16-ary DBOK, # 16-ary DBOK with # (15,13) R/S FEC, # 4x4-ary DBOK |
| Data Rate(s) |  |  | 10 Mbit/s (primary) and 18, 8.7, 2, 1 Mbit/s |
| Sensitivity |  |  | -88 dBm @10 Mbit/s -90 dBm @8.7 Mbit/s -83 dBm @18 Mbit/s |
| Reference submissions |  |  | 9750.ppt 9751.ppt 9752.ppt 9753.ppt 9782.ppt 9783.ppt 97116.doc 97117.doc 97118.doc 97119.doc 97120.doc 97128.ppt 97129.ppt 97130.ppt 97131.ppt 98016.doc 98017.doc 98018.doc 98019.doc 98117.ppt 98018r1.doc 98117r1.ppt 98206.ppt 98207.ppt 98209.ppt 98016r1.doc 98117r2.ppt |

Receiver structure:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Receiver structure description |  |  | # ML for Rayleigh channel; correlators for demodulation; correlator for acquisition; matched filter for CSMA; non-coherent receiver. |
| RF/IF complexity relative to current low rate PHYs. |  |  | Same as low rate PHYs |
| Baseband processing complexity. relative to current low rate PHYs. (Gate Count, MIPS) |  |  | Similar: approximately 40k gates including channel matched filter |
| Equaliser Complexity and performance impact (if applicable). |  |  | Not required; equalizer or MLSE techniques could be added for enhancement |
| Antenna Diversity and performance impact. |  |  | 1 or 2 dB improvement in SNR; adds 4 s to preamble. |

Multipath and Noise performance:

|  |  |  | # Micrilor |
| --- | --- | --- | --- |
| # PER vs. multipath RMS delay spread (no noise). Delay spread @ 10% PER for 64 and 1000 byte packets. |  |  | ## 10 Mbit/s ## 64 bbyte: 360 ns ## 1 kbyte: 280 ns |
| PER vs. thermal noise w/ multipath @ 10% PER. Eb/No @ 20% PER for 64 and 1000 byte packets. |  |  | ## 10 Mbit/s 64 bbyte @ 360 ns: 26 dB 1 kbyte @ 280 ns: 24.5 dB |
| PER vs. thermal noise (no multipath). Eb/No @ 10% PER for 64 and 1000 byte packets. |  |  | ## 10 Mbit/s ## 64 byte: 5.5dB ## 1 kbyte: 6.7dB ## 18 Mbit/s ## 64 byte: 8.5dB ## 1 kbyte: 9.7dB ## 8.7 Mbit/s ## 64 byte: 4.5dB ## 1 kbyte: 5.0dB |

Carrier and Data frequency accuracy:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Required Carrier frequency accuracy. |  |  | # 20 ppm; Recommend specify @ 10 ppm |
| Degradation at worst case carrier frequency offset. |  |  | <.2 dB @ 20 ppm; negligible @ 10 ppm; no tracking required in any case |
| Data clock frequency accuracy. |  |  | 10 ppm to reduce stress on tracking |
| Degradation at worst case data clock frequency offset. |  |  | < .3 dB @ 1/8-chip time-tracking error |

Overhead related parameters:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Preamble length |  |  | # 24 us |
| Does the preamble length include receive antenna diversity? Yes or no. |  |  | Yes |
| Does the preamble length include equaliser training? Yes or no. |  |  | Includes training of channel matched filter |
| Slot time. |  |  | 10 us |
| CCA mechanism description. |  |  | Detect 1-/2-Mbps DS or FH at –80 dBm |
| Co-Channel signal detection time. |  |  | 2 us for all DS TBD for FH |
| RX/TX turnaround time. |  |  | 2 us |
| SIFS. |  |  | 10 us |

Spectral efficiency, Cell density related parameters:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Channelization scheme |  |  | # = low rate PHY (frequency); # plus many code channels |
| Cell planing scheme |  |  | = low rate PHY 3 independent frequency channels; code channels used to isolate BSAs for frequency re-use |
| Adjacent channel interference rejection. |  |  | 35 dB |
| Co-channel interference rejection. |  |  | 12 dB |
| S/J where CW interference gives 10% PER. |  |  | 2 dB |
| Other interference immunity tests. |  |  | Same |
| Co-Channel signal detection time. |  |  | 2 us |
| Total number of channels in 2.4GHz band. |  |  | Identical to low rate DSSS PHY; 3 colocated channels for FCC or ETSI |
| Aggregate throughput. |  |  | Dependent on cell topology. e.g. three channels in one cell gives 3 * throughput or 30 Mbit/s; at short range could be 3*18=54 Mbit/s |

# Misc. critical performance factors:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Phase noise sensitivity |  |  | # N/A; noncoherent Rx |
| RF PA backoff |  |  | 2-5 dB |
| DC power consumption |  |  | Same as low rate PHY PCMCIA form factor TX < 300mA @ 3V RX < 250A @ 3V |

Interoperability:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Interoperability / Co-existence strategy with current low rate PHYs |  |  | # Prefix legacy preamble to enable CCA by FH or DS 1-/2-Mbps equipment. Multisignal CCA (ref: 97/128) enables CCA of FH and DS legacy transmissions. Thus, interoperable or coexisting when required, but single high-rate preamble. |
| Is the proposal Interoperable at the data level? |  |  | Yes |
| Is the proposal Interoperable at the antenna level? |  |  | Yes |
| Performance penalty due to Interoperability / Coexistence. |  |  | 192 us preamble added for DS interoperability mode when needed; not part of high-rate PHY. |

General Information:

|  |  |  | Micrilor |
| --- | --- | --- | --- |
| # Has the submission of the required IEEE letter covering IP been made? Yes or No |  |  | # Yes |
| Applicable patent numbers |  |  | Pending |
| Point of contact |  |  | Dr. Stanley Reible Micrilor, Inc. 17 Lakeside Office Park Wakefield, MA 01880 Tel: 781-246-0103 Fax: 781-246-0157 |