IEEE P802.11
Wireless LANs

Comparison of MAC PICS versus WPAN requirements

Date:	September 16, 1998

Author:	Tim Blaney
	Commcepts
	10052 Alta Sierra Dr., Suite C, Grass Valley, CA, 95949, USA
	Phone: +1 530 273-3482
	Fax:     +1 530 273-3543
	e-Mail: tim@wireless-network.com

Greg Ennis
	Ennis Associates
	16331 Englewood Ave., Los Gatos, CA, 95032, USA
	Phone: +1 408 358-5544
	Fax:     +1 408 358-5541
	e-Mail: gennis@mindspring.com

Bob O'Hara
	Informed Technology, Inc.
	1750 Nantucket Circle, Suite 138, Santa Clara, CA, 95054-3822, USA
	Phone: +1 408 986-9586
	Fax:     +1 408 727-2654
	e-Mail: bob@informed-technology.com

Abstract

In this submission we consider a comparison of the MAC portion of the existing IEEE 802.11 PICS as it relates to the requirements of a WPAN application. The intent of this submission is to assist the WPAN SG in understanding how the existing 802.11 MAC can be adapted to work as a MAC for a WPAN.

Discussion

In our investigation of the 802.11 MAC, as it applies to WPAN applications, we concluded that a small number of changes to the 802.11 MAC could be made that would tailor it more closely for meeting the WPAN requirements.  These changes include

- Providing stations with the ability to rotate the AP responsibility, including a mechanism that will recognize when an AP is lost

- Adding power management modes to the AP

- Removing the RTS/CTS handshake

- Significantly slimming the MIB, and

- Eliminating support for IBSS

These changes extend the current capabilities of 802.11 and simplify the implementation of 802.11 stations.

There are still two "classes" of 802.11 devices in a WPAN

- Devices capable of becoming a WPAN AP, and

- Devices that are independent stations (not capable of becoming a WPAN AP)

These device classes are likely to differ somewhat in complexity and cost.  Our idea of a WPAN consists of a "few"

devices capable of being WPAN APs and the rest being independent stations.

The MAC portion of the PICS in Annex A of the 802.11 standard is reproduced below, twice.  The first instance of the PICS is completed with the IUT being a hypothetical WPAN AP.  The second instance of the PICS is completed with the IUT being a WPAN station.  Each PICS shows where the WPAN devices differ from current 802.11 devices.  It is at these points of difference, where WPAN devices would be non-compliant with the current 802.11 standard and new conditional items would be used to create the new IUT classes for WPAN AP and WPAN station.  These new items would be CF6 for a WPAN AP and CF7 for a WPAN station.

Following is the dual listing of the Annex:

A.	Annex - Protocol Implementation Conformance Statement (PICS) Proforma (normative)

A.4	PICS Proforma—IEEE Std 802.11-1997†

A.4.3	IUT Configuration (AP capable)

| Item | IUT Configuration | Status | Support |
| --- | --- | --- | --- |
|  | What is the configuration of the IUT? |  |  |
| * CF1 | Access Point (AP) | O.1 | Yes No |
| * CF2 | Independent Station (NOT an AP) | O.1 | Yes No |
| * CF3 | Frequency Hopping Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes No |
| * CF4 | Direct Sequence Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes No |
| * CF5 | Infrared PHY Layer | O.2 | Yes No |

A.4.4	MAC Protocol

A.4.4.1	MAC Protocol Capabilities

| Item | Protocol Capability | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC protocol capabilities supported? |  |  |
| PC1 | Authentication Service | M | Yes No |
| PC1.1 | Authentication State | M | Yes No |
| PC1.2 | Open System Authentication | M | Yes No |
| PC1.3 | Shared Key Authentication | PC2:M | Yes No N/A |
| * PC2 | WEP Privacy Algorithm | O | Yes No |
| PC2.1 | WEP Encryption Procedure | PC2:M | Yes No N/A |
| PC2.2 | WEP Decryption Procedure | PC2:M | Yes No N/A |
| PC2.3 | Security Services Management | M | Yes No |
| PC3 | Distributed Coordination Function | M | Yes No |
| PC3.1 | Net Allocation Vector (NAV) Function | M | Yes No |
| PC3.2 | Interframe Space Usage & Timing | M | Yes No |
| PC3.3 | Random Backoff Function | M | Yes No |
| PC3.4 | DCF Access Procedure | M | Yes No |
| PC3.5 | Random Backoff Procedure | M | Yes No |
| PC3.6 | Recovery Procedures and Retransmit Limits | M | Yes No |
| PC3.7 | RTS/CTS Procedure | M | Yes No |
| PC3.8 | Directed MPDU Transfer | M | Yes No |
| PC3.9 | Broadcast & Multicast MPDU Transfer | M | Yes No |
| PC3.10 | MAC Level Acknowledgment | M | Yes No |
| PC3.11 | Duplicate Detection and Recovery | M | Yes No |
| * PC4 | Point Coordinator | CF1:O | Yes No N/A |
| PC4.1 | Maintenance of CFP Structure & Timing | PC4:M | Yes No N/A |
| PC4.2 | PCF MPDU Transfer from Point Coordinator | PC4:M | Yes No N/A |
| * PC4.3 | PCF MPDU Transfer to Point Coordinator | PC4:O | Yes No N/A |
| PC4.4 | Overlapping PC Provisions | PC4:M | Yes No N/A |
| PC4.5 | Polling List Maintenance | PC4.3:M | Yes No N/A |
| * PC5 | CF-Pollable | CF2:O | Yes No N/A |
| PC5.1 | Interpretation of CFP Structure & Timing | PC5:M | Yes No N/A |
| PC5.2 | PCF MPDU Transfer to/from CF-Pollable STA | PC5:M | Yes No N/A |
| PC5.3 | Polling List Update | PC5:M | Yes No N/A |
| PC6 | Fragmentation | M | Yes No |
| PC7 | Defragmentation | M | Yes No |
| PC8 | MAC Data Service | M | Yes No |
| PC8.1 | Reorderable-Multicast Service Class | M | Yes No |
| PC8.2 | Strictly-Ordered Service Class | O | Yes No |
| PC9 | Multirate Support | M | Yes No |
| * PC10 | Multiple Outstanding MSDU Support | O | Yes No |
| PC10.1 | Multiple Outstanding MSDU Transmission Restrictions | PC10:M | Yes No N/A |
| PC11 | Timing Synchronization | M | Yes No |
| PC11.1 | Timing in an Infrastructure Network | CF1:M | Yes No N/A |
| PC11.2 | Timing in an Independent BSS | CF2:M | Yes No N/A |
| PC11.3 | Beacon Generation Function | M | Yes No |
| PC11.4 | TSF Synchronization & Accuracy | M | Yes No |
| PC11.5 | Infrastructure BSS Initialization | CF1:M | Yes No N/A |
| PC11.6 | Independent BSS Initialization | CF2:M | Yes No N/A |
| PC11.7 | Passive Scanning | CF2:M | Yes No N/A |
| PC11.8 | Active Scanning | CF2:M | Yes No N/A |
| PC11.9 | Probe Response | M | Yes No |
| PC11.10 | Hop Synchronization Function | CF3:M | Yes No N/A |
| PC12 | Infrastructure Power Management | M | Yes No |
| PC12.1 | Station Power Management Modes | CF2:M | Yes No N/A |
| PC12.2 | TIM Transmission | CF1:M | Yes No N/A |
| PC12.3 | Access Point Function During Contention Period | CF1:M | Yes No N/A |
| PC12.4 | Access Point Function During CFP | PC4:M | Yes No N/A |
| PC12.5 | Receive Function During Contention Period | CF2:M | Yes No N/A |
| PC12.6 | Receive Function During CFP | PC5:M | Yes No N/A |
| PC12.7 | Aging Function | CF1:M | Yes No N/A |
| PC13 | Independent BSS Power Management | CF2:M | Yes No N/A |
| PC13.1 | Initialization of Power Management | CF2:M | Yes No N/A |
| PC13.2 | STA Power State Transitions | CF2:M | Yes No N/A |
| PC13.3 | ATIM and Frame Transmission | CF2:M | Yes No N/A |
| PC14 | Association & Reassociation | M | Yes No |
| PC14.1 | Association State | M | Yes No |
| PC14.2 | STA Association Procedure | CF2:M | Yes No N/A |
| PC14.3 | AP Association Procedure | CF1:M | Yes No N/A |
| PC14.4 | STA Reassociation Procedure | CF2:M | Yes No N/A |
| PC14.5 | AP Reassociation Procedure | CF1:M | Yes No N/A |
| PC15 | Management Information Base | M | Yes No |
| PC15.1 | SMT Object Class | M | Yes No |
| * PC15.2 | Privacy Package | PC2:M | Yes No N/A |
| PC15.3 | MAC Object Class | M | Yes No |
| * PC15.4 | MAC Statistics Package | O | Yes No |
| PC15.3 | Resource Type Object Class | M | Yes No |

A.4.4.2	MAC Frames

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is transmission of the following MAC frames supported? |  |  |
| FT1 | Association Request | CF2:M | Yes No N/A |
| FT2 | Association Response | CF1:M | Yes No N/A |
| FT3 | Reassociation Request | CF2:M | Yes No N/A |
| FT4 | Reassociation Response | CF1:M | Yes No N/A |
| FT5 | Probe Request | CF2:M | Yes No N/A |
| FT6 | Probe Response | M | Yes No |
| FT7 | Beacon | M | Yes No |
| FT8 | ATIM | CF2:M | Yes No N/A |
| FT9 | Disassociation | M | Yes No |
| FT10 | Authentication | M | Yes No |
| FT11 | Deauthentication | M | Yes No |
| FT12 | PS-Poll | CF2:M | Yes No N/A |
| FT13 | RTS | M | Yes No |
| FT14 | CTS | M | Yes No |
| FT15 | ACK | M | Yes No |
| FT16 | CF-End | PC4:M | Yes No N/A |
| FT17 | CF End+CF-Ack | PC4:M | Yes No N/A |
| FT18 | Data | M | Yes No |
| FT19 | Data + CF-Ack | (PC4 or PC5):M | Yes No N/A |
| FT20 | Data + CF-Poll | PC4.3:M | Yes No N/A |
| FT21 | Data + CF-Ack+CF-Poll | PC4.3:M | Yes No N/A |
| FT22 | Null | M | Yes No |
| FT23 | CF-Ack (no data) | (PC4 or PC5):M | Yes No N/A |
| FT24 | CF-Poll (no data) | PC4.3:M | Yes No N/A |
| FT25 | CF-Ack+CF-Poll (no data) | PC4.3:M | Yes No N/A |

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is reception of the following MAC frames supported? |  |  |
| FR1 | Association Request | CF1:M | Yes No N/A |
| FR2 | Association Response | CF2:M | Yes No N/A |
| FR3 | Reassociation Request | CF1:M | Yes No N/A |
| FR4 | Reassociation Response | CF2:M | Yes No N/A |
| FR5 | Probe Request | M | Yes No |
| FR6 | Probe Response | M | Yes No |
| FR7 | Beacon | M | Yes No |
| FR8 | ATIM | CF2:M | Yes No N/A |
| FR9 | Disassociation | M | Yes No |
| FR10 | Authentication | M | Yes No |
| FR11 | Deauthentication | M | Yes No |
| FR12 | PS-Poll | CF1:M | Yes No N/A |
| FR13 | RTS | M | Yes No |
| FR14 | CTS | M | Yes No |
| FR15 | ACK | M | Yes No |
| FR16 | CF-End | M | Yes No |
| FR17 | CF End+CF-Ack | M | Yes No |
| FR18 | Data | M | Yes No |
| FR19 | Data + CF-Ack | M | Yes No |
| FR20 | Data + CF-Poll | PC5:M | Yes No N/A |
| FR21 | Data + CF-Ack+CF-Poll | PC5:M | Yes No N/A |
| FR22 | Null | M | Yes No |
| FR23 | CF-Ack (no data) | (PC4 or PC5):M | Yes No N/A |
| FR24 | CF-Poll (no data) | PC5:M | Yes No N/A |
| FR25 | CF-Ack+CF-Poll (no data) | PC5:M | Yes No N/A |

A.4.4.3	Frame Exchange Sequences

| Item | Frame Exchange Sequence | Status | Support |
| --- | --- | --- | --- |
|  | Are the following Frame Sequences Supported? |  |  |
| FS1 | Basic Frame Sequences | M | Yes No |
| FS2 | CF-Frame Sequences | (PC4 or PC5):M | Yes No N/A |

A.4.4.4	MAC Addressing Functions

| Item | MAC Address Function | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC Addressing Functions Supported? |  |  |
| AD1 | STA Universal Individual IEEE802 Address | M | Yes No |
| AD2 | BSS Identifier Generation | M | Yes No |
| AD3 | Receive Address Matching | M | Yes No |

A.4.3	IUT Configuration (Station only)

| Item | IUT Configuration | Status | Support |
| --- | --- | --- | --- |
|  | What is the configuration of the IUT? |  |  |
| * CF1 | Access Point (AP) | O.1 | Yes No |
| * CF2 | Independent Station (NOT an AP) | O.1 | Yes No |
| * CF3 | Frequency Hopping Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes No |
| * CF4 | Direct Sequence Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes No |
| * CF5 | Infrared PHY Layer | O.2 | Yes No |

A.4.4	MAC Protocol

A.4.4.1	MAC Protocol Capabilities

| Item | Protocol Capability | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC protocol capabilities supported? |  |  |
| PC1 | Authentication Service | M | Yes No |
| PC1.1 | Authentication State | M | Yes No |
| PC1.2 | Open System Authentication | M | Yes No |
| PC1.3 | Shared Key Authentication | PC2:M | Yes No N/A |
| * PC2 | WEP Privacy Algorithm | O | Yes No |
| PC2.1 | WEP Encryption Procedure | PC2:M | Yes No N/A |
| PC2.2 | WEP Decryption Procedure | PC2:M | Yes No N/A |
| PC2.3 | Security Services Management | M | Yes No |
| PC3 | Distributed Coordination Function | M | Yes No |
| PC3.1 | Net Allocation Vector (NAV) Function | M | Yes No |
| PC3.2 | Interframe Space Usage & Timing | M | Yes No |
| PC3.3 | Random Backoff Function | M | Yes No |
| PC3.4 | DCF Access Procedure | M | Yes No |
| PC3.5 | Random Backoff Procedure | M | Yes No |
| PC3.6 | Recovery Procedures and Retransmit Limits | M | Yes No |
| PC3.7 | RTS/CTS Procedure | M | Yes No |
| PC3.8 | Directed MPDU Transfer | M | Yes No |
| PC3.9 | Broadcast & Multicast MPDU Transfer | M | Yes No |
| PC3.10 | MAC Level Acknowledgment | M | Yes No |
| PC3.11 | Duplicate Detection and Recovery | M | Yes No |
| * PC4 | Point Coordinator | CF1:O | Yes No N/A |
| PC4.1 | Maintenance of CFP Structure & Timing | PC4:M | Yes No N/A |
| PC4.2 | PCF MPDU Transfer from Point Coordinator | PC4:M | Yes No N/A |
| * PC4.3 | PCF MPDU Transfer to Point Coordinator | PC4:O | Yes No N/A |
| PC4.4 | Overlapping PC Provisions | PC4:M | Yes No N/A |
| PC4.5 | Polling List Maintenance | PC4.3:M | Yes No N/A |
| * PC5 | CF-Pollable | CF2:O | Yes No N/A |
| PC5.1 | Interpretation of CFP Structure & Timing | PC5:M | Yes No N/A |
| PC5.2 | PCF MPDU Transfer to/from CF-Pollable STA | PC5:M | Yes No N/A |
| PC5.3 | Polling List Update | PC5:M | Yes No N/A |
| PC6 | Fragmentation | M | Yes No |
| PC7 | Defragmentation | M | Yes No |
| PC8 | MAC Data Service | M | Yes No |
| PC8.1 | Reorderable-Multicast Service Class | M | Yes No |
| PC8.2 | Strictly-Ordered Service Class | O | Yes No |
| PC9 | Multirate Support | M | Yes No |
| * PC10 | Multiple Outstanding MSDU Support | O | Yes No |
| PC10.1 | Multiple Outstanding MSDU Transmission Restrictions | PC10:M | Yes No N/A |
| PC11 | Timing Synchronization | M | Yes No |
| PC11.1 | Timing in an Infrastructure Network | CF1:M | Yes No N/A |
| PC11.2 | Timing in an Independent BSS | CF2:M | Yes No N/A |
| PC11.3 | Beacon Generation Function | M | Yes No |
| PC11.4 | TSF Synchronization & Accuracy | M | Yes No |
| PC11.5 | Infrastructure BSS Initialization | CF1:M | Yes No N/A |
| PC11.6 | Independent BSS Initialization | CF2:M | Yes No N/A |
| PC11.7 | Passive Scanning | CF2:M | Yes No N/A |
| PC11.8 | Active Scanning | CF2:M | Yes No N/A |
| PC11.9 | Probe Response | M | Yes No |
| PC11.10 | Hop Synchronization Function | CF3:M | Yes No N/A |
| PC12 | Infrastructure Power Management | M | Yes No |
| PC12.1 | Station Power Management Modes | CF2:M | Yes No N/A |
| PC12.2 | TIM Transmission | CF1:M | Yes No N/A |
| PC12.3 | Access Point Function During Contention Period | CF1:M | Yes No N/A |
| PC12.4 | Access Point Function During CFP | PC4:M | Yes No N/A |
| PC12.5 | Receive Function During Contention Period | CF2:M | Yes No N/A |
| PC12.6 | Receive Function During CFP | PC5:M | Yes No N/A |
| PC12.7 | Aging Function | CF1:M | Yes No N/A |
| PC13 | Independent BSS Power Management | CF2:M | Yes No N/A |
| PC13.1 | Initialization of Power Management | CF2:M | Yes No N/A |
| PC13.2 | STA Power State Transitions | CF2:M | Yes No N/A |
| PC13.3 | ATIM and Frame Transmission | CF2:M | Yes No N/A |
| PC14 | Association & Reassociation | M | Yes No |
| PC14.1 | Association State | M | Yes No |
| PC14.2 | STA Association Procedure | CF2:M | Yes No N/A |
| PC14.3 | AP Association Procedure | CF1:M | Yes No N/A |
| PC14.4 | STA Reassociation Procedure | CF2:M | Yes No N/A |
| PC14.5 | AP Reassociation Procedure | CF1:M | Yes No N/A |
| PC15 | Management Information Base | M | Yes No |
| PC15.1 | SMT Object Class | M | Yes No |
| * PC15.2 | Privacy Package | PC2:M | Yes No N/A |
| PC15.3 | MAC Object Class | M | Yes No |
| * PC15.4 | MAC Statistics Package | O | Yes No |
| PC15.3 | Resource Type Object Class | M | Yes No |

A.4.4.2	MAC Frames

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is transmission of the following MAC frames supported? |  |  |
| FT1 | Association Request | CF2:M | Yes No N/A |
| FT2 | Association Response | CF1:M | Yes No N/A |
| FT3 | Reassociation Request | CF2:M | Yes No N/A |
| FT4 | Reassociation Response | CF1:M | Yes No N/A |
| FT5 | Probe Request | CF2:M | Yes No N/A |
| FT6 | Probe Response | M | Yes No |
| FT7 | Beacon | M | Yes No |
| FT8 | ATIM | CF2:M | Yes No N/A |
| FT9 | Disassociation | M | Yes No |
| FT10 | Authentication | M | Yes No |
| FT11 | Deauthentication | M | Yes No |
| FT12 | PS-Poll | CF2:M | Yes No N/A |
| FT13 | RTS | M | Yes No |
| FT14 | CTS | M | Yes No |
| FT15 | ACK | M | Yes No |
| FT16 | CF-End | PC4:M | Yes No N/A |
| FT17 | CF End+CF-Ack | PC4:M | Yes No N/A |
| FT18 | Data | M | Yes No |
| FT19 | Data + CF-Ack | (PC4 or PC5):M | Yes No N/A |
| FT20 | Data + CF-Poll | PC4.3:M | Yes No N/A |
| FT21 | Data + CF-Ack+CF-Poll | PC4.3:M | Yes No N/A |
| FT22 | Null | M | Yes No |
| FT23 | CF-Ack (no data) | (PC4 or PC5):M | Yes No N/A |
| FT24 | CF-Poll (no data) | PC4.3:M | Yes No N/A |
| FT25 | CF-Ack+CF-Poll (no data) | PC4.3:M | Yes No N/A |

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is reception of the following MAC frames supported? |  |  |
| FR1 | Association Request | CF1:M | Yes No N/A |
| FR2 | Association Response | CF2:M | Yes No N/A |
| FR3 | Reassociation Request | CF1:M | Yes No N/A |
| FR4 | Reassociation Response | CF2:M | Yes No N/A |
| FR5 | Probe Request | M | Yes No |
| FR6 | Probe Response | M | Yes No |
| FR7 | Beacon | M | Yes No |
| FR8 | ATIM | CF2:M | Yes No N/A |
| FR9 | Disassociation | M | Yes No |
| FR10 | Authentication | M | Yes No |
| FR11 | Deauthentication | M | Yes No |
| FR12 | PS-Poll | CF1:M | Yes No N/A |
| FR13 | RTS | M | Yes No |
| FR14 | CTS | M | Yes No |
| FR15 | ACK | M | Yes No |
| FR16 | CF-End | M | Yes No |
| FR17 | CF End+CF-Ack | M | Yes No |
| FR18 | Data | M | Yes No |
| FR19 | Data + CF-Ack | M | Yes No |
| FR20 | Data + CF-Poll | PC5:M | Yes No N/A |
| FR21 | Data + CF-Ack+CF-Poll | PC5:M | Yes No N/A |
| FR22 | Null | M | Yes No |
| FR23 | CF-Ack (no data) | (PC4 or PC5):M | Yes No N/A |
| FR24 | CF-Poll (no data) | PC5:M | Yes No N/A |
| FR25 | CF-Ack+CF-Poll (no data) | PC5:M | Yes No N/A |

A.4.4.3	Frame Exchange Sequences

| Item | Frame Exchange Sequence | Status | Support |
| --- | --- | --- | --- |
|  | Are the following Frame Sequences Supported? |  |  |
| FS1 | Basic Frame Sequences | M | Yes No |
| FS2 | CF-Frame Sequences | (PC4 or PC5):M | Yes No N/A |

A.4.4.4	MAC Addressing Functions

| Item | MAC Address Function | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC Addressing Functions Supported? |  |  |
| AD1 | STA Universal Individual IEEE802 Address | M | Yes No |
| AD2 | BSS Identifier Generation | M | Yes No |
| AD3 | Receive Address Matching | M | Yes No |

† Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.