IEEE Draft P802.11c/D2

IS ISO/IEC 10038 : 1993 (IEEE 802.1d) Information Technology - Telecommunications and information exchange between systems - Local area networks - Media access control (MAC) bridges - Supplement for support by IEEE 802.11

Sponsor

LAN MAN Standards Committee

of the

IEEE Computer Society

The purpose of this standard is to add the necessary information to IEEE Std. 802.1D to map the IEEE 802.11 MAC parameters onto 802.1d MAC parameters.  The text revisions below are referenced to the 802.1p/D8 draft.

This draft expires on March 31, 1998.

Copyright 1998 by the Institute of Electrical and Electronics Engineers, Inc.

345 East 47th Street

New York, NY 10017, USA

All rights reserved.

This is an unapproved draft of a proposed IEEE Standard, subject to change. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of IEEE standardization activities. If this document is to be submitted to ISO or IEC, notification shall be given to the IEEE Copyright Administrator. Permission is also granted for member bodies and technical committees of ISO and IEC to reproduce this document for purposes of developing a national position. Other entities seeking permission to reproduce portions of this document for these or other uses must contact the IEEE Standards Department for the appropriate license.

Use of information contained in this unapproved draft is at your own risk.

IEEE Standards Department

Copyright and Permissions

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ 08855-1331, USA

IEEE Standards for Local and Metropolitan Area Networks:

IS ISO/IEC 10038 : 1993 (IEEE 802.1d) Information Technology - Telecommunications and information exchange between systems - Local area networks - Media access control (MAC) bridges - Supplement for support by IEEE 802.11

Support by IEEE 802.11 Wireless LANs

Supplement to IEEE Draft Std. 802.1D

The purpose of this standard is to add the necessary information to IEEE Std. 802.1d to map the IEEE 802.11 MAC parameters onto 802.1d MAC parameters.  The text revisions below are referenced to the 802.1d/D15 draft.

Editing instructions for incorporating the revisions defined by this supplement into the indicated subclauses of the base standard (802.1d/D15) are shown in bold italic type.

3.4 IEEE 802 Local Area Network (LAN)

Page 21, line 40,  insert the following between “ 8802-9 (IS-LAN)” and  “ISO/IEC 8802-12”

", IEEE Std 802.11-1997"

2  References

Page 19, line 38: add the following  reference:

IEEE Std 802.11-1997, IEEE Standard for Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications.

6.  Support of the MAC service

Page 38 : add a new subclause 6.5.7 as follows

6.5.7 Support by IEEE Std 802.11 (Wireless LANs )

A Bridge may only connect to an 802.11 Portal, which in turn connects to an 802.11 Distribution System. For the purposes of bridging, the SAP of the Portal is identical to the SAP of the MAC. An instance of an 802.11 Distribution System may be implemented from 802 LAN components.  802.11 STAs attach to the Distribution System via an 802.11 Access Point.  Bridging to an 802.11 Independent BSS is not permitted.  For a description of the 802.11 architecture, see Clause 5 of IEEE Std 802.11-1997.

The wireless LAN access method is specified in IEEE Std 802.11-1997. Clause 7 of that standard specifies frame formats, Clause 9 specifies the medium access control sublayer function, and Clause 11 specifies the mandatory medium access control sublayer management function

On receipt of an MA_UNITDATA.request primitive the portal constructs and transmits a MAC frame, in accordance with the frame formats and procedures specified in IEEE Std. 802.11 clauses 6, 7, 9 and Annex C, using the parameters supplied as specified below.

On receipt of a valid MAC frame (see IEEE 802.11 clause 9), the portal generates an MA_UNITDATA.indication primitive with parameter values derived from the frame fields as specified below.

The frame_type parameter only takes the value user_data_frame.  When processing MSDU_from_LLC, the frame_type of user_data_frame must be translated according to parameters specified in Clause 7.1.3.1 of 802.11 and is explicitly encoded in MAC frames.

The mac_action parameter only takes the value request_with_no_response and is not explicitly encoded in MA_UNITDATA.request frame.

The destination_address parameter is encoded in of the MAC frame as the DA described in Table 4 of Clause 7.2.2 of 802.11.

The source_address parameter is encoded in the MAC frame as the SA described in Table 4 of clause 7.2.2 of 802.11.

The routing information parameter (IEEE Std 802.11-1997, Clauses 6.2.1.1, and 6.2.1.2) is set to null.

The mac_service_user_data parameter is encoded in the Frame Body  field (IEEE Std 802.11-1997, Clause 7.1.3.5) of the MAC frame. The length of the MSDU must be less than or equal to 2304 octets.

The user_priority parameter is not encoded in the MAC Frame.   The user_priority parameter provided in an MA_UNITDATA.indication primitive always takes the value of zero. (IEEE Std 802.11-1997, 6.2.1).

The frame_check_sequence parameter is encoded in the Frame Check Sequence (FCS) field of the MAC frame (IEEE Std 802.11-1997 ).  The FCS is calculated over all the fields of the MAC header and the frame body field.  The calculation is completed using a standard generator polynomial of degree 32 in accordance with IEEE Std 802.11-1997, 7.1.3.6 FCS.

The access_priority parameter is not encoded in the MAC frame.

No special action, above that specified in IEEE Std 802.12-1995, is required for the support of the MAC Internal Sublayer Service by the demand priority access method.

- Mapping Priority

In Table 7-3 on page 55, add a column for 802.11 that maps all priorities to zero (0).

14.4.2.1.3  Outputs

Item ( b ), page 232 line 35 : insert before "ISO 9314"

"; IEEE Std 802.11"

# Annex A  PICS Proforma

A.5  Major capabilities and options

Section 1, item (1a), page 292 line 16:  insert the following item and renumber those following:

1a.8	Wireless LAN IEEE 802.11