IEEE P802.11
Wireless LANs

Frequency Hopping Sequences for Australia, Israel and Canada (outdoors)

Date:	January 1998

Author:	Naftali Chayat
	BreezeCom
	Atidim Tech. Park, Bldg. 1, Tel-Aviv 61131, Israel
	Phone: +972-3-6456262
	Fax:     +972-3-6456290
	e-Mail: naftalic@breezecom.co.il

Follow proposed additions to 802.11 text as to include hopping sequences for the regulatory domains of Australia, Israel and Canada (for outdoor use).

=  [b(i) + x] mod (47) + 2	in Australia with b(i) defined in Error: Reference source not found.

=  [b(i) + x] mod (35) + 21	in Israel with b(i) defined in Error: Reference source not found.

=  [b(i) + x] mod (30) + 52	in Canada (outdoors) with b(i) defined in Error: Reference source not found.

| i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 11 | 32 | 21 | 46 | 31 | 7 | 41 | 8 |
| 2 | 10 | 12 | 27 | 22 | 17 | 32 | 23 | 42 | 26 |
| 3 | 2 | 13 | 4 | 23 | 11 | 33 | 14 | 43 | 37 |
| 4 | 31 | 14 | 30 | 24 | 33 | 34 | 44 | 44 | 9 |
| 5 | 43 | 15 | 42 | 25 | 3 | 35 | 13 | 45 | 45 |
| 6 | 18 | 16 | 35 | 26 | 41 | 36 | 21 | 46 | 6 |
| 7 | 24 | 17 | 12 | 27 | 20 | 37 | 38 | 47 | 28 |
| 8 | 39 | 18 | 34 | 28 | 36 | 38 | 5 |  |  |
| 9 | 1 | 19 | 15 | 29 | 22 | 39 | 40 |  |  |
| 10 | 19 | 20 | 29 | 30 | 16 | 40 | 25 |  |  |

Table A:Base Hopping Sequence b(i) for Australia

| i | b(i) | i | b(i) | i | b(i) | I | b(i) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 11 | 24 | 21 | 24 | 31 | 9 |
| 2 | 13 | 12 | 4 | 22 | 22 | 32 | 23 |
| 3 | 28 | 13 | 21 | 23 | 32 | 33 | 33 |
| 4 | 20 | 14 | 3 | 24 | 18 | 34 | 14 |
| 5 | 29 | 15 | 11 | 25 | 30 | 35 | 19 |
| 6 | 6 | 16 | 26 | 26 | 5 |  |  |
| 7 | 25 | 17 | 5 | 27 | 10 |  |  |
| 8 | 26 | 18 | 12 | 28 | 2 |  |  |
| 9 | 31 | 19 | 1 | 29 | 8 |  |  |
| 10 | 17 | 20 | 7 | 30 | 27 |  |  |

Table B: Base Hopping Sequence b(i) for Israel

| i | b(i) | i | b(i) | i | b(i) |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 11 | 27 | 21 | 14 |
| 2 | 6 | 12 | 19 | 22 | 4 |
| 3 | 20 | 13 | 7 | 23 | 26 |
| 4 | 28 | 14 | 22 | 24 | 13 |
| 5 | 11 | 15 | 19 | 25 | 23 |
| 6 | 24 | 16 | 18 | 26 | 3 |
| 7 | 1 | 17 | 29 | 27 | 17 |
| 8 | 15 | 18 | 10 | 28 | 25 |
| 9 | 2 | 19 | 21 | 29 | 16 |
| 10 | 12 | 20 | 18 | 30 | 5 |

Table C: Base Hopping Sequence b(i) for Canada

For Australia:

x =  {0,3,6,9,12,15,18,21,24,27,30,33,36,39,42}	Set 1

x =  {1,4,7,10,13,16,19,22,25,28,31,34,37,40,43}	Set 2

x =  {2,5,8,11,14,17,20,23,26,29,32,35,38,41,44}	Set 3

For Israel:

x =  {0,3,6,9,12,15,18,21,24,27,30}	Set 1

x =  {1,4,7,10,13,16,19,22,25,28,31}	Set 2

x =  {2,5,8,11,14,17,20,23,26,29,32}	Set 3

For Canada (outdoors):

x =  {0,3,6,9,12,15,18,21,24,27}		Set 1

x =  {1,4,7,10,13,16,19,22,25,28}		Set 2

x =  {2,5,8,11,14,17,20,23,26,29}		Set 3