IEEE P802.11
Wireless LANs

IEEE P802.11
Wireless LANs

Status of TGa (5 GHz high speed PHY) of 802.11 after March 98 Plenary Meeting

Naftali Chayat, TGa Chair

In March 1998 Task Group A of 802.11 gathered in Irvine, CA, with 120 people attending. Of the 6 PHY proposals in existence after January meeting one was withdrawn (Micrilor) and two pairs were merged: NTT and Lucent Technology merged their OFDM proposals, and BreezeCom and NEC merged their single carrier proposal. Two weeks prior to the meeting the proposers provided performance estimates for their proposal according to an agreed template. However, due to reinterpretation of regulatory requirements, reconsideration of power amplifier modeling and some missing data the Task Group postponed the choice of the modulation method to May meeting. In the meanwhile, the submitters will provide the additional data related to their proposals.

## Comparison of proposals

| # Company | # Lucent Tech. + NTT | # BreezeCom + NEC | # RadioLAN |
| --- | --- | --- | --- |
| Modulation method | OFDM DBPSK, DQPSK or 16-QAM in each subcarrier | Offset Quadrature Modulation (OQPSK/OQAM) | Differential Pulse Position Modulation (16-DPPM, 4-DPPM) |
| Pulse shaping features | 48 subcarriers out of 64 | 50% Square-Root Raised Cosine | 50% Square-Root Raised Cosine |
| Error Correction Coding | Convolutional K=7, R=1/2 or R=3/4 Inter-carrier interleaving | Hamming (31,26), uncoded option | Uncoded, option for Reed Solomon(15.13) coded with interleaving |
| Rates supported | 5 Mbit/s (DBPSK, R=1/2) 10 Mbit/s (DQPSK, R=1/2) 15 Mbit/s (DQPSK, R=3/4) 20 Mbit/s (16-QAM, R=1/2) 30 Mbit/s (16-QAM, R=3/4) | 21 Mbit/s (2-level, OQPSK, coded) 25 Mbit/s (2-level, OQPSK, uncoded) 42 Mbit/s (4-level, OQAM, coded) 50 Mbit/s (4-level, OQAM, uncoded) | 10 Mbit/s (4-DPPM) 20 Mbit/s (16-DPPM) 20 Mbit/s (16-DPPM, coded) |
| Number of channels in U-NII band | 5 in 100 MHz, 10 in 200 MHz 15 MHz channel spacing | 4 in 100 MHz, 9 in 200 MHz 20 MHz channel spacing | 3 in 100 MHz, 7 in 200 MHz 30 MHz channel spacing |
| Applicable documents | 97/92, 97/123, 97/137, 98/02, 98/03, 98/12, 98/71r1, 98/72, 98/73, 98/74, | 98/76r1, 98/109, 98/144 | 97/145r1, 98/38, 98/75, 98/132, 98/133 |