IEEE P802.11
Wireless LANs

IEEE P802.11
Wireless LANs

PLCP preamble and system identification proposals

Date:	March 8, 1999

Author:	Tomoki Ohsawa, 
	NEC Corporation
	4-1-1, Miyazaki Miyamae-ku, Kawasaki 216 Japan
	Phone: +81-471-85-6732
	Fax: +44-856-2230
	e-Mail: ohsawa@ptl.abk.nec.co.jp

Subject: PLCP preamble and system identificatoin proposals

Date: March 8, 1999

Dear Mr. Victor Hayes,

On the discussion at Ethernet Wireless Access Working Group (EWA WG) of MMAC-PC dated of 23rd of February, we would like to propose a couple of things regarding to a co-existence among standards. The following result should be valid in the condition that we have a common description among the standards. We understand that our proposal will provide a beneficial changes to realize a global standard and a common physical layer of 5Ghz band in the world, which are IEEE802.11a, ETSI BRAN and Ethernet Wireless and Wireless ATM Working Groups of MMAC-PC.

- In order to have a common preamble structure, we have received a request from Wireless ATM working group of MMAC-PC. They basically agreed on having a same preamble structure with IEEE802.11. However, in order to realize a simple way for processing for their MAC frame structure, they proposed to add one short training preamble symbol.

- Therefore, the preamble will become as 10 repetitions of “short training symbol” instead of that of 9 repetitions in the draft.

- We have basically accepted this proposal for Ethernet Wireless Access working group though it will generate a little bit lager overhead.

- The WATM working group has already submitted this proposal to ETSI BRAN. The Ethernet Wireless Access working group would like to propose it to change the IEEE802.11a draft.

- As it is clear, those standards take different MAC mechanisms. IEEE802.11 and Ethernet Wireless Access working group of MMAC-PC takes a CSMA/CA based algorithm. ETSI BRAN and Wireless ATM working group of MMAC-PC chose a TDMA/TDD type of access method. They can recognize each other what kind of system is being operated in an area of a channel.

We thought that it will be nice to have a recognition mechanism on PHY layer among the standards. From this point of view, we would like propose to have a common field on physical layer level of header. In this sense, we would like to propose to insert system ID field in the PLCP header by using a service field of the PLCP header.

This makes a benefit for co-existence in terms of avoidance of interference to an operated system. Therefore, we are requesting the same proposal to WATM WG of MMAC-PC and ETSI BRAN.

The MMAC Ethernet Wireless Access WG again thanks the IEEE802.11 TGa for its activities of standardization.

Yours sincerely