IEEE P802.11
Wireless LANs

Proposed comment resolution fot outstanding NO-votes

Date:	March 30, 1999

Author:	Vic Hayes
	Lucent Technologies
	Zadelstede 1-10
	3431 JZ  Nieuwegein, the Netherlands
	Phone: +31 30 609 7528
	Fax: +31 30 609 7556
	e-Mail: vichayes@lucent.com or v.hayes@ieee.org

Abstract

This paper is a proposal for the material that will accompany our 802.11 draft to Sponsor Ballot.

I have added remarks that voters sent me since the closure of the resolution meetings as far as I found the remarks relevant as well as additions to place the timing of the comments in perspective to the recirculation ballots.

Finally, I reworded the response to Mr Bagby’s comments as well as made the response shorter by removing the parts that I believ are superfluous for the purpose.

This paper accompanies my YES-Vote to the 802.11b Letter Ballot 18.

# No comment from Mr. J. Fischer

## Reason for disapproval:

The PBCC (i.e. coded) mode should be required, not optional.  This issue is not related to the debate of having “options” in the standard, but to needing the PBCC mode because it is the only way the standard can be generally useful to the industry.  The CCK modulation is inherently very weak by today’s communications standards. If the PBCC is not used then the only way to make this waveform useful is with a severe measure of equalization.  Therefore the only way to make this standard a useful one depends on a companies implementation, not on the standard waveform itself.  By making the PBCC a requirement then the standard waveform itself will have inherent utility.

Recommended change:

Make this mode required for a standard implementation.

## Reason why 802.11 could not accept recommendation:

Due to market considerations CCK has been adopted as a mandatory modulation. PBCC has been added as an option to allow a potentially higher performance upgrade. Use of the CCK or PBCC modulation allows complete interoperability through the use of the same PLCP header.

NOTE:	This comment (reworded)/resolution was generated at resolving comments on :Letter Ballot 16. Two subsequent recirculation ballots did not attract new no votes.

# No comment from Mr. J. Cafarella

## Reason for disapproval:

The FH option contained in the draft violates the PAR restriction to a single PHY. Anyone can build a dual- mode transceiver if desired, but specifying how to do this violates our PAR.

Separate from the fact that our PAR restricts the high- rate solution to a single PHY, it is important to realize that the FH PHY is limited by regulatory agencies (at least in the US) to low data rates, while DS signaling can effect much higher rates for reasonable E B /N 0 values. It makes no sense to constrain any aspect of the future technology.

## Reworded comment from Mr. Cafarella after closure of the comment resolution meeting

My "NO" votes on TGb in the last two ballots were based upon my belief that the FH aspect of the high-rate standard should be removed.  My opinion has not changed, nor have I heard any convincing argument to the contrary.  It remains because parties have "made a deal," not because it really makes sense.  I believe that we are violating our PAR restriction to a single PHY, and I also believe that the accommodation of FH will soon be of little practical concern.

## Recommended change:

Remove FH material from HR DSSS PHY standard

## Reason why 802.11 could not accept recommendation:

The FH option was eliminated in favor of a channel agility capability.

NOTE:	This comment/resolution was generated  on letter ballot 16, voter did not respond to the 2 subsequent recirculation ballots to adjust his comment. There were no changes in votes to support Mr Cafarella’s position.

# No comment from Mr. D. Bagby

## Reason for disapproval:

The PHY specification contains options.

802.11 has voted that options shall be minimized and included only when absolutely necessary (see previous meeting minutes). The presence of following options mandate a No vote:

- Short PLCP frame format

- FH PLCP frame format

- DSSS/ PBCC Data Modulation and Modulation rate

## Recommended change:

- Delete or make mandatory the short preamble option.

- Make mandatory the FH option.

- Delete the PBCC option

## Reason why 802.11 could not accept recommendation:

Partially accepted, the FH PLCP frame format option has been deleted.

IEEE802.11 Task Group B has considered the status of the short preamble, deletion and the status of the FH option and the status of the PBCC option  at length but respectfully declines the proposed changes.

The group understands and appreciates fully IEEE802.11’s agreed position on options within the standard and its charter to produce a single IEEE802.11 high rate PHY. It is our belief that we have not violated either requirement. Our reasoning is based on both logical argument and considering and comparing to prior policy in other task groups under the same committee working to the same agreed guidelines. Several motions were put forth with the exact concerns expressed here and were voted down by the group.

Consideration of this comment started with the question of whether the draft standard as published represents a single PHY. To resolve this question one has to agree on what defines a single PHY. One way to define this is to consider that the specification represents a single PHY if all implementations interoperate successfully. When tested against this criterion the published draft does represent a single PHY. Where there are options, sufficient thought has been given to ensure that these do not sacrifice interoperability.