IEEE P802.11
Wireless LANs

Title:						HIPERLAN type 2 PHY Decision List

Date:	May 3, 1999

Author:	Jamshid Khun-Jush
	Ericsson
	Nordostpark12,

D-90411 Nürnberg

Germany
	Phone: +49 911 5217260
	Fax:  +49 911 5217950
	e-Mail: jamshid.khun-jush@eedn.ericsson.se

Abstract

This document summarizes the agreements achieved between three different communities BRAN HIPERLAN type 2, MMAC WATM WG and IEEE802.11a regarding the harmonization of physical layer for different system currently being standardized in Europe, Japan and USA. The table shows that the physical layers have been harmonized to a large extent and

| Parameter | # H/2 Current working assumption or decision | Harmonised with IEEE? | Harmonised with MMAC-WATM-WG? |
| --- | --- | --- | --- |
| Channel spacing | ## 20 MHz (decided) | ## Yes | Yes |
| Sampling rate | 20 Msample/s (decided) | Yes | Yes |
| FFT length | 64 (decided) | Yes | Yes |
| Number of used sub-carriers | 48 data sub-carriers and 4 pilot sub-carriers (decided) | Yes | Yes |
| Sub-carrier modulation | BPSK, QPSK, 16QAM, optionally 64 QAM (decided) | # Yes | Yes |
| Signal constellation and bit mapping | Decided | Yes | Yes |
| Demodulation | Coherent (decided) | Yes | Yes |
| FEC mother code | Convolutional code, rate 1/2 (decided) | Yes | Yes |
| Code termination | Method (decided), additional puncturing patterns to be decided at BRAN#13.5 | No | Yes |
| Guard interval | 800 ns corresponding to 16 time samples (decide) | Yes | Yes |
| PHY modes & code rates | 7 modes (decided) 6, 9, 12, 18, 27, 36, 54 Mbps | Mainly yes 6, 9, 12, 18, 24, 48, 54 Mbps | Yes |
| Oscillator accuracy | +/- 20 ppm (decided) | Yes | Yes |
| Linkage of Oscillators | Generation of RF and baseband timing from the same reference oscillator | Yes | Yes |
| Generator polynomial for data scrambling | X^7+ X^4 + 1 (decided) | Yes | Yes |
| Scrambling of pilot sub-carriers | Generator polynomial X^7+ X^4 + 1 (decided), scheme not decided (two alternatives, IEEE and BRAN specific ones) | Generator polynomial Yes, scheme not clear | Generator polynomial Yes, scheme not clear (IEEE scheme adopted) |
| Preamble structure (based on quarter symbols, 16 samples) | BCH preamble, DL preamble (FCH, SCH, LCH) and short version and long version of UL preamble (SCH, LCH, RCH) (decided) | BCH preamble and the long version UL preamble have the same length as IEEE | BCH preamble the same length (content under discussion), DL Yes, UL under discussion |
| Interleaving | OFDM symbol wise (decided), permutation (working assumption) | Method Yes, Permutation No | Method Yes, Permutation No |
| Pulse shaping | TX power mask + modulation constellation accuracy (method decided, IEEE TX power mask and IEEE values for modulation accuracy working assumption) | Method Yes, Modulation accuracy values Yes, TX power mask Yes | Method Yes, Modulation accuracy values Yes, TX power mask No due to specific Japanese regulation matters |