IEEE P802.11
Wireless LANs

FH Hopping Patterns for New Regulatory Domains

Date:	July 5, 1999

Author:	Dean Kawaguchi
	Symbol Technologies
	2145 Hamilton Ave, MS 160
	Voice: (408)369-2629 
	Fax: (408)369-2740
	e-Mail: deank@psd.symbol.com

Abstract

This submission proposes new channel sets and hopping patterns for use of the Frequency Hopping PHY in regulatory domains that are currently not included in the 802.11 standard. The current domains defined in 802.11 are USA/Europe, Japan, France, and Spain.  The new regulatory domains addressed include Korea, Israel, Mexico, and Belgium-outdoor.

Proposal

The summary of current domains and the new proposed domains are summarized in Table 1.

| Regulatory Domain | Domain Number | Regulatory Band Limits (MHz) | Operating Range (MHz) | Number of Channels | Hop Table | Sets & Patterns |
| --- | --- | --- | --- | --- | --- | --- |
| # Current Domains |  |  |  |  |  |  |
| USA/ETSI |  | 2400-2483.5 | 2402-2480 | 79 | HST-79 | 3 x 26 |
| Japan |  | 2471-2497 | 2473-2495 | 23 | delta | 3 x 4 |
| Spain |  | 2445-2475 | 2447-2473 | 27 | HST-27 | 3 x 9 |
| France |  | 2446.5-2483.5 | 2448-2482 | 35 | HST-35 | 3 x 11 |
| # New Domains |  |  |  |  |  |  |
| Korea |  | 2452-2478 | 2454-2476 | 23 | delta | 3 x 4 |
| Israel |  | ? | 2420-2454 | 35 | HST-35 | 3 x 11 |
| Mexico |  | 2450-2483.5 | 2452-2480 | 29 | HST-29 | 3 x 9 |
| Belgium-outdoor |  | 2460-2483.5 | 2462-2481 | 20 | HST-20 | 3 x 6 |

| Hop Sequence Tables | Pattern Tables |
| --- | --- |
| HST-79 { 0, 23, 62, 8, 43, 16, 71, 47, 19, 61, 76, 29, 59, 22, 52, 63, 26, 77, 31, 2, 18, 11, 36, 72, 54, 69, 21, 3, 37, 10, 34, 66, 7, 68, 75, 4, 60, 27, 12, 25, 14, 57, 41, 74, 32, 70, 9, 58, 78, 45, 20, 73, 64, 39, 13, 33, 65, 50, 56, 42, 48, 15, 5, 17, 6, 67, 49, 40, 1, 28, 55, 35, 53, 24, 44, 51, 38, 30, 46 } | Set 1 = { 0, 3, 6, 9, 12, 15, 18, 21, 24, 27, 30, 33, 36, 39, 42, 45, 48, 51, 54, 57, 60, 63, 66, 69, 72, 75 } Set 2 = { 1, 4, 7, 10, 13, 16, 19, 22, 25, 28, 31, 34, 37, 40, 43, 46, 49, 52, 55, 58, 61, 64, 67, 70, 73, 76 } Set 3 = { 2, 5, 8, 11, 14, 17, 20, 23, 26, 29, 32, 35, 38, 41, 44, 47, 50, 53, 56, 59, 62, 65, 68, 71, 74, 77 } |
| HST-35 { 17, 5, 18, 32, 23, 7, 16, 4, 13, 33, 26, 10, 31, 20, 29, 22, 12, 6, 28, 14, 25, 0, 8, 1, 15, 3, 11, 30, 24, 9, 27, 19, 2, 21, 34 } | Set 1 = { 0, 3, 6, 9, 12, 15, 18, 21, 24, 27, 30 } Set 2 = { 1, 4, 7, 10, 13, 16, 19, 22, 25, 28, 31 } Set 3 = { 2, 5, 8, 11, 14, 17, 20, 23, 26, 29, 32 } |
| HST-27 { 13, 4, 24, 18, 5, 12, 3, 10, 25, 19, 8, 23, 15, 22, 9, 21, 0, 6, 14, 1, 20, 7, 16, 2, 11, 17, 26 } | Set 1 = { 0, 3, 6, 9, 12, 15, 18, 21, 24 } Set 2 = { 1, 4, 7, 10, 13, 16, 19, 22, 25 } Set 3 = { 2, 5, 8, 11, 14, 17, 20, 23, 26 } |
| HST-20 { 9, 18, 3, 13, 4, 14, 2, 11, 0, 8, 17, 10, 1, 16, 7, 15, 5, 12, 19, 6 } | Set 1 = { 0, 3, 6, 9, 12, 15 } Set 2 = { 1, 4, 7, 10, 13, 16 } Set 3 = { 2, 5, 8, 11, 14, 17 } |
| HST-29 { 14, 4, 26, 19, 6, 13, 3, 10, 27, 21, 8, 25, 11, 24, 18, 9, 23, 12, 5, 20, 0, 16, 1, 15, 7, 22, 2, 17, 28 } | Set 1 = { 0, 3, 6, 9, 12, 15, 18, 21, 24 } Set 2 = { 1, 4, 7, 10, 13, 16, 19, 22, 25 } Set 3 = { 2, 5, 8, 11, 14, 17, 20, 23, 26 } |
| Delta | Set 1 = { 6, 9, 12, 15 } Set 2 = { 7, 10, 13, 16 } Set 3 = { 8, 11, 14, 17 } |