- Standards Working Group IEEE 802

- Local and Metropolitan Area Network Standards Committee

Homepage at http://grouper.ieee.org/groups/802/

Reply to:	Vic Hayes, Chair, IEEE P802.11

Lucent Technologies Nederland B.V.

Zadelstede 1-10

3431 JZ  Nieuwegein, the Netherlands

phone: +31 30 609 7528

fax: +31 30 609 7556

e-mail: v.hayes@ieee.org

July 19, 1999

Magalie R. Salas, Esquire

Secretary

Federal Communications Commission

445 12th St. SW

Washington DC 20554

Re:	 ET Docket No. 98-42 --- Ex Parte Communication

Dear Ms. Salas:

IEEE 802, the LAN/MAN Standards Committee (“the Committee”), is writing in regard to ET Docket No. 98-42: 1998 Biennial Regulatory Review, Amendment of Part 18 of the Commission’s Rules to Update Regulations for RF Lighting Devices. The Committee has already submitted comments in the Commission’s Notice of Proposed Rulemaking (“the Notice”), FCC 98-42 on several previous occasions. The Committee respectfully submits this further statement in the matter specifically regarding the comments from Fusion Lighting in their letter of May 28th, 1999, and to the subsequently revised proposal for a limit on in-band emissions for RF lighting devices as proposed by the Part 15/Mobile Satellite Services (MSS) Interests.

Headquarters Office	Publications Office	European Office	Asian Office

1730 Massachusetts Avenue, N.W.	10662 Los Vaqueros Circle	13 Avenue de l'Aquilon (rez A)	Ooshima Building

Washington, D.C. 20036-1903	Los Alamitos, CA 90720	1200-Brussels, Belguim	2-19-1 Minami Aoyama

(202) 371 0101	Phone:(714) 821 8380	Phone: +32 2 770 21 98	Minato-ku

TWX: 7108250437 IEEE Compso	TWX: 7108250437 IEEE Compso	TWX: 25387 AVVALB	Tokyo 107 Japan

FAX: (202) 728 9614	FAX: (714) 821 4010	FAX: +32 3 660 30 14	Phone: +81 3 408 3118

FAX: +81 3 408 3553

ET Docket No. 98-42 --- Ex Parte Communication	July 9, 1999-07-13

Comments from IEEE Project 802		Page 2 of 3

Based on that review, we offer the following comments:

- We support expansion of region of peak emissions from 5 MHz to 20 MHz.  We further support the location of this band in the 2460 to 2480 MHz region, as this will minimize adverse impact on existing FHSS and DSSS Part 15 systems, and promote joint use of the 2.45 GHz ISM band by both RF lighting devices and Part 15/MSS communications equipment.

- We are of the opinion that the limit for emissions in the 2400 - 2460 MHz and 2480 - 2500 MHz bands should remain at 1mV/m @ 3m as previously stated in the  second proposal.  The measurement method described in the March 15, 1999 letter from Mr. Ray LaForge to Mr. Michael Ury indicates a video bandwidth setting of 10 Hz.  For magnetrons displaying the type of spectral splatter described above, a limit of 10 mV/m is not sufficiently low to preclude the use of half-wave or full-wave rectified power supplies that have inadequate ripple suppression to prevent significant and harmful frequency transients of the magnetrons.

ET Docket No. 98-42 --- Ex Parte Communication	July 9, 1999-07-13

Comments from IEEE Project 802		Page 3 of 3

Based on these considerations, we recommend the current Part 15/MSS proposed limitation on in-band emissions be revised as follows:

- Region I (2400 - 2460 MHz):   Emissions shall be limited to 1 mV/m @ 3 m(avg)

- Region II (2460 - 2480 MHz):  Emissions shall be limited to 330 mV/m @ 3m (avg)

- Region III (2480 - 2500 MHz):  Emissions shall be limited to 1 mV/m @ 3 m (avg)

We thank you for your continued attention in this matter.  We remain hopeful that a solution that is mutually agreeable to all parties in this matter can be reached.

Respectfully,

James T. Carlo (jcarlo@ti.com)		Vic Hayes (vichayes@lucent.com)

Chair, IEEE 802 LAN/MAN Standards	Chair, IEEE 802.11, Wireless LANs

Texas Instruments		Lucent Technologies Nederland B.V.

9208 Heatherdale Drive			Zadelstede 1-10

Dallas TX 75234		3431 JZ Nieuwegein, the Netherlands

Cc:

- Chairman William E. Kennard

Commissioner Susan Ness

Commissioner Harold Furchgott-Roth

Commissioner Michael K. Powell

Commissioner Gloria Trastani

Dale Hatfield

Julius P. Knapp

Karen Rackley

John A. Reed

Anthony Serafini

Deborah Rudolph, IEEE, USA

Dr. Ned Sauthoff, IEEE, USA