IEEE P802.11
Wireless LANs

An Improved Rate Signalling

Date:	January 11, 1999

Author:	Naftali Chayat and Tal Kaitz 
	BreezeCom
	Atidim Technology Park, Tel Aviv 61131  Israel
	Phone: 972 –3-6456262
	Fax: 972-3-6546290
	e-Mail: {Talk, Naftalic}@ Breezecom.co.il

An Improved Rate Signalling

- The reliability of the rate-signaling scheme is crucial for the performance of the 802.11a Wireless LAN.

- We propose a new reliable scheme based on bi-orthogonal Hadamard coding and OFDM modulation.

- No overhead relative to current scheme

- Simple to implement.

## Overview of current scheme

- 4 bits are conveyed by QPSK modulating the sequences S1 and S2.

- t10 serves as a guard interval.

- Overall length 0.8uS+0.8uS+0.8uS=2.4uS.

- In AWGN: same error rate as rate ½ BPSK OFDM. (6Mb/s).

- Non satisfactory performance under severe multipath conditions.

# Proposed Scheme

- 4 bits are conveyed by the sequence S1 spanning 1.6uS.

- Additional guard interval of 0.8uS.

- Overall length 2.4uS – same as before.

- Coding and Modulation

- 3 LSB select one row of Hadamard 8 matrix.

- The MSB selects sign.

- The 8 binary symbols are repeated 3 times to form 24 vector.

- The vector is multiplied with a cover sequence.

- The result is used to modulate the even subcarriers of a 64 point OFDM symbol. The time domain vector has two identical halves.

- The time domain vector is cyclically extended and a window is applied to truncate it to length 2.4uS.

#### Decoding and Demodulation

##### The 32 samples signal is cyclically extended to provide 64 samples.

- A 64-point FFT is used to demodulate.

- The even subcarriers are multiplied by the cover sequence.

- The subcarriers are combined to produce an 8-point vector.

- A Fast Hadamard Transform is applied.

- The location of peak determines 3 MSBs. The sign of the peak determines the MSB.

- Both modulation and demodulation require existing H/W: namely the 64  point FFT/IFFT

- Coding and decoding require an 8 point fast Hadamard transform.

### Performance in flat AWGN

- Let Es denote the power per spectral line. Then for rate ½ k=7  BPSK we have d2free=5*4Es=20Es.

- For bi-orthogonal coding we have d2free=2*8*3*Es=48Es.

- The bi-orthogonal scheme is better by 3.8dB than the coding scheme of the section.

### Performance in severe multipath

Simulation results:  64bytes packets, 6Mb/s.  Green : errors in rate field. Blue: errors in data.

Peak to Average Power Ratio

The cover sequence assures good PAP ratios for all codewords.

| Codeword | PAP [dB] |
| --- | --- |
| 1 | 3.2 dB |
| 2 | 3.7 dB |
| 3 | 4.6 dB |
| 4 | 3.7 dB |
| 5 | 3.2 dB |
| 6 | 4.2 dB |
| 7 | 4.6 dB |
| 8 | 3.7 dB |

PAP = 3.2dB… 4.6dB.

Extension to 5 bits

- Due to the proliferation of codes and data rates, (1/3 , 9/16 etc.) there is a need to convey more then 4 bits.

- The proposed scheme can be easily extended to support  5 bits.

- Performed by QPSK modulating the H8 row.

- Decoding by complex 8-point FHT.

- Same minimum free distance of  48*Es. Double number of minimum distance neighbours => Slight degradation in performance.

Conclusions

- A reliable method for transmitting the 4 bits of the rate field.

- Requires no overhead relative to the current scheme.

- Much lower error rate than the data section even in the most reliable mode, both under flat channel and under severe multipath conditions.

- Simple to implement. Uses existing modulation and demodulation mechanisms.

- Requires Fast Hadamard Transform to be implemented.

- Simple extension to 5 bits.