IEEE P802.11
Wireless LANs

To:	Mr. Vic Hayes, Chairman IEEE 802.11, Standard Working Groups for WLAN

Cc:	Mr. Naftali Chayat, 802.11 TGa Chairman

Mr. Tadao Kobayashi Chairman of HSWA Subcommittee of MMAC-PC

Mr. K. Koga, MMAC, Japan

Date:	April 20, 1998

Subject:	Alignment of preamble between IEEE 802.11a and HIPERLAN type 2

# Dear IEEE 802.11 Officers and Members,

The Plenary of the ETSI Project BRAN would like to inform you on the outcome of the BRAN#13 meeting in April 1999 regarding the technical specifications of the HIPERLAN type 2 physical layer. In a document sent together with this letter, the status of physical layer alignment among three communities IEEE802.11, MMAC and HIPERLAN-2 is given. One of the most important issues is the preamble design that we tried to get harmonised at the IEEE/BRAN joint meeting in Orlando, FL. Unfortunately, this effort was not successful.

As stated in our Liaison Statement in October 1998, HIPERLAN-2 system needs three different preambles due to the centralised MAC protocol applied to this system. The first one is for the downlink channel BCH (Broad cast CHannel) at the beginning of a MAC frame. From the functionality point of view it is similar to the PLCP preamble in IEEE802.11a, i.e. it could be applied for AGC, timing estimation, frequency offset and channel estimation as well as antenna diversity selection. The second preamble is needed for other downlink channels: FCH (Frame control CHannel) and data transport channels SCH (Short transport CHannel) and LCH (Long transport CHannel) and could be mainly used for updating existing channel estimation or performing a new channel estimation. It might also be used for time/frequency synchronisation purposes. The third preamble is needed for uplink channels: data transport channels SCH and LCH as well as RCH (Random access CHannel). It should support timing and frequency synchronisation, channel estimation, and if required by an access point, other additional functionalities as switched antenna diversity selection.

Two proposals were submitted to the HIPERLAN-2 Physical Layer Group. Both proposals were discussed and compared based on a set of criteria agreed at the last BRAN meeting for selection purposes. We could achieve a merged solution given below that includes the advantages of both proposals.

- BCCH preamble

A16

RA16

A16

RA16

IA16

B16

B16

B16

B16

IB16

C

Figure 1. BCH preamble

The symbol A16 uses 12 subcarriers at positions +/-2, +/-6, +/-10, +/-14, +/-18, +/-22, and is defined as:

A16-26…26 =sqrt(2)*{ 0, 0, 0, 0, 1-j, 0, 0, 0, 1+j, 0, 0, 0, -1+j, 0, 0, 0, -1-j, 0, 0, 0, 1-j, 0, 0, 0, -1-j,  0, 0,

0, 1-j, 0, 0, 0, -1-j,0, 0, 0, 1-j, 0, 0, 0,-1-j, 0, 0, 0, -1+j, 0, 0, 0, 1+j, 0, 0, 0, 0}

The sign reversal of every second A16 symbol is automatically achieved by the specified subcarrier loading. The last repetition of the A field, called IA16, is a copy of the preceding RA16.

The symbol B16 consisting of 12 subcarriers at positions +/-4, +/-8, +/-12, +/-16, +/-20, +/-24 and is defined as:

B16-26…26 = sqrt(2)*{0, 0, 1+j, 0, 0, 0, -1+j, 0, 0, 0, -1-j, 0, 0, 0, 1-j, 0, 0, 0, -1-j, 0, 0, 0, 1-j, 0, 0, 0, 0,

0, 0, 0, 1-j, 0, 0, 0, -1-j, 0, 0, 0, 1-j, 0, 0, 0, -1-j, 0, 0, 0, -1+j, 0, 0, 0, 1+j, 0, 0}

The last repetition of the B field, called IB16, is a sign inverted copy of B16. Both A16 and B16 have been designed in such a way that the resulting time domain waveforms have a low PAPR (Peak-to-Average-Power-Ratio) and a smal dynamic range. For A16, the PAPR is 2.13 dB and the dynamic range is 6.13 dB, and for B16 the PAPR is 2.24 dB and the dynamic range is 7.01 dB. The C part is the preamble applied to other downlink channels given below. The length of BCH preamble is 16 us and is equal to that of IEEE802.11a PLCP preamble and MMAC preamble. The rational behind the introduction of fields A and B in the preamble is to reduce the fals alarm probability by disinguishing between downlink and uplink preambels of HIPERLAN-2 and also by distiguishing between HIPERLAN-2 BCH preamble and an IEEE802.11a like preamble, when both devices are operating in the same geographical area at the same time.

- FCH, SCH and LCH preamble for other downlink channels

C32

C64

C64

Figure 2. FCH/SCH/LCH preamble.

The symbols C64 is equal to the T1/T2 symbol defined in IEEE802.11 draft standard, and is defined as:

C64-26…26 = {1, 1, -1, -1, 1, 1, -1, 1, -1, 1, 1, 1, 1, 1, 1, -1, -1, 1, 1, -1, 1, -1, 1, 1, 1, 1, 0, 1, -1, -1, 1, 1, -1, 1, -1, 1, -1, -1, -1, -1, -1, 1, 1, -1, -1, 1, -1, 1, -1, 1, 1, 1, 1}

- RCH, SCH and LCH preamble for uplink channels

B16

B16

B16

B16

IB16

C32

C64

C64

(a)

B16

B16

B16

B16

B16

B16

B16

B16

B16

IB16

C

(b)

Figure 3. SCH/LCH/RCH preambles: (a) Option 1, (b) Option 2.

The C part is the downlink preamble given in 2) and the B16 symbol is the same B16 symbol used in BCH preamble given in 1). Option 2 is introduced to enable efficient smart antenna solutions at the access point. By ordering longer preambles in uplink channels, an access point assures that it has enough time and information to perform e.g. antenna switching. Which one of the preambles is used by a wireless terminal in an uplink channel will be decided by the access point and could be signalled by a respective bit in the MAC frame to the corresponding wireless terminal.

The solution BRAN community achieved for HIPERLAN-2 preamble could be considered as a further step toward a world wide harmonised physical layer for different systems currently being standardised in Europe, Japan and the USA for the operation in the 5 GHz band. The positions of subcarrier loading in B16 symbol are similar to that of the short symbols of IEEE802.11 a PCLP preamble. The reason, why the later ones are not used, is the higher PAPR (3.01 dB) and especially higher dynamic range (30.82 dB). We believe that the replacement of your short symbols by B16 symbol used in the HIPERLAN-2 is not only in the sense of further harmonisation of preambles, but also could have performance benefits for IEEE802.11a. Therefore we would like to ask you taking this into account in your forthcoming meeting in May 1999 in Japan. The sign inversion of the last short symbol in IEEE802.11a PLCP preamble is another item that might be considered in this meeting. The BRAN HL2 PHY group has identified the sign inverted last repetition of the short symbols is beneficial for improving timing detection accuracy, simplifying the synchronisation processing, increasing the receiver implementation flexibility (e.g. auto-correlation based or cross-correlation based) and providing unique identification possibilities of the last short symbol repetition. Two members of our expert team will attend the meeting to discuss these issues with the 802.11 members. They would like also to address some other minor topics like interleaving permutation and pilot sub-carriers scrambling that need more careful consideration before harmonisation.

Sincerely,

Jan Kruys, Chairman ETSI Project BRAN

Jamshid Khun-Jush, Co-ordinator of HIPERLAN type 2 Standard Area and PHY TS Rapporteur

Date:	April 15, 1999

Source:	PHY TS Rapporteur

Title:	HIPERLAN Decision List

Agenda item:

| Parameter | # H/2 Current working assumption or decision | Harmonised with IEEE? | Harmonised with MMAC-WATM-WG? |
| --- | --- | --- | --- |
| Channel spacing | ## 20 MHz (decided) | ## Yes | Yes |
| Sampling rate | 20 Msample/s (decided) | Yes | Yes |
| FFT length | 64 (decided) | Yes | Yes |
| Number of used sub-carriers | 48 data sub-carriers and 4 pilot sub-carriers (decided) | Yes | Yes |
| Sub-carrier modulation | BPSK, QPSK, 16QAM, optionally 64 QAM (decided) | # Yes | Yes |
| Signal constellation and bit mapping | Decided | Yes | Yes |
| Demodulation | Coherent (decided) | Yes | Yes |
| FEC mother code | Convolutional code, rate ½ (decided) | Yes | Yes |
| Code termination | Method (decided), additional puncturing patterns to be decided at BRAN#13.5 | No | Yes |
| Guard interval | 800 ns corresponding to 16 time samples (decide) | Yes | Yes |
| PHY modes & code rates | 7 modes (decided) 6, 9, 12, 18, 27, 36, 54 Mbps | Mainly yes 6, 9, 12, 18, 24, 48, 54 Mbps | Yes |
| Oscillator accuracy | +/- 20 ppm (decided) | Yes | Yes |
| Linkage of Oscillators | Generation of RF and baseband timing from the same reference oscillator | Yes | Yes |
| Generator polynomial for data scrambling | X^7+ X^4 + 1 (decided) | Yes | Yes |
| Scrambling of pilot sub-carriers | Generator polynomial X^7+ X^4 + 1 (decided), scheme not decided (two alternatives, IEEE and BRAN specific ones) | Generator polynomial Yes, scheme not clear | Generator polynomial Yes, scheme not clear (IEEE scheme adopted) |
| Preamble structure (based on quarter symbols, 16 samples) | BCH preamble, DL preamble (FCH, SCH, LCH) and short version and long version of UL preamble (SCH, LCH, RCH) (decided) | BCH preamble and the long version UL preamble have the same length as IEEE | BCH preamble the same length (content under discussion), DL Yes, UL under discussion |
| Interleaving | OFDM symbol wise (decided), permutation (working assumption) | Method Yes, Permutation No | Method Yes, Permutation No |
| Pulse shaping | TX power mask + modulation constellation accuracy (method decided, IEEE TX power mask and IEEE values for modulation accuracy working assumption) | Method Yes, Modulation accuracy values Yes, TX power mask Yes | Method Yes, Modulation accuracy values Yes, TX power mask No due to specific Japanese regulation matters |