IEEE P802.11
Wireless LANs

802.11 MAC Enhancements, Draft Requirements

Date:	January 12, 2000

Author:	Tim Godfrey, John Fakatselis
	Intersil
	Palm Bay FL
	913-706-3777 321-724-7000 
	
	e-Mail: tgodfrey@choicemicro.com jfakat01@intersil.com

### Performance Test Matrix

| Scenario |  | Channel PER |  |  | Overlapped BSS | Output Parameter (top of MAC) |  |  | Baseline MAC performance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | # streams | 0 | 10% | 25% |  | Throughput per stream/application | Latency within stream (MAC to MAC delay) | Jitter within stream |  |
| MPEG1 @2.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 1 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 2 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 25% duty. |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 4 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 50% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Network Game |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 6 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 4 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Video Conference |  |  |  |  |  |  |  |  |
| HDTV @20M | 1 |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |

- Overlapped BSS – two identical test configurations operating in the same area on the same channel.

- Baseline MAC Performance – the results of running the test with the current 802.11 MAC. Allows for validation of simulation environments.

# Definition of Class structure:

## Audio

### MP3 (up to 128Kbps)

### Redbook audio (1.45Mbps)

### Dolby Digital, and others  (up to 768Kbps)

## Video

### MPEG1 (up to 3Mbps)

### MPEG2 (4 to 11Mbps)

### HDTV (20Mbps and higher)

## Voice

### High Quality Telephony (32 to 64Kbps)

### Low Quality Telephony (6 to 16Kbps)

## Data

### Bursty (POP3 client, WWW, large numbers of small packets)

### Bulk (File Transfer, multi-megabits of large packets)

## Interactive Multimedia

### Head to Head Games (networked multiplayer games – ex Doom)

### Video Conferencing (H.323, Netmeeting)

# Definition of Source or Service (input pipe)

## 1 to 4MBps (ADSL, WLL)

## 4 to 20MBps (Cable Modem, LMDS)

## above 20MBps (enterprise infrastructure, point to point wireless bridging)

# Topology Structures (testing to be performed for each of these 2 cases)

## At least a 5 node network, one BSS.

## Two co-located BSS’s on the same frequency.