IEEE P802.11
Wireless LANs

TGe Functional Requirements

Date:

July 13, 2000

Author:

Tim Godfrey, Jesse Walker
Intersil, Intel
Phone: 913-706-3777, 503-712-1849
Fax: 913-664-2545
e-Mail: tgodfrey@choicemicro.com, jesse.walker@intel.com

# General Requirements (**)

## Any changes to the standard must be optional. This standard can not make a device conformant to the existing 802.11 standard non-conformant.

## Any changes to the standard must remain compatible with legacy equipment (both APs and stations, and both DCF and PCF modes).

### Association acceptence decisions must remain a policy decision of the AP or station and must not become requirements in the standard.

### Changes to frame formats must be compatible with existing formats.

#### Capabilities must fit in remaining bits of CIF

#### Extensions to existing frames must use the information element data structure or existing reserved bits.

#### New frame subtypes of existing types should be used in preference to the currently reserved fourth frame type.

### New frame formats should be kept to the minimum required to meet the requirements.

# External Requirements

## Higher Layers

### Do not duplicate functions provided by higher layer standards, except where the nature of the wireless medium breaks an assumption of the higher layer standard.

# QoS Requirements

## Corrections and enhancements to the PCF that may be required to best meet QoS performance objectives must be incorporated.

## Differential handling of MSDUs supplied to the MAC with additional priorities and classes of service.

## Bounded delay, prioritized acess, and bounded latency per MDSU (allocatable services), power management bypass mechanisms (which has priority in iBSS and BSS may need a mechanism for separable handsets).

## MAC SAP support for 802.1D/802.1q.

## Transmit Power Control.

## To provide the hooks in the MAC to obtain remote channel information.

# Security Requirements

## General

### The standard must add at least one extension to the authentication algorithms that provides mutual authentication in both Infrastructure and Independent BSSs.

### In the standard, security requirements are independent of QoS requirements. However, implementers should be aware of the potential interactions.

#### The extensions to the standard should not be constrained by QoS requirements.

#### It is an implementer decision as to which algorithms are to be used over and above the baseline standard and whether that choice is compatible with QoS requirements.

## Authentication

### Security framework must be able to prevent unauthorized access by unauthenticated peers over the link.

### Security framework must allow for mutual authentication of STA and AP.

### Security framework must allow for authentication of the source of each packet, to prevent link hijacking or undetected insertion of rogue packets into the link.

## Privacy

### Security framework must protect network traffic from eavesdropping to a reasonable level compatible with the state of the art.

## Keys

### Security framework must allow key distribution or derivation of per-link or per-session keys

### Security framework must strongly protect keys and passwords from recovery by eavesdropper

## Extensibility, Compatibility, and Interoperability

### Negotiation of authentication and privacy algorithms must be incorporated.

#### The following negotiations must be supported:

##### authentication algorithm

##### privacy algorithm

##### data integrity algorithm

##### key establishment algorithm

##### one way hash function for sub key derivation algorithm

##### key expiration

#### Inability to complete negotiations must be able to cause a failure to authenticate.

### A flexible mechanism for adding interoperable security algorithms must be incorporated, so that the standard does not need to be revised to use new algorithms in the future.

### The standard should specify one set of algorithms as mandatory when security extensions are implemented.

## Security framework must scale to:

### Simple environments (etc., home, SOHO)

### Ad hoc wireless LANs

### Enterprise environments (e.g., office campuses, factories)

### Public environments (e.g., hotels, public services)