Received:	16 March 2000

| Joint Rapporteurs Group 8A-9B (Task 4) | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8A-9B/TEMP/75-E 17 March 2000 English only |

working document

methodology for assessing the required spectrum for 
generic broadband nwa networks

# 1	Background

At the 6th JRG 8A-9B meeting a draft work plan was agreed for spectrum requirements for Broadband Nomadic Wireless Access (BWA) systems, which encompasses service applications with a net bit rate larger than 10 Mbit/s that use RLAN technology for low mobility public and private networks. One of the items for consideration in the draft work plan was:

•	to ascertain the required amount of frequency spectrum for generic broadband NWA networks assuming maximum deployment scenarios in clear spectrum.

The first step towards this is to devise a methodology relevant to BWA systems for assessing the required spectrum need. This document presents various methodologies; it is also intended as a working document. To this end, a number of open problems and necessary action points are listed in the present version. The document is based on Document 8A-9B/201.

The methodology could possibly involve the following elements:

•	the identification of a number of demanding scenarios in terms of the required throughput of a RLAN terminal;

•	estimation of the average and burst capacity (bits/s) per user for each of these scenarios with their typical applications;

•	determination of the minimum amount of spectrum (number of channels) needed to achieve the required capacity.

The most difficult scenarios will determine, to a large extent, the spectrum needed to successfully operate RLAN systems. The following scenarios, or reference cases, have been selected provisionally:

•	an office environment;

•	an industrial plant;

•	a public environment;

•	a domestic deployment.

A number of applications and activities are then associated with each scenario, the goal is to arrive at an estimated capacity requirement per RLAN user. The next step is to map the capacity requirement on the corresponding amount of spectrum needed. If one deals with a centralized (cellular) system, a first stab could be to assume a regular hexagonal cell pattern and compute the required number of channels based on the signal-to-interference ratio, which is in turn determined by the required bite rate for a given scenario. The resulting spectrum requirements for the above reference cases should be treated as typical values to guide future spectrum designations.

The next section provides a general introduction to RLANs and describes the above scenarios. Section 3 then further elucidates the scenarios chosen and a number of typical activities associated with each scenario are listed. In Section 4, possible methods for estimating the required spectrum need based on the required bit rates (for each scenario) are described. Annex 1 supplies estimated bit rates (subject to changes) for the activities listed in Section 3.

# 2	Introduction on Radio LANs

[Wireless networks have enjoyed an increased demand from the general public as well as from business and other professional users. The networks in existence today range from cellular phones to high speed digital networks supporting high speed computer communications. They operate in licensed as well as in unlicensed frequency bands. Examples of RLANs, their operating frequencies and applications can be found in DNR ITU-R N.[8a9b T4/DD] (Document 8/58).

At the same time, wired telecommunications networks have shown a remarkable evolution towards higher transmission rate and support for multimedia applications rather than simple voice oriented services.]

NOTE - Introductory text on the RLAN services and typical requirements in terms of bit rate.

Multimedia applications are becoming popular and are already beginning to demand wireless transport with a high quality of service. Multimedia applications shall be taken into account when defining the RLAN family.

Multimedia covers anything from basic messaging through to audio, video or any combination thereof. At the transport layer multimedia consists of two types of information flow; firstly the delivery of fixed packages of information and secondly the delivery of a stream of information which can be described by a certain data rate and delay tolerance.

As users get accustomed to this QoS in their wired systems they are going to demand the same QoS on wireless systems. RLAN shall support IP and ATM applications and QoS.

## 2.1	Relation to IMT-2000

The range of bit rates, with a maximum of 2 Mbit/s, supported by IMT-2000 is geared primarily towards voice and low quality video as well as data services. However, because of spectrum limitations as well as for economic reasons, IMT-2000 will not be able to meet the bandwidth demands of true, high resolution multimedia communications. These require bit rates in the range of 10 Mb/s. The required bandwidth is not available in the presently planned IMT-2000 frequency range and it is likely that the cost to users of such bandwidth would be excessive. Furthermore, it is not clear that there exists demand for such high speed services beyond the premises of a business or other organization. On premises, short range wireless networks that do not share spectrum with IMT-2000 are much more attractive and flexible as a solution to multimedia wireless networking. RLANs fill that need. The following figure clarifies the relationship between RLANs and IMT-2000:

Figure 1

The relationship between RLANs and IMT-2000

## 2.2	Application Scenarios for RLAN

[This subclause describes different scenarios in which RLANs may be used.

•	Infrastructure replacement scenario, i.e. when could be used instead of cabling.

•	Cordless access scenario, in which users need to use RLANs in different locations at different times possibly maintaining connectivity while in transit.

•	Wireless access to infrastructure scenario.

•	Specialized portable applications scenario, i.e. user uses a PDA type device mainly for specialized applications, e.g. maintenance or surveillance.

•	Domestic premises scenario, i.e. RLANs are used in the home environment.

•	Wireless manufacturing automation scenario, i.e. RLANs are used in a factory or a large assembly/building facility.

•	Inter-network communication scenario.]

NOTE - Introductory text needed.

### 2.2.1	Office RLAN scenario

[Text needs to be modified] RLANs can be used for wired infrastructure replacement in a number of scenarios including the replacement of wired premises networks. Typical cases could be temporary office installations or installations into spaces where building characteristics or protection prohibit the extensive use of a cabling.

[The infrastructure to be replaced includes stationary backbone networks operating at high speeds as well as wireless network terminations.]

Terminals typically connected to infrastructure networks are designed for fixed use. Such a terminal could, for example, be a workstation, a PC or any other purpose specific terminal. The applications are typically broadband applications. In this scenario the user device is mostly stationary and the main benefit derived from RLANs is the wireless dimension. Thus, RLANs shall provide or approximate fixed network QoS to a stationary user. The user should not be able to notice the difference between using the wireless system and a wired system.

Table 1

An example of an office RLAN scenario

| Attribute |  |
| --- | --- |
| End-user equipment | PC or workstation |
| Usage environment | Offices etc. |
| Range | Up to 50 metres for indoor systems |
| QoS expectation | Same as fixed installation |
| Applications | Same as fixed installation |
| Mobility requirement | Limited |
| Coverage | Continuous within the premises |
| User Density | High [in the office] |

### 2.2.2	Public access scenario

In this scenario, the RLAN user needs to perform his or her work at different locations at different times. The main end-user equipment would be a portable computer. Typically such a user would carry a portable computer to various places within the office and then use the computer while stationary. Typical places for using the RLAN system outside an office room would be meeting rooms, dining facilities, patient wards, classrooms and auditoria as well as waiting rooms/halls. A cordless user will also access the public network, through base stations installed in locations such as railway stations, airports and shopping centres. In some cases, connectivity has to be maintained while the user is in transit from one location to another.

The terminals in this scenario are movable. A typical terminal could be built around a laptop computer and a RLAN card. The mobile node will in many cases be a battery driven device so that an economic consumption of power is required.

Table 2

An example of a public access scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Portable computer, e.g. Notebook or Palmtop |
| Usage environment | Offices, schools, hospitals, airports, railway stations, shopping centres, etc. |
| Range | Up to 50 metres for indoor systems Up to 150 metres for outdoor systems |
| QoS expectation | Similar to fixed installation |
| Applications | Similar to fixed installation |
| Mobility requirement | None during use |
| Coverage | Continuous in hot spots |
| User Density | High (e.g. in a meeting room) |
| [Power consumption] | [Low] |

### 2.2.3	[Specialized portable applications scenario]

In the third scenario a user has a small (possibly dedicated) system like a PDA to access services. The applications are typical broadband applications, which shall be supported for mobile users with an acceptable QoS by the mobility functions in the network, e.g. handover. The QoS expected from the RLAN system in this scenario could however be somewhat lower than the QoS of a fixed system. The user can be assumed to realize that a small loss in QoS is the price paid for the mobility gained. For example, the connection might tolerate a short interruption because of a handover (resulting in momentary disturbance in the video picture), etc.

The terminal in this scenario is a mobile handheld terminal e.g. a PDA with a wireless network card or a dedicated mobile node such as a WEBpads, Gamepad, WEBteevee. The applications are mostly dedicated mobile applications that are capable of operating at a lower QoS, as they would use mobile specific features to compensate for some mobile related problems.

The mobile node will in many cases be a battery driven device so that an economic consumption of power is required.

Table 3

An example of a specialized portable scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Hand portable unit, PDA |
| Usage environment | Anywhere within or near private premises |
| Range | Up to 50 metres for indoor systems Up to 150 metres for outdoor systems |
| QoS expectation | Modest, but maintained during movement |
| Applications | Dedicated, could be mobile specific |
| Mobility | Walking speed or slow vehicle (e.g. forklift) |
| Coverage | Continuous |
| User Density | Low |
| Power consumption | Very low |

NOTE - Not critical from a spectrum viewpoint, should possibly be removed. Is this scenario attractive since it seems to require an operator to offer contiguous coverage?

### 2.2.4	Domestic Entertainment scenario

NOTE - This section will be updated following analysis of domestic entertainment scenarios.

In the domestic network scenario, many appliances, e.g. PC laptop, printer/fax machines, security systems, home appliances, digital HDTV/SDTV sets, digital Video Cassette Recorder (VCR), speakers and more could be linked in various ways. A typical scenario would be:

1)	An entertainment cluster (video and sound) located in the living room transmitting to television sets located in the living room, kitchen and bedroom.

2)	A music system in the living room transmitting to speakers located in the living room, bedroom or dining room.

3)	Security features outside the home such as wireless security camera or remote sensors. These could either be located on the external walls of the property or at the boundary wall or a remote building such as a garage or recreation facility.

4)	Outdoor speakers for barbecue/party. Assuming that the music system is located in the entertainment cluster in the living room, the transmission path and length may extend into the garden.

From the above, it is obvious that the domestic network shall allow access to external networks e.g. digital television or be capable of working with no external links e.g. a music system with remote speakers. This system should be easily installable by non-technical people.

A domestic network generally covers a much smaller area than either factory or office environments. The rooms in a domestic premises tend to be smaller when compared to work environments and have more compartmentalized structure (storage spaces and en-suites).

Table 4

An example of a domestic premises scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Computer, television, entertainment cluster, security systems, etc. |
| Usage environment | Domestic premises, i.e. small rooms with high attenuation |
| Range | Up to 15 metres |
| QoS expectation | [Consistent with real-time multimedia services] |
| Applications | Real-time multimedia, TBD |
| Mobility requirement | Walking speed |
| Coverage | Continuous within premises |
| User Density | Low |

NOTE 1 - User density requires further explanations.

NOTE 2 - QoS: video cannot set the requirements nor be the basis for spectrum requirement.

### 2.2.5	Industrial and transportation scenario

In manufacturing scenarios such as process automation, commissioning systems, baggage transfer, distribution systems, warehouse storage and retrieval services we have a large number of intelligent transportation elements which may move autonomously and automatically in a factory hall, a storage building, or in an airport. Such a system should cover approximately an area of 250 m × 250 m. Delay values and data losses are critical. The ability to support highly reliable real-time control and alarm data as well as other time bounded services is mandatory. Power consumption and the physical size of the communication device are not as critical as in other scenarios.

Table 5

An example of an industrial scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Intelligent transportation elements, autonomous, automatic vehicles, surveillance systems, monitors |
| Usage environment | Factory halls, airports, storehouses, industrial environments |
| Range | Up to 50 metres. Shadowing, highly variable radio channels |
| QoS expectation | Low delay, [high error sensitivity, time bounded, real-time, short packets] |
| Applications | Mobile, file transfer, control, alarms, surveillance, monitoring |
| Mobility requirement | < 10 m/s |
| Coverage | Continuous within premises |
| User Density | High (variable) |
| Power consumption | Not critical |

NOTE - Not critical from a spectrum standpoint, just as the "Specialized …"

Change … This document describes the applications of broadband RLANs and derives the spectrum needed to supports these applications. The methodology used makes estimations of the average and burst capacity needed per user in Mbit/sec. Using the expected radio performance of these broadband RLAN devices, the spectrum in MHz is derived that would be needed to support the most difficult application environments (large offices and large public installations).

# 3	Reference cases for assessing the spectrum required

To assess the amount of required spectrum for RLANs [a number of particularly demanding application scenarios], reference cases, have been chosen. These have been picked out of larger number of possible scenarios.

NOTE - Explanation of the general structure of the section.

## 3.1	Proposal for the estimation of user data rates

RLANs may be used in a number of different applications and in different scenarios. These scenarios determine to a large extent the spectrum needed to successfully operate RLANs. For details of these Environments and Scenarios, the reader is referred to Annex 1.

The following sections detail the application requirements for the scenarios described in Annex 1.

Each scenario is broken down further into typical activities and shows estimated data rate requirements for each activity. The following steps are used to determine the bit rate requirements for each scenario:

1)	The uplink and downlink average data rate is estimated for each activity (Internet browsing, for example).

2)	The peak data rate for each activity is estimated.

3)	The usage of each activity is estimated in order to obtain a weighted average data rate (using item 1) for each scenario.

Reference to Annex with figures

These figures will be used shortly to compute estimates of the spectrum required to support typical office use of RLANs.

## 3.2	Application requirements for the reference cases

### 3.2.1	Office RLAN deployment scenario

The following activities are expected in an office deployment scenario for RLAN over the next two or three years. A list and brief description of office related activities that could be supported by RLANs follows:

NOTE - Typical data rates are needed for each activity.

Multimedia conference (large video displays)

High quality video/audio channels with multiparty data links for the transmission of still images as well as the exchange of computer data including shared multi-user applications.

Telephone/Audio

From toll quality telephone service to higher quality audio.

General networked computing applications

Examples of applications are: Client-server, Processing, Printing, E-mail, Messaging, Fax, Groupware, Games and Simulations, Network file systems, etc. The transfers are generally asymmetric and highly bursty. The data rate requirements are quite dependent on the level of mobility, i.e. the quality should be very similar to that one offered by a fixed LAN on a static mobile node, and temporarily degraded while on the move. Moreover, the bit rate should correspond to the processing speed of the terminal i.e. PDA, portable computer or workstation.

The requirements for a wired-LAN QoS are upper bounds and may be considered as a basis for a wireless LAN.

Multimedia database

Encyclopaedia browsing, medical diagnosis records, electronic newspaper, bulletin board, World Wide Web, manuals, etc. Includes asymmetric, resource demanding applications and bursty non-real-time data.

Security and monitoring

Surveillance video/audio, Industrial or office security service, Alarms, etc.

Internet and Intranet Browsing

The Internet has gained prominence far beyond the expectations expressed by experts only a few years ago. Today businesses of all kinds make extensive use of Internet and Intranet as a means to disseminate information about their products and services. Similarly, government institutions are getting ready to put their information on the Net. With the emergence of electronic payment the Net will become a commercial environment as well. For many international organizations, including ERO and ETSI, the Net has become an indispensable tool. As a consequence users spend hours a

day "surfing" the Net to find and exchange information. This information is typically not just text form but includes extensive graphics as well as, in some cases, video and audio sequences.

Teleworking

Less prominent but gaining ground is the notion of teleworking. Teleworking may mean working at home but being in contact with colleagues at work and with customers through video/voice/data sessions. It also means collaboration between geographically separated persons, possibly a group of them. Here too, the ability of telecommunications to deliver high quality video and sound as well as real time data allows users to avoid costly and time consuming travel. Application developers have caught on to this opportunity. A variety of "screen sharing" tools is being developed that provide users with the means to work together in real time on the same electronic documents while being in eye and ear contact. Much like the Net browsers opened up the demand for Internet services so these sharing tools will create a large demand for teleworking services.

### 3.2.2	Public RLAN deployment scenario

The applications are the same as in the office scenario, since public access will mostly be available in geographically small (hot spot) areas. However, the usage of each application may be different. These hot spot areas will be at e.g. airports, hospitals, conference sites etc., i.e. areas in which the same access network can be used for both public (guests, customers etc.) and private (employees of the operator) access. For simplicity, the same figures can be used for pure public access networks, e.g. in city centres.

### 3.2.3	[Industrial RLAN deployment scenarios]

A list of industrial activities that can be supported by RLAN follows:

Gatelink

Gatelink is a typical example of multimedia networking in an industrial environment. The applications are in aircraft maintenance support, software loading of airborne systems, passenger service and entertainment, pilot briefing and backup of aircraft maintenance systems. The data rate requirements of Gatelink are not analysed further in the present document.

Manufacturing Applications

In process automation, commissioning systems, baggage transfer and distribution systems we will find a mixture of services. Services will include non-real-time data for file transfer, software and configuration data download, as well as very time critical (real time) data transfer for control and alarm data. Also a mixture of conversational multimedia services for surveillance and monitoring purposes is needed.

Industrial Remote control

Remote control of some device. High quality asymmetric video/audio (MPEG-1 or MPEG-2, possibly multichannel and/or stereo picture), control information and computer data.

Industrial monitoring

Industrial monitoring is a specific application in industrial environments. The applications are for instance monitoring of oil pipelines or monitoring of production processes and resources like tanks in chemistry plants. Data is typically generated by a sensor, is very small as well as specific and has very stringent delay bound and variance. Normally the bandwidth needs are low. However, in certain circumstances (for instance fire or explosions) a very bursty and strongly correlated traffic can be generated by hundreds (thousands) of sensors which has to be handled by the network according to the QoS requirements.

### 3.2.4	Domestic RLAN deployment scenario

NOTE - This section will be updated following analysis of domestic entertainment scenarios.

# 4	Calculation of the spectrum requirement

NOTE 4.1 - This section describes methods that map the data-rate requirements to spectrum requirements. Traffic models are needed.

The typical environment for the use of RLANs is characterized by very high user density - e.g. up to 1 user per 10 m2 - supported by a significant number of access points. Therefore, in many cases, the propagation environment is dominated by multipath effects and by interference from adjacent RLANs operating either co-channel or on an adjacent channel. In such an environment the amount of spectrum required is driven by the required carrier to interference ratio which in turn depends on the bit rate required for a given environment or type of application.

The multipath effects determine the technology used and have little direct influence on the spectrum required.

[NOTE - Even where RLANs are used for access to public infrastructure networks like IMT-2000, the propagation environment is likely to be indoor. However, the scale of such an indoor environment may be such that free space propagation dominates over most if not all of the area covered.]

The parameters to be considered are at least: [these apply for the method described below]

–	bit rate;

–	carrier-to-interference ratio;

–	access point spacing;

–	user density per m2.

NOTE 4.2 - The following text describes one particular method to estimate the spectrum requirement for a centralized cellular system. Other methods may be devised for this architecture, and particularly for other architectures such as ad hoc networks (peer-to-peer).

## 4.1	Use of a regular hexagonal cell plan

NOTE 4.3 - This needs more explanation.

For a given number of available channels, a channel plan must be created. A commonly used channel plan is the symmetrical hexagonal channel plan. For such channel plans, it can be shown [2] that

DR=3K	(Eq. 1)

where R is the cell radius, D is the reuse distance (i.e. the distance between two co-channel cells), and K is the number of available channels. A symmetrical hexagonal channel plan can only be obtained for certain values of K, but the expression above is in the following used for all integer values of K.

The carrier to interference level for a user on the cell edge in a large system can be calculated [2] as:

SIR≈P1αNPDRα=DRαN=3Kα2N	(Eq. 2)

where P is the transmitter power,  is the propagation exponent and N is the number of simultaneously active interference. In a symmetrical hexagonal channel plan, a total of 6 interferers are located at the distance D/R from the user. As it is unlikely that all of these interferers are active simultaneously, N=3 is used here, meaning that half of the interferers are active.

Thus, the SIR level for a user depends on the number of available channels and the propagation exponent. It shall be noted that a symmetrical hexagonal channel plan may not always be possible to achieve, especially when an automatic frequency allocation algorithm is used. In many cases, the SIR will therefore be worse than what is estimated here.

When the SIR value is calculated, the achievable throughput for a user can be determined from link layer simulations. In [1], the packet error rate as a function of SIR is given. The results for HIPERLAN 2 are shown in the figure below.

From the packet error rate (PER), the link throughput can be calculated as Data rate*(1-PER) [1].

The link throughput is the average throughput that can be achieved when an AP transmits (or receives) 100% of the time. This value is then divided between the MTs connected to the AP. Further, the net throughput/MT is obtained by multiplying the throughput with the protocol efficiency. Finally, the full time equivalent bit rate is the bit rate that can be achieved during the fraction of time a MT is active.

NOTE - The above does not include the effects of bit rate dependency in the protocol overhead. This will be added in a subsequent version.

Example. The hexagonal channel plan applied to an office building. The propagation exponent is  = 3.5, and K = 8 channels are assumed to be available for a single operator.

Input:

Propagation exponent 3.5

Number of channels 8

m2 / user 10

APrange 22

Number of APs 40

Protocol overhead 0.2

Network access factor 0.3

Results:

C/I level (dB) 19.4 (from Eq. 1)

Table 6

Throughput per user (HIPERLAN 2) for an office environment

| Data rate in Mbit/s | PER | Throughput in AP (Mbit/s) | Throughput per MT (kbit/s) | Net Throughput per MT, incl. overhead (kbit/s) | Full time equiv. bit rate per user (kbit/s) |
| --- | --- | --- | --- | --- | --- |
| 6 | 0.00 | 6.00 | 31.0 | 24.8 | 83 |
| 9 | 0.00 | 8.99 | 46.4 | 37.1 | 124 |
| 12 | 0.00 | 12.00 | 62.0 | 49.6 | 165 |
| 18 | 0.01 | 17.88 | 92.3 | 73.9 | 246 |
| 27 | 0.02 | 26.41 | 136.4 | 109.1 | 364 |
| 36 | 0.10 | 32.36 | 167.1 | 133.7 | 446 |
| 54 | 0.44 | 30.16 | 155.8 | 124.6 | 415 |

For the given input, the average throughput per user (column 6) peaks for the 36 Mb/s PHY mode and equals 446 kbit/s. The coverage area (assumed to be square shaped) per access point is calculated as 4*(APrange)2. From Table A.1 (Annex 1), it is seen that the required average throughput per user is 570 kbit/s, i.e. the requirement is not fulfilled in this case. For K=16, the throughput will peak at 667 kbit/s/user for the 54 Mb/s PHY mode (an option for HIPERLAN 2 systems), and the bit rate requirement will be fulfilled. This example shows how the achievable throughput increases when more channels are available.

NOTE - The above example assumes that all users ("on average") can use the same PHY mode. Simulations may be used to account for the fact that different users employ different PHY modes depending on their prevailing propagation conditions and interference conditions.

NOTE - The impact of interference from other adjacent (uncoordinated) systems is not accounted for.

## 4.2	[Examples cases]

The spectrum requirements presented below are based on the required useful data rates analysed above for a large office area with access to a wired network scenario, taking into account the relevant spectrum re-use factor for different data rates and the spectral efficiency of the modulations that can be used. The resulting spectrum requirements should be treated as typical values to guide future spectrum designations.

### 4.2.1	Wireless access networks for office use

The office reference scenario consists of a large office building with 2 400 users. Although there are large buildings with more users, this scenario is used as typical of a large scale deployment of RLANs. Similar figures will apply to hospitals, schools, shopping centres, convention centres etc.

–	Total area covered: 24 000 m2, (approx. 160 metres × 160 metres, 3 floors of 40 x 200 metres, or 10 floors of 40 x 60 metres).

–	Number of users: 2 400 (at 10 square metres per user).

–	AP range: 20 m

–	Propagation exponent: 3.5.

–	Average throughput required: 570 kbit/s per user (see Table 11).

In this case, at least 14 channels would be required to provide the desired data rate of 570 kbit/s per user.

### 4.2.2	Wireless access networks for public use – Conference hall

The conference hall (or exhibition hall) reference scenario consists of an open area with line of sight propagation.

–	Building size: 200 x 120 m.

–	Number of users: 1 200 (at 20 square metres per user).

–	Number of APs: 40.

–	AP range: 20 m.

–	Propagation exponent: 2.0.

–	Average throughput required: 750 kbit/s per user (see Table 11).

In this case, at least 16 channels would be required to provide the desired data rate of 750 kbit/s per user.

This reference case has also been evaluated by system simulations (see Appendix B). The simulations indicate that 14 channels would be needed in this reference case. The difference in the results partly arise from the fact that the simulations are carried out in a system of limited size (40 APs), but the analytical results are obtained from a large scale deployment.

### 4.2.3	Domestic Entertainment Scenario

To be added.

## 4.3	References

[1]	J. Kun-Jush et al., "Structure and Performance of the HiperLan/2 Physical Layer", VTC'99 Fall (Amsterdam).

[2]	Lee, William C.Y. "Mobile Communications Engineering - Theory and applications", McGraw-Hill Telecommunications, Second edition 1997.

Annex 1

Examples of predicted average data rates for the reference cases

In this Annex, examples of predicted average data rates for the office, public and industrial scenarios are given. The following steps are used to determine the bit rate requirements for each scenario

1)	The uplink and downlink average data rate is estimated for each activity (Internet browsing, for example).

2)	The usage of each activity is estimated in order to obtain a weighted average data rate (using item 1) for each scenario.

## A.1	Office RLAN deployment scenario

Table A.1 shows the estimated data rate per RLAN device for each activity listed in Section 3.2.1. The average data rate is calculated according to the steps above. Peak rates are also given. The data in the table can be used in different ways depending on the method to obtain the required amount of spectrum.

Table A.1

Predicted average data rate per RLAN, office deployment

## A.2	Public RLAN deployment scenario

In Table A.2 the required data rates for the applications in a public deployment scenario are shown. Just as in the office scenario, the average data rate per person is also listed. The listed applications are the same as in the office scenario, since public access will mostly be available in geographically small areas. For simplicity, the same figures can be used for pure public access networks, e.g. in city centres.

Table A.2

Predicted average data rate per RLAN, public deployment

## A.3	Industrial RLAN deployment scenarios

Table A.3 provides a breakdown of the data rate capacity required to support a typical industrial deployment of a RLAN network on a piece of industrial plant or machinery assumed to contain 50 separate RLAN equipments operating in a single radio locale defined by the operating radio range.

Table A.3

Predicted average data rates per RLAN for an industrial deployment

## A.4	Summary of data rate requirements for RLAN deployments

A summary of the data rate requirements based on the example deployments listed above and analysed in Tables 7, 8, 9 and 10, is given in Table 11. The table includes some reasonable assumptions for the numbers of RLAN terminals that would exist in each deployment and shows how the total data rate is calculated in each case. The table also includes factors for the efficiency of the network protocol (e.g. TCP/IP) and for the protocol efficiency of the air interface which takes into account the signalling traffic generated by the operation of the RLAN MAC protocol which reduces the available channel capacity.

Table 11

Summary of data rate requirements for RLAN deployments

_________________