| Received: 10 March 2000 | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Delayed Contribution Document 8A-9B/201-E 13 March 2000 English only |

Telia, Nokia and Lucent

spectrum requirement analysis for the implementation
of broadband nwa networks

# 1	1ntroduction

At the last JRG 8A-9B meeting a draft workplan was agreed for spectrum requirements for Broadband Nomadic Wireless Access Systems.  One of the items for consideration in the draft workplan was:

“To ascertain the required amount of frequency spectrum for generic broadband NWA networks assuming maximum deployment scenarios in clear spectrum”

This paper gives a spectrum analysis on the minimum amount of clear spectrum we believe is required for the implementation of broadband NWA networks.

# 2	Introduction on Radio LANs

This document uses the term RLANs to denote broadband RLANs used in a variety of applications. These RLANs are also referred to a nomadic wireless access devices (see PDNR….).

Wireless networks have enjoyed an increased demand from the general public as well as from business and other professional users. Wireless networks in existence today range from cellular phones to high speed digital networks supporting high speed computer communications. They operate in licensed as well as in unlicensed frequency bands.

At the same time, wired telecommunications networks have shown a remarkable evolution towards higher transmission rate and support for multi-media applications rather than simple voice oriented services.

Examples of RLANs, their operating frequencies and applications can be found in DNR ITU-R N[8/58]

IMT-2000 developments in other types of wireless networks have increased the scope and potential applications of such networks. A primary example is IMT2000 which, in its various forms, supports a wide range of communications services, from cordless services to wide area cellular services. The range of bit rates, with a maximum of 2 Mbit/s, supported by IMT-2000 is geared primarily towards voice and low quality video as well as data services. However, because of spectrum limitations as well as for economic reasons, IMT-2000 will not be able to meet the bandwidth demands of true, high resolution multi-media communications. These require bit rates in the range of 10 Mb/s. The required bandwidth is not available in the presently planned IMT-2000 frequency range and it is likely that the cost to users of such bandwidth would be excessive. Furthermore, it is not clear that there exists demand for such high speed services beyond the premises of a business or other organization. On premises, short range wireless networks that do not share spectrum with IMT-2000 are much more attractive and flexible as a solution to multi-media wireless networking. RLANs fill that need. The following figure clarifies the relationship between RLANs and IMT-2000:

Figure 1

The relationship between RLANs and IMT-2000

It should be noted that the above does indicates that users may well perceive benefits form being able to access IMT-2000 based services from RLAN devices and vice versa.

This document is concerned with the spectrum needs for broadband RLANs that operate in  the 5 GHz range.

This document describes the applications of broadband RLANs and derives the spectrum needed to supports these applications. The methodology used makes estimations of the average and burst capacity needed per user in Mbit/sec. Using the expected radio performance of these broadband RLAN devices, the spectrum in MHz is derived that would be  needed to support the most difficult application environments (large offices and large public installations).

# 3	Requirements

This clause deals with the general requirements that underlie the development of the RLANstandards for wireless broadband access.

## 3.1	Application Environments and Scenarios

RLANs may be used in a number of different applications and in different scenarios. These scenarios determine to a large extent the spectrum needed to successfully operate RLANs. For details of these Environments and Scenarios, the reader is referred to Annex A.

The following sections detail the application requirements for the scenarios described in Annex A..

## 3.2	Application requirements

There are many applications that together form the requirements for wired as well as wireless systems. Many of these will be covered in later clauses, but a general application type, the multimedia application deserves special attention. Multimedia applications are becoming popular and are already beginning to demand wireless transport with a high quality of service. Multimedia applications shall be taken into account when defining the RLAN family.

Multimedia covers anything from basic messaging through to audio, video or any combination thereof. At the transport layer multimedia consists of two types of information flow; firstly the delivery of fixed packages of information and secondly the delivery of a stream of information which can be described by a certain data rate and delay tolerance.

IP has been designed to cater specifically for data packet traffic with no specific QoS guarantees, i.e. best effort. However, new applications and protocols have been and are being developed which demand or provide QoS guarantees over IP networks. Examples of this are integrated services using RSVP and differentiated services. As users get accustomed to this level of service in their wired systems they are going to demand the same QoS on wireless systems. RLAN shall support IP applications and QoS.

ATM is a transport mechanism, which has been designed to cater specifically for multimedia by being able to support very different kinds of connections with different QoS parameters. New applications will be developed which fully exploit the capabilities of the ATM transport technology, especially the availability of high bandwidth. Also for ATM, as users get accustomed to this level of service they are going to demand the same QoS on wireless systems. RLAN shall support ATM applications and QoS.

The following subclauses describe a number of scenarios for RLAN deployment. Two main scenarios are described, corresponding to an office and an industrial application. Each scenario is broken down further into typical activities and shows estimated data rate requirements for each activity. The purpose of this analysis is to provide a thorough basis for an estimate of RLAN spectral requirements.

### 3.2.1	Office RLAN deployment scenario

The following activities are expected in an office deployment scenario for RLANover the next two or three years. The required data rates for each activity are given in a spreadsheet (table 7). Table7 also shows the calculation of an average data rate required to support the listed activities for each person in the office. These figures will be used shortly to compute estimates of the spectrum required to support typical office use of RLANs. A list and brief description of office related activities that could be supported by RLANs follows:

Multimedia conference (large video displays)

High quality video/audio channels with multiparty data links for the transmission of still images as well as the exchange of computer data including shared multi-user applications.

Telephone/Audio

From toll quality telephone service to higher quality audio.

General networked computing applications

Examples of applications are: Client-server, Processing, Printing, E-mail, Messaging, Fax, Groupware, Games and Simulations, Network file systems, etc. The transfers are generally asymmetric and highly bursty. The data rate requirements are quite dependent on the level of mobility, i.e. the quality should be very similar to that one offered by a fixed LAN on a static mobile node, and temporarily degraded while on the move. Moreover, the bit rate should correspond to the processing speed of the terminal i.e. PDA, portable computer or workstation.

The requirements for a wired-LAN QoS are upper bounds and may be considered as a basis for a wireless LAN:

Multimedia database

Encyclopaedia browsing, medical diagnosis records, electronic newspaper, bulletin board, World Wide Web, manuals, etc. Includes asymmetric, resource demanding applications and bursty non-real-time data.

Security and monitoring

Surveillance video/audio, Industrial or office security service, Alarms, etc.

Internet and Intranet Browsing

The Internet has gained prominence far beyond the expectations expressed by experts only a few years ago. Today businesses of all kinds make extensive use of Internet and Intranet as a means to disseminate information about their products and services. Similarly, government institutions are getting ready to put their information on the Net. With the emergence of electronic payment the Net will become a commercial environment as well. For many international organizations, including ERO and ETSI, the Net has become an indispensable tool. As a consequence users spend hours a day "surfing" the Net to find and exchange information. This information is typically not just text form but includes extensive graphics as well as, in some cases, video and audio sequences.

Teleworking

Less prominent but gaining ground is the notion of teleworking. Teleworking may mean working at home but being in contact with colleagues at work and with customers through video/voice/data sessions. It also means collaboration between geographically separated persons, possibly a group of them. Here too, the ability of telecommunications to deliver high quality video and sound as well as real time data allows users to avoid costly and time consuming travel. Application developers have caught on to this opportunity. A variety of "screen sharing" tools is being developed that provide users with the means to work together in real time on the same electronic documents while being in eye and ear contact. Much like the Net browsers opened up the demand for Internet services so these sharing tools will create a large demand for teleworking services.

Table 7

Predicted average data rate per  RLAN, office deployment

### 3.2.2	Industrial RLAN deployment scenarios

Table 8 provides a breakdown of the data rate capacity required to support a typical industrial deployment of a RLAN network on a piece of industrial plant or machinery assumed to contain 50 separate RLAN equipments operating in a single radio locale defined by the operating radio range. A more general list of industrial activities that can be supported by RLAN follows:

Gatelink

Gatelink is a typical example of multimedia networking in an industrial environment. The applications are in aircraft maintenance support, software loading of airborne systems, passenger service and entertainment, pilot briefing and backup of aircraft maintenance systems. The data rate requirements of Gatelink are not analysed further in the present document.

Manufacturing Applications

In process automation, commissioning systems, baggage transfer and distribution systems we will find a mixture of services. Services will include non-real-time data for file transfer, software and configuration data download, as well as very time critical (real time) data transfer for control and alarm data. Also a mixture of conversational multimedia services for surveillance and monitoring purposes is needed.

Industrial Remote control

Remote control of some device. High quality asymmetric video/audio (MPEG-1 or MPEG-2, possibly multichannel and/or stereo picture), control information and computer data.

Industrial monitoring

Industrial monitoring is a specific application in industrial environments. The applications are for instance monitoring of oil pipelines or monitoring of production processes and resources like tanks in chemistry plants. Data is typically generated by a sensor, is very small as well as specific and has very stringent delay bound and variance. Normally the bandwidth needs are low. However, in certain circumstances (for instances fire or explosions) a very bursty and strongly correlated traffic can be generated by hundreds (thousands) of sensors which has to be handled by the network according to the QoS requirements.

Table 8

Predicted average data rates per RLAN for an industrial deployment

### 3.2.3	Public RLAN deployment scenario

In table 9 the required data rates for the applications in a public deployment scenario are shown. Just as in the office scenario, the average data rate per person is also listed. The listed applications are the same as in the office scenario, since public access will mostly be available in geographically small (hot spot) areas. These hot spot areas will be at e.g. airports, hospitals, conference sites etc., i.e. areas in which the same access network can be used for both public (guests, customers etc.) and private (employees of the operator) access. For simplicity, the same figures can be used for pure public access networks, e.g. in city centres.

Table 9

Predicted average data rate per RLAN, public deployment

### 3.2.4	Other RLAN deployment scenarios

Note: this section will be updated following analysis of domestic entertainment scenarios.

RLAN can support many other activities and deployment scenarios other than those listed above. A number of the more prominent examples of alternative RLAN deployments are described below and a TV, radio or recording studio deployment containing about 60 separate RLAN equipment is analysed further in table 10:

-	Audio distribution.

-	High quality audio.

-	High quality Audio distribution.

-	High quality e.g. delivery of audio or wireless equipment for programme production (possible multiparty).

-	Database services.

Inventory of available goods, On-floor customer services in shops, Menu of the company cafeteria, Telephone and contact information directory, etc. This deployment scenario is identified, but not analysed further in the present document.

Table 10

Predicted average data rates for broadcast or recording studio RLAN deployments

## 3.3	Summary of data rate requirements for RLAN deployments

A summary of the data rate requirements based on the example deployments listed above and analysed in tables 7, 8, 9 and 10, is given in table 11. The table includes some reasonable assumptions for the numbers of RLAN terminals that would exist in each deployment and shows how the total data rate is calculated in each case. The table also includes factors for the efficiency of the network protocol (e.g. TCP/IP) and for the protocol efficiency of the air interface which takes into account the signalling traffic generated by the operation of the RLAN MAC protocol which reduces the available channel capacity.

Table 11

Summary of data rate requirements for RLAN deployments

# 4	Spectrum requirements

## 4.1	Spectrum Requirements

The typical environment for the use of RLANs is characterised by very high user density – e.g. up to 1 user per 10 m2 – supported by a significant number of access points. Therefore, in many cases, the propagation environment is dominated by multipath effects and by interference from adjacent RLANs operating either co-channel or on an adjacent channel. In such an environment the amount of spectrum required is driven by the required carrier to interference ratio which in turn depends on the bit rate required for a given environment or type of application.

The multipath effects determine the technology used and have little direct influence on the spectrum required.

Note: Even where RLANs are used for access to public infrastructure networks like IMT-2000, the propagation environemnt is likely to be indoor. However, the scale of such an indoor environment may be such that free space propagation dominates over most if not all of the area covered.

The main parameters to be considered are:

-	bitrate

-	carrier to interference ratio

-	access point spacing

-	user density per m2

These parameters are used in the following tables to derive the achievable bitrate per user; these values can be used to judge how much spectrum is needed to address a specific application scenario.

## 4.2	Calculation of needed spectrum

For a given number of available channels, a channel plan must be created. A commonly used channel plan is the symmetrical hexagonal channel plan. For such channel plans, it can be shown [2] that

DR=3K,	(Eq. 1)

where R is the cell radius, D is the reuse distance (i.e. the distance between two co-channel cells), and K is the number of available channels. A symmetrical hexagonal channel plan can only be obtained for certain values of K, but the expression above is in the following used for all integer values of K.

The carrier to interference level for a user on the cell edge in a large system can be calculated [2] as:

SIR≈P1αNPDRα=DRαN=3Kα2N	(Eq. 2)

where P is the transmitter power,  is the propagation exponent and N is the number of simultaneously active interference. In a symmetrical hexagonal channel plan, a total of 6 interferers are located at the distance D/R from the user. As it is unlikely that all of these interferers are active simultaneously, N=3 is used here, meaning that half of the interferers are active.

Thus, the SIR level for a user depends on the number of available channels and the propagation exponent. It shall be noted that a symmetrical hexagonal channel plan may not always be possible to achieve, especially when an automatic frequency allocation algorithm is used. In many cases, the SIR will therefore be worse than what is estimated here.

When the SIR value is calculated, the achievable throughput for a user can be determined from link layer simulations. In [1], the packet error rate as a function of SIR is given. The results are shown in the figure below.

From the packet error rate (PER), the link throughput can be calculated as Data rate*(1-PER) [1].

The link throughput is the average throughput that can be achieved when an AP transmits (or receives) 100% of the time. This value is then divided between the MTs connected to the AP. Further, the net throughput/ MT is obtained by multiplying the throughput with the protocol efficiency. Finally, the full time equivalent bitrate is the bitrate that can be achieved during the fraction of time a MT is active.

Note: the above does not include the effects of bitrate dependency in the protocol overhead. This will be added in a subsequent version.

## 4.3	Numerical examples

The following 3 examples show the effects of changing the number of channels, the propagation coefficient and the user density.

Note: these examples include the effects of bitrate dependency in the protocol overhead. This will be added in a subsequent version.

Example 1: Office building (=3.5) with 8 channels

Input:

Propagation exponent   3,5
Number of channels   8
m2 / user   10
APrange   22
Number of APs   40
Protocol overhead   0,2
Network access factor   0,3

Results:   
C/I level (dB)   19,4

| Data rate in Mbit/s | PER | Throughput in AP (Mbit/s) | Throughput per MT (Kbit/s) | Net Throughput per MT, incl overhead (Kbit/s) | Full time equiv bitrate per user (Kbit/s) |
| --- | --- | --- | --- | --- | --- |
| 6 | 0,00 | 6,00 | 31,0 | 24,8 | 83 |
| 9 | 0,00 | 8,99 | 46,4 | 37,1 | 124 |
| 12 | 0,00 | 12,00 | 62,0 | 49,6 | 165 |
| 18 | 0,01 | 17,88 | 92,3 | 73,9 | 246 |
| 27 | 0,02 | 26,41 | 136,4 | 109,1 | 364 |
| 36 | 0,10 | 32,36 | 167,1 | 133,7 | 446 |
| 54 | 0,44 | 30,16 | 155,8 | 124,6 | 415 |

For the given input, the average throughput per user (column 6) peaks for the 36 Mb/s PHY mode and equals 446 kbit/s. From Table 11, it is seen that the required average throughput per user is 570 kbit/s, i.e. the requirement is not fulfilled in this case.

Example 2: Office building (=3.5) with 16 channels

Input:

Propagation exponent   3,5
Number of channels   16
m2 / user   10
APrange   22
Number of APs   40
Protocol overhead   0,2
Network access factor   0,3

Results:

C/I level (dB)   24,7

| Data rate in Mbit/s | PER | Throughput in AP (Mbit/s) | Throughput per MT (Kbit/s) | Net Throughput per MT, incl overhead (Kbit/s) | Full time equiv bitrate per user (Kbit/s) |
| --- | --- | --- | --- | --- | --- |
| 6 | 0,00 | 6,00 | 31,0 | 24,8 | 83 |
| 9 | 0,00 | 9,00 | 46,5 | 37,2 | 124 |
| 12 | 0,00 | 12,00 | 62,0 | 49,6 | 165 |
| 18 | 0,00 | 18,00 | 93,0 | 74,4 | 248 |
| 27 | 0,00 | 26,97 | 139,3 | 111,5 | 372 |
| 36 | 0,01 | 35,68 | 184,3 | 147,4 | 491 |
| 54 | 0,10 | 48,44 | 250,2 | 200,2 | 667 |

For the given input, the average throughput per user (column 6) peaks for the 54 Mb/s PHY mode and equals 667 kbit/s. From Table 11, it is seen that the required average throughput per user is 570 kbit/s, i.e. the requirement is now fulfilled. This example shows how the achievable throughput is increased when more channels are available.

Example 3: Exhibition hall (=2.0) with 16 channels

Input:

Propagation exponent   2
Number of channels   16
m2 / user   15
APrange   15|
Number of APs   40
Protocol overhead   0,2
Network access factor   0,3

Results:

C/I level (dB)   12,0

| Data rate in Mbit/s | PER | Throughput in AP (Mbit/s) | Throughput per MT (Kbit/s) | Net Throughput per MT, incl overhead (Kbit/s) | Full time equiv bitrate per user (Kbit/s) |
| --- | --- | --- | --- | --- | --- |
| 6 | 0,01 | 5,96 | 99,3 | 79,4 | 265 |
| 9 | 0,06 | 8,48 | 141,4 | 113,1 | 377 |
| 12 | 0,03 | 11,61 | 193,6 | 154,8 | 516 |
| 18 | 0,17 | 15,02 | 250,4 | 200,3 | 668 |
| 27 | 0,36 | 17,41 | 290,2 | 232,1 | 774 |
| 36 | 0,62 | 13,59 | 226,5 | 181,2 | 604 |
| 54 | 0,92 | 4,30 | 71,6 | 57,3 | 191 |

In this example, the propagation exponent is lowered (LOS propagation), which leads to a significantly decreased SIR. Thus, the achievable throughput per AP is much lower than in the office building. Although the AP spacing is decreased to 15m, the average throughput per user is not higher than 774 kbit/s (27 Mbit/s PHY mode). In this case, the requirement for a public environment (750 kbit/s see Table 11) is just fulfilled. Note further that the packet error rate (PER) for the 27 Mbit/s PHY mode is 36%, i.e. to high for practical applications. This means that the practical usable bitrate is even lower than what is indicated by these results.

## 4.4	Reference cases

The spectrum requirements presented below are based on the required useful data rates analysed above for a large office area with access to a wired network scenario, taking into account the relevant  spectrum re-use factor for different data rates and the spectral efficiency of the modulations that can be used. The resulting  spectrum requirements should be treated as typical values to guide  future spectrum designations.

### 4.4.1	Wireless access networks for office use

The office reference scenario consists of a large office building with 2400 users. Although there are large buildings with more users, this scenario is used as typical of a large scale deployment of RLANs. Similar figures will apply to hospitals, schools, shopping centers, convention centers etc.

-	Total area covered: 24 000 m2, (approx. 160 metres × 160 metres, 3 floors of 40x 200 metres, or 10 floors of 40 x 60 metres).

-	Number of users: 2400 (at 10 square metres per user).

-	AP range: 20m

-	Propagation exponent: 3.5

-	Average throughput required: 570 Kbit/s per user (see table 11).

In this case, at least 14 channels would be required to provide the desired data rate of 570 kbit/s per user.

### 4.4.2	Wireless access networks for public use – Conference hall

The conference hall (or exhibition hall) reference scenario consists of an open area with line of sight propagation.

-	Building size: 200 x 120m

-	Number of users: 1200 (at 20 square meters per user)

-	Number of APs: 40

-	AP range: 20 m

-	Propagation exponent: 2.0

-	Average throughput required: 750 kbit/s per user (see table 11)

In this case, at least 16 channels would be required to provide the desired data rate of 750 kbit/s per user.

This reference case has also been evaluated by system simulations, (see Appendix B). The simulations indicate that 14 channels would be needed in this reference case. The difference in the results partly arise from the fact that the simulations are carried out in a system of limited size (40 APs), but the analytical results are obtained from a large scale deployment.

### 4.4.3	Domestic Entertainment Scenario

### to be added

## 4.5	References

[1]  	J. Kun-Jush et al, “Structure and Performance of the HiperLan/2 Physical Layer”, VTC’99 Fall (Amsterdam).

[2] 	Lee, William C.Y. “Mobile Communications Engineering - Theory and applications”, McGraw-Hill Telecommunications,  Second edition 1997

# 5	Conclusion

As shown by the above analysis based on desired user bit rates in different propagation environments, 16 channels are needed to meet the requirements, when the whole available bandwidth is allocated to a single operator. In many case some of the available channels will be interfered by radars or other equipment, implying that extra additional would be needed to fulfil the requirements in practical situations.

Given that the RLAN channel width as defined by ETSI, IEEE and MMAC is 20 MHz, the minimum amount of spectrum required is 320 MHz. In order to meet out of band emission levels, the channels at the end of a band have to be at least 20 MHz away from the band edge. That implies that the total spectrum required would be 320 MHz + N*20 MHz of buffer spectrum. In case the spectrum is split up over two subbands, the total required would be at least 360 MHz of clean spectrum. If part of this spectrum is to be shared with other terrestrial services that could cause interference, the required spectrum would increase with at least the bandwith typical of the interfering system if the interfereing systems are widely spaced – e.g. ground based radars. However, in practice more than one of these interfering systems might be present and thus reduce the spectrum available to RLANs.

Note: The CEPT has recognised this and designated a total of 455 MHz of spectrum to RLANs as defined by ETSI (HIPERLANs).

Annex A

Application Environments and Scenarios

### A.1 Application Environments

The following subclauses describe a number of application environments. The common denominator of these environments is that:

-	they are used in a geographically limited area;

-	they support multimedia services.

### A.1.1	Types of Broadband RLAN application environments

Domestic Premises Network (DPN) environment

The DPN environment covers the home and its immediate vicinity; it typically includes a localized radio extension to a broadband network. It is characterized by spot coverage areas, perhaps individual cells, one per home or building. Support for mobility beyond the coverage area is outside the scope of the present document.

Business Premises Network (BPN) environment

The BPN environment covers a network covering e.g. a company area, university campus hospital, industrial premises, airports, train stations, etc. It may offer access, switching and management functions within an arbitrary large coverage area serviced by multi-cellular wireless communications facilities. Thus, functions like handover and paging may be necessary within this environment.

### A.1.2	Types of networks

may be used in a number of ways, for example:

Wireless Access to Public Network

RLANs may be used to gain access to a public network, for example, to provide Telepoint services.

Wireless Access to Private Network

may be used to gain access to a private network, for example, business premises or campus networks.

Temporary Network

may be used to create temporary networks, independent of an established wired local network. Such a network may be used semi-permanently, as an alternative for a wired network, and for ad-hoc purposes, for example for people to communicate and work on documents during a meeting.

### A.1.3	Usage environments

The table below shows various examples of usage and applications in the networks types given above. A number of these application environments are analysed in the following subclauses.

Table 1

Examples of usage environments

|  | Wireless Access to Public Networks | Wireless Private Networks (access- and infrastructure) | Temporary networks |
| --- | --- | --- | --- |
| DPN | - education - security, (sensors) - multimedia, e.g. radio CATV access point - mobile access to IP, ATM or IMT-2000 network | - education security (surveillance and sensors) - domestic cordless multi-media distribution | - education - meetings - fairs - exhibition |
| BPN | - emergency networks telepoint - education - security, (sensors) - multimedia, e.g. radio CATV access point - mobile access to IP, ATM or IMT-2000 network | - manufacturing - office automation - education - financial transactions - medical/hospital - security (surveillance and sensors) - broadcast studios - maintenance of large objects - stock control - aircraft gate link | - large meetings - offices - maintenance of large objects - industrial - emergency networks - exhibition |

## A.2	 Application scenarios

This subclause describes different scenarios in which RLANs may be used.

- Infrastructure replacement scenario, i.e. when  could be used instead of cabling.

- Cordless access scenario, in which users need to use RLANs in different locations at different times possibly maintaining connectivity while in transit.

- Wireless access to infrastructure scenario.

- Specialized portable applications scenario, i.e. user uses a PDA type device mainly for specialized applications, e.g. maintenance or surveillance.

- Domestic premises scenario, i.e. RLANs are used in the home environment.

- Wireless manufacturing automation scenario, i.e. RLANs are used in a factory or a large assembly / building facility.

- Inter network communication scenario.

### A.2.1	Infrastructure replacement scenario

RLANs can be used for wired infrastructure replacement in a number of scenarios including the replacement of wired premises networks. Typical cases could be temporary office installations or installations into spaces where building characteristics or protection prohibit the extensive use of a cabling.

The infrastructure to be replaced includes stationary backbone networks operating at high speeds as well as wireless network terminations.

Terminals typically connected to infrastructure networks typically are designed for fixed use. Such a terminal could, for example, be a workstation, a PC or any other purpose specific terminal. The applications are typically broadband applications. In this scenario the user device is mostly stationary and the main benefit derived from RLANs is the wireless dimension. Thus, RLANs shall provide or approximate fixed network QoS to a stationary user. The user should not be able to notice the difference between using the wireless system and a wired system.

Table 2

An example of a wired infrastructure replacement scenario

| Attribute |  |
| --- | --- |
| End-user equipment | PC or work station |
| Usage environment | Offices etc. |
| Range | Up to 50 meters for indoor systems; |
| QoS expectation | Same as desktop |
| Applications | Same as desktop |
| Mobility | Limited |
| Coverage | Continuous |
| User Density | High |

### A.2.2	Cordless access scenario

In this scenario, the RLANuser needs to perform his or her work at different locations at different times. The main end-user equipment would be a portable computer. Typically such a user would carry a portable computer to various places within the office and then use the computer while stationary. Typical places for using the RLANsystem outside a office room would be meeting rooms, dining facilities, patient wards, class rooms and auditoria as well as waiting rooms/halls. A cordless user will also access the public network, through base stations installed in locations such as railway stations, airports and shopping centres. In some cases, connectivity has to be maintained while the user is in transit from one location to another.

The terminals in this scenario are movable. A typical terminal could be built around a laptop computer and a RLANcard. The mobile node will in many cases be a battery driven device so that an economic consumption of power is required.

Table 3

An example of a cordless access scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Portable computer, e.g. Notebook or Palmtop. |
| Usage environment | Offices, schools, hospitals, airports, railway stations, shopping centres, etc. |
| Range | Up to 50 meters for indoor systems; Up to 150 meters for outdoor systems. |
| QoS expectation | Similar to desktop |
| Applications | Similar to desktop |
| Mobility | none during use |
| Coverage | Continuous |
| User Density | High (e.g. in a meeting room) |
| Power consumption | Low |

### A.2.3	Specialized portable applications scenario

In the third scenario a user has a small (possibly dedicated) system like a PDA to access services. The applications are typical broadband applications, which shall be supported for mobile users with an acceptable QoS by the mobility functions in the network, e.g. handover. The QoS expected from the RLAN system in this scenario could however be somewhat lower than the QoS of a fixed system. The user can be assumed to realize that a small loss in QoS is the price paid for the mobility gained. For example, the connection might tolerate a short interruption because of a handover (resulting in momentary disturbance in the video picture) etc.

The terminal in this scenario is a mobile handheld terminal e.g. a PDA with a wireless network card or a dedicated mobile node such as a WEBpads, Gamepad, WEBteevee. The applications are mostly dedicated mobile applications that are capable of operating at a lower QoS, as they would use mobile specific features to compensate for some mobile related problems.

The mobile node will in many cases be a battery driven device so that an economic consumption of power is required.

Table 4

An example of a specialized portable scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Hand portable unit, PDA |
| Usage environment | Anywhere within or near private premises |
| Range | Up to 50 meters for indoor systems; Up to 150 meters for outdoor systems. |
| QoS expectation | Modest, but maintained during movement |
| Applications | Dedicated, could be mobile specific |
| Mobility | Walking speed or slow vehicle (e.g. forklift) |
| Coverage | Continuous |
| User Density | Low |
| Power consumption | Very low |

### A.2.4	Domestic Entertainment scenario

Note: this section will be updated following analysis of domestic entertainment scenarios.

In the domestic network scenario, many appliances, e.g. PC laptop, printer/fax machines, security systems, home appliances, digital HDTV/SDTV sets, digital Video Cassette Recorder (VCR), speakers and more could be linked in various ways. A typical scenario would be:

1)	An entertainment cluster (video and sound) located in the living room transmitting to television sets located in the living room, kitchen and bedroom.

2)	A music system in the living room transmitting to speakers located in the living room, bedroom or dining room.

3)	Security features outside the home such as wireless security camera or remote sensors. These could either be located on the external walls of the property or at the boundary wall or a remote building such as a garage or recreation facility.

4)	Outdoor speakers for barbecue/party. Assuming that the music system is located in the entertainment cluster in the living room, the transmission path and length may extend into the garden.

From the above, it is obvious that the domestic network shall allow access to external networks e.g. digital television or be capable of working with no external links e.g. a music system with remote speakers. This system should be easily installable by non-technical people.

A domestic network generally covers a much smaller area than either factory or office environments. The rooms in a domestic premises tend to be smaller when compared to work environments and have more compartmentalized structure (storage spaces and en-suites).

Table 5

An example of a domestic premises scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Computer, television, entertainment cluster, security systems, etc. |
| Usage environment | Domestic premises, i.e. small rooms with high attenuation |
| Range | Up to 15 meters |
| QoS expectation | Consistent with real-time multi-media services |
| Applications | Real-time multi-media |
| Mobility | Walking speed |
| Coverage | Continuous |
| User Density | Low |

### A.2.5	Industrial and transportation scenario

In manufacturing scenarios such as process automation, commissioning systems, baggage transfer, distribution systems, warehouse storage and retrieval services we have a large number of intelligent transportation elements which may move autonomously and automatically in a factory hall, a storage building, or in an airport. Such a system should cover approximately an area of 250 m × 250 m. Delay values and data losses are critical. The ability to support highly reliable real-time control and alarm data as well as other time bounded services is mandatory. Power consumption and the physical size of the communication device are not as critical as in other scenarios.

Table 6

An example of a manufacturing scenario

| Attribute |  |
| --- | --- |
| End-user equipment | Intelligent transportation elements, autonomous, automatic vehicles, surveillance systems, monitors |
| Usage environment | Factory halls, airports, storehouses, industrial environments |
| Range | Up to 50 meters. Shadowing, highly variable radio channels |
| QoS expectation | Low delay, high error sensitivity, time bounded, real-time, short packets |
| Applications | Mobile, file transfer, control, alarms, surveillance, monitoring |
| Mobility | < 10 m/s |
| Coverage | Continuous |
| User Density | High (variable) |
| Power consumption | Not critical |

Appendix B

System simulation

Simulation environment for the exhibition hall scenario

The investigated exhibition hall scenario consisted of a large building with one floor and no inner walls, see Figure 1. A line-of-sight (LOS) propagation model was used. It is assumed that a large deployment consisting of 40 APs is installed in the building. This scenario was chosen since it represents a quite demanding case spectrum wise, but still is anticipated to be valid in several cases of dense deployment. Besides the case of an exhibition hall, a similar situation could be the case in e.g. an airport or a shopping mall where a public service is provided by more than one operator.

The following simulation is based on the HIPERLAN/2 specification.

The mobile terminals (MTs) were randomly placed in the building according to a uniform distribution. The simulation technique was static: each iteration corresponded to a traffic situation unrelated to the previous one, i.e. “snapshots” were taken of the situation in the building. In average, 64 MTs are located in each cell. The probability that an AP is active (transmitting or receiving) in an iteration is 50%. 75% of the data traffic is assumed to be carried in the downlink direction. Each MT is assumed to carry data 30 % of the time, under which the peak data rate requirements shall be fulfilled.

Only the quality in the downlink is analysed, as the majority of the data traffic is assumed to be carried in the downlink direction. The downlink interference arise both from other downlinks (APs) and uplinks (MTs) due to the used TDD technology in H/2. Prior to the simulations, a channel plan is obtained by a distributed Dynamic Frequency Selection (DFS) algorithm, i.e. each AP performs measurements of the interference and selects the channel with least interference. The frequency selection is repeated until a stable channel plan is obtained, which is kept throughout the simulation.

External interference was not taken into account. Further, power control was not applied. The important simulation parameters are summarised in Table 1.

Figure 1

The simulated exhibition hall and the positions of the 40 APs (*)

Table 1

Simulation Parameters

| Number of APs | 40 |
| --- | --- |
| AP duty cycle | 50 % |
| Number of MTs/ cell | 64 |
| Network Access factor | 30 % |
| Downlink data traffic | 75 % |
| LOS propagation model | 47+20log(d) |
| Site to site distance | 24 m |
| AP/MT power | 23 dBm |
| Noise power | -90 dBm |
| Antennas | Omni (0 dBi) |
| Handover margin | 5 dB |
| Adjacent channel suppression | 25 dB |
| Power control | No |
| Protocol efficiency | 80 % |
| Lognormal fading standard deviation | 2 dB |

Requirements

It is required that 90% of the users shall have a peak data rate of 750 kbit/s. (see table 11 in the main document).

Results

The C/(I+N) distribution is shown in figure 2 below. The achievable data rate for a user with a given link quality is estimated by link layer simulations [B.1]. This is done with the simple expression Throughput = data rate * (1-PER), where the PER as a function of C/I is shown in Figure 3 below. The data rate for the 10% worst users are calculated and presented in table 2. The data rate is calculated with the assumption that the physical layer mode providing the highest throughput is selected for each user (ideal link adaptation).

The net data rate is obtained by multiplying the gross data rate with the protocol efficiency. Further, the peak data rate is obtained by dividing the average net data rate with the MT activity factor.

Example:

The results for 10 available channels are shown in the leftmost column in Table 2. The 10th percentile of C/I = 10 dB is obtained form Figure 1. From the link layer simulations in [B.1], the packet error rate (PER) for this C/I value can be obtained for each of the defined PHY modes in BRAN. From the PER, the AP throughput can be estimated from Throughput = Data rate * (1-PER), assuming ideal link adaptation. In this case, the AP throughput peaks at 12 Mbit/s for the 18 Mbit/s PHY mode. The gross data rate per user (187 kbit/s) is obtained by dividing the AP throughput  (12 Mbit/s) with the number of MTs in the cell (64). Further, the average net data rate (150 kbit/s) is obtained by multiplying the gross data rate with the protocol efficiency (80%). Finally, the peak data rate per user (500 kbit/s) is obtained by dividing the average net data rate per user with the MT activity factor (30%).

Figure 2

C/(I+N) distribution with 40 APs

Figure 3

Packet error rate (PER) as a function of C/I for a channel with 50 ns delay spread.

Table 2

Results for the 10% worst users

|  | Number of channels |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 10 | 12 | 14 | 16 | 18 |
| 10 percentile of C/I | 9.7 dB | 11.6 dB | 13.3 dB | 14.3 dB | 15.5 dB |
| Used PHY mode | 18 Mbit/s | 27 Mbit/s | 27 Mbit/s | 27 Mbit/s | 36 Mbit/s |
| Throughput/ AP | 12 Mbit/s | 15 Mbit/s | 18 Mbit/s | 21 Mbit/s | 23 Mbit/s |
| Gross Data rate/user (average) | 187 kbit/s | 234 kbit/s | 281 kbit/s | 328 kbit/s | 359 kbit/s |
| Net data rate/user (average) | 150 kbit/s | 187 kbit/s | 225 kbit/s | 262 kbit/s | 287 kbit/s |
| Net data rate/user (peak) | 500 kbit/s | 625 kbit/s | 750 kbit/s | 875 kbit/s | 958 kbit/s |

According to the requirements in the previous section, 14 channels are required to fulfil the requirements on peak data rate.

Summary and Discussion

Open areas with high AP density provide with a quite demanding situation spectrum wise. In the scenario given in this paper around 14 channels were needed to reach the requirements on user data.

References

[B.1]	J Kun-Jush et al, “Structure and Performance of the HiperLan/2 Physical Layer”, VTC’ 99 Fall (Amsterdam)

_______________