IEEE P802.11
Wireless LANs

IEEE802.11a-1999 Corrigenda

Date:	July 13, 2000

Author:	Hitoshi Takanashi / Naftali Chayat
	NTT MCL / BreezeCom
	250 Cambridge Ave. #300, Palo Alto, CA 94306
	Phone: 650 833 3634
	Fax: 650 326 1878
	e-Mail: takanashi@nttmcl.com

Abstract

This document reports errors found in the IEEE Std 802.11a – 1999 and suggests appropriate corrections that are supposed to be revised by the IEEE 802 editor.

Errors and Corrections

In the IEEE Std 802.11a-1999, as published by IEEE on Dec 30, 1999, errors were discovered relatively to the P802.11a Draft 7.0, which was approved by the Working Group and the Standards Board.

The errors are all in the informative Annex G. In particular, Tables G.2 and G.24 contain major errors, and table G.3 contains typographic errors. All errors are editorial found in only IEEE Std 802.11a-1999, but no errors found in P802.11a Draft 7.0. Thus we don’t have to open a corrigendum for these errors.

The IEEE publishers are working on publishing an errata and a corrected version of IEEE Std 802.11a. This work is expected to complete by mid-August 2000. In the meanwhile, the users are advised to use the Annex G from 802.11a Draft 7.0 or refer to this document.

Errata - Table G. 3

|  | IEEE 802.11a-1999 (wrong) |  | P802.11a Draft 7.0 (Correct) |  |
| --- | --- | --- | --- | --- |
| Coefficient | Re | Im | Re | Im |
| #2 | 0.013 | -0.079 | -0.013 | -0.079 |
| #15 | 0.002 | 0.132 | 0.002 | -0.132 |

The table below is the replacement for table G.24 that has major errors. This table was re-formatted for the sake of easy maintenance:

- The header row entries are centered

- The numeric entries are right-aligned

- The line between columns 3-4, 6-7 and 9-10 is thicker (this comes instead of empty cells in the published standard).

- There are thicker line before the rows starting with numbers 0, 160, 320 and 400. This serves as a visual clue to a segmentation of the table into logically distinct parts.

The tables G.2 and G.3 in the IEEE Std 802.11a-1999 needs to be corrected as shown in this document. The tables G.4, G.6 and G.12 (“time domain” tables) with same sample ordering and formatting for the sake of uniformity are also included in this document. This document will be forwarded to the IEEE 802 Editor.

G. 24 (Correct)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.023 | 0.023 | 1 | -0.132 | 0.002 | 2 | -0.013 | -0.079 | 3 | 0.143 | -0.013 |
| 4 | 0.092 | 0.000 | 5 | 0.143 | -0.013 | 6 | -0.013 | -0.079 | 7 | -0.132 | 0.002 |
| 8 | 0.046 | 0.046 | 9 | 0.002 | -0.132 | 10 | -0.079 | -0.013 | 11 | -0.013 | 0.143 |
| 12 | 0.000 | 0.092 | 13 | -0.013 | 0.143 | 14 | -0.079 | -0.013 | 15 | 0.002 | -0.132 |
| 16 | 0.046 | 0.046 | 17 | -0.132 | 0.002 | 18 | -0.013 | -0.079 | 19 | 0.143 | -0.013 |
| 20 | 0.092 | 0.000 | 21 | 0.143 | -0.013 | 22 | -0.013 | -0.079 | 23 | -0.132 | 0.002 |
| 24 | 0.046 | 0.046 | 25 | 0.002 | -0.132 | 26 | -0.079 | -0.013 | 27 | -0.013 | 0.143 |
| 28 | 0.000 | 0.092 | 29 | -0.013 | 0.143 | 30 | -0.079 | -0.013 | 31 | 0.002 | -0.132 |
| 32 | 0.046 | 0.046 | 33 | -0.132 | 0.002 | 34 | -0.013 | -0.079 | 35 | 0.143 | -0.013 |
| 36 | 0.092 | 0.000 | 37 | 0.143 | -0.013 | 38 | -0.013 | -0.079 | 39 | -0.132 | 0.002 |
| 40 | 0.046 | 0.046 | 41 | 0.002 | -0.132 | 42 | -0.079 | -0.013 | 43 | -0.013 | 0.143 |
| 44 | 0.000 | 0.092 | 45 | -0.013 | 0.143 | 46 | -0.079 | -0.013 | 47 | 0.002 | -0.132 |
| 48 | 0.046 | 0.046 | 49 | -0.132 | 0.002 | 50 | -0.013 | -0.079 | 51 | 0.143 | -0.013 |
| 52 | 0.092 | 0.000 | 53 | 0.143 | -0.013 | 54 | -0.013 | -0.079 | 55 | -0.132 | 0.002 |
| 56 | 0.046 | 0.046 | 57 | 0.002 | -0.132 | 58 | -0.079 | -0.013 | 59 | -0.013 | 0.143 |
| 60 | 0.000 | 0.092 | 61 | -0.013 | 0.143 | 62 | -0.079 | -0.013 | 63 | 0.002 | -0.132 |
| 64 | 0.046 | 0.046 | 65 | -0.132 | 0.002 | 66 | -0.013 | -0.079 | 67 | 0.143 | -0.013 |
| 68 | 0.092 | 0.000 | 69 | 0.143 | -0.013 | 70 | -0.013 | -0.079 | 71 | -0.132 | 0.002 |
| 72 | 0.046 | 0.046 | 73 | 0.002 | -0.132 | 74 | -0.079 | -0.013 | 75 | -0.013 | 0.143 |
| 76 | 0.000 | 0.092 | 77 | -0.013 | 0.143 | 78 | -0.079 | -0.013 | 79 | 0.002 | -0.132 |
| 80 | 0.046 | 0.046 | 81 | -0.132 | 0.002 | 82 | -0.013 | -0.079 | 83 | 0.143 | -0.013 |
| 84 | 0.092 | 0.000 | 85 | 0.143 | -0.013 | 86 | -0.013 | -0.079 | 87 | -0.132 | 0.002 |
| 88 | 0.046 | 0.046 | 89 | 0.002 | -0.132 | 90 | -0.079 | -0.013 | 91 | -0.013 | 0.143 |
| 92 | 0.000 | 0.092 | 93 | -0.013 | 0.143 | 94 | -0.079 | -0.013 | 95 | 0.002 | -0.132 |
| 96 | 0.046 | 0.046 | 97 | -0.132 | 0.002 | 98 | -0.013 | -0.079 | 99 | 0.143 | -0.013 |
| 100 | 0.092 | 0.000 | 101 | 0.143 | -0.013 | 102 | -0.013 | -0.079 | 103 | -0.132 | 0.002 |
| 104 | 0.046 | 0.046 | 105 | 0.002 | -0.132 | 106 | -0.079 | -0.013 | 107 | -0.013 | 0.143 |
| 108 | 0.000 | 0.092 | 109 | -0.013 | 0.143 | 110 | -0.079 | -0.013 | 111 | 0.002 | -0.132 |
| 112 | 0.046 | 0.046 | 113 | -0.132 | 0.002 | 114 | -0.013 | -0.079 | 115 | 0.143 | -0.013 |
| 116 | 0.092 | 0.000 | 117 | 0.143 | -0.013 | 118 | -0.013 | -0.079 | 119 | -0.132 | 0.002 |
| 120 | 0.046 | 0.046 | 121 | 0.002 | -0.132 | 122 | -0.079 | -0.013 | 123 | -0.013 | 0.143 |
| 124 | 0.000 | 0.092 | 125 | -0.013 | 0.143 | 126 | -0.079 | -0.013 | 127 | 0.002 | -0.132 |
| 128 | 0.046 | 0.046 | 129 | -0.132 | 0.002 | 130 | -0.013 | -0.079 | 131 | 0.143 | -0.013 |
| 132 | 0.092 | 0.000 | 133 | 0.143 | -0.013 | 134 | -0.013 | -0.079 | 135 | -0.132 | 0.002 |
| 136 | 0.046 | 0.046 | 137 | 0.002 | -0.132 | 138 | -0.079 | -0.013 | 139 | -0.013 | 0.143 |
| 140 | 0.000 | 0.092 | 141 | -0.013 | 0.143 | 142 | -0.079 | -0.013 | 143 | 0.002 | -0.132 |
| 144 | 0.046 | 0.046 | 145 | -0.132 | 0.002 | 146 | -0.013 | -0.079 | 147 | 0.143 | -0.013 |
| 148 | 0.092 | 0.000 | 149 | 0.143 | -0.013 | 150 | -0.013 | -0.079 | 151 | -0.132 | 0.002 |
| 152 | 0.046 | 0.046 | 153 | 0.002 | -0.132 | 154 | -0.079 | -0.013 | 155 | -0.013 | 0.143 |
| 156 | 0.000 | 0.092 | 157 | -0.013 | 0.143 | 158 | -0.079 | -0.013 | 159 | 0.002 | -0.132 |
| 160 | -0.055 | 0.023 | 161 | 0.012 | -0.098 | 162 | 0.092 | -0.106 | 163 | -0.092 | -0.115 |
| 164 | -0.003 | -0.054 | 165 | 0.075 | 0.074 | 166 | -0.127 | 0.021 | 167 | -0.122 | 0.017 |
| 168 | -0.035 | 0.151 | 169 | -0.056 | 0.022 | 170 | -0.060 | -0.081 | 171 | 0.070 | -0.014 |
| 172 | 0.082 | -0.092 | 173 | -0.131 | -0.065 | 174 | -0.057 | -0.039 | 175 | 0.037 | -0.098 |
| 176 | 0.062 | 0.062 | 177 | 0.119 | 0.004 | 178 | -0.022 | -0.161 | 179 | 0.059 | 0.015 |
| 180 | 0.024 | 0.059 | 181 | -0.137 | 0.047 | 182 | 0.001 | 0.115 | 183 | 0.053 | -0.004 |
| 184 | 0.098 | 0.026 | 185 | -0.038 | 0.106 | 186 | -0.115 | 0.055 | 187 | 0.060 | 0.088 |
| 188 | 0.021 | -0.028 | 189 | 0.097 | -0.083 | 190 | 0.040 | 0.111 | 191 | -0.005 | 0.120 |
| 192 | 0.156 | 0.000 | 193 | -0.005 | -0.120 | 194 | 0.040 | -0.111 | 195 | 0.097 | 0.083 |
| 196 | 0.021 | 0.028 | 197 | 0.060 | -0.088 | 198 | -0.115 | -0.055 | 199 | -0.038 | -0.106 |
| 200 | 0.098 | -0.026 | 201 | 0.053 | 0.004 | 202 | 0.001 | -0.115 | 203 | -0.137 | -0.047 |
| 204 | 0.024 | -0.059 | 205 | 0.059 | -0.015 | 206 | -0.022 | 0.161 | 207 | 0.119 | -0.004 |
| 208 | 0.062 | -0.062 | 209 | 0.037 | 0.098 | 210 | -0.057 | 0.039 | 211 | -0.131 | 0.065 |
| 212 | 0.082 | 0.092 | 213 | 0.070 | 0.014 | 214 | -0.060 | 0.081 | 215 | -0.056 | -0.022 |
| 216 | -0.035 | -0.151 | 217 | -0.122 | -0.017 | 218 | -0.127 | -0.021 | 219 | 0.075 | -0.074 |
| 220 | -0.003 | 0.054 | 221 | -0.092 | 0.115 | 222 | 0.092 | 0.106 | 223 | 0.012 | 0.098 |
| 224 | -0.156 | 0.000 | 225 | 0.012 | -0.098 | 226 | 0.092 | -0.106 | 227 | -0.092 | -0.115 |
| 228 | -0.003 | -0.054 | 229 | 0.075 | 0.074 | 230 | -0.127 | 0.021 | 231 | -0.122 | 0.017 |
| 232 | -0.035 | 0.151 | 233 | -0.056 | 0.022 | 234 | -0.060 | -0.081 | 235 | 0.070 | -0.014 |
| 236 | 0.082 | -0.092 | 237 | -0.131 | -0.065 | 238 | -0.057 | -0.039 | 239 | 0.037 | -0.098 |
| 240 | 0.062 | 0.062 | 241 | 0.119 | 0.004 | 242 | -0.022 | -0.161 | 243 | 0.059 | 0.015 |
| 244 | 0.024 | 0.059 | 245 | -0.137 | 0.047 | 246 | 0.001 | 0.115 | 247 | 0.053 | -0.004 |
| 248 | 0.098 | 0.026 | 249 | -0.038 | 0.106 | 250 | -0.115 | 0.055 | 251 | 0.060 | 0.088 |
| 252 | 0.021 | -0.028 | 253 | 0.097 | -0.083 | 254 | 0.040 | 0.111 | 255 | -0.005 | 0.120 |
| 256 | 0.156 | 0.000 | 257 | -0.005 | -0.120 | 258 | 0.040 | -0.111 | 259 | 0.097 | 0.083 |
| 260 | 0.021 | 0.028 | 261 | 0.060 | -0.088 | 262 | -0.115 | -0.055 | 263 | -0.038 | -0.106 |
| 264 | 0.098 | -0.026 | 265 | 0.053 | 0.004 | 266 | 0.001 | -0.115 | 267 | -0.137 | -0.047 |
| 268 | 0.024 | -0.059 | 269 | 0.059 | -0.015 | 270 | -0.022 | 0.161 | 271 | 0.119 | -0.004 |
| 272 | 0.062 | -0.062 | 273 | 0.037 | 0.098 | 274 | -0.057 | 0.039 | 275 | -0.131 | 0.065 |
| 276 | 0.082 | 0.092 | 277 | 0.070 | 0.014 | 278 | -0.060 | 0.081 | 279 | -0.056 | -0.022 |
| 280 | -0.035 | -0.151 | 281 | -0.122 | -0.017 | 282 | -0.127 | -0.021 | 283 | 0.075 | -0.074 |
| 284 | -0.003 | 0.054 | 285 | -0.092 | 0.115 | 286 | 0.092 | 0.106 | 287 | 0.012 | 0.098 |
| 288 | -0.156 | 0.000 | 289 | 0.012 | -0.098 | 290 | 0.092 | -0.106 | 291 | -0.092 | -0.115 |
| 292 | -0.003 | -0.054 | 293 | 0.075 | 0.074 | 294 | -0.127 | 0.021 | 295 | -0.122 | 0.017 |
| 296 | -0.035 | 0.151 | 297 | -0.056 | 0.022 | 298 | -0.060 | -0.081 | 299 | 0.070 | -0.014 |
| 300 | 0.082 | -0.092 | 301 | -0.131 | -0.065 | 302 | -0.057 | -0.039 | 303 | 0.037 | -0.098 |
| 304 | 0.062 | 0.062 | 305 | 0.119 | 0.004 | 306 | -0.022 | -0.161 | 307 | 0.059 | 0.015 |
| 308 | 0.024 | 0.059 | 309 | -0.137 | 0.047 | 310 | 0.001 | 0.115 | 311 | 0.053 | -0.004 |
| 312 | 0.098 | 0.026 | 313 | -0.038 | 0.106 | 314 | -0.115 | 0.055 | 315 | 0.060 | 0.088 |
| 316 | 0.021 | -0.028 | 317 | 0.097 | -0.083 | 318 | 0.040 | 0.111 | 319 | -0.005 | 0.120 |
| 320 | 0.109 | 0.000 | 321 | 0.033 | -0.044 | 322 | -0.002 | -0.038 | 323 | -0.081 | 0.084 |
| 324 | 0.007 | -0.100 | 325 | -0.001 | -0.113 | 326 | -0.021 | -0.005 | 327 | 0.136 | -0.105 |
| 328 | 0.098 | -0.044 | 329 | 0.011 | -0.002 | 330 | -0.033 | 0.044 | 331 | -0.060 | 0.124 |
| 332 | 0.010 | 0.097 | 333 | 0.000 | -0.008 | 334 | 0.018 | -0.083 | 335 | -0.069 | 0.027 |
| 336 | -0.219 | 0.000 | 337 | -0.069 | -0.027 | 338 | 0.018 | 0.083 | 339 | 0.000 | 0.008 |
| 340 | 0.010 | -0.097 | 341 | -0.060 | -0.124 | 342 | -0.033 | -0.044 | 343 | 0.011 | 0.002 |
| 344 | 0.098 | 0.044 | 345 | 0.136 | 0.105 | 346 | -0.021 | 0.005 | 347 | -0.001 | 0.113 |
| 348 | 0.007 | 0.100 | 349 | -0.081 | -0.084 | 350 | -0.002 | 0.038 | 351 | 0.033 | 0.044 |
| 352 | 0.062 | 0.000 | 353 | 0.057 | 0.052 | 354 | 0.016 | 0.174 | 355 | 0.035 | 0.116 |
| 356 | -0.051 | -0.202 | 357 | 0.011 | 0.036 | 358 | 0.089 | 0.209 | 359 | -0.049 | -0.008 |
| 360 | -0.035 | 0.044 | 361 | 0.017 | -0.059 | 362 | 0.053 | -0.017 | 363 | 0.099 | 0.100 |
| 364 | 0.034 | -0.148 | 365 | -0.003 | -0.094 | 366 | -0.120 | 0.042 | 367 | -0.136 | -0.070 |
| 368 | -0.031 | 0.000 | 369 | -0.136 | 0.070 | 370 | -0.120 | -0.042 | 371 | -0.003 | 0.094 |
| 372 | 0.034 | 0.148 | 373 | 0.099 | -0.100 | 374 | 0.053 | 0.017 | 375 | 0.017 | 0.059 |
| 376 | -0.035 | -0.044 | 377 | -0.049 | 0.008 | 378 | 0.089 | -0.209 | 379 | 0.011 | -0.036 |
| 380 | -0.051 | 0.202 | 381 | 0.035 | -0.116 | 382 | 0.016 | -0.174 | 383 | 0.057 | -0.052 |
| 384 | 0.062 | 0.000 | 385 | 0.033 | -0.044 | 386 | -0.002 | -0.038 | 387 | -0.081 | 0.084 |
| 388 | 0.007 | -0.100 | 389 | -0.001 | -0.113 | 390 | -0.021 | -0.005 | 391 | 0.136 | -0.105 |
| 392 | 0.098 | -0.044 | 393 | 0.011 | -0.002 | 394 | -0.033 | 0.044 | 395 | -0.060 | 0.124 |
| 396 | 0.010 | 0.097 | 397 | 0.000 | -0.008 | 398 | 0.018 | -0.083 | 399 | -0.069 | 0.027 |
| 400 | -0.139 | 0.050 | 401 | 0.004 | 0.014 | 402 | 0.011 | -0.100 | 403 | -0.097 | -0.020 |
| 404 | 0.062 | 0.081 | 405 | 0.124 | 0.139 | 406 | 0.104 | -0.015 | 407 | 0.173 | -0.140 |
| 408 | -0.040 | 0.006 | 409 | -0.133 | 0.009 | 410 | -0.002 | -0.043 | 411 | -0.047 | 0.092 |
| 412 | -0.109 | 0.082 | 413 | -0.024 | 0.010 | 414 | 0.096 | 0.019 | 415 | 0.019 | -0.023 |
| 416 | -0.087 | -0.049 | 417 | 0.002 | 0.058 | 418 | -0.021 | 0.228 | 419 | -0.103 | 0.023 |
| 420 | -0.019 | -0.175 | 421 | 0.018 | 0.132 | 422 | -0.071 | 0.160 | 423 | -0.153 | -0.062 |
| 424 | -0.107 | 0.028 | 425 | 0.055 | 0.140 | 426 | 0.070 | 0.103 | 427 | -0.056 | 0.025 |
| 428 | -0.043 | 0.002 | 429 | 0.016 | -0.118 | 430 | 0.026 | -0.071 | 431 | 0.033 | 0.177 |
| 432 | 0.020 | -0.021 | 433 | 0.035 | -0.088 | 434 | -0.008 | 0.101 | 435 | -0.035 | -0.010 |
| 436 | 0.065 | 0.030 | 437 | 0.092 | -0.034 | 438 | 0.032 | -0.123 | 439 | -0.018 | 0.092 |
| 440 | 0.000 | -0.006 | 441 | -0.006 | -0.056 | 442 | -0.019 | 0.040 | 443 | 0.053 | -0.131 |
| 444 | 0.022 | -0.133 | 445 | 0.104 | -0.032 | 446 | 0.163 | -0.045 | 447 | -0.105 | -0.030 |
| 448 | -0.110 | -0.069 | 449 | -0.008 | -0.092 | 450 | -0.049 | -0.043 | 451 | 0.085 | -0.017 |
| 452 | 0.090 | 0.063 | 453 | 0.015 | 0.153 | 454 | 0.049 | 0.094 | 455 | 0.011 | 0.034 |
| 456 | -0.012 | 0.012 | 457 | -0.015 | -0.017 | 458 | -0.061 | 0.031 | 459 | -0.070 | -0.040 |
| 460 | 0.011 | -0.109 | 461 | 0.037 | -0.060 | 462 | -0.003 | -0.178 | 463 | -0.007 | -0.128 |
| 464 | -0.059 | 0.100 | 465 | 0.004 | 0.014 | 466 | 0.011 | -0.100 | 467 | -0.097 | -0.020 |
| 468 | 0.062 | 0.081 | 469 | 0.124 | 0.139 | 470 | 0.104 | -0.015 | 471 | 0.173 | -0.140 |
| 472 | -0.040 | 0.006 | 473 | -0.133 | 0.009 | 474 | -0.002 | -0.043 | 475 | -0.047 | 0.092 |
| 476 | -0.109 | 0.082 | 477 | -0.024 | 0.010 | 478 | 0.096 | 0.019 | 479 | 0.019 | -0.023 |
| 480 | -0.058 | 0.016 | 481 | -0.096 | -0.045 | 482 | -0.110 | 0.003 | 483 | -0.070 | 0.216 |
| 484 | -0.040 | 0.059 | 485 | 0.010 | -0.056 | 486 | 0.034 | 0.065 | 487 | 0.117 | 0.033 |
| 488 | 0.078 | -0.133 | 489 | -0.043 | -0.146 | 490 | 0.158 | -0.071 | 491 | 0.254 | -0.021 |
| 492 | 0.068 | 0.117 | 493 | -0.044 | 0.114 | 494 | -0.035 | 0.041 | 495 | 0.085 | 0.070 |
| 496 | 0.120 | 0.010 | 497 | 0.057 | 0.055 | 498 | 0.063 | 0.188 | 499 | 0.091 | 0.149 |
| 500 | -0.017 | -0.039 | 501 | -0.078 | -0.075 | 502 | 0.049 | 0.079 | 503 | -0.014 | -0.007 |
| 504 | 0.030 | -0.027 | 505 | 0.080 | 0.054 | 506 | -0.186 | -0.067 | 507 | -0.039 | -0.027 |
| 508 | 0.043 | -0.072 | 509 | -0.092 | -0.089 | 510 | 0.029 | 0.105 | 511 | -0.144 | 0.003 |
| 512 | -0.069 | -0.041 | 513 | 0.132 | 0.057 | 514 | -0.126 | 0.070 | 515 | -0.031 | 0.109 |
| 516 | 0.161 | -0.009 | 517 | 0.056 | -0.046 | 518 | -0.004 | 0.028 | 519 | -0.049 | 0.000 |
| 520 | -0.078 | -0.005 | 521 | 0.015 | -0.087 | 522 | 0.149 | -0.104 | 523 | -0.021 | -0.051 |
| 524 | -0.154 | -0.106 | 525 | 0.024 | 0.030 | 526 | 0.046 | 0.123 | 527 | -0.004 | -0.098 |
| 528 | -0.061 | -0.128 | 529 | -0.024 | -0.038 | 530 | 0.066 | -0.048 | 531 | -0.067 | 0.027 |
| 532 | 0.054 | -0.050 | 533 | 0.171 | -0.049 | 534 | -0.108 | 0.132 | 535 | -0.161 | -0.019 |
| 536 | -0.070 | -0.072 | 537 | -0.177 | 0.049 | 538 | -0.172 | -0.050 | 539 | 0.051 | -0.075 |
| 540 | 0.122 | -0.057 | 541 | 0.009 | -0.044 | 542 | -0.012 | -0.021 | 543 | 0.004 | 0.009 |
| 544 | -0.030 | 0.081 | 545 | -0.096 | -0.045 | 546 | -0.110 | 0.003 | 547 | -0.070 | 0.216 |
| 548 | -0.040 | 0.059 | 549 | 0.010 | -0.056 | 550 | 0.034 | 0.065 | 551 | 0.117 | 0.033 |
| 552 | 0.078 | -0.133 | 553 | -0.043 | -0.146 | 554 | 0.158 | -0.071 | 555 | 0.254 | -0.021 |
| 556 | 0.068 | 0.117 | 557 | -0.044 | 0.114 | 558 | -0.035 | 0.041 | 559 | 0.085 | 0.070 |
| 560 | 0.001 | 0.011 | 561 | -0.099 | -0.048 | 562 | 0.054 | -0.196 | 563 | 0.124 | 0.035 |
| 564 | 0.092 | 0.045 | 565 | -0.037 | -0.066 | 566 | -0.021 | -0.004 | 567 | 0.042 | -0.065 |
| 568 | 0.061 | 0.048 | 569 | 0.046 | 0.004 | 570 | -0.063 | -0.045 | 571 | -0.102 | 0.152 |
| 572 | -0.039 | -0.019 | 573 | -0.005 | -0.106 | 574 | 0.083 | 0.031 | 575 | 0.226 | 0.028 |
| 576 | 0.140 | -0.010 | 577 | -0.132 | -0.033 | 578 | -0.116 | 0.088 | 579 | 0.023 | 0.052 |
| 580 | -0.171 | -0.080 | 581 | -0.246 | -0.025 | 582 | -0.062 | -0.038 | 583 | -0.055 | -0.062 |
| 584 | -0.004 | -0.060 | 585 | 0.034 | 0.000 | 586 | -0.030 | 0.021 | 587 | 0.075 | -0.122 |
| 588 | 0.043 | -0.080 | 589 | -0.022 | 0.041 | 590 | 0.026 | 0.013 | 591 | -0.031 | -0.018 |
| 592 | 0.059 | 0.008 | 593 | 0.109 | 0.078 | 594 | 0.002 | 0.101 | 595 | -0.016 | 0.054 |
| 596 | -0.059 | 0.070 | 597 | 0.017 | 0.114 | 598 | 0.104 | -0.034 | 599 | -0.024 | -0.059 |
| 600 | -0.081 | 0.051 | 601 | -0.040 | -0.069 | 602 | -0.069 | 0.058 | 603 | -0.067 | 0.117 |
| 604 | 0.007 | -0.131 | 605 | 0.009 | 0.028 | 606 | 0.075 | 0.117 | 607 | 0.118 | 0.030 |
| 608 | -0.041 | 0.148 | 609 | 0.005 | 0.098 | 610 | 0.026 | 0.002 | 611 | -0.116 | 0.045 |
| 612 | -0.020 | 0.084 | 613 | 0.101 | 0.006 | 614 | 0.205 | -0.064 | 615 | 0.073 | -0.063 |
| 616 | -0.174 | -0.118 | 617 | -0.024 | 0.026 | 618 | -0.041 | 0.129 | 619 | -0.042 | -0.053 |
| 620 | 0.148 | -0.126 | 621 | -0.030 | -0.049 | 622 | -0.015 | -0.021 | 623 | 0.089 | -0.069 |
| 624 | -0.119 | 0.011 | 625 | -0.099 | -0.048 | 626 | 0.054 | -0.196 | 627 | 0.124 | 0.035 |
| 628 | 0.092 | 0.045 | 629 | -0.037 | -0.066 | 630 | -0.021 | -0.004 | 631 | 0.042 | -0.065 |
| 632 | 0.061 | 0.048 | 633 | 0.046 | 0.004 | 634 | -0.063 | -0.045 | 635 | -0.102 | 0.152 |
| 636 | -0.039 | -0.019 | 637 | -0.005 | -0.106 | 638 | 0.083 | 0.031 | 639 | 0.226 | 0.028 |
| 640 | 0.085 | -0.065 | 641 | 0.034 | -0.142 | 642 | 0.004 | -0.012 | 643 | 0.126 | -0.043 |
| 644 | 0.055 | 0.068 | 645 | -0.020 | 0.077 | 646 | 0.008 | -0.056 | 647 | -0.034 | 0.046 |
| 648 | -0.040 | -0.134 | 649 | -0.056 | -0.131 | 650 | 0.014 | 0.097 | 651 | 0.045 | -0.009 |
| 652 | -0.113 | -0.170 | 653 | -0.065 | -0.230 | 654 | 0.065 | -0.011 | 655 | 0.011 | 0.048 |
| 656 | -0.091 | -0.059 | 657 | -0.110 | 0.024 | 658 | 0.074 | -0.034 | 659 | 0.124 | 0.022 |
| 660 | -0.037 | 0.071 | 661 | 0.015 | 0.002 | 662 | 0.028 | 0.099 | 663 | -0.062 | 0.068 |
| 664 | 0.064 | 0.016 | 665 | 0.078 | 0.156 | 666 | 0.009 | 0.219 | 667 | 0.147 | 0.024 |
| 668 | 0.106 | 0.030 | 669 | -0.080 | 0.143 | 670 | -0.049 | -0.100 | 671 | -0.036 | -0.082 |
| 672 | -0.089 | 0.021 | 673 | -0.070 | -0.029 | 674 | -0.086 | 0.048 | 675 | -0.066 | -0.015 |
| 676 | -0.024 | 0.002 | 677 | -0.030 | -0.023 | 678 | -0.032 | 0.020 | 679 | -0.002 | 0.212 |
| 680 | 0.158 | -0.024 | 681 | 0.141 | -0.119 | 682 | -0.146 | 0.058 | 683 | -0.155 | 0.083 |
| 684 | -0.002 | -0.030 | 685 | 0.018 | -0.129 | 686 | 0.012 | -0.018 | 687 | -0.008 | -0.037 |
| 688 | 0.031 | 0.040 | 689 | 0.023 | 0.097 | 690 | 0.014 | -0.039 | 691 | 0.050 | 0.019 |
| 692 | -0.072 | -0.141 | 693 | -0.023 | -0.051 | 694 | 0.024 | 0.099 | 695 | -0.127 | -0.116 |
| 696 | 0.094 | 0.102 | 697 | 0.183 | 0.098 | 698 | -0.040 | -0.020 | 699 | 0.065 | 0.077 |
| 700 | 0.088 | -0.147 | 701 | -0.039 | -0.059 | 702 | -0.057 | 0.124 | 703 | -0.077 | 0.020 |
| 704 | 0.030 | -0.120 | 705 | 0.034 | -0.142 | 706 | 0.004 | -0.012 | 707 | 0.126 | -0.043 |
| 708 | 0.055 | 0.068 | 709 | -0.020 | 0.077 | 710 | 0.008 | -0.056 | 711 | -0.034 | 0.046 |
| 712 | -0.040 | -0.134 | 713 | -0.056 | -0.131 | 714 | 0.014 | 0.097 | 715 | 0.045 | -0.009 |
| 716 | -0.113 | -0.170 | 717 | -0.065 | -0.230 | 718 | 0.065 | -0.011 | 719 | 0.011 | 0.048 |
| 720 | -0.026 | -0.021 | 721 | -0.002 | 0.041 | 722 | 0.001 | 0.071 | 723 | -0.037 | -0.117 |
| 724 | -0.106 | -0.062 | 725 | 0.002 | 0.057 | 726 | -0.008 | -0.011 | 727 | 0.019 | 0.072 |
| 728 | 0.016 | 0.059 | 729 | -0.065 | -0.077 | 730 | 0.142 | -0.062 | 731 | 0.087 | 0.025 |
| 732 | -0.003 | -0.103 | 733 | 0.107 | -0.152 | 734 | -0.054 | 0.036 | 735 | -0.030 | -0.003 |
| 736 | 0.058 | -0.020 | 737 | -0.028 | 0.007 | 738 | -0.027 | -0.099 | 739 | 0.049 | -0.075 |
| 740 | 0.174 | 0.031 | 741 | 0.134 | 0.156 | 742 | 0.060 | 0.077 | 743 | -0.010 | -0.022 |
| 744 | -0.084 | 0.040 | 745 | -0.074 | 0.011 | 746 | -0.163 | 0.054 | 747 | -0.052 | -0.008 |
| 748 | 0.076 | -0.042 | 749 | 0.043 | 0.101 | 750 | 0.058 | -0.018 | 751 | 0.003 | -0.090 |
| 752 | 0.059 | -0.018 | 753 | 0.023 | -0.031 | 754 | 0.007 | -0.017 | 755 | 0.066 | -0.017 |
| 756 | -0.135 | -0.098 | 757 | -0.056 | -0.081 | 758 | 0.089 | 0.154 | 759 | 0.120 | 0.122 |
| 760 | 0.102 | 0.001 | 761 | -0.141 | 0.102 | 762 | 0.006 | -0.011 | 763 | 0.057 | -0.039 |
| 764 | -0.059 | 0.066 | 765 | 0.132 | 0.111 | 766 | 0.012 | 0.114 | 767 | 0.047 | -0.106 |
| 768 | 0.160 | -0.099 | 769 | -0.076 | 0.084 | 770 | -0.049 | 0.073 | 771 | 0.005 | -0.086 |
| 772 | -0.052 | -0.108 | 773 | -0.073 | 0.129 | 774 | -0.129 | -0.034 | 775 | -0.153 | -0.111 |
| 776 | -0.193 | 0.098 | 777 | -0.107 | -0.068 | 778 | 0.004 | -0.009 | 779 | -0.039 | 0.024 |
| 780 | -0.054 | -0.079 | 781 | 0.024 | 0.084 | 782 | 0.052 | -0.002 | 783 | 0.028 | -0.044 |
| 784 | 0.040 | 0.018 | 785 | -0.002 | 0.041 | 786 | 0.001 | 0.071 | 787 | -0.037 | -0.117 |
| 788 | -0.106 | -0.062 | 789 | 0.002 | 0.057 | 790 | -0.008 | -0.011 | 791 | 0.019 | 0.072 |
| 792 | 0.016 | 0.059 | 793 | -0.065 | -0.077 | 794 | 0.142 | -0.062 | 795 | 0.087 | 0.025 |
| 796 | -0.003 | -0.103 | 797 | 0.107 | -0.152 | 798 | -0.054 | 0.036 | 799 | -0.030 | -0.003 |
| 800 | 0.039 | -0.090 | 801 | 0.029 | 0.025 | 802 | 0.086 | -0.029 | 803 | 0.087 | -0.082 |
| 804 | 0.003 | -0.036 | 805 | -0.096 | -0.089 | 806 | -0.073 | -0.046 | 807 | 0.105 | -0.020 |
| 808 | 0.193 | 0.018 | 809 | -0.053 | -0.073 | 810 | -0.118 | -0.149 | 811 | 0.019 | -0.019 |
| 812 | -0.042 | 0.026 | 813 | 0.041 | 0.009 | 814 | 0.028 | -0.076 | 815 | -0.038 | -0.068 |
| 816 | -0.011 | 0.010 | 817 | -0.134 | -0.064 | 818 | 0.069 | -0.067 | 819 | 0.057 | 0.006 |
| 820 | -0.134 | 0.098 | 821 | 0.152 | 0.036 | 822 | 0.041 | -0.085 | 823 | -0.099 | -0.049 |
| 824 | 0.089 | -0.099 | 825 | -0.046 | 0.018 | 826 | -0.112 | 0.135 | 827 | -0.064 | 0.018 |
| 828 | -0.022 | 0.053 | 829 | 0.041 | 0.077 | 830 | -0.021 | 0.145 | 831 | 0.007 | 0.179 |
| 832 | 0.059 | 0.041 | 833 | 0.023 | 0.064 | 834 | 0.062 | 0.022 | 835 | 0.110 | -0.081 |
| 836 | -0.016 | -0.054 | 837 | -0.014 | -0.017 | 838 | 0.171 | 0.008 | 839 | 0.070 | -0.027 |
| 840 | -0.015 | 0.002 | 841 | -0.012 | 0.053 | 842 | -0.125 | 0.009 | 843 | -0.040 | 0.012 |
| 844 | 0.036 | 0.114 | 845 | 0.007 | 0.090 | 846 | -0.016 | -0.082 | 847 | -0.008 | -0.013 |
| 848 | 0.091 | 0.030 | 849 | 0.072 | -0.068 | 850 | 0.051 | 0.063 | 851 | -0.004 | 0.049 |
| 852 | -0.130 | -0.048 | 853 | -0.121 | 0.061 | 854 | -0.095 | 0.078 | 855 | 0.011 | 0.005 |
| 856 | 0.049 | 0.001 | 857 | -0.014 | -0.011 | 858 | 0.009 | -0.063 | 859 | -0.031 | 0.040 |
| 860 | -0.011 | 0.004 | 861 | -0.033 | -0.111 | 862 | -0.115 | 0.137 | 863 | -0.025 | 0.049 |
| 864 | 0.020 | -0.160 | 865 | 0.029 | 0.025 | 866 | 0.086 | -0.029 | 867 | 0.087 | -0.082 |
| 868 | 0.003 | -0.036 | 869 | -0.096 | -0.089 | 870 | -0.073 | -0.046 | 871 | 0.105 | -0.020 |
| 872 | 0.193 | 0.018 | 873 | -0.053 | -0.073 | 874 | -0.118 | -0.149 | 875 | 0.019 | -0.019 |
| 876 | -0.042 | 0.026 | 877 | 0.041 | 0.009 | 878 | 0.028 | -0.076 | 879 | -0.038 | -0.068 |
| 880 | -0.006 | 0.005 |  |  |  |  |  |  |  |  |  |

Table G.2 (Correct)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -32 | 0.0 | 0.0 | -31 | 0.0 | 0.0 | -30 | 0.0 | 0.0 | -29 | 0.0 | 0.0 |
| -28 | 0.0 | 0.0 | -27 | 0.0 | 0.0 | -26 | 0.0 | 0.0 | -25 | 0.0 | 0.0 |
| -24 | 1.472 | 1.472 | -23 | 0.0 | 0.0 | -22 | 0.0 | 0.0 | -21 | 0.0 | 0.0 |
| -20 | -1.472 | -1.472 | -19 | 0.0 | 0.0 | -18 | 0.0 | 0.0 | -17 | 0.0 | 0.0 |
| -16 | 1.472 | 1.472 | -15 | 0.0 | 0.0 | -14 | 0.0 | 0.0 | -13 | 0.0 | 0.0 |
| -12 | -1.472 | -1.472 | -11 | 0.0 | 0.0 | -10 | 0.0 | 0.0 | -9 | 0.0 | 0.0 |
| -8 | -1.472 | -1.472 | -7 | 0.0 | 0.0 | -6 | 0.0 | 0.0 | -5 | 0.0 | 0.0 |
| -4 | 1.472 | 1.472 | -3 | 0.0 | 0.0 | -2 | 0.0 | 0.0 | -1 | 0.0 | 0.0 |
| 0 | 0.0 | 0.0 | 1 | 0.0 | 0.0 | 2 | 0.0 | 0.0 | 3 | 0.0 | 0.0 |
| 4 | -1.472 | -1.472 | 5 | 0.0 | 0.0 | 6 | 0.0 | 0.0 | 7 | 0.0 | 0.0 |
| 8 | -1.472 | -1.472 | 9 | 0.0 | 0.0 | 10 | 0.0 | 0.0 | 11 | 0.0 | 0.0 |
| 12 | 1.472 | 1.472 | 13 | 0.0 | 0.0 | 14 | 0.0 | 0.0 | 15 | 0.0 | 0.0 |
| 16 | 1.472 | 1.472 | 17 | 0.0 | 0.0 | 18 | 0.0 | 0.0 | 19 | 0.0 | 0.0 |
| 20 | 1.472 | 1.472 | 21 | 0.0 | 0.0 | 22 | 0.0 | 0.0 | 23 | 0.0 | 0.0 |
| 24 | 1.472 | 1.472 | 25 | 0.0 | 0.0 | 26 | 0.0 | 0.0 | 27 | 0.0 | 0.0 |
| 28 | 0.0 | 0.0 | 29 | 0.0 | 0.0 | 62 | 0.0 | 0.0 | 31 | 0.0 | 0.0 |

Table G.3 (Correct)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.046 | 0.046 | 1 | -0.132 | 0.002 | 2 | -0.013 | -0.079 | 3 | 0.143 | -0.013 |
| 4 | 0.092 | 0.000 | 5 | 0.143 | -0.013 | 6 | -0.013 | -0.079 | 7 | -0.132 | 0.002 |
| 8 | 0.046 | 0.046 | 9 | 0.002 | -0.132 | 10 | -0.079 | -0.013 | 11 | -0.013 | 0.143 |
| 12 | 0.000 | 0.092 | 13 | -0.013 | 0.143 | 14 | -0.079 | -0.013 | 15 | 0.002 | -0.132 |
| 16 | 0.046 | 0.046 | 17 | -0.132 | 0.002 | 18 | -0.013 | -0.079 | 19 | 0.143 | -0.013 |
| 20 | 0.092 | 0.000 | 21 | 0.143 | -0.013 | 22 | -0.013 | -0.079 | 23 | -0.132 | 0.002 |
| 24 | 0.046 | 0.046 | 25 | 0.002 | -0.132 | 26 | -0.079 | -0.013 | 27 | -0.013 | 0.143 |
| 28 | 0.000 | 0.092 | 29 | -0.013 | 0.143 | 30 | -0.079 | -0.013 | 31 | 0.002 | -0.132 |
| 32 | 0.046 | 0.046 | 33 | -0.132 | 0.002 | 34 | -0.013 | -0.079 | 35 | 0.143 | -0.013 |
| 36 | 0.092 | 0.000 | 37 | 0.143 | -0.013 | 38 | -0.013 | -0.079 | 39 | -0.132 | 0.002 |
| 40 | 0.046 | 0.046 | 41 | 0.002 | -0.132 | 42 | -0.079 | -0.013 | 43 | -0.013 | 0.143 |
| 44 | 0.000 | 0.092 | 45 | -0.013 | 0.143 | 46 | -0.079 | -0.013 | 47 | 0.002 | -0.132 |
| 48 | 0.046 | 0.046 | 49 | -0.132 | 0.002 | 50 | -0.013 | -0.079 | 51 | 0.143 | -0.013 |
| 52 | 0.092 | 0.000 | 53 | 0.143 | -0.013 | 54 | -0.013 | -0.079 | 55 | -0.132 | 0.002 |
| 56 | 0.046 | 0.046 | 57 | 0.002 | -0.132 | 58 | -0.079 | -0.013 | 59 | -0.013 | 0.143 |
| 60 | 0.000 | 0.092 | 61 | -0.013 | 0.143 | 62 | -0.079 | -0.013 | 63 | 0.002 | -0.132 |

Table G.4 (Re-formatted)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.023 | 0.023 | 1 | -0.132 | 0.002 | 2 | -0.013 | -0.079 | 3 | 0.143 | -0.013 |
| 4 | 0.092 | 0.000 | 5 | 0.143 | -0.013 | 6 | -0.013 | -0.079 | 7 | -0.132 | 0.002 |
| 8 | 0.046 | 0.046 | 9 | 0.002 | -0.132 | 10 | -0.079 | -0.013 | 11 | -0.013 | 0.143 |
| 12 | 0.000 | 0.092 | 13 | -0.013 | 0.143 | 14 | -0.079 | -0.013 | 15 | 0.002 | -0.132 |
| 16 | 0.046 | 0.046 | 17 | -0.132 | 0.002 | 18 | -0.013 | -0.079 | 19 | 0.143 | -0.013 |
| 20 | 0.092 | 0.000 | 21 | 0.143 | -0.013 | 22 | -0.013 | -0.079 | 23 | -0.132 | 0.002 |
| 24 | 0.046 | 0.046 | 25 | 0.002 | -0.132 | 26 | -0.079 | -0.013 | 27 | -0.013 | 0.143 |
| 28 | 0.000 | 0.092 | 29 | -0.013 | 0.143 | 30 | -0.079 | -0.013 | 31 | 0.002 | -0.132 |
| 32 | 0.046 | 0.046 | 33 | -0.132 | 0.002 | 34 | -0.013 | -0.079 | 35 | 0.143 | -0.013 |
| 36 | 0.092 | 0.000 | 37 | 0.143 | -0.013 | 38 | -0.013 | -0.079 | 39 | -0.132 | 0.002 |
| 40 | 0.046 | 0.046 | 41 | 0.002 | -0.132 | 42 | -0.079 | -0.013 | 43 | -0.013 | 0.143 |
| 44 | 0.000 | 0.092 | 45 | -0.013 | 0.143 | 46 | -0.079 | -0.013 | 47 | 0.002 | -0.132 |
| 48 | 0.046 | 0.046 | 49 | -0.132 | 0.002 | 50 | -0.013 | -0.079 | 51 | 0.143 | -0.013 |
| 52 | 0.092 | 0.000 | 53 | 0.143 | -0.013 | 54 | -0.013 | -0.079 | 55 | -0.132 | 0.002 |
| 56 | 0.046 | 0.046 | 57 | 0.002 | -0.132 | 58 | -0.079 | -0.013 | 59 | -0.013 | 0.143 |
| 60 | 0.000 | 0.092 | 61 | -0.013 | 0.143 | 62 | -0.079 | -0.013 | 63 | 0.002 | -0.132 |
| 64 | 0.046 | 0.046 | 65 | -0.132 | 0.002 | 66 | -0.013 | -0.079 | 67 | 0.143 | -0.013 |
| 68 | 0.092 | 0.000 | 69 | 0.143 | -0.013 | 70 | -0.013 | -0.079 | 71 | -0.132 | 0.002 |
| 72 | 0.046 | 0.046 | 73 | 0.002 | -0.132 | 74 | -0.079 | -0.013 | 75 | -0.013 | 0.143 |
| 76 | 0.000 | 0.092 | 77 | -0.013 | 0.143 | 78 | -0.079 | -0.013 | 79 | 0.002 | -0.132 |
| 80 | 0.046 | 0.046 | 81 | -0.132 | 0.002 | 82 | -0.013 | -0.079 | 83 | 0.143 | -0.013 |
| 84 | 0.092 | 0.000 | 85 | 0.143 | -0.013 | 86 | -0.013 | -0.079 | 87 | -0.132 | 0.002 |
| 88 | 0.046 | 0.046 | 89 | 0.002 | -0.132 | 90 | -0.079 | -0.013 | 91 | -0.013 | 0.143 |
| 92 | 0.000 | 0.092 | 93 | -0.013 | 0.143 | 94 | -0.079 | -0.013 | 95 | 0.002 | -0.132 |
| 96 | 0.046 | 0.046 | 97 | -0.132 | 0.002 | 98 | -0.013 | -0.079 | 99 | 0.143 | -0.013 |
| 100 | 0.092 | 0.000 | 101 | 0.143 | -0.013 | 102 | -0.013 | -0.079 | 103 | -0.132 | 0.002 |
| 104 | 0.046 | 0.046 | 105 | 0.002 | -0.132 | 106 | -0.079 | -0.013 | 107 | -0.013 | 0.143 |
| 108 | 0.000 | 0.092 | 109 | -0.013 | 0.143 | 110 | -0.079 | -0.013 | 111 | 0.002 | -0.132 |
| 112 | 0.046 | 0.046 | 113 | -0.132 | 0.002 | 114 | -0.013 | -0.079 | 115 | 0.143 | -0.013 |
| 116 | 0.092 | 0.000 | 117 | 0.143 | -0.013 | 118 | -0.013 | -0.079 | 119 | -0.132 | 0.002 |
| 120 | 0.046 | 0.046 | 121 | 0.002 | -0.132 | 122 | -0.079 | -0.013 | 123 | -0.013 | 0.143 |
| 124 | 0.000 | 0.092 | 125 | -0.013 | 0.143 | 126 | -0.079 | -0.013 | 127 | 0.002 | -0.132 |
| 128 | 0.046 | 0.046 | 129 | -0.132 | 0.002 | 130 | -0.013 | -0.079 | 131 | 0.143 | -0.013 |
| 132 | 0.092 | 0.000 | 133 | 0.143 | -0.013 | 134 | -0.013 | -0.079 | 135 | -0.132 | 0.002 |
| 136 | 0.046 | 0.046 | 137 | 0.002 | -0.132 | 138 | -0.079 | -0.013 | 139 | -0.013 | 0.143 |
| 140 | 0.000 | 0.092 | 141 | -0.013 | 0.143 | 142 | -0.079 | -0.013 | 143 | 0.002 | -0.132 |
| 144 | 0.046 | 0.046 | 145 | -0.132 | 0.002 | 146 | -0.013 | -0.079 | 147 | 0.143 | -0.013 |
| 148 | 0.092 | 0.000 | 149 | 0.143 | -0.013 | 150 | -0.013 | -0.079 | 151 | -0.132 | 0.002 |
| 152 | 0.046 | 0.046 | 153 | 0.002 | -0.132 | 154 | -0.079 | -0.013 | 155 | -0.013 | 0.143 |
| 156 | 0.000 | 0.092 | 157 | -0.013 | 0.143 | 158 | -0.079 | -0.013 | 159 | 0.002 | -0.132 |
| 160 | 0.023 | 0.023 |  |  |  |  |  |  |  |  |  |

Table G.6 (Re-formatted)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -0.078 | 0.000 | 1 | 0.012 | -0.098 | 2 | 0.092 | -0.106 | 3 | -0.092 | -0.115 |
| 4 | -0.003 | -0.054 | 5 | 0.075 | 0.074 | 6 | -0.127 | 0.021 | 7 | -0.122 | 0.017 |
| 8 | -0.035 | 0.151 | 9 | -0.056 | 0.022 | 10 | -0.060 | -0.081 | 11 | 0.070 | -0.014 |
| 12 | 0.082 | -0.092 | 13 | -0.131 | -0.065 | 14 | -0.057 | -0.039 | 15 | 0.037 | -0.098 |
| 16 | 0.062 | 0.062 | 17 | 0.119 | 0.004 | 18 | -0.022 | -0.161 | 19 | 0.059 | 0.015 |
| 20 | 0.024 | 0.059 | 21 | -0.137 | 0.047 | 22 | 0.001 | 0.115 | 23 | 0.053 | -0.004 |
| 24 | 0.098 | 0.026 | 25 | -0.038 | 0.106 | 26 | -0.115 | 0.055 | 27 | 0.060 | 0.088 |
| 28 | 0.021 | -0.028 | 29 | 0.097 | -0.083 | 30 | 0.040 | 0.111 | 31 | -0.005 | 0.120 |
| 32 | 0.156 | 0.000 | 33 | -0.005 | -0.120 | 34 | 0.040 | -0.111 | 35 | 0.097 | 0.083 |
| 36 | 0.021 | 0.028 | 37 | 0.060 | -0.088 | 38 | -0.115 | -0.055 | 39 | -0.038 | -0.106 |
| 40 | 0.098 | -0.026 | 41 | 0.053 | 0.004 | 42 | 0.001 | -0.115 | 43 | -0.137 | -0.047 |
| 44 | 0.024 | -0.059 | 45 | 0.059 | -0.015 | 46 | -0.022 | 0.161 | 47 | 0.119 | -0.004 |
| 48 | 0.062 | -0.062 | 49 | 0.037 | 0.098 | 50 | -0.057 | 0.039 | 51 | -0.131 | 0.065 |
| 52 | 0.082 | 0.092 | 53 | 0.070 | 0.014 | 54 | -0.060 | 0.081 | 55 | -0.056 | -0.022 |
| 56 | -0.035 | -0.151 | 57 | -0.122 | -0.017 | 58 | -0.127 | -0.021 | 59 | 0.075 | -0.074 |
| 60 | -0.003 | 0.054 | 61 | -0.092 | 0.115 | 62 | 0.092 | 0.106 | 63 | 0.012 | 0.098 |
| 64 | -0.156 | 0.000 | 65 | 0.012 | -0.098 | 66 | 0.092 | -0.106 | 67 | -0.092 | -0.115 |
| 68 | -0.003 | -0.054 | 69 | 0.075 | 0.074 | 70 | -0.127 | 0.021 | 71 | -0.122 | 0.017 |
| 72 | -0.035 | 0.151 | 73 | -0.056 | 0.022 | 74 | -0.060 | -0.081 | 75 | 0.070 | -0.014 |
| 76 | 0.082 | -0.092 | 77 | -0.131 | -0.065 | 78 | -0.057 | -0.039 | 79 | 0.037 | -0.098 |
| 80 | 0.062 | 0.062 | 81 | 0.119 | 0.004 | 82 | -0.022 | -0.161 | 83 | 0.059 | 0.015 |
| 84 | 0.024 | 0.059 | 85 | -0.137 | 0.047 | 86 | 0.001 | 0.115 | 87 | 0.053 | -0.004 |
| 88 | 0.098 | 0.026 | 89 | -0.038 | 0.106 | 90 | -0.115 | 0.055 | 91 | 0.060 | 0.088 |
| 92 | 0.021 | -0.028 | 93 | 0.097 | -0.083 | 94 | 0.040 | 0.111 | 95 | -0.005 | 0.120 |
| 96 | 0.156 | 0.000 | 97 | -0.005 | -0.120 | 98 | 0.040 | -0.111 | 99 | 0.097 | 0.083 |
| 100 | 0.021 | 0.028 | 101 | 0.060 | -0.088 | 102 | -0.115 | -0.055 | 103 | -0.038 | -0.106 |
| 104 | 0.098 | -0.026 | 105 | 0.053 | 0.004 | 106 | 0.001 | -0.115 | 107 | -0.137 | -0.047 |
| 108 | 0.024 | -0.059 | 109 | 0.059 | -0.015 | 110 | -0.022 | 0.161 | 111 | 0.119 | -0.004 |
| 112 | 0.062 | -0.062 | 113 | 0.037 | 0.098 | 114 | -0.057 | 0.039 | 115 | -0.131 | 0.065 |
| 116 | 0.082 | 0.092 | 117 | 0.070 | 0.014 | 118 | -0.060 | 0.081 | 119 | -0.056 | -0.022 |
| 120 | -0.035 | -0.151 | 121 | -0.122 | -0.017 | 122 | -0.127 | -0.021 | 123 | 0.075 | -0.074 |
| 124 | -0.003 | 0.054 | 125 | -0.092 | 0.115 | 126 | 0.092 | 0.106 | 127 | 0.012 | 0.098 |
| 128 | -0.156 | 0.000 | 129 | 0.012 | -0.098 | 130 | 0.092 | -0.106 | 131 | -0.092 | -0.115 |
| 132 | -0.003 | -0.054 | 133 | 0.075 | 0.074 | 134 | -0.127 | 0.021 | 135 | -0.122 | 0.017 |
| 136 | -0.035 | 0.151 | 137 | -0.056 | 0.022 | 138 | -0.060 | -0.081 | 139 | 0.070 | -0.014 |
| 140 | 0.082 | -0.092 | 141 | -0.131 | -0.065 | 142 | -0.057 | -0.039 | 143 | 0.037 | -0.098 |
| 144 | 0.062 | 0.062 | 145 | 0.119 | 0.004 | 146 | -0.022 | -0.161 | 147 | 0.059 | 0.015 |
| 148 | 0.024 | 0.059 | 149 | -0.137 | 0.047 | 150 | 0.001 | 0.115 | 151 | 0.053 | -0.004 |
| 152 | 0.098 | 0.026 | 153 | -0.038 | 0.106 | 154 | -0.115 | 0.055 | 155 | 0.060 | 0.088 |
| 156 | 0.021 | -0.028 | 157 | 0.097 | -0.083 | 158 | 0.040 | 0.111 | 159 | -0.005 | 0.120 |
| 160 | 0.078 | 0 |  |  |  |  |  |  |  |  |  |

Table G.12 (Re-formatted)

| ## | Re | Im | ## | Re | Im | ## | Re | Im | ## | Re | Im |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.031 | 0.000 | 1 | 0.033 | -0.044 | 2 | -0.002 | -0.038 | 3 | -0.081 | 0.084 |
| 4 | 0.007 | -0.100 | 5 | -0.001 | -0.113 | 6 | -0.021 | -0.005 | 7 | 0.136 | -0.105 |
| 8 | 0.098 | -0.044 | 9 | 0.011 | -0.002 | 10 | -0.033 | 0.044 | 11 | -0.060 | 0.124 |
| 12 | 0.010 | 0.097 | 13 | 0.000 | -0.008 | 14 | 0.018 | -0.083 | 15 | -0.069 | 0.027 |
| 16 | -0.219 | 0.000 | 17 | -0.069 | -0.027 | 18 | 0.018 | 0.083 | 19 | 0.000 | 0.008 |
| 20 | 0.010 | -0.097 | 21 | -0.060 | -0.124 | 22 | -0.033 | -0.044 | 23 | 0.011 | 0.002 |
| 24 | 0.098 | 0.044 | 25 | 0.136 | 0.105 | 26 | -0.021 | 0.005 | 27 | -0.001 | 0.113 |
| 28 | 0.007 | 0.100 | 29 | -0.081 | -0.084 | 30 | -0.002 | 0.038 | 31 | 0.033 | 0.044 |
| 32 | 0.062 | 0.000 | 33 | 0.057 | 0.052 | 34 | 0.016 | 0.174 | 35 | 0.035 | 0.116 |
| 36 | -0.051 | -0.202 | 37 | 0.011 | 0.036 | 38 | 0.089 | 0.209 | 39 | -0.049 | -0.008 |
| 40 | -0.035 | 0.044 | 41 | 0.017 | -0.059 | 42 | 0.053 | -0.017 | 43 | 0.099 | 0.100 |
| 44 | 0.034 | -0.148 | 45 | -0.003 | -0.094 | 46 | -0.120 | 0.042 | 47 | -0.136 | -0.070 |
| 48 | -0.031 | 0.000 | 49 | -0.136 | 0.070 | 50 | -0.120 | -0.042 | 51 | -0.003 | 0.094 |
| 52 | 0.034 | 0.148 | 53 | 0.099 | -0.100 | 54 | 0.053 | 0.017 | 55 | 0.017 | 0.059 |
| 56 | -0.035 | -0.044 | 57 | -0.049 | 0.008 | 58 | 0.089 | -0.209 | 59 | 0.011 | -0.036 |
| 60 | -0.051 | 0.202 | 61 | 0.035 | -0.116 | 62 | 0.016 | -0.174 | 63 | 0.057 | -0.052 |
| 64 | 0.062 | 0.000 | 65 | 0.033 | -0.044 | 66 | -0.002 | -0.038 | 67 | -0.081 | 0.084 |
| 68 | 0.007 | -0.100 | 69 | -0.001 | -0.113 | 70 | -0.021 | -0.005 | 71 | 0.136 | -0.105 |
| 72 | 0.098 | -0.044 | 73 | 0.011 | -0.002 | 74 | -0.033 | 0.044 | 75 | -0.060 | 0.124 |
| 76 | 0.010 | 0.097 | 77 | 0.000 | -0.008 | 78 | 0.018 | -0.083 | 79 | -0.069 | 0.027 |
| 80 | -0.109 | 0.000 |  |  |  |  |  |  |  |  |  |