IEEE P802.11
Wireless LANs

802.11 MAC Enhancements, Draft Requirements

Date:	January 11, 2000

Author:	Tim Godfrey, John Fakatselis
	Intersil
	Palm Bay FL
	913-706-3777 321-724-7000 
	
	e-Mail: tgodfrey@choicemicro.com jfakat01@intersil.com

# Application Requirements

| Function | Applications | Comments |
| --- | --- | --- |
| QoS | Voice |  |
|  | Audio |  |
|  | Video |  |
|  | Interactive multimedia |  |
|  | Ad-hoc networks to be supported |  |
|  | Videoconferencing |  |
| Privacy | Audio and Video content security / copy protection |  |
| Authentication | Roaming support (IAPP) |  |
| Other | Load Balancing, Conveyance of unique vendor information, PHY power control and coordination. |  |

# Functional Requirements

| Function | Requirement |  | Comments |
| --- | --- | --- | --- |
| QoS | Efficiently transport digital audio/video streams, including one-to-many distribution |  |  |
|  | Support for multiple priorities and classes of service |  |  |
|  | Protocols MUST support efficient transport of encoded voice data in terms of bandwidth, reliability and delay. |  |  |
|  | The MAC enhancements must provide support for real-time and non-real-time services. It SHOULD be possible to support the emerging Quality of Service (QoS) efforts |  |  |
|  | The MAC enhancements must support resolving contention and bandwidth allocation. |  |  |
|  | The MAC enhancements SHOULD allow for the support for different “scales” of 802.11 networks. (IBSS, single BSS, ESS) |  |  |
|  | The MAC enhancements may allow for classes of service where acknowlegements are not mandatory |  |  |
|  | MAC enhancements MUST support classes of service (CoS) with various quality of service (QoS) guarantees to support the applications the 802.11 system may transport. |  |  |
|  | MAC enhancements MUST define interfaces and procedures that accommodate the needs of the applications with respect to allocation and prioritization of bandwidth. |  |  |
|  | An 802.11 AP acting as an inter-working function (IWF) between a QoS-capable LAN or WAN and an 802.11 system MUST participate in signaling protocols to set up QoS parameters for connection-oriented services. |  |  |
|  | the QoS parameters for the MAC enhancements MUST be chosen and interface primitives defined that allow for applications to negotiate QoS “through” an 802.11 system. |  |  |
|  | The MAC enhancements must interoperate with existing wired QoS standards. |  |  |
|  | The MAC enhancements may provide the ability to account for time- and bandwidth-utilization and the various QoS parameters for each station. |  |  |
| Privacy | MAC enhancements should allow for new encryption algorithms, in addition to the existing WEP specified by 802.11. |  |  |
|  | MAC enhancements should allow for a strong cryptographic algorithm to be employed that is internationally applicable for privacy. |  |  |
|  | MAC enhancements should support various security strength levels |  |  |
|  | MAC enhancements should comply with existing multimedia security standards. |  |  |
| Authentication | The MAC enhancements may require authentication of associating stations. This initial authentication must prevent unauthorized access to the BSS. |  |  |
|  | Passwords and secret keys MUST NOT be passed “in the clear” through the air interface. |  |  |
| Other | Support adjustable power, modulation, and other parameters for rapid channel changes. |  |  |
|  | Allow for proprietary vendor information while maintaining interoperability, except for items already covered in the standard. |  |  |
|  | Allow for implementation of Load Balancing algorithms |  |  |

# Performance Requirements

| Function | MAC Payload rate |  | Maximum Bit Error Rate |  | Maximum Delay |  | Other |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QoS | High Quality Narrowband/Voice Frequency Telephony (Vocoder MOS < 4.0) |  |  |  |  |  |  |  |  |  |
| Privacy |  |  |  |  |  |  |  |  |  |  |
| Authentication |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |

<Table for Compatibility and Interoperability>