IEEE P802.11
Wireless LANs

802.11 MAC Enhancements, Draft Requirements

Date:	January 12, 2000

Author:	Tim Godfrey, John Fakatselis
	Intersil
	Palm Bay FL
	913-706-3777 321-724-7000 
	
	e-Mail: tgodfrey@choicemicro.com jfakat01@intersil.com

# Application Requirements

| Function | Applications | Comments |
| --- | --- | --- |
| QoS | Voice |  |
|  | Audio |  |
|  | Video |  |
|  | Interactive multimedia |  |
|  | Ad-hoc networks to be supported |  |
|  | Videoconferencing |  |
| Privacy | Audio and Video content security / copy protection |  |
| Authentication | Roaming support (IAPP) |  |
| Other | Load Balancing, Conveyance of unique vendor information, PHY power control and coordination. |  |

# Functional Requirements

| Function | Requirement |  | Comments |
| --- | --- | --- | --- |
| QoS | Efficiently transport digital audio/video streams, including one-to-many distribution |  |  |
|  | Support for multiple priorities and classes of service |  |  |
|  | Protocols MUST support efficient transport of encoded voice data in terms of bandwidth, reliability and delay. |  |  |
|  | The MAC enhancements must provide support for real-time and non-real-time services. It SHOULD be possible to support the emerging Quality of Service (QoS) efforts |  |  |
|  | The MAC enhancements must support resolving contention and bandwidth allocation. |  |  |
|  | The MAC enhancements SHOULD allow for the support for different “scales” of 802.11 networks. (IBSS, single BSS, ESS) |  |  |
|  | The MAC enhancements may allow for classes of service where acknowlegements are not mandatory |  |  |
|  | MAC enhancements MUST support classes of service (CoS) with various quality of service (QoS) guarantees to support the applications the 802.11 system may transport. |  |  |
|  | MAC enhancements MUST define interfaces and procedures that accommodate the needs of the applications with respect to allocation and prioritization of bandwidth. |  |  |
|  | An 802.11 AP acting as an inter-working function (IWF) between a QoS-capable LAN or WAN and an 802.11 system MUST participate in signaling protocols to set up QoS parameters for connection-oriented services. |  |  |
|  | the QoS parameters for the MAC enhancements MUST be chosen and interface primitives defined that allow for applications to negotiate QoS “through” an 802.11 system. |  |  |
|  | The MAC enhancements must interoperate with existing wired QoS standards. |  |  |
|  | The MAC enhancements may provide the ability to account for time- and bandwidth-utilization and the various QoS parameters for each station. |  |  |
| Privacy | MAC enhancements should allow for new encryption algorithms, in addition to the existing WEP specified by 802.11. |  |  |
|  | MAC enhancements should allow for a strong cryptographic algorithm to be employed that is internationally applicable for privacy. |  |  |
|  | MAC enhancements should support various security strength levels |  |  |
|  | MAC enhancements should comply with existing multimedia security standards. |  |  |
| Authentication | The MAC enhancements may require authentication of associating stations. This initial authentication must prevent unauthorized access to the BSS. |  |  |
|  | Passwords and secret keys MUST NOT be passed “in the clear” through the air interface. |  |  |
| Other | Support adjustable power modulation, and other parameters for rapid channel changes. |  |  |
|  | Allow for proprietary vendor information while maintaining interoperability, except for items already covered in the standard. |  |  |
|  | Allow for implementation of Load Balancing algorithms |  |  |
|  | Stability of the network shall be maintained when new nodes and connections are added or removed. |  |  |
|  | Mechanisms to coordinate co-located and overlapping BSS’s shall be provided. |  |  |
|  | Mechanisms shall be provided to handle network overload. |  |  |

# Performance Requirements

| Function | Application | MAC Payload rate (per channel) | Maximum Bit Error Rate | Maximum Delay | Other |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QoS | High Quality Narrowband/Voice Frequency Telephony* | 32 kbps – 64 kbps | 10-6 BER | 5mS nominal, 10mS maximum |  |  | TBD – we need to research what BER is actually necessary for this class |
|  | Lower Quality Narrowband/Voice Frequency Telephony | 6 kbps – 16 kbps | 10-4 BER | 5mS nominal, 10mS maximum |  |  |  |
|  | CD Quality Audio | 1.5 Mbps | 10-6 BER (better BER may be necessary for “CD quality”) | < 100mS |  | Jitter is important <TBD> |  |
|  | Time Critical Packet Services: H.323 MPEG1 (high end of BW range supports high quality audio) | 4-13 kbps (voice) and 32kbps-1.5 Mbps (audio/video) | BER 10-8 | 5mS nominal, 10mS maximum (for full duplex voice/video applications) |  |  | (Need to deal with differing time to live for different application classes) |
|  | Non- Time Critical Services: IP, IPX, FR… Bulk data transfer etc.. | Up to maximum PHY rate | BER 10-8 | N/A |  | Yes, but limited to 802.11 PHY rates |  |
|  | MPEG video unicast broadcast, audio/video streaming | Broadband video typically around 6 Mbps, HDTV 19.68 Mbps | ## BER 10-8 | TBD (nominal delay of 90ms, but selectable) (need to specify inter-packet jitter) |  |  |  |
| Privacy | Bulk Data Transfer | Encryption does not affect throughput |  |  |  | Support for at least 128 bit keys | In compliance with export laws |
|  | Audio and Video content |  |  |  |  | Preserves copy protection |  |
| Authentication |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |

- Telephony is defined to be real time, interactive conversational mode.

Compatibility and Interoperability Requirements

| Existing 802.11 standard | Must be completely interoperable with existing stations implementing the base 802.11 DCF and PCF functionality. |
| --- | --- |
| Higher layer standards | Provide the necessary support for higher layer Inter Working Functions. (EG end to end connection setup and QoS control) |

### Performance Test Matrix

| Scenario |  | Channel PER |  |  | Overlapped BSS | Output Parameter (top of MAC) |  |  | Baseline MAC performance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | # streams | 0 | 10% | 25% |  | Throughput per stream/application | Latency within stream (MAC to MAC delay) | Jitter within stream |  |
| MPEG1 @2.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 1 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 2 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 25% duty. |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 4 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 50% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Network Game |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 6 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 4 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Video Conference |  |  |  |  |  |  |  |  |
| HDTV @20M | 1 |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |

# Definition of Class structure:

## Audio

### MP3 (up to 128Kbps)

### Redbook audio (1.45Mbps)

### Dolby Digital, and others  (up to 768Kbps)

## Video

### MPEG1 (up to 3Mbps)

### MPEG2 (4 to 11Mbps)

### HDTV (20Mbps and higher)

## Voice

### High Quality Telephony (32 to 64Kbps)

### Low Quality Telephony (6 to 16Kbps)

## Data

### Bursty (POP3 client, WWW, large numbers of small packets)

### Bulk (File Transfer, multi-megabits of large packets)

## Interactive Multimedia

### Head to Head Games (networked multiplayer games – ex Doom)

### Video Conferencing (H.323, Netmeeting)

# Definition of Source or Service (input pipe)

## 1 to 4MBps (ADSL, WLL)

## 4 to 20MBps (Cable Modem, LMDS)

## above 20MBps (enterprise infrastructure, point to point wireless bridging)

# Topology Structures (testing to be performed for each of these 2 cases)

## At least a 5 node network, one BSS.

## Two co-located BSS’s on the same frequency.