IEEE P802.11
Wireless LANs

802.11 MAC Enhancements, Draft Requirements

Date:	March 9, 2000

Author:	Tim Godfrey, John Fakatselis
	Intersil
	Palm Bay FL
	321-724-7000

Including edits from the Ad-Hoc Group for MAC Enhancements Requirements
at the 802.11 March 2000 Plenary, Albuquerque, NM

# Application Requirements

| Function | Applications | Comments |
| --- | --- | --- |
| QoS | Voice |  |
|  | Audio |  |
|  | Video |  |
|  | Interactive multimedia |  |
|  | Peer-to-peer communications without intermediary (incl. Ad hoc networks) |  |
|  | Videoconferencing |  |
| Privacy | Audio and Video content security / copy protection equivalent to DSS and DVD security standards, Protection against voice and data eavesdropping and media stream theft over a shared channel |  |
| Authentication | Roaming support and support for connection handoff Support for various forms of Authentication, including current 802.11 Open System and Shared Key as well as Unary ID, Shared ID, Open Enrollment, Guest Enrollment. |  |
| Other | Load Balancing, Conveyance of unique vendor information, PHY power control and coordination, Dynamic frequency control, provision for end-to-end QoS indication/negotiation. |  |

# Functional Requirements

| Function | Requirement | Comments |
| --- | --- | --- |
| QoS | Efficiently transport digital audio/video streams, including one-to-many (multicast) distribution |  |
|  | Support for multiple priorities and classes of service |  |
|  | Protocols MUST support efficient transport of encoded voice data in terms of bandwidth, reliability and delay. |  |
|  | The MAC enhancements must provide support for real-time and non-real-time services. It SHOULD be possible to support the emerging IETF and 802.1 Quality of Service (QoS) efforts |  |
|  | The MAC enhancements must support resolving contention and bandwidth allocation including support for guaranteed connection contracts. |  |
|  | The MAC enhancements SHOULD allow for the support for different “scales” of 802.11 networks. (IBSS, single BSS, ESS, overlapped BSS) |  |
|  | The MAC enhancements may allow for classes of service where acknowlegements are not mandatory, or delayed or are grouped |  |
|  | MAC enhancements MUST support classes of service (CoS) with various quality of service (QoS) guarantees to support the applications the 802.11 system may transport. |  |
|  | MAC enhancements MUST define interfaces and procedures that accommodate the needs of the applications with respect to allocation and prioritization of bandwidth. |  |
|  | An 802.11 AP acting as an inter-working function (IWF) between a QoS-capable LAN or WAN and an 802.11 system MUST participate in signaling protocols to set up QoS parameters for connection-oriented services for relay of QoS from end-to-end and over intermediate links. |  |
|  | The QoS parameters for the MAC enhancements MUST be chosen and interface primitives defined that allow for applications to negotiate QoS “through” an 802.11 system for relay of QoS from end-to-end or/and over intermediate links. |  |
|  | The MAC enhancements must interoperate with existing wired QoS standards. |  |
|  | The MAC enhancements may provide the ability to account for time- and bandwidth-utilization, stream synchronization (including multicast) and the various QoS parameters for each station. |  |
| Privacy | MAC enhancements should allow for new encryption algorithms, in addition to the existing WEP specified by 802.11. |  |
|  | MAC enhancements should allow for a strong cryptographic algorithm to be employed that is internationally applicable for privacy. |  |
|  | MAC enhancements should support various security strength levels |  |
|  | MAC enhancements should comply with existing multimedia security standards. |  |
|  | MAC Enhancements should protect the encrypted data from modification or forgery. |  |
| Authentication | The MAC enhancements may require authentication of associating stations. This initial authentication must prevent unauthorized access to the BSS. |  |
|  | Passwords and secret keys MUST NOT be passed “in the clear” through the air interface. |  |
| Other | Support adjustable power modulation, and other parameters for rapid channel changes. |  |
|  | Allow for proprietary vendor information while maintaining interoperability, except for items already covered in the standard. |  |
|  | Allow for implementation of Load Balancing algorithms |  |
|  | Stability of the network shall be maintained when new nodes and connections are added or removed. |  |
|  | Mechanisms to coordinate co-located and overlapping BSS’s shall be provided. |  |
|  | Mechanisms shall be provided to handle network overload. |  |

# Performance Requirements

| Function | Application | MAC Payload rate (per channel) | Maximum Error Rate – Application Requirement | Maximum Delay | Other |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QoS | High Quality Narrowband/Voice Frequency Telephony* | >16 kbps – 64 kbps | 10-6 BER* | 5mS to 20mS access delay; tbd mS max framing delay |  | Measured from portal to end device | *TBD – we need to research what BER or FER is actually necessary for this application |
|  | Lower Quality Narrowband/Voice Frequency Telephony | 6 kbps – 16 kbps | 10-4 BER | 5mS to 20mS access delay; tbd mS max framing delay |  | Measured from portal to end device |  |
|  | CD Quality Audio (16 bit PCM) | 1.5 Mbps | 10-8 BER | < 100mS (same as three-frame video latency) |  | Jitter is important <TBD> Will be similar to that speced for a CD player |  |
|  | Time Critical Packet Services: H.323 MPEG1 (high end of BW range supports high quality audio) | 4-13 kbps (voice) and 32kbps-1.5 Mbps (audio/video) | BER 10-8 | 5mS to 20mS (for full duplex voice/video applications) |  | Video will need to be specified separately | (Need to deal with differing time to live for different application classes) |
|  | Interactive Multimedia | tbd Mbps | tbd BER | < 80mS |  | tbd | Need to look at voice-over requirements for gaming |
|  | Non- Time Critical Services: IP, IPX, FR… Bulk data transfer etc.. | Up to maximum PHY rate | BER 10-8 as delivered to higher layer protocol | N/A |  | Yes, but limited to 802.11 PHY rates |  |
|  | MPEG video unicast, multicast and broadcast, audio/video streaming | Broadband video typically around 6 Mbps, | ## BER 10-8 | TBD (nominal delay of 90ms, but selectable) (need to specify inter-packet jitter) |  |  |  |
|  | HDTV video unicast, multicast and broadcast, audio/video streaming | HDTV video 19.68 Mbps | ## BER 10-8 | TBD (nominal delay of 90ms, but selectable) (need to specify inter-packet jitter) |  |  |  |
| Privacy | Bulk Data Transfer | Encryption does not affect throughput, except for key management/setup |  |  |  | Support for at least 128 bit keys | In compliance with export laws |
|  | Audio and Video content |  |  |  |  | Preserves copy protection |  |
| Authentication |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |

- Telephony is defined to be real time, interactive conversational mode.

Compatibility and Interoperability Requirements

| Existing 802.11 standard | Must be completely interoperable with existing stations implementing the base 802.11 DCF and PCF functionality. |
| --- | --- |
| Higher layer standards | Provide the necessary support for higher layer Inter Working Functions. (EG end to end connection setup and QoS control) |

### Performance Test Matrix

| Scenario |  | Channel PER |  |  | Overlapped BSS | Output Parameter (top of MAC) |  |  | Baseline MAC performance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | # streams | 0 | 10% | 25% |  | Throughput per stream/application | Latency within stream (MAC to MAC delay) | Jitter within stream |  |
| MPEG1 @2.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 1 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 2 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 25% duty. |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 4 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 2 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 50% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Network Game |  |  |  |  |  |  |  |  |
| MPEG2 @4.5M | 1 |  |  |  |  |  |  |  |  |
| MP3 @ 128K | 2 |  |  |  |  |  |  |  |  |
| Hi Qual Voice @ 64K | 6 |  |  |  |  |  |  |  |  |
| Bursty Data (POP3, WWW) | 4 sessions |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |
| Interactive Multimedia | 1 Video Conference |  |  |  |  |  |  |  |  |
| HDTV @20M | 1 |  |  |  |  |  |  |  |  |
| Bulk Data | 1 file xfer @ 75% duty. |  |  |  |  |  |  |  |  |

# Definition of Class structure:

## Audio

### MP3 (up to 128Kbps)

### Redbook audio (1.45Mbps)

### Dolby Digital, and others  (up to 768Kbps)

## Video

### MPEG1 (up to 3Mbps)

### MPEG2 (4 to 11Mbps)

### HDTV (20Mbps and higher)

## Voice

### High Quality Telephony (32 to 64Kbps)

### Low Quality Telephony (6 to 16Kbps)

## Data

### Bursty (POP3 client, WWW, large numbers of small packets)

### Bulk (File Transfer, multi-megabits of large packets)

## Interactive Multimedia

### Head to Head Games (networked multiplayer games – ex Doom)

### Video Conferencing (H.323, Netmeeting)

# Definition of Source or Service (input pipe)

## 1 to 4MBps (ADSL, WLL)

## 4 to 20MBps (Cable Modem, LMDS)

tbd to tbdMBps (VDSL)

## above 20MBps (enterprise infrastructure, point to point wireless bridging)

# Topology Structures (testing to be performed for each of these 2 cases)

## At least a 5 node network, one BSS.

## Two co-located BSS’s on the same frequency.