IEEE P802.11
Wireless LANs

An estimate of the minimum number of channels for full capacity 54 Mbit/s 802.11a in a dense cellular structured network

Date:	May 9, 2000

Author:	David Skellern
	Radiata Communications Inc.
	Level 2, 1 Julius Avenue
	North Ryde, NSW 2113, Australia
	Phone: +61 2 8874 5404
	Fax: +61 2 8874 5401
	e-Mail: daves@radiata.com

Abstract

An estimate of the minimum number of channels needed to achieve full 54 Mbit/s capacity in a dense cellular structured 802.11a network is obtained under the assumption that the only source of interference is from cochannel use.  More channels will be needed to achieve full capacity in any realistic situation.

# Introduction

This submission draws on the results of previous work on co-channel interference modelling to obtain an estimate of the minimum number of channels needed to support the full rate of 54 Mbit/s in a dense environment of 802.11a networks.  The method is based on modelling the dense environment as a cellular structure with an access point at the centre of each cell. The number of channels needed for full coverage of an area is calculated by considering the carrier to interference ratio constraints in the optimistic case where the only interference arises from the presence of cochannel cells.

The number of channels, N, in this case can be shown to be given by:

N=1ΔαZ1γ+12

where

is the ceiling function

	is a geometry model factor equal to 2 for square cells and 3 for hexagonal cells

	is the strength of the interferer relative to a cochannel interferer (i.e. = 1 for a cochannel interferer)

Z	is the C/I (Carrier to Interference ratio) performance threshold (in linear form)

	is the propagation path loss exponent.

_

# An Estimate of N for the 802.11a 54 Mbit/s Mode

Calculation of the number of channels requires the values of , Z and , all of which could be the subject of long debate and experiment.   is 1 under the assumption of cochannel interference only.

The theoretical value of Z for the 54 Mbit/s mode over an AWGN channel is close to 22dB.  To this must be added some allowance for the more realistic case of a non-Gaussian channel plus an allowance for implementation loss.  A realistic value for Z could be in the range 26 to 30dB (giving Z, as a linear ratio, in the range 398 to 1000).

The range of values for the path loss exponent might typically be from 2 (free space loss and also the limit of an indoor three ray propagation model) to, say, 3.5.  Note that a worse result (ie large number of channels) arises for lower .  An average figure for outdoor use may be under 3.

The value of the geometry factor depends on the environment.  =2 (square cells) is likely choice for indoor use, while =3 may be more realistic for outdoor operation.

Using these values, the minimum number of channels can be calculated to be in the range 533 (square cells, g=2, Z=1000) to 24 (hexagonal cells, g=3, Z=398).  For a mid-range set of parameters, say, g=2.5, D=3, Z=631 (28dB), the number of channels needed is 68.

It should be stressed that these estimates are the MINIMUM number of channels based on consideration of cochannel interference alone.

B C Jones & D J Skellern, “HIPERLAN System Performance Under DCA and FCA”, 8th IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC ’97), 1–4 September 1997, Helsinki, Finland, pp 1216-1220.  (PDF copy at http://www.ics.mq.edu.au/~brendan/phd/publications/pimrc97.pdf)