IEEE P802.11
Wireless LANs

Performance Metrics and Evaluation Criteria for the 802.11-QoS Simulation platform

Date:	June 19, 2000

Author:	Gregory H. Parks,
	Sharewave, Inc
	+1 (916) 939-9400x3211 FAX +1 (916) 939-9437
	e-Mail: Greg.Parks@Sharewave.com

- Version history:

June 5, 2000, Greg Parks: This document currently contains the collective list extracted from doc.: IEEE 802.11-00/125 and obtained during the brainstorm among the participants of phone conference on June 5, 2000 (Peter Eccelsine, Tim Godfrey, Kieth Amann).

June 19, 2000, Greg Parks, revision 1: This document was modified with the collective input obtained during  the brainstorm among the participants of phone conference on June 12, 2000 (Kieth Aman, Greg Chesson).

THIS IS A WORK IN PROGRESS AND BY NO MEANS SHOULD BE CONSTRUED AS COMPLETE.

- Simulation Metrics and Criteria

| AREA | Priority 1 (High) 2 3 4 5 (Low) |  |  |  |  |  |  | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | tbd | tbd | tbd | tbd | tbd | tbd | Overall rate |  |
| # General | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Simulation Model: the model to be used is OpNet based 802.11 model as modified by IEEE 802.11 Simulation ad hoc group and as further modified by each of the members wishing to submit simulation results of their respective proposals. Parameterization detail for the base IEEE 802.11 model will be included in this document. |  |  |  |  |  |  |  |  |
| ## Traffic Generator: the traffic generator to be used in order to create the loading models described in the document are tbd. |  |  |  |  |  |  |  |  |
| ## Measurement Points: the primary measurement point is on the channel using the channel model |  |  |  |  |  |  |  |  |
| # Static Loading Models | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## 11Mbps Model 1: model representing some Enterprise environments: consists of 6 toll-quality voice streams at all times with the addition of bulk data at a rate that achieves 50% and 85% loading of the network |  |  |  |  |  |  |  |  |
| ## 11Mbps Model 2: model representing some Home environments: consists of 3 toll-quality voice streams at all times and 1 toll-quality video streams at all times with the addition of bulk data at a rate that achieves 50% and 85% loading of the network |  |  |  |  |  |  |  |  |
| ## 54Mbps Model 3: model representing some Enterprise environments: consists of 6 toll-quality voice streams at all times with the addition of bulk data at a rate that achieves 50% and 85% loading of the network |  |  |  |  |  |  |  |  |
| ## 54Mbps Model 4: model representing some Home environments: consists of 3 toll-quality voice streams at all times, toll-quality video streams and 1 toll-quality HDTV streams at all times with the addition of bulk data at a rate that achieves 50% and 85% loading of the network |  |  |  |  |  |  |  |  |
| ## NOTE: 50% and 85% loading are defined as the amount of air time occupied |  |  |  |  |  |  |  |  |
| ## NOTE: a question remains as to whether Model 1 and Model 2 represent a rich enough enterprise environment |  |  |  |  |  |  |  |  |
| ## Note: these dynamic loading models are to use the topological models defined in section 4. |  |  |  |  |  |  |  |  |
| ## Model 5 representing some IEEE 1394 environments: consists of tbd (details forthcoming) |  |  |  |  |  |  |  |  |
| # Dynamic Loading Models | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## 11Mbps Model 1: model representing some Enterprise environments: consists of 6 toll-quality voice streams at all times with the addition of bulk data at a rate that achieves 85% loading of the network. Once this load is quiescent at the 85% loading, a 7th toll quality voice stream will be added between the two STAs. |  |  |  |  |  |  |  |  |
| ## 11Mbps Model 2: model representing some Home environments: consists of 3 toll-quality voice streams at all times and 1 toll-quality video streams at all times with the addition of bulk data at a rate that achieves 85% loading of the network. Once this load is quiescent at the 85% loading, a 4th toll quality voice stream will be added between the two STAs. |  |  |  |  |  |  |  |  |
| ## 54Mbps Model 3: model representing some Enterprise environments: consists of 6 toll-quality voice streams at all times with the addition of bulk data at a rate that achieves 85% loading of the network. Once this load is quiescent at the 85% loading, a 7th toll quality voice stream will be added between the two STAs. |  |  |  |  |  |  |  |  |
| ## 54Mbps Model 4: model representing some Home environments: consists of 3 toll-quality voice streams at all times, toll-quality video streams and 1 toll-quality HDTV streams at all times with the addition of bulk data at a rate that achieves 85% loading of the network. Once this load is quiescent at the 85% loading, a 4th toll quality voice stream will be added between the two STAs. |  |  |  |  |  |  |  |  |
| ## Note: the voice stream is added with the intent of discovering (a) whether the connection can be made (b) how long the connection takes to set up and (c) how the change in bandwidth usage affects the rest of the quiescent system. |  |  |  |  |  |  |  |  |
| ## Note: these dynamic loading models are to use the topological models defined in section 4. |  |  |  |  |  |  |  |  |
| # Topological Models | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Node Topology: 3 nodes: 1 AP plus 2 STAs, which will includes streams from and to each STA as well as streams between the two STAs |  |  |  |  |  |  |  |  |
| ## Second topology with larger number of nodes remains open for discussion pending understanding what we would really learn |  |  |  |  |  |  |  |  |
| ## Stream Topology: Topology 1 representing some enterprise environments: consists of 2 toll-quality voice streams between the AP and each STA and 2 toll-quality voice stream between the two STAs. Bulk data will be distributed in a total of six best effort streams between each of the STA pairs. A 3rd toll-quality voice stream is added between the two STAs for dynamic load testing. |  |  |  |  |  |  |  |  |
| ## Stream Topology: Topology 2 representing some home environments: consists of 1 toll-quality voice stream between the AP and each STA and 1 toll-quality voice stream between the two STAs, and 1 toll-quality video or alternatively 1 toll-quality HDTV stream(sbetween the two STA(s) depending on the PHY rate. Bulk data will be distributed in a total of six best effort streams between each of the STA pairs. A 2nd toll-quality voice stream is added between the two STAs for dynamic load testing. |  |  |  |  |  |  |  |  |
| # Physical Layer | XXX | XXX | XXX | XXX | XXX |  | XXX | XXX |
| ## Transmission Rate: Transmission rates will be at the IEEE 802.11b rate of 11Mbps and the IEEE 802.11a rate of 54Mbps |  |  |  |  |  |  |  |  |
| ## Error Rates: the error model and the error rate remain open for discussion |  |  |  |  |  |  |  |  |
| # Throughput | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Throughput will be determined from the data carried during packet rate achieved at defined loadings measured on the channel |  |  |  |  |  |  |  |  |
| # Latency | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Definition: Latency is defined to be the duration between the time a packet is enqueued at the input to the source STA and dequeued at the output from the destination STA |  |  |  |  |  |  |  |  |
| ## Specification for Voice: overall round trip max delay specified by ITU is 300 mS; the budget for the MAC is tbd |  |  |  |  |  |  |  |  |
| ## Specification for MPEG2: |  |  |  |  |  |  |  |  |
| ## Specification for MPEG4 (HDTV) |  |  |  |  |  |  |  |  |
| ## Specification for Bulk Data: |  |  |  |  |  |  |  |  |
| # Jitter | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Definition: Jitter is defined to be the statistical distribution of variation in the arrival time of an individual packet at the destination. Jitter applies largely to constant packet rate traffic, and should be no larger than ½ the time between adjacent packets in a stream assuming the packet under study is centered in time between the two adjacent packets |  |  |  |  |  |  |  |  |
| ## Specification for Voice: |  |  |  |  |  |  |  |  |
| ## Specification for MPEG2: |  |  |  |  |  |  |  |  |
| ## Specification for MPEG4 (HDTV) |  |  |  |  |  |  |  |  |
| ## Specification Bulk Data: |  |  |  |  |  |  |  |  |
| # Traffic Models | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Toll-quality voice: defined using two ADPCM 32Kbit/s streams each consisting of 128 32Byte packets per second (256 Kbit/s duplex) |  |  |  |  |  |  |  |  |
| ## Toll-quality video: defined using one MPEG2 (3 or 4)Mbit/s playback stream consisting of tbd 288Byte packets per second |  |  |  |  |  |  |  |  |
| ## Toll-quality HDTV: defined using MPEG4 one (3 or 4)Mbit/s playback stream consisting of tbd tbdByte packets per second as the base CBR stream with the addition of one (3 or 4) Mbit/s VBR stream characteristics tbd. HDTV may either be this combined peak rate of 6 or 8 Mbps or twice this rate (12 or 16 Mbps): which is is must be defined |  |  |  |  |  |  |  |  |
| ## Bulk data: defined using TCP/IP traffic pattern consisting of 66% 64Byte packets, 20% 1500Byte packets and 14% random sized packets. Rate of packets will be adjusted according to the loading required. |  |  |  |  |  |  |  |  |
| ## NOTE: The size of all packets will be adjusted to take into account the size of the packet in the air for each phy rate thereby accounting for radio preambles, headers and mandatory gaps |  |  |  |  |  |  |  |  |
| # Authentication | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Open Question: does this actually having a bearing on the performance of the MAC? NOTE: the group feels there is no reason why this should be modeled |  |  |  |  |  |  |  |  |
| # Handoff | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Same channel handoff |  |  |  |  |  |  |  |  |
| ## Different channel handoff |  |  |  |  |  |  |  |  |
|  | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| # Priorities | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Open Question: how do we differentiate priorities in the traffic model and how many priorities should be exercised |  |  |  |  |  |  |  |  |
| # Other | XXX | XXX | XXX | XXX | XXX | XXX | XXX | XXX |
| ## Overlapping BSS |  |  |  |  |  |  |  |  |
| ## Dynamic Frequency SelectionNOTE: the group feels there is no need to model this function. |  |  |  |  |  |  |  |  |

| Person | Comments |
| --- | --- |
| tbd | x.x) TBD |
| tbd | x.x) TBD |