|  | INTERNATIONAL TELECOMMUNICATION UNION RADIOCOMMUNICATION STUDY GROUPS | Document 8/58-E 26 February 1999 Original: English |
| --- | --- | --- |

Source:	Document 8A/10(Rev.1)

Working Party 8A

Draft new Recommendation ITU-R M.[8A9B-T4/DD]*, **

characteristics of broadband radio local area networks (rlans)

(Questions ITU-R 212/8 and ITU-R 142/9)

# Summary

This Recommendation provides preferred technical parameters including multiple access and modulation schemes, as well as general guidance for system design of broadband RLANs for mobile applications. Some of them are still under study and will be incorporated in later revisions. The term "broadband" RLAN in this Recommendation means a transmission capacity higher than the order of 10 Mbit/s.

The ITU Radiocommunication Assembly,

considering

a)	that broadband RLANs will be widely used for semi-fixed (transportable) and portable computer equipment for a variety of broadband applications;

b)	that broadband RLAN standards currently being developed will be compatible with current wired LAN standards;

c)	that it is desirable to establish guidelines for broadband RLANs in various frequency bands;

d)	that broadband RLANs should be implemented with careful consideration to compatibility with other radio applications;

e)	that the above guidelines should not limit the effectiveness of broadband RLANs but be used to enhance their development,

recommends

1	that for guidance on preferred methods of multiple access and modulation techniques for broadband RLANs in mobile applications Table 2 can be referred to;

2	that for guidance on broadband RLAN applications currently under development, Table 3 can be referred to;

3	that for guidance on the characteristics of broadband RLANs, Annex 1 can be referred to;

4	that for guidance on modulation schemes using OFDM for broadband RLANs, Annex 2 can be referred to;

5	that for detailed guidance on remote access schemes for RLANs in mobile applications Annex 3 can be referred to;

6	that for other information on RLANs Recommendation refer to Recommendation ITU-R F.1244.

NOTE 1 - Acronyms and terminology used in this Recommendation is given in Table 1.

Table 1

Acronyms and terms used in this Recommendation

| AFC AGA AP ARA ARP ATM BPSK BRAN CCK CSMA/CA DHCP DQPSK DS ETSI FDD FDMA FFT FH FSK FWA GI GMSK HBR IFFT IF IP ISI LBR LMS LSI MAC OFDM PPP PSK QAM QPSK RF RLS SOHO SSMA TCP TDMA TDD WATM | Automatic Frequency Control Automatic Gain Amp Access Point Apple Remote Access Authentication Request Packet Asynchronous Transfer Mode Binary Phase Shift Keying Broadband Radio Networks Complementary Code Keying Carrier Sensing Multiple Access with Collision Avoidance Dynamic Host Configuration Protocol Differential Quaternary Phase Shift Keying Direct Sequence European Telecommunications Standards Institute Frequency Division Duplex Frequency Division Multiple Access Fast Fourier Transform Frequency Hopping Frequency Shift Keying Fixed Wireless Access Guard Interval Gaussian Minimum Shift Keying High Bit Rate HiperLAN 1 for data period only Inverse Fast Fourier Transform Intermediate frequency Internet Protocol Inter Symbol Interference Low Bit Rate HiperLAN 1 for signalling period only Least Mean Square Large Scale Integrated circuits Machine Access Control Orthogonal Frequency Division Multiplexing Point-to-Point Protocol Phase Shift Keying Quadrature Amplitude Modulation Quaternary Phase Shift Keying Radio Frequency Recursive Least Squares Small Office Home Office Spread Spectrum Multiple Access Transmission Control Protocol Time Division Multiple Access Time Division Duplex Wireless Asynchronous Transfer Mode |
| --- | --- |

| Access method Bit rate Channelization Frequency band Modulation Tx power | Scheme used to provide multiple access to a channel The rate of transfer of bit information from one network device to another Bandwidth of each channel and number of channels that can be contained in the RF Bandwidth allocation Nominal operating spectrum of application The method used to put digital information on an RF carrier (Transmitter power) - RF power in watts produced by the transmitter. |
| --- | --- |

TABLE 2

Methods of multiple access and modulation techniques

| Frequency band | Multiple access | Modulation technique |
| --- | --- | --- |
| UHF | CSMA/CA FDMA TDMA SSMA-DS SSMA-FH | CCK (Complementary Code Keying) |
| SHF | CSMA/CA FDMA TDMA-FDD TDMA-TDD TDMA/EY-NPMA | GMSK/FSK BPSK-OFDM QPSK-OFDM 8-PSK-OFDM 16-QAM-OFDM 64-QAM-OFDM |

Table 3

Technical parameters for broadband RLAN applications

| Network standard | IEEE Project 802.11b | IEEE Project 802.11a (NOTE 1) | ETSI BRAN HiperLAN 1 ETS 300-652 | ETSI BRAN HiperLAN 2 (NOTE 1) (NOTE 2) |
| --- | --- | --- | --- | --- |
| Access method | CSMA/CA, SSMA | CSMA/CA | TDMA/EY-NPMA | TDMA/TDD |
| Modulation | CCK (8 complex chip spreading) | 64 QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM | GMSK/FSK | 64-QAM-OFDM 16-QAM-OFDM QPSK-OFDM BPSK-OFDM |
| Data rate | 1, 2, 5.5 and 11 Mbit/s | 6, 9, 12, 18, 24, 36, 48 and 54 Mbit/s | 23 Mbit/s (HBR) 1.4 Mbit/s (LBR) | 6, 9, 12, 18, 27, 36, 48 and 54 Mbit/s |
| Frequency band | 2 400 - 2 483.5 MHz | 5 150 - 5 250 MHz 5 725 - 5 825 MHz 5 250 - 5 350 MHz (NOTE 8) | 5 150 to 5 300 MHz Limited in some countries to 5 150 to 5 250 MHz (NOTE 8) | 5 GHz bands are currently under study in CEPT (NOTE 8) |
| Channelization | 25/30 MHz spacing 3 channels | 20 MHz channel spacing | 23.5294 MHz (HBR) 3 channels in 100 MHz and 5 channels in 150 MHz 1.4 MHz (LBR) | 20 MHz channel spacing 4 channels in 100 MHz |
| Tx power | 1 000 mW e.i.r.p. (NOTE 3) 100 mW e.i.r.p. (NOTE 5) 10 mW/MHz e.i.r.p. density (NOTE 6) | 5 150 to 5 250 MHz 10 mW/MHz 200 mW e.i.r.p. in 20 MHz channel 5 250 - 5 350 MHz 1 W e.i.r.p. 5 725 - 5 825 MHz 4 W e.i.r.p. (NOTE 7) | Three different classes of power levels depending on country administration 1 Watt e.i.r.p., 100 mW e.i.r.p., 10 mW e.i.r.p. (NOTE 4) | Current power limits for various bands are under study in CEPT |
| Sharing considerations | a) CDMA allows orthogonal spectrum spreading b) CSMA/CA provides "listen before talk" access etiquette | a) OFDM provides low power spectral density b) CSMA/CA provides "listen before talk" access etiquette c) In 5 150 - 5 250 MHz e.i.r.p. density limit should be subject to [PDNR M.[Doc. 8A-9B/ TEMP/22(Rev.1)]] | In 5 150 - 5 250 MHz e.i.r.p. density limit should be subject to PDNR M.[Doc. 8A-9B/TEMP/22 (Rev.1)] | a) OFDM provides low power spectral density b) In 5 150 - 5 250 MHz e.i.r.p. density limit should be subject to [PDNR M.[Doc. 8A-9B/TEMP/22 (Rev.1)]] |

NOTE 1 - Common parameters for the physical layer are now under study between IEEE 802.11a and ETSI BRAN HIPERLAN 2. NOTE 2 - WATM (Wireless ATM) and advanced IP with QoS (Ipv^, RSVP) are intended for use over ETSI BRAN HIPERLAN 2 physical transport. NOTE 3 - This requirement refers to FCC 15.247 in the United States. NOTE 4 - Some restrictions on max output power are under study in the band 5 150 - 5 250 MHz within CEPT. NOTE 5 - This requirement refers to EUROPE ETS 300-328. NOTE 6 - This requirement refers to JAPAN MPT ordinance for Regulating Radio Equipment, Article 49-20. NOTE 7 - All values from FCC amendment of the Commission's Rules to Docket No. 96-102 provide for operation of unlicensed NII (RM-8648) devices in the 5 GHz frequency range (RM-865) NOTE 8 - For the band 5 150 to 5 250 MHz, Radio Regulations No. S5.447 applies.

ANNEX 1

General guidance for broadband RLAN system design

# 1	Introduction

Emerging broadband RLAN standards will allow compatibility with wired LANs such as IEEE 802.3, 10BASE-T, 100BASE-T and 51.2 Mbit/s ATM at comparable data rates. Some broadband RLANs have been developed to be compatible with current wired LANs and are intended to function as an wireless extension of wired LANs using TCP/IP and ATM protocols. This will allow operation without the "bottle neck" that occurs with current wireless LANs. Recent bandwidth allocations by some administrations will promote development of broadband RLANs.

A feature provided by broadband RLANs not provided by wired LANs is portability. New laptop and palmtop computers are very portable and have the ability when connected to a wired LAN to provide interactive services. However, when they are connected to wired LANs one loses the portability feature. Broadband RLANs allow portable computing devices to remain portable and operate at maximum potential.

Private on-premise, computer networks are not covered by traditional definitions of fixed and mobile wireless access and should be considered. The nomadic user of the future will no longer be bound to a desk. Instead, they will be able to carry their computing devices with them and maintain contact with the wired LAN in a facility.

# 1.1	Characteristics of broadband RLANs

Speeds of notebook computers and hand held computing devices are increasing steadily. Many of these devices are able to provide interactive communications between users on a wired network but sacrifice portability when connected. Multimedia applications and services require broadband communications facilities not only for wired terminals but also for portable and personal

communications devices. Wired local area network standards, i.e. IEEE 802.3ab 1000BASE-T, are in development that will be able to transport high rate, multimedia applications. To maintain portability, future wireless LANs will need to transport higher data rates. Broadband RLANs are generally defined as those that can provide data throughput greater than 2 Mbit/s.

## 1.2	Mobility

Broadband RLANs may be either pseudo fixed as in the case of a desktop computer that may be transported from place to place or portable as in the case of a laptop or palm top devices working on batteries. Relative velocity between devices remains low. In warehousing applications, RLANs may be used to maintain contact with lift trucks at speeds of up to 6 metres per second. RLAN devices are generally not designed to be used at automotive or higher speeds.

## 1.3	Operational environment and considerations of interface

Broadband RLANs are predominantly deployed inside buildings, in offices, factories, warehouses, etc. For RLAN devices to be deployed inside buildings, emissions will be attenuated by the structure.

RLANs utilize low power levels because of the short distance nature of inside building operation. Power spectral density requirements are based on a basic service area of a single RLAN defined by a circle with a radius from 10 to 50 metres. When larger networks are required, RLANS may be logically concatenated via bridge or router function to form larger networks without increasing their composite power spectral density.

One of the most useful RLAN features is the connection of mobile computer users to their own LAN network without wires. In other words, a mobile user can be connected to its own LAN subnetwork anywhere within the RLAN service area. The service area may expand to other locations under different LAN subnetworks, enhancing the mobile user's convenience.

Annex 2 of this document describes several remote access network techniques to enable the RLAN service area to extend to other RLANs under different subnetworks. Among these techniques, the mobile VLAN technique is a most promising enhancement.

To achieve the coverage areas specified above, it is assumed that RLANs require a peak power spectral density of approximately 12.5 mW/MHz in the 5 GHz operating frequency range. For data transmission, some standards use higher power spectral density for initialization. The required power spectral density is proportional to the square of the operating frequency. The large scale, average power spectral density will be substantially lower than the peak value. RLAN devices share the frequency spectrum on a time basis. Activity ratio will vary depending on the usage, in terms of application and period of the day.

Broadband RLAN devices are normally deployed in high density configurations and use an etiquette such as "listen before talk" and dynamic channel assignment to facilitate spectrum sharing between devices.

## 1.4	System architecture

Broadband RLANs are nearly always point-to-multipoint architecture. Point-to-multipoint applications commonly use omnidirectional, down looking antennas. The multipoint architecture employs two system configurations:

1.4.1	Point-to-multipoint centralized system (multiple devices connecting to a central device or access point (AP) via a radio interface).

1.4.2	Point-to-multipoint non-centralized system (multiple devices communicating in a small area on an ad hoc basis).

1.4.3	Occasionally, fixed point-to-point devices are implemented between buildings in a campus environment. Point-to-point systems commonly use directional antennas that allow greater distance between devices with a narrow lobe angle. This allows band sharing via channel reuse with a minimum of interference with other applications.

ANNEX 2

Preferred modulation techniques in broadband wireless LANs

# 2	Introduction

RLAN systems are being marketed all over the world. There are several major standards for broadband wireless LAN systems. ETSI (European Telecommunications Standards Institute) already developed HiperLAN Type-1 standard. Another discussion is currently very active in IEEE 802.11, which established a RLAN standard for the 2.4 GHz band. These standards will stimulate economical RLAN equipment.

Broadband wireless LAN systems make it possible to move a computer within a certain area such as an office, a factory, and SOHO (Small Office Home Office) with high data rates of more than 20 Mbit/s. As a consequence of the great progress in this field, computer users are demanding free movement with bit rates equivalent to those of conventional wired LANs such as 10BASE-T Ethernet. This new demand raises significant issues of a stable physical layer for broadband radio transmission. There are two major candidates for this purpose: the one is an equalization scheme and the other is a multicarrier scheme.

This document presents features of both schemes and comparison between them. A stable high bit rate, physical layer, which employs DQPSK-OFDM (Orthogonal Frequency Division Multiplexing) with convolutional encoding, is recommended.

## 2.1	Physical layer to realize high bit rate and stable wireless networks

The broadband radio channel is known to be frequency selective, causing Inter Symbol Interference (ISI) in the time domain and deep notches in the frequency domain. To realize a high speed, wireless access system under frequency selective fading channels, a possible method is to shorten the symbol period. A second way is to use bandwidth efficiently by multi-level modulation. The third way is to employ multicarrier modulation. The first and second solutions show serious drawbacks in multipath environments. In the first solution, as the symbol period decreases, ISI becomes a severe problem. Therefore, equalization techniques will be necessary. The second solution reduces the symbol distance in the signal space and hence the margin for thermal noise or interference is decreased, leading to intolerable performance degradation for high speed, wireless access systems. The third solution, the multicarrier method, is to increase the symbol period in

order to compensate for ISI resulting from multi-path propagation. As promising methods for multipath countermeasures, the first solution of single carrier with equalizer and the third solution using multicarrier methods (OFDM) are discussed below.

## 2.2	Single carrier with equalizer

In radiocommunications, the transmission is affected by the time-varying multipath propagation characteristics of the radio channel. To compensate for these time-varying characteristics, it is necessary to use adaptive channel equalization. There are two main groups into which adaptive equalizers can be subdivided; the Least Mean Square (LMS) equalizer and the Recursive Least Squares (RLS) equalizer. The LMS algorithm is the most commonly used equalization algorithm because of its simplicity and stability. Its main disadvantage is its relatively slow convergence. LMS converges in 100 - 1 000 symbols. A faster equalization technique is known as a RLS method. There exist various versions of RLS with somewhat different complexity and convergence trade-off. RLS is more difficult to implement than LMS, but converges in fewer symbols compared with LMS methods. Although much research has been conducted on RLS and MLS equalizers in the cellular systems, RLS and MLS are still a research topic in the points of fast convergence, stability and complexity for high speed wireless access applications.

## 2.3	Multicarrier Orthogonal Frequency Division Multiplexing (OFDM)

With multicarrier transmission schemes the nominal frequency band is split up into a suitable number of sub-carriers each modulated by QPSK modulation, etc. with a low data rate. In general, when dimensioning a multi carrier system, the maximum path delay should be shorter than the symbol time. An OFDM modulation scheme is one of the promising multicarrier methods. The power spectrum of this modulation is shown in Figure 1. The development of fast and power saving Large Scale Integrated circuits (LSI) and effective algorithms (Fast Fourier Transform: FFT) for signal processing today allows a cost-effective realization of OFDM schemes. The advantages of this system are given by a satisfactory spectral efficiency and in the reduced effort for equalization of the received signal. In the case of limited delay spread (<~300 ns) of the multipath signals it is possible to dispense with an equalizer.

The multicarrier transmission scheme employed with OFDM causes envelope fluctuation like additive white Gaussian noise and the effect on the interference environment is negligible.

figure 1

Spectrum of OFDM

## 2.4	Comparison between OFDM and equalizer

As discussed in the IEEE 802.11 working group and ETSI BRAN, the OFDM scheme outperforms the equalizer scheme in the following points:

2.4.1	Hardware complexity of OFDM is lower compared with equalizers to combat with a multi-path-fading channel such as outdoors-wireless environment.

2.4.2	Spectral efficiency of OFDM is better compared to GMSK or Offset QPSK with equalizers.

2.4.3	No equalizer training is needed, saving extra complexity and training overhead.

2.4.4	OFDM can support fallback operation with simple hardware.

2.4.5	Larger diversity gain is achieved compared with equalizer.

## 2.5	Configuration of OFDM system

A simplified block diagram of the OFDM transmitter and receiver is shown in Figure 2. The data to be transmitted are coded by convolutional coding (r=3/4, k=7) and serial-parallel converted and the data modulates the allocated subcarrier by DQPSK modulation. An Inverse Fast Fourier Transform (IFFT) of the modulated sub-symbols generates the OFDM signals. Guard Interval (GI) signals are added to the output signals of the IFFT. The GI added OFDM signals are shaped by roll-off amplitude weighting to reduce outband emission. Finally, the OFDM signals modulate Intermediate Frequency (IF). At the receiver side, received signals are amplified by the Automatic Gain Amp (AGA) and converted to the baseband signals. At this stage, frequency error due to instability of the RF oscillators is compensated by AFC (Automatic Frequency Control) and the timing of packet arrival is detected. After this synchronization processing, the GI signals are removed and the OFDM signals are de-multiplexed by the FFT circuit. The output signals of the FFT circuit are fed to the de-mapping circuit and demodulated. Finally, a Viterbi decoder decodes the demodulated signals.

Figure 2

Configuration of DQPSK-OFDM with convolutional coding

## 2.6	Computer simulation

Major simulation parameters and the OFDM symbol format are shown in Table 3 and Figure 3, respectively. Figure 4 shows that to achieve the packet error rate of 10%, the required Eb/No is about 20 dB under the frequency selective fading channel with 300 ns delay spread. The proposed physical layer approach allows us to use this high bit rate RLAN system not only in indoor areas but also outdoor areas such as universities, factories, and shopping malls, etc.

Table 3

Major simulation parameters

Figure 3

OFDM symbol format

Figure 4

Packet error rate versus Eb/No

## 2.7	Conclusion of Annex 2

This annex presented that OFDM is an promising way to realize a high bit rate (more than 20 Mbit/s) and stable wireless physical layer. IEEE has chosen this OFDM scheme as a physical layer 802.11 TGa and ETSI BRAN HYPERLAN 2.

Annex 3

Remote access techniques in RLANs

# 1	Introduction

One of the most beneficial usages of RLANs is that the RLAN terminals can be used without any additional operation at other company offices where they move. In order to realize such usage, it is very important to establish network techniques to virtually connect the RLAN terminals that are in other offices (other subnetworks) to their own subnetwork.

There are several approaches to support such remote access for RLAN terminals.

In following sections, these techniques will be explained, and compared in the aspects of service performance and system composition.

## 3.1	Remote access techniques

### 3.1.1	Dial-up connection

Currently, the simplest way to connect a terminal from a remote place is a dial-up method. It does not need LAN environments, but is possible wherever telephone network is available, using a modem or an ISDN adapter. Normally, the user sets up a telephone line in his home office, and connects a modem to a dial-up server. A mobile PC with a modem card can be connected to the home network server by a public wired or wireless telephone. In this connection PPP (Point-to-Point Protocol) [1], or ARA (Apple Remote Access) is mainly used.

On the other hand, the dial-up method has the following restrictions:

–	additional software is necessary on mobile terminals;

–	the network interface changes;

–	communication bit rate is low;

–	connection fee is generally expensive.

### 3.1.2	DHCP (Dynamic Host Configuration Protocol)

DHCP [2] is a technique using a new network address at a remote network. DHCP is originally a protocol for the auto-configuration of terminal network interfaces. It enables mobile RLAN terminals to connect to the home network via the Internet by searching for a DHCP server and obtaining a new address.

For DHCP, the following restrictions exist:

–	additional software is necessary on mobile RLAN terminals;

–	only TCP/IP is available;

–	it is unavailable for networks with private IP addresses.

### 3.1.3	Mobile IP

Mobile IP [3] is a technique that supports terminal mobility in networks. In mobile IP, IP packets transmitted to a mobile RLAN terminal are encapsulated by a Home Agent into other IP packets, and are forwarded to the Foreign Agent. In this way, the mobile RLAN terminal can be used at the home network. Because mobile IP works on the Internet, communication cost is low even for international communication.

However, the following are its restrictions:

–	additional software is necessary on mobile RLAN terminals;

–	only TCP/IP is available;

–	it is unavailable for networks with private IP addresses.

### 3.1.4	VLAN (Virtual LAN)

Recent advances in VLAN allow us to construct subnetworks or LAN segments independent of physical network topology, by using switching hubs, ATM switches, or routers. The main purpose of VLAN is to adopt the followings independently of the physical locations:

–	unified administration;

–	security;

–	private IP address or multi-protocol;

–	broadcast.

Some of them allow us to construct wide area VLANs, which are also called Internet VPNs [4]. The wide area VLAN is a very recent technique and the standardization works are now under study in the IETF. In this technique, VLAN functions are necessary on remote network routers, or mobile RLAN terminals themselves.

When the function is on a router, advance registration is necessary. This means that access to Intranet is available only in limited remote networks. When the function is on a mobile RLAN terminal, additional software is necessary.

### 3.1.5	Mobile VLAN

Among the various mobile environment requirements, the mobile VLAN technique [5] was developed to support the following features:

–	low-cost communication;

–	no operation for connection at the RLAN terminal;

–	multi-protocol, private IP address;

–	ubiquitous communication;

–	high security.

In mobile VLAN, the MAC (Medium Access Control) frame transmitted by a mobile RLAN terminal moves to a remote network. Next, it is encapsulated into an IP packet by the server at the remote network. The IP packet is then transferred to its home network (MAC over IP). When the server at the home network de-encapsulates the received IP packet to the original MAC frame. Therefore, the mobile RLAN terminal can use the home network environment at the remote network.

Mobile VLAN has such functions as terminal location registration, address resolution, authentication, and recognition of disconnection. In order to connect with no operation at the RLAN terminal, all of these functions are performed on the network side.

## 3.2	Evaluation

Table 4 summarizes the serviceability of the techniques mentioned above. The mobile VLAN realizes low cost communication, connection with no operation at a RLAN terminal, support for multi-protocols, and ubiquitous communication without losing other technical advantages.

The appendix outlines the mobile VLAN system, which is considered most promising to support RLAN terminal mobility.

Table 4

Comparison of the mobility support techniques

|  | Mobile VLAN | Dial-up connection | DHCP | Mobile IP | Wide area VLAN (in router) |
| --- | --- | --- | --- | --- | --- |
| Transport Network | Internet | PSTN ISDN | Internet | Internet | Internet |
| Communication cost | low | high | low | low | low |
| Network interface modification | no | yes | no | no | no |
| Network address modification | no | no | yes | no | no |
| Additional software on terminal | no | yes | yes | yes | no |
| Multi-protocol | available | unavailable | unavailable | unavailable | available |
| Private IP address | available | available | unavailable | unavailable | available |
| Ubiquitous communication | available | available | available | available | unavailable |

appendix 1 to annex 3

Outline of mobile VLAN system

# 1	System composition

The functions needed for the mobile VLAN techniques are address resolution, terminal authentication, location registration for recognition of disconnection, and MAC frame encapsulation/de-capsulation. The first two factors, i.e. address resolution and terminal authentication, are necessary over the entire network. The location registration function is required only in remote networks. The MAC frame encapsulation/de-capsulation is necessary in both home networks and remote networks. Consequently, the usage of three kinds of servers may be proposed: the Management Server (MS), the Home Server (HS), and the Client Server (CS), as shown in Figure 5. One MS serves the whole network. It manages terminal authentication data and terminal location data, and resolves addresses. One HS is located in one home network, where it encapsulates and forwards MAC frames for mobile terminals. One CS is located in one remote network, where it recognizes mobile terminals, requests terminal authentication to the MS, establishes connection to the HS, and encapsulates MAC frames.

Figure 5

System composition of mobile VLAN

# 2	Major techniques of mobile VLAN

In this section, the major techniques of mobile VLAN are introduced based on sequence charts.

## 2.1	Terminal authentication, location registration, connection

MAC addresses and the corresponding HS IP addresses have to be registered in advance in the MS. IP addresses of all HSs and CSs are also registered. TCP connections to all HSs and CSs are established. The mobile terminal can be connected to remote networks that are connected to the CSs. After connection, when the terminal sends a packet, e.g. an Authentication Request Packet (ARP), the CS captures the packet as a MAC frame. The CS sends the source MAC address to the MS, and the MS authenticates that the terminal is from the corresponding home network.

Upon authentication, the MS registers the terminal location to itself, and notifies the CS and corresponding HS of terminal movement. Then, the CS establishes a TCP connection for MAC frame forwarding to the HS.

Because the destination HS differs depending on the source address of the MAC frame, a CS can belong to many HSs.

Figure 6

Sequence chart for terminal authentication, location registration, and connection

## 2.2	Encapsulation/de-encapsulation

After TCP connection is established, the CS captures MAC frames with source MAC address of the mobile terminal, and the HS captures MAC frames with destination MAC address of the mobile terminal. Then they encapsulate MAC frames into IP packets. If they receive encapsulated MAC frames via the TCP connection, they de-encapsulate them and transmit extracted MAC frames to the LAN. If a MAC frame for another mobile terminal is captured, they encapsulate it again and send it to the corresponding CS. In this way, many CSs can belong to one HS.

Figure 7

Sequence chart for encapsulation/de-encapsulation

## 2.3	Recognition of terminal disconnection

The CS has a timer, and if reception of MAC frames from the mobile terminal stops for a certain period, it recognizes this as disconnection.

Figure 8

Sequence chart for terminal disconnection

References

[1] 	IETF RFC1661, 1548 - The Point-to-Point Protocol, 1994.

[2] 	IETF RFC1541, 1531 - Dynamic Host Configuration Protocol, 1993.

[3] 	IETF INTERNET DRAFT - IP Mobility Support Rev.17, 1996.

[4] 	IETF RFC1701 - Generic Routing Encapsulation, 1994.

_____________

*	 	This Recommendation was jointly developed by Radiocommunication Study Groups 8 and 9, and future revision should be undertaken jointly.

*	* This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 7, and Radiocommunication Study Groups 3 and 4.