IEEE P802.11
Wireless LANs

Inter-Access Point Protocol - 
Centralized Dynamic Route Resolution Protocol over UDP/IP

Date:	July 4, 2000

Author:	Amre El-Hoiydi
	CSEM
	Jaquet-Droz 1, 2000 Neuchâtel, Switzerland
	Phone: +41 32 7205 188
	Fax: +41 32 7205 720
	e-Mail: aeh@csem.ch

Abstract

This document presents a proposal for an inter access point protocol using a centralized route resolution protocol and a frame forwarding protocol over UDP/IP. In order to permit a simple working mode without needing any manual configuration, the access point should run the simple MAC bridging function as long as no route resolution server address has been entered.

1	Introduction	3

2	MAC Bridge Mode	3

3	Dynamic Route Resolution Mode	3

3.1	Network architecture	3

3.2	Route Resolution	4

3.3	Route Update	5

3.4	Reduction of the forwarding traffic	5

4	References	6

# Introduction

This document presents a proposal for the IAPP protocol based on a combination of the options 3 and 4 presented in the paper [1]. The basic idea is to have the two following modes of operation: a MAC bridging mode and a centralized route resolution mode. The MAC bridging mode provides a simple and efficient out-of-box working mode, but suffer from some limitations (maximum frame size and backbone network architecture). These limitations can optionally be overcome by the second mode of operation implementing a distribution system over UDP/IP using a centralized dynamic route resolution protocol.

# MAC Bridge Mode

To permit a mode of operation right out-of-box, i.e. without any needed configuration, the access point should perform the MAC bridging function when no dynamic route resolution server address has been entered. As shown in Figure 1, the integration function (portal function) is present in the access points. There is no need of a distribution function.

Figure 1 MAC Bridge

Advantages:

 This mode of operation is very simple and efficient for any wireless network whose backbone can be limited to a single broadcast medium. The backbone can be composed of multiple Ethernets interconnected by bridges and/or repeaters. No mobility management signalling is needed.

Disadvantages:

 The backbone cannot be composed of LANs interconnected by IP routers.

 MAC frames having the maximum frame body size of 2312 defined in 802.11 cannot be bridged into 802.3. The maximal size of 802.11 MAC frames must be reduced.

# Dynamic Route Resolution Mode

## Network architecture

The network architecture, shown in Figure 2, includes two access points and one portal interconnected by an IP network. The cells covered by the access points (ESS) and the wired LAN to which the portal is connected are bridged together by the distribution system.

MAC

Figure 2 Distribution System over UDP/IP

The 802.11 MAC frames are forwarded between the access points and portal encapsulated into UDP packets. Each access point and the portal maintains a routing table, with entries like as shown in Figure 3:

| Destination MAC address |  | Next Node |
| --- | --- | --- |
| MAC Addr. STA1 |  | IP Addr. AP1 |
| MAC Addr. STA2 |  | IP Addr. AP2 |
| MAC Addr. Desktop Computer 1 |  | IP Addr Portal |

Figure 3 Routing Table

It is proposed that the mobility management protocol used to keep the routing tables up-to-date will use a centralized dynamic routing resolution scheme. This solution is inspired from the protocol used to resolve the virtual path and connection number from the IP address in IP over ATM networks [2]. Compared to a scheme where the routing information is periodically multicast, such a dynamic scheme offers the advantage to scale the mobility management signaling overhead to that needed to react to the mobility of the stations. In a wireless network with stations that are stationary for a while and accessing constant resources (NFS server, mail server, IP gateway), no mobility management signaling will be needed anymore.

The route resolution server (RSS) function must be in one node of the distribution system, for example the portal. The IP address of the route resolution server must be entered in each access point.

## Route Resolution

The route resolution protocol is shown in Figure 4. When an access point receives an association message from a station, it sends a RoutingIndication messages to the RSS and receives a RoutingResponse back. Thank to the RoutingIndication messages sent by the access points, the RSS knows in which BSS each MAC address of the wireless network can be reached. When an access point receives a frame for a station (STA1) with a destination MAC address (STA2) absent from its routing table, it sends a RoutingRequest to the RSS, and receives a RoutingReply back, containing the IP address of the access point to which the MAC frame must be forwarded (AP2). This routing entry is added in the local routing table and the subsequent frames destined to the same MAC address are directly forwarded.

If the RSS (the portal) knows that some MAC address is on the wired LAN, it will respond with a RoutingResponse message giving its own IP address. If the RSS do not know the MAC address for which a route resolution is asked, it will reply with a RoutingResponse message giving its own IP address and indicating that the MAC address is not known. The access point can then forward the packet to the portal anyway, for bridging into the LAN, hoping that the destination MAC address is there.

Figure 4 Route Resolution

## Route Update

When a station moves into the coverage area of another base station, it will send a ReAssociationRequest message (see Figure 5). In order to update the routing tables of all other access points and of the portal, the access point sends a RoutingIndication to the RSS. The RSS, which is the only one to know the IP address of all access points, multicast a RoutingIndication message to all access point, or, in order to reduce the signaling overhead, to only those who previously inquired about the route to reach this moving station. A RoutingResponse acknowledgement of this message is expected by the RSS from the previous access point who was is charge of the station. This acknowledgement is needed to guarantee that the old access point has received the RoutingIndication message. The old access point shall then forward the packets that are still received for that MAC address to the new access point (or possibly discard them). After some time, if the old access point still receives packets for that MAC address, it shall sent a RoutingIndication message to the access point sending these packets to inform it of the new route, assuming that it has not received the initial RoutingIndication message from the RSS.

Figure 5 Route Update

## Reduction of the forwarding traffic

In the network architecture presented in Figure 2, all traffic between a wireless station and a wired computed has to be forwarded over the distribution system encapsulated into UDP/IP packets before to be bridged onto the wired LAN. In a network where most access points and wired computers are on the same broadcast medium (see Figure 6), forwarding the traffic destined to a wired computer to the portal will double the traffic on the wired LAN compared to the bridging solution. An enhancement of the UDP/IP solution could be to use the integration function in each access point in parallel to the distribution function. If the RSS knows that the destination MAC address is in the same wired LAN as the access point, the RoutingResponse message will indicate to the access point to bridge the traffic directly. This knowledge of which MAC address is present on the same LAN can be gained by listening or through active inquires.

Figure 6 Network with most nodes on the same LAN

Comments are welcome at the email: aeh@csem.ch

# References

[1]	Amre El-Hoiydi, "Implementation Options for the Distribution System in the 802.11 Wireless LAN Infrastructure Network", IEEE International Conference on Communications - ICC 2000, New Orleans, Louisiana, USA, June 18-22, 2000.

[2]	M. Laubach, J. Halpern, RFC-2225, "Classical IP and ARP over ATM", April 1998.