IEEE P802.11
Wireless LANs

SMASG Functional Requirements

Recommendations

Date:

October 2, 2000

Author:

Steven D. Gray,  Simon Black plus others to come
Nokia 
Phone: 
Fax: 
e-Mail:

# General Requirements

## A device implementing TGh functionality shall meet the functional requirements of ERC/DEC/(99)23 (IEEE802.11-00/171), "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)"

## No functionality specified by TGh shall cause an existing IEEE 802.11 MAC and IEEE802.11a PHY mechanisms to become non-conformant.

## All devices that implement functionality specified by TGh shall remain compatible with legacy equipment (both APs and stations, and both DCF and PCF modes).

## Changes to the existing standard and new functionality introduced to meet TGh requirements shall be kept to a minimum.

# Transmit Power Control Requirements

## Applicable Regulatory Requirements

### Support transmit power control requirements as defined in ERC/DEC/(99)23, "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)".

## Functional Requirements

### A protocol mechanism shall be defined to allow an Access Point , or members of an IBSS to communicate a per-BSS transmit power level for use by all members of the BSS.

### The supported transmit power levels shall yield a 3 dB power mitigation.

## MAC Requirements

### Frame Definitions

#### Define a new management information element to indicate BSS transmit power level. Modify the definition of Beacon and Probe Response Management Frames to include the BSS transmit power level information element. (?)

#### Define a mechanism to allow transmit power information to be communicated inPCF data frames.

### MAC Protocol Elements

#### Define a mechanism at an STA to allow aCurrentTXPowerLevel to be set from the MLME on receipt of appropriate management information elements.

#### Define the BSS initial power level set at MLME-START.

## PHY Requirements

### Modify the MIB parameter aNumberSupportedPowerLevels to match the number of possible power settings

### Define appropriate power settings for use in the 5470-5725 MHz bands.

# Spectrum Management Requirements

## Applicable Regulatory Requirements

### Support dynamic frequency requirements as defined in ERC/DEC/(99)23, "ERC Decision of November 1999 on the harmonised frequency bands to be designed for the introduction of High Performance Radio Local Area Networks (HIPERLANS)"

## Functional Requirements

### Mechanisms shall be defined to allow an AP and an STA to assess channel state

### Mechanisms shall be defined to allow an STA to report the state of a given channel

### Mechanisms shall be defined to allow an AP to request an STA to report the state of a given channel

### Review and enhance if necessary mechanisms in existing MAC protocol that allow an infrastructure BSS to change channel in a synchronised manner compatible with all other IEEE802.11 MAC management protocols

### Mechanisms shall be defined to allow an STA to measure the state of any specified channel while associated without loss of unicast data.

## MAC Requirements

### Frame Definitions

#### Add a new management frame from the AP to a STA requesting the STA to measure the state or quality of any admissible channel

#### Add a new management frame from an STA to the AP reporting to the requesting AP the state or quality of the requested channel

#### Add a new information element to indicate channel switch information; modify the definition of beacon and probe response to include the new definition.

### General MAC

#### Define constraints for new channel selection algorithm

#### Define new MAC-PHY primitives for communicating channel state data

#### Review and enhance mechanisms if required for BSS frequency agility

## PHY Requirements

### Add channels in the 5470-5725 MHz bands

### Specify mechanisms and PHY primitives for assessing  and communicating channel state to the MAC.