IEEE P802.11
Wireless LANs

| CR for CIDs related to NSTR Capability signaling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2021-02-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dibakar Das | Intel |  |  | dibakar.das@intel.com |
| Laurent Cariou |  |  |  | laurent.cariou@intel.com |
| Minyoung Park |  |  |  | minyoung.park@intel.com |
| Po-kai Huang |  |  |  | po-kai.huang@intel.com |
| Dmitry Akhmetov |  |  |  | dmitry.akhmetov@intel.com |
| Dan Bravo |  |  |  |  |
| Yunbo Li | Huawei |  |  |  |
| Young Hoon Kwon | NXP |  |  |  |
| Zhiqiang Han | ZTE |  |  |  |
| Alfred Asterjadhi | Qualcomm |  |  |  |
| Gaurang Naik |  |  |  |  |
| Payam Torab | Facebook |  |  |  |

Abstract

This document proposes to resolve the following CIDs: 1078, 1475, 2981.

Rev0: initial version

Rev1: accommodate comments from Yunbo and Young Hoon

REV2: accommodate comments from Zhiqiang: clarify the index of the bitmap solution.

Rev6: updates based on comments received during the call

REV10: the signaling for bitmap indication is moved to Per-STA Control field.

| CID | Page | Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1078 | 142 | 29 | 35.3.13.4 | Fix the TBD | Add a (sub)field in the common portion of multi-link element (basic variant) to signal the MLD's STR capabilities with respect to a pair of links | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r10 |
| 1475 | 142 | 29 | 35.3.13.4 | In R1 timeframe an EHT non-AP STA MLD just needs to signal which of its STA/link pairs are STR/NSTR | Allow a non-AP MLD to signal its STR/NSTR capabilities as a bitmap in the Per STA profile of an ML element. | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r10 |
| 2981 | 142 | 29 | 35.3.13.4 | To provide information on NSTR link pair, I think the Multi-Link element is the right place. And to minimize the field length, it may be better to set the maximum number of NSTR link pairs. | As in comment. | Revised. We added a subfield in the Link Info field of Multi-Link element described in 9.4.2.295b Multi-Link element to signal whether each NSTR link pair is STR/NSTR. We also updated the text in 35.3.13.4 Capability signaling to describe usage of this new field. TGbe editor to make the changes with the CID tag (#1078, 1475, 2981) in doc.: IEEE 802.11-21/0222r10 |

Discussion:

All the comments are related about how to signal the NSTR/STR link pair capability which is currently missing in draft 0.3. We propose the following simplified signaling:

- Have a bit in Per-STA Control field to signal if there is at least one pair of NSTR link pairs formed by this link.

- If that bit is set to 1, include a bitmap inside the corresponding per-STA Profile that contains the STR/NSTR link pair signaling. Basically, each STA profile included in the Basic ML element will contain a bitmap that signals the STR/NSTR indication for each pair of links of which that STA Profile (/link) is a member. The bit positions are indexed based on the relative link IDs.

- The Common Info field contains a subfield representing #of radios (“ Maximum Number Of Simultaneous Links” in Figure 9-788ex) which is set to greater than or equal to 1 for a multi-radio MLD.

- The Per-STA Control field contains a subfield indicating whether the bitmap is 1 or 2 octets long. Note that per the format of Link ID field, there can be up to 15 links overall.

- The signaling is symmetric that is, if link pairs (1,2) are NSTR, then (2,1) is clearly NSTR as well.

TGbe editor: Revise Figure 9-788eg and the text starting in P115L32 of draft 0.4 as follows:

B0                Bx           Bx+1                       Bx+2          Bx+3  B15

| Type | MLD MAC Address Present | MLD Capabilities Present | Reserved |
| --- | --- | --- | --- |

Bits:                x+1                     1                                   1                   13-x

Figure 9-788eg—Multi-Link Control field (#1078, 1475, 2981)

The MLD MAC Address Present subfield is set to 1 if the MLD MAC Address field is present in the Common Info field. Otherwise, the subfield is set to 0.

The MLD Capabilities Present subfield is set to 1 if the MLD Capabilities subfield is present in the Common Info field. Otherwise, the subfield is set to 0 (#1078, 1475, 2981).

TGbe editor: Revise the text starting in P116L6 of draft 0.4 as follows:

9.4.2.295b.2 Basic variant Multi-Link element (#1078, 1475, 2981)

The condition for the presence of the MLD Capabilities subfield in the Common Info field is defined in 35.3.13.4 (Capability signaling).

TGbe editor: Revise Figure 9-788ej and modify the text starting in P116L47 of draft 0.4 as follows:

The format of the Per-STA Control field is defined in Figure 9-788ej (Per-STA Control field format).

B0              B3     B4                  B5                     B6             B7             Bx

| Link ID | Complete Profile | NSTR Link Pair Present | NSTR Bitmap Size | Reserved |
| --- | --- | --- | --- | --- |

Bits:                   4                1                            1                      1                x-7

Figure 9-788ej—Per-STA Control field format

The Link ID subfield specifies a value that uniquely identifies the link where the reported STA is operating on.

The Complete Profile subfield is set to 1 when the Per-STA Profile subelement of the Multi-Link element is complete as defined in 35.3.2.2 (Complete or partial per-STA profile). Otherwise the subfield is set to 0.

If the value of the Maximum Number Of Simultaneous Links subfield in the MLD Capabilities field is greater than 0, the NSTR Link Pair Present subfield in a Per-STA Control field indicates if at least one NSTR link pair is present in the MLD that contains the link corresponding to that STA. It is set to 1 if there is at least one such link pair; otherwise,

If the Complete Profile subfield is equal to 1 and the NSTR Link Pair Present subfield is equal to 1 in the Per-STA Control field, then each Per-STA Profile subelement contains an NSTR Indication Bitmap field whose size is indicated by the NSTR Bitmap Size subfield; otherwise, the NSTR Indication Bitmap field is not present. The NSTR Bitmap Size subfield in a Per-STA Control field is set to 1 if the length of the corresponding NSTR Indication Bitmap subfield is 2 octets and is set to 0 if the length of the corresponding NSTR Indication Bitmap field is 1 octet. The NSTR Indication Bitmap subfield in a Per-STA Control field is reserved if the NSTR Link Pair Present subfield in that field is 0.

Each bit Bj  (j≠i) in the NSTR Indication Bitmap subfield included in the Per-STA Profile subelement with Link ID subfield equal to i  (where 0 ≤i <15) is set to 1 if the link pair corresponding to link IDs equal to i and j is NSTR and the Basic variant Multi-Link element contains a Per-STA Profile subelement with link ID j; otherwise it is set to 0. Bit Bi in the NSTR Indication Bitmap subfield included in the Per-STA Profile subelement with Link ID subfield equal to i is reserved.

TGbe editor: Add the following text before the paragraph starting in P211L4 of draft 0.4 as follows:

35.3.13.4 Capability signaling (#1078, 1475, 2981)

An MLD shall set the MLD Capabilities Present subfield in the Multi-Link Control field of the Basic variant Multi-Link element to 1 when carried in a (Re)Association Request frame or (Re)Association Response frame.

An MLD shall set the MLD Capabilities Present subfield in the Multi-Link Control field of the Basic variant Multi-Link element to 0 when carried in an Authentication frame.

A non-AP MLD shall set the NSTR Link Pair Present subfield value to 1 in a Per-STA Control field that corresponds to Link ID i (where 0 ≤i <15) only if it is a multi-radio MLD and the corresponding non-AP STA has at least one NSTR link pair formed by the link with Link ID i; otherwise, it shall set the subfield value to 0. An AP MLD shall set the NSTR Link Pair Present subfield value in each Per-STA Control field to 0.

TGbe editor: Add the following paragraph in P209L62 of draft 0.4 as follows:

35.3.13.2 Simultaneous transmit and receive (STR) operation

An MLD shall be capable of STR over any two pairs of links where one link operates in the 2.4 GHz band and the other operates in the 5 GHz or higher frequency bands.