IEEE P802.11
Wireless LANs

| 802.11 REVmd Proposed Resolutions for 11aj related comments in REVmd LB236 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-11-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Shiwen He | SEU |  |  | shiwenhe@seu.edu.cn |
| Jiamin Chen | Hisilicon |  |  | Jiamin.chen@mail01.huawei.com |

Abstract

This document contains proposed resolutions to some 11aj related comments in REVmd LB236

CMMG related comments:

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2048 | 25.12.1 | 3517 | 53 | T | The transmit procedure ignore (Mandatory) AGC and TRN fields | Indicate that the transmission does not end at the last bit of the PSDU if AGC and TRN are appended |  |

Proposed resolution: Revised

A new is added at the end of L55, P. 3517 in Clause 25.12.1, i.e., “If the value of Training Length in CMMG SIG field is nonzero, the transmission does not end at the last bit of the PSDU that followed by AGC and TRN subfields.”

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2024 | 25.3.2.3.6 | 3454 | 54 | T | Receiver Sensitivity for MCS 0 is missing from table 25-2 | Add receiver Sensitivity for MCS 0. I believe is should be around -81 and -78 |  |

Proposed resolution: Revised

A new line is inserted in Table 25-2 to indicate the Receiver Sensitivity for MCS 0.

Table 25-2—Receiver sensitivity

| MCS | CBW 540 MHz | CBW 1080 MHz |
| --- | --- | --- |
| 0 | -81 | -78 |
| 1 | -74 | -71 |
| … | …. | …. |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2027 | 25.3.6 | 3464 | 54 | T | $\sum_{i=N-K}^{N-1}D^i$ - it would be nice to add at the end of the sentence that this is equivalent to setting the shfit register bits to 1. | Add at the end of the sentence ". (This is equivalent to setting the shift register to all ones)." |  |

Proposed resolution: Revised

The sentence in L. 54, P. 3464, Clause 25.3.6 is revised as “ are initialized values that are added modulo 2 to the first K bits of the input bit sequence. This is equivalent to setting the shift register to all ones.”

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2029 | 25.3.13 | 3472 | 59 | T | This text follows the RCPI clause as it was in the original 11ad standard. Over time the text in 20.3.10 changed. I prefer that the text in 20.3.10 alligns with the text here, the group may decide otherwise | allign the text in 25.3.13 with other RCPI subclauses |  |

Proposed resolution: Revised

The contents of 25.3.13 is revised as follows:

25.3.13 Received channel power indicator (RCPI) measurement

The RCPI is a measure of the received RF power in the selected channel. This parameter shall be measured by the PHY of the received RF power in the channel measured over the data portion of the received frame.

The RCPI encoding is defined in 15.4.6.6 (Received Channel Power Indicator Measurement).

RCPI shall equal the received RF power with an accuracy of ± 5 dB (95% confidence interval) within the

specified dynamic range of the receiver. The received RF power shall be determined assuming a receiver

noise equivalent bandwidth equal to the channel width multiplied by 1.1. The relative error between RF

power measurements made within a 1 second interval should be less than ± 1 dB.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2032 | 25.2.2 | 3450 | 59 | T | "SPREADING_FACTOR" is missing from the TXVECTOR and RXVECTOR | Add a parameter "SPREADING_FACTOR" to the TX/RXVECTOR representing the equivalent CMMG field |  |

Proposed resolution: Revised

A new line before the last second line is added in Table 25-1—TXVECTOR and RXVECTOR parameters(11aj) to explain the parameter "SPREADING_FACTOR"

Table 25-1—TXVECTOR and RXVECTOR parameters(11aj)

| Parameter | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- |
| … | … | … | … |
| LAST_RSSI | In the TXVECTOR, LAST_RSSI indicates the received power level of the last packet with a valid PHY SIG that was received an SIFS period before transmission of the current packet; otherwise, it is 0 (see 10.3.2.3.3 (SIFS)). In the RXVECTOR, LAST_RSSI indicates the value of the LAST_RSSI field from the PCLP header of the received packet. Valid values are integers in the range from 0 to 15: - Values of 2 to 14 represent power levels (–71+value×2) dBm. - A value of 15 represents power greater than or equal to –42 dBm. - A value of 1 represents power less than or equal to –68 dBm. - A value of 0 indicates that the previous packet was not received an SIFS period before the current transmission. | Y | Y |
| SPREADING_FACTOR | Indicate the spread factor used in the Control mode | Y | Y |
| Turnaround | Set to 1 or 0 as specified in 10.3.2.3.3 (SIFS). | Y | Y |
| NOTE—In the “TXVECTOR” and “RXVECTOR” columns, the following apply: Y = Present N = Not present O = Optional |  |  |  |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2036 | 25.3.5.3.1 | 3461 | 55 | T | Figures 25-4, 25-5, and 25-6 show sequences of length 256 used in the preamble while the sequences are actually of length 32 | Fix figures 25-4, 25-5, 25-6 |  |

Proposed resolution: Revised

Figure 25-4 to 25-8 are revised as follows:

Figure 25-4—Control mode preamble

Figure 25-5—CMMG SC mode preamble for CBW 540 MHz

Figure 25-6—CMMG SC mode preamble for CBW 1080 MHz

Figure 25-7—CMMG OFDM mode preamble for CBW 540 MHz

Figure 25-8—CMMG OFDM mode preamble for CBW 1080 MHz

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2037 | 25.3.5.3.1 | 3461 | 51 | T | "duplicate style" - I don't understand what does that mean. | Add a link to 25.3.10 |  |

Proposed resolution: Revised

The NOTE in L. 51 p. 3461 Clause 25.3.5.3.1 CEF of control mode is revised as:

NOTE—All fields are transmitted with SC mode transmission. For 1080 MHz SC transmission, the STFs, the CEFs, and the SIG fields are transmitted with duplicated style 25.3.10 (Duplication transmission on a 1080 MHz channel).

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2044 | 25.3.9.1 | 3469 | 17 | T | Add a link to the MCS table because it is far away and hard to find | As in comment |  |

Proposed resolution: Revised

The description in L. 17 P. 3469 in Clause 25.3.1 is revised as:

Table 25-7—Fields in the CMMG SIG field

| Bit | Field | Number of bits | Description |
| --- | --- | --- | --- |
| … | … | … | … |
| B50–B54 | MCS | 5 | Index into the modulation and coding scheme in 25.15 *Parameters for CMMG MCSs). |
| … | … | … | … |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2045 | 25.7.2.3 | 3509 | 17 | T | "A value of 0├ù0 in the PPDU Type(#1379) field indicates a BRP-RX PPDU" The text here contadicts the text in 25.3.9.1 (CMDG SIG). The value used there is "10" or 0x2 | Change 0x0 to 0x2 or preferably "0b10" |  |

Proposed resolution: Revised

The corresponding description is revised as:

25.7.2.3 BRP packet SIG fields

A value of 2 in the Packet Type field indicates a BRP-TX packet (TRN-T field is present).

A value of 3 in the Packet Type field indicates a BRP-RX packet (TRN-R field is present).

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | 25.2 | 3452 | 50 | T | "RX_START_OF_FRAME_OFFSET" - using units of 10 nanosecond does make sense when the sampling rate is ~1GHz | replace 10 nanosecond with 100picosecond |  |

Proposed resolution: Revised

In L. 50 P. 3452 Clause 25.2, 10 nanosecond is replaced with 100 picosecond.

Table 25-1—TXVECTOR and RXVECTOR parameters

| Parameter | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- |
| … | … | … | … |
| RX_START_OF_FRAME_OFFSET | 0 to 232–1. An estimate of the offset (in 100 picosecond units) from the point in time at which the start of the preamble corresponding to the incoming frame arrived at the receive antenna port to the point in time at which this primitive is issued to the MAC. | Y | Y |
| … | … | … | … |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2047 | 25.7.2.4 | 3510 | 1 | T | "Any transmit signal transients that occur due to this TX AWV configuration change" - which TX AWV configuration change? None were discussed in this sub clause | Move this sentence after the first sentence of the next paragraph |  |

Proposed resolution: Rejected

This description and the place are similar with that in 20.9.2.2.5 AGC field.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2542 | 25 |  |  | T | A non-directional multi-gigabit PHY was not seen as necessary/practicable in the 60G band, so doesn't seem necessary/practicable in the 55G band | Delete Clause 25 |  |

Proposed resolution: Rejected

Though multiple gigahertz bandwidth can be supported in millimetre frequency band, it is still hard to realize the goal of transmission data rate of tens of gigabits. Multiple input multiple output is extensively regarded as a powerful technology to realize high transmission data rate transmission. The technology adopted Clause 25 can support simultaneously directional multi-gigabit PHY and non-directional multi-gigabit PHY. Furthermore, it can realize the goal of transmission data rate of tens of gigabits and is adopted by the emerging standard IEEE 802.11ay.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2049 | 25.13.1 | 3521 | 6 | T | The receive procedure points to a receive state machine in figure 25-38, however, figure 25-38 is the PHY receive procedure. There is no receive state machine. | Add a receive state machine. |  |

Proposed resolution: Revised

The sentence in L. 6-7 P. 3521 Clause 25.13.1 is revised as “A typical state machine implementation of the receive PHY is given in Figure 25-39. This receive procedure and state machine do not describe the operation of optional features, such as STBC.”

The typical Rx state machine figure is added and is numbered as “Figure 39 Typical Rx state machine”.

Figure 39 Typical Rx state machine

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2126 | 25.3.3 | 3456 | 6 | T | How is Table 22-8 related to this parameter? I think it should be Table 21-5 Timing-related constants. | At cited location, change "Table 22-8" to "Table 21-5 Timing-related constants". |  |
| 2355 | 25.3.3 | 3456 | 6 | T | T_SYML can obviously not be T_SYML or T_SYMS | Change the entry in the Parameter cell to "T_SYM" |  |

Proposed resolution: Revised

The description in L. 6 P. 3456 Clause 25.3.3 is typos and is corrected as follows:

Table 25-3—Time-related parameters

| Parameter | CBW 540 MHz | CBW 1080 MHz | Description |
| --- | --- | --- | --- |
| … | … | … | … |
| TSYM | TSYML or TSYMS depending on the used GI |  | Symbol interval |
| … | … | … | … |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2127 | 25.6.8.11 | 3502 | 57 | T | There are two definitions of TGI: 3502.50 and 3502.57. | Removed the second definition. |  |

Proposed resolution: Revised

The second definition of the TGI is removed.

TGI is the guard interval duration

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2128 | 25.6.8.12 | 3504 | 27 | T | There are two definitions of TGI: 3504.20 and 3504.27. | Removed the second definition. |  |

Proposed resolution: Revised

The second definition of the TGI is removed.

TGI is the guard interval duration

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2373 | 25.5.3.2 | 3478 | 12 | T | "This is need only for the CMMG SC mode MIMO mechanism." is not grammatical, and is not clear either | Change the cited text at the referenced location to "This only applies to the CMMG SC mode MIMO mechanism." |  |

Proposed resolution: Revised

The revised description is cited as:

25.5.3.2 Construction of CMMG SC mode SIG

g) CSD: Apply CSD for each transmit chain as described in 25.5.5.4.5 (Symbol blocking, CSD, and GI insertion). This only applies to the CMMG SC mode MIMO mechanism.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2374 | 25.6.4.2 | 3490 | 12 | T | "This is need only for the CMMG OFDM MIMO mode mechanism." is not grammatical, and is not clear either | Change the cited text at the referenced location to "This only applies to the CMMG OFDM mode MIMO mechanism." |  |

Proposed resolution: Revised

The revised description is cited as:

25.6.4.2 Construction of CMMG OFDM mode SIG

f) CSD: Apply CSD for each transmit chain as described in 25.5.5.4.5 (Symbol blocking, CSD, and GI insertion). This only applies to the CMMG OFDM mode MIMO mechanism.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2375 | 25 |  |  | T | The term "MIMO mode" is used but such a mode is not described | Change "MIMO mode" to "MIMO" throughout |  |

Proposed resolution: Revised

The term “MIMO mode” is replaced with “MIMO” throughout Clause 25.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2387 | 25.3.9.1 | 3467 | 41 | T | Table 25-7---Fields in the CMMG SIG field needs the same changes as made under CID 1351. However Assaf reports that it "requires (a lot of) more work because the scrambling is not mentioned in the encoding process." | Ask Assaf to kindly to the more work |  |

Proposed resolution: Rejected

The specific scrambling is not need to be described detailed in the standard.

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2046 | 25.7.2.3 | 3509 | 41 | T | "A value of 0├ù1 in the PPDU Type(#1379) field indicates a BRP-TX PPDU" The text here contradicts the text in 25.9.3.1 (CDMG SIG) The value used there is "11" or 0x3 | Change 0x1 to 0x3 or preferably to "0b11" |  |

Proposed resolution: Revised

The corresponding description is revised as:

25.7.2.3 BRP packet SIG fields

A value of 2 in the Packet Type field indicates a BRP-TX packet (TRN-T field is present).

A value of 3 in the Packet Type field indicates a BRP-RX packet (TRN-R field is present).

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2413 | 9.4.1.61 | 960 | 23 | T | " in a matrix" -- need to say which matrix | Change "in a matrix" to "in the compressed beamforming feedback matrix". Also in Table 9-57 |  |
| 2413 | 9.4.1.61 | 960 | 18 | T | " in a matrix" -- need to say which matrix | Change "in a matrix" to "in the compressed beamforming feedback matrix". Also in Table 9-57 |  |

Proposed resolution: Revised

The corresponding description is revised as:

Table 9-57—Subfields of the MIMO Control field

| Subfield | Description |
| --- | --- |
| Nc Index | Indicates the number of columns in the compressed beamforming feedback matrix minus 1: Set to 0 for Nc = 1 Set to 1 for Nc = 2 Set to 2 for Nc = 3 Set to 3 for Nc = 4 |
| Nr Index | Indicates the number of rows in the compressed beamforming feedback matrix minus 1: Set to 0 for Nr = 1 Set to 1 for Nr = 2 Set to 2 for Nr = 3 Set to 3 for Nr = 4 |

Table 9-87—Subfields of the CMMG MIMO Control field

| Subfield | Description |
| --- | --- |
| Nc Index | Indicates the number of columns in the compressed beamforming feedback matrix minus 1: Set to 0 for Nc = 1 Set to 1 for Nc = 2 Set to 2 for Nc = 3 Set to 3 for Nc = 4 |
| Nr Index | Indicates the number of rows in the compressed beamforming feedback matrix minus 1: Set to 0 for Nr = 1 Set to 1 for Nr = 2 Set to 2 for Nr = 3 Set to 3 for Nr = 4 |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2031 | 10.6.8.1 | 1759 | 25 | T | This subcluases discusses the use of the CMMG Control Modulation class but ignores the fact that MCS 0 has 4 different spreading factors - resulting in different rates. I believe the choice of spreading factor should not be left to the implementor | Add at the end of the first sentence that the spreading factor should be set 1 |  |

Proposed resolution: Revised

The first sentence in the L. 25, P. 1759 in Clause 10.6.8.1 is rewritten as:

“The CMMG Control modulation class has only one MCS, which is CMMG MCS 0 defined in Clause 25 (China millimeter-wave multi-gigabit (CMMG) PHY specification) with the spreading factor should be set to 13.”

CDMG related comments:

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2678 | 9.6.21.3 | 1629 | 17 | T | The CDMG specific IE in the BRP and other frames are presented in case the interacting STA are CDMG capable and are not present overwise. It shall be clearly stated in the definition of the frame. | Add to the definition of the elements: "The IE is optionally present if the frame is transmitted by the STA and to the STA that is CDMG capable as indicated by the presence of the CDMG Capabilities element(9.4.2.21) in the Beacon, Probe Request, and Probe Response, Association Request and Association Response frames the STA last transmitted" |  |

Proposed resolution: Revised

Accept in principle, propose the changes as follows:

“9.4.2.220 Dynamic Bandwidth Control element

The Dynamic Bandwidth Control element defines the information from the CDMG AP or PCP to support the dynamic bandwidth control mechanism described in 10.65 (DBC mechanism for CDMG STAs(11aj)) and the CDMG AP or PCP clustering mechanism described in 10.65 (DBC mechanism for CDMG STAs(11aj)). The Dynamic Bandwidth Control element is included in a DMG Beacon, Announce, or Notification Period Response frame transmitted to the peer STA and the AP or PCP of a BSS. The Dynamic Bandwidth Control element is present in a DMG Beacon, Announce, or Notification Period Response frame transmitted by a CDMG AP or PCP to one or more CDMG STAs. The format of the Dynamic Bandwidth Control element is shown in Figure 9-726 (Dynamic Bandwidth Control element format(11aj)).

…”

“9.4.2.221 CDMG Extended Schedule element(11aj)

The CDMG Extended Schedule element is defined in Figure 9-728 (CDMG Extended Schedule element format(11aj)). Like the Extended Schedule element (9.4.2.229 (CMMG Capabilities element(11aj))), the AP or PCP can split the Allocation fields in the CDMG Extended Schedule element into more than one CDMG Extended Schedule element entry in the same DMG Beacon frame or Announce frame. Despite this splitting, the set of CDMG Extended Schedule element entries conveyed within a DMG Beacon or Announce frame is considered to be a single schedule for the beacon interval and, in this standard, is referred to simply as CDMG Extended Schedule element unless otherwise noted. The Allocation fields are ordered by increasing allocation start time with allocations beginning at the same time arbitrarily ordered. The CDMG Extended Schedule element is present in a DMG Beacon frame or Announce frame transmitted by a CDMG AP or PCP to one or more CDMG STAs.

…”

“9.4.2.222 SSW Report element(11aj)

The SSW Report element is used by a CDMG non-AP and non-PCP STA to report beamforming training information to a CDMG AP or PCP (see 11.31.1 (General)) or used by a responder STA to deliver the backup sector information to an initiator STA. The format of the SSW Report element is as illustrated in Figure 9-732 (SSW Report element format(11aj)).

…”

“9.4.2.224 Extended Cluster Report element(11aj)

The Extended Cluster Report element is used to report the cluster synchronization and control information to the cluster probe requesting CDMG AP or PCP by the S-AP or S-PCP of a CDMG AP or PCP cluster. The Extended Cluster Report element is also used by an S-AP to report the cluster information of the S-Aps within the same CCSS for an AP or PCP that intends to join the centralized cluster. This element can be included in a DMG Beacon, Announce, or Probe Response frame transmitted by the CDMG AP or PCP to the CDMG S-AP or S-PCP. The Extended Cluster Report element is shown in Figure 9-736 (Extended Cluster Report element format(11aj)).

…”

“9.4.2.225 Cluster Switch Announcement element(11aj)

A CDMG AP or PCP transmits the Cluster Switch Announcement element to its original cluster before switching from a cluster to another. One of the synchronization pair APs or PCPs also transmits the Cluster Switch Announcement element to its peer CDMG AP or PCP before joining a cluster. The Cluster Switch Announcement element can be included in the DMG Beacon frame or Announce frame transmitted by a CDMG AP or PCP and received by other member CDMG Aps or member CDMG PCPs. The format of the Cluster Switch Announcement element is shown in Figure 9-737 (Cluster Switch Announcement element format(11aj)).

…”

“9.4.2.226 Enhanced Beam Tracking element(11aj)

The Enhanced Beam Tracking element is used to configure the alternative link of a CDMG STA, track the alternative link of the enhanced beam tracking initiator and responder, and switch to an alternative link from the current link. The element can be included in a BRP, DMG Beacon, Information Request, or Information Response frame transmitted by a CDMG STA to one or more CDMG STAs.

…”

“9.4.2.227 SPSH Report element(11aj)

The SPSH Report element is used for an AP or PCP in a cluster to report the possibility of spatial sharing and coexistence between links in other BSSs and links in its own BSS. The SPSH Report element is transmitted in the DMG Beacon frame by a CDMG AP or PCP to other CDMG STAs and is formatted as illustrated in Figure 9-741 (SPSH Report element format(11aj)). Because the length parameter supports only 255 octets of payload in an element, the AP or PCP can split the SPSH List fields into more than one SPSH Report element entry in the same DMG Beacon or Announce frame.

…”

“9.4.2.228 Clustering Interference Assessment element(11aj)

The S-AP or S-PCP in a CDMG AP or PCP cluster uses a Clustering Interference Assessment element to

enable spatial sharing in the cluster and sets channel quality measurement and control parameters of spatial sharing for their member APs or member PCPs. The Clustering Interference Assessment element can be included in the DMG Beacon frames transmitted by the CDMG S-AP or S-PCP to their member CDMG AP or PCP, as shown in Figure 9-743(Clustering Interference Assessment element format(11aj)).

…”

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2120 | 9.6.21.2 | 1628 | 50 | T | Rows "23/Cluster Probe", "24/Extended Cluster Report", and "25/Cluster Switch Announcement" are incomplete. | Fill in the third column "Notes" for cited rows. |  |

Proposed resolution: Revised

Change Table 9-485 as follows:

| Table 9-485—Announce frame Action field format |  |  |
| --- | --- | --- |
| Order Information Notes | Order Information Notes | Order Information Notes |
| … | … | … |
| 23(11aj) | Cluster Probe | The Cluster Probe element is defined in 9.4.2.223 (Cluster Probe element). The Cluster Probe element is optionally present. |
| 24(11aj) | Extended Cluster Report | The Extended Cluster Report element is defined in 9.4.2.224 (Extended Cluster Report). The Extended Cluster Report element is optionally present. |
| 25(11aj) | Cluster Switch Announcement | The Cluster Switch Announcement element is defined in 9.4.2.225 (Cluster Switch Announcement element). The Cluster Switch Announcement element is optionally present. |
| … | … | … |

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2602 | 24.4.3.2.1 | 236 | 2 | T | Table 24-6 should be aligned with Table 20-11 (DMG v. CDMG) | As it says in the comment |  |

Proposed resolution: Reject

Table 20-11—DMG control mode header fields

…

Compared to Table 20-11, two reserved bits in Table 24-6 are used as field “Robust PHY Mode Indication/Reserved bits” to indicate the Robust PHY Mode, which is introduced in 802.11aj to enlarge the data PHY coverage for small antenna array mobile devices. Therefor Table 24-6 cannot be aligned with Table 20-11. More information can be found in the following documents regarding technical proposal of the robust PHY mode and the discussion about backward compatibility issue between DMG and CDMG STAs.

https://mentor.ieee.org/802.11/dcn/16/11-16-0456-00-00aj-overview-of-802-11aj-60ghz-and-its-backward-compatibility-features.ppt

https://mentor.ieee.org/802.11/dcn/16/11-16-0719-01-00aj-proposed-resolution-to-cid-100-101-102-etc-in-lb217.docx