| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Replacement Security Text |  |
| Date Submitted | January 19, 2005 |  |
| Source | René Struik Certicom Corp. 5520 Explorer Drive, 4th Floor Mississauga, ON, Canada L4W 5L1 | Voice: +1 (905) 501-6083 Fax: +1 (905) 507-4230 E-mail: rstruik@certicom.com |
| Re: | IEEE 802.15.4-2003 |  |
| Abstract | This document provides an overview of replacement security text (rough draft!). |  |
| Purpose | Facilitate adoption and incorporation of security improvements to the current IEEE 802.15.4-2003 specification. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

- Table of contents

Table of contents	2

1	References to security comments on 802.15.4-2003 and its predecessors	3

2	Replacement Security Text	4

2.1	Security-Related PIB Attributes	4

3	Frame Security	7

3.1	Auxiliary Frame Header Format	7

3.1.1	Frame counter field	7

3.1.2	Security control field	7

3.1.3	Key Identification Field	9

3.1.4	Constructing the Processed Auxiliary Frame Header	9

3.2	Security Parameters	10

3.2.1	CCM* Mode of Operation and Parameters	10

3.2.2	Security Material	10

3.2.3	CCM* Nonce	10

3.3	Security Processing Steps	11

3.3.1	Outgoing Frame Operations	11

3.3.2	Incoming Frame Operations	12

4	Security Status Codes	15

4.1	Status Values	15

ANNEX A – Basic notions	16

A.1.1	Strings and string operations	16

A.1.2	Integers, octets, and their representation	16

A.1.3	Entities	16

ANNEX B – CCM* mode of operation	17

ANNEX C – Security Building Blocks	18

C.1.1	Block-cipher	18

C.1.2	Mode of operation	18

ANNEX D – References	19

- References to security comments on 802.15.4-2003 and its predecessors

IEEE 802.15.4 security documents (submitted by René Struik):

November 14, 2002:

02474r0P802-15_TG4-Security-and-Security-Architectural-Recommendations-for-the-IEEE-802.15.4-Low-Rate-WPAN

November 14, 2002:

02474r1P802-15_TG4-Security-and-Security-Architectural-Recommendations-for-the-IEEE-802.15.4-Low-Rate-WPAN

November 15, 2002:

02468r0P802-15_TG4-Draft-D17-Clause 7.6-Security-Recommendation-for-IEEE-802.15.4-Low-Rate-WPAN

November 15, 2002:

02469r0P802-15_TG4-Draft-D17-Annex B-Security-Recommendation-for-IEEE-802.15.4-Low-Rate-WPAN

January 16, 2003:

02474r2P802-15_TG4-Security-and-Security-Architectural-Recommendations-for-the-IEEE-802.15.4-Low-Rate-WPAN

July 24, 2003:

15-03-0320-00-0040P802-15_TG4-Suggestions-for-Improvement-of-the-IEEE-802.15.4-WPAN-Standard

July 25, 2003:

15-03-0320-01-0040P802-15_TG4-Suggestions-for-Improvement-of-the-IEEE-802.15.4-WPAN-Standard

November 11, 2003:

15-03-0320-02-0040P802-15_TG4-Suggestions-for-Improvement-of-the-IEEE-802.15.4-WPAN-Standard

July 22, 2003:

03284r0P802-15_TG4-Suggestions-for-Improvement-of-the-IEEE-802.15.4-Standard

July 18, 2003:

03285r0ZB-Suggestions-for-Improvement-of-the-IEEE-802.15.4-Standard

July 24, 2003:

03285r1ZB-Suggestions-for-Improvement-of-the-IEEE-802.15.4-Standard

May 11, 2004:

15-04-0245-00-004b-suggestions-improvement-ieee-802-15-4-2003-wpan-standard

July 15, 2004:

15-04-0245-01-004b-suggestions-improvement-ieee-802-15-4-2003-wpan-standard

July 15, 2004:

15-04-0245-01-004b-suggestions-improvement-ieee-802-15-4-2003-wpan-standard

September 16, 2004:

15-04-0537-00-004b-Formal-Specification-of-CCM-Star-Mode-of-Operation

Academic paper:

N. Sastry, D. Wagner, Security Considerations for IEEE 802.15.4 Networks, University of California, Berkely, CA, submitted for publication.

- Replacement Security Text

## Security-Related PIB Attributes

Reference 802.15.4-2003: Tables 72 and 73.

The security-related PIB attributes are required to manage security. The security-related PIB-attributes are presented in Table 1 through ).

Table 1  PIB security attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| LinkKeySet | TBD | Set of link key descriptor entries. See Table 2. | Variable | A set of link key descriptor entries, each containing key information and attributes. | Null set |
| DeviceSet | TBD | Set of device descriptor entries. See | Variable | A set of device descriptor entries, each containing device information and security attributes. | Null set |
| FrameCounter | TBD | Set of 4 octets | 0x00000000-0xFFFFFFFF | The frame counter used to ensure sequential freshness of frames sent by this device. | 0x00000000 |
| SecurityLevelSet | TBD | Set of security level descriptors | Variable | A set of security level descriptor entries for this device, depending on frame types and subtypes, each with information as to the minimum security level required/expected for this particular frame type/subtype. | Null Set |
|  |  |  |  | Table 6 |  |

Table 2 Elements of the link key descriptor

| Name | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| KeySrcAddress | Device address | Any valid 64-bit address | Extended device address. | - |
| KeySeqNumber | Octet | 0x00-0xFF | The value 0x00 indicates that this key is a peer-to-peer key; the other values indicate that this is a key shared among a group of devices. | - |
| MembershipSet | Set of key membership descriptor values. See | Variable | Set of devices of which messages were received using this link key, including blacklist info (to indicate whether received frame counter for that device was exhausted, if freshness is checked). | Null set |
| Key | Set of 16 octets | - | The actual value of the key. | - |

Table 3 Elements of the key membership descriptor

| Name | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| SrcAddress | Device address | Any valid 64-bit address | Extended device address. | Device specific |
| BlackListStatus | Boolean | TRUE or FALSE | Indicator as to whether the device indicated by SrcAddress previously communicated with this link key, prior to exhaustion of the frame counter, if freshness is checked. (Yes if TRUE; No if FALSE). If TRUE, this indicates that the source device cannot further use this key, since it exhausted its use of the frame counter used with this key. | FALSE |

Table 4 Elements of the security level descriptor

| Name | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| FrameType/subtype |  |  | See 7.2.1.1.1, Table 67 |  |
| Security Minimum | Integer | 0x00-0x07 | The minimal required/expected security level for outgoing and incoming MAC frames for this device, for the indicated frame type | 0x06 |

Table 5 Elements of the device descriptor

| Name | Type | Range | Description | Default |
| --- | --- | --- | --- | --- |
| SrcAddress | Device address | Any valid 64-bit address | Extended device address. | Device specific |
| FrameCounter | Set of 4 octets | 0x00000000-0xFFFFFFFF | The frame counter used to ensure sequential freshness of frames received from the device indicated by SrcAddress. | 0x00000000 |
| Security Level Set | Set of security level descriptors | Variable | A set of security level descriptor entries for this device, depending on frame types and subtypes, each with information as to the minimum security level required/expected for this particular frame type/subtype. |  |
|  |  |  | Table 6 |  |

# Frame Security

Reference 802.15.4-2003: Clause 7.5.8.

This clause describes the security-related processing steps at the MAC layer. These layers shall use the auxiliary frame header, as specified in 3.1, to convey security settings in a secured frame. Outgoing and incoming frames shall be handled according to the procedures specified in sections 3.2.1 and 3.2.2, respectively.

## Auxiliary Frame Header Format

The auxiliary frame header includes a counter field, a security control field, and a key identification field, and shall be formatted as illustrated by Figure 1. The security control field specifies the settings used for the security processing and indicates the makeup of the key identification field. If security is disabled, the counter shall be omitted.)

| 0/4 | Octets: 0/1 | 0/5/9 |
| --- | --- | --- |
| Frame counter field | Security control field | Key identification field |

Figure 1 Auxiliary frame header format

### Frame counter field

The counter field is used to provide for frame freshness and to prevent processing of duplicate frames. Its value is determined from the frame counter of the device that is the source of the frame.

- Security control field

The security control field is the empty string if the 1-bit security subfield contained in the frame control field is set to 0, since then security is turned off (see Clause 7.2.1.1.2 of 802.15.4-2003). Otherwise, the security control field is a 1-octet field and shall be formatted as shown in Figure 2.

| Bit: 0-2 | 3-5 | 5-7 |
| --- | --- | --- |
| Security level | Minimum Security Level | Key source addressing mode |

Figure 2 Security control field format

#### Security level

The security level identifier indicates how an outgoing frame is to be secured, respectively, how an incoming frame purportedly has been secured: it indicates whether or not the payload is encrypted and to what extent data authenticity over the frame is provided, as reflected by the length of the message integrity code (MIC). The bit-length of the MIC may take the values 0, 32, 64 or 128 and determines the probability that a random guess of the MIC would be correct. The security properties of the security levels are listed in Table 6.

#### Minimum Security level

The minimum security level identifier indicates the minimum security level according to which frames of this type are required to be secured from the sender’s perspective.

Table 6 Security levels available to the MAC layer

| Security level identifier | Security Control Field (Figure 2) | Security Attributes | Data Encryption | Frame Integrity (length M of MIC, in number of octets) |
| --- | --- | --- | --- | --- |
| 0x00 | ‘000’ | None | OFF | NO (M = 0) |
| 0x01 | ‘001’ | MIC-32 | OFF | YES (M=4) |
| 0x02 | ‘010’ | MIC-64 | OFF | YES (M=8) |
| 0x03 | ‘011’ | MIC-128 | OFF | YES (M=16) |
| 0x04 | ‘100’ | ENC | ON | NO (M = 0) |
| 0x05 | ‘101’ | ENC-MIC-32 | ON | YES (M=4) |
| 0x06 | ‘110’ | ENC-MIC-64 | ON | YES (M=8) |
| 0x07 | ‘111’ | ENC-MIC-128 | ON | YES (M=16) |

#### Key identification mode

The key identification mode is a 2-bit field indicating whether or not the key by which the frame is secured is derived implicitly or explicitly and used to identify the addressing mode fo the source within the key identification field if derived explicitly. This subfield shall be set to ‘00’ if the key is derived implicitly (from addressing information associated with the processed frame header) and shall be set to one of the other 3 values if the key is derived explicitly. The key identification field of the auxiliary frame header (Figure 1) shall only be present if this subfield has the value 01.

#### Key source addressing mode

The key source addressing mode field is a 2-bit subfield that is used to identify the addressing mode of the key source within the key identification field and shall be set to one of the values indicated in Table 7.

Table 7 Values of the key-source address mode

| Key source addressing mode B1 b0 | Integer Value |  | Description | Key source address length (octets) |  |
| --- | --- | --- | --- | --- | --- |
| 0 0 | 0x01 |  | Implicit key identification (no key source address, no key sequence counter | n.a. |  |
| 0 1 | 0x00 |  | The key source address is implicitly determined from the address of the coordinator. | 0 |  |
| 1 0 | 0x02 |  | The key source address is the right-concatenation of the PAN ID and the 16-bit short address of the source entity. | 4 |  |
| 1 1 | 0x03 |  | The key source address is the extended 64-bit address of the source entity. | 8 |  |

### Key Identification Field

The key identification field shall only be present if the key identification mode subfield (see 3.1.1.3) has value unequal to 0. If so, the key identification field explicitly indicates the key that shall be used for securing outgoing frames or that purportedly has been used to secure incoming frames. The key identification field shall be formatted as indicated in Figure 3. The key identification field consists a key source address and a key sequence number. The key source address subfield shall indicate the address of the key originator, represented in the format indicated by the key source addressing mode subfield (see 3.1.1.4). The key sequence number may allow unique identification of different keys originating from the same key source. It is the responsibility of the key originator to make sure that actively used keys that it issues have distinct key sequence numbers.

| Octets: 0/2/4/8 | 0/1 |
| --- | --- |
| Key source address | Key sequence number |

Figure 3 Format for the key identification fields

### Security Parameters

This clause specifies the parameters used for the CCM* security operations.

### CCM* Mode of Operation and Parameters

Applying security to a frame on a particular security level corresponds to a particular instantiation of the AES-CCM* mode of operation as specified in section C.1.2. The AES-CCM* mode of operation is an extension of the AES-CCM mode that is used in the 802.15.4-2003 MAC specification and provides capabilities for authentication, encryption, or both.

The cryptographic key shall be extracted from the security material in the MAC PIB as described in 3.1.4, and the nonce shall be formatted as specified in 3.1.5.

Table 6 gives the relationship between the security level subfield of the security control field (i.e., see Figure 2), the security level identifier, and the CCM* encryption/authentication properties used for these operations.

### Security Material

For each entity to which a device sends or receives secured frames there is an entry in the MAC PIB. This entry contains the implicit or explicit address of the entity (if explicit it also contains a key identifier – see 3.1.1.3) and the corresponding security material associated with that entity. The security material shall consist of an AES key, a frame counter for outgoing frames, and an external frame counter for incoming frames (see Figure 4).

The frame counter is the running counter that shall be used to construct the nonce field specified in clause 3.1.5. This counter shall be incremented each time a secure frame is transmitted, as specified in 3.2.1and it shall not roll over. The fact that this frame counter value changes for every frame helps to ensure that the CCM* nonce is unique (and thus guarantees semantic security) and allows the recipient to use the counter to ensure freshness or to detect duplicates. The recipient of a frame shall use the frame counter of the originator of the message to verify the freshness of an incoming frame, as specified in 3.2.2.

| Security Element | Size (Octets) | Description |
| --- | --- | --- |
| AES Symmetric Key | 16 | The cryptographic key used to secure incoming and outgoing frames. |
| Frame Counter (for outgoing frames) | 4 | The frame counter used by a device when originating a frame (same as the FrameCounter parameter given in Table 1). |
| External Frame Counter (for incoming frames) | 4 | The frame counter used by a device to verify freshness of incoming frames (same as the FrameCounter parameter given in ). This counter is only available if the CheckFreshness parameter in is enabled. |

Figure 4 CCM* security-related material

    - CCM* Nonce

The nonce input used for the CCM* encryption and authentication transformation and for the CCM* decryption and authentication checking transformation consists of data explicitly included in the frame and data that both devices can independently obtain. Figure 5 specifies the order and length of the subfields of the CCM* nonce. The source address is the extended 64-bit MAC address SrcAddress of the device originating the frame. The frame counter is the outgoing frame counter determined by the source device. The nonce's security level identifier is as defined in Table 6 and shall correspond to the security control field (see Figure 2) of the frame being processed.

| Octets: 8 | 4 | 1 |
| --- | --- | --- |
| Source address | Frame counter | Security Control Field |

Figure 5 CCM* nonce

## Security Processing Steps

The operations performed on outgoing and incoming frames are given in 3.2.1 and 3.2.2, respectively.

### Outgoing Frame Operations

The outgoing frame operations vary depending on the security level. When security processing of an outgoing frame is needed, the following operations shall be performed:

- Determine the security level identifier SecurityLevel and use Table 6 to determine the octet length M of the authentication field. If the particular security level is not supported, set SecurePayload equal to the empty set and fail with a status of UNSUPPORTED_SECURITY_LEVEL.2.1).

- Obtain the frame counter to be used for security processing as follows:

Obtain the frame counter (FrameCounter) associated with the source address of the outgoing frame;

- If FrameCounter cannot be obtained or if FrameCounter has as value the 4-octet representation of the integer 232-1, set SecurePayload equal to the empty set and fail with a status of COUNTER_ERROR;

- If the security level indicates “no security”, set the SecurePayload equal to Payload and proceed to step 3 below.

- Obtain the keying material to be used for security processing as follows:

- If the key identification mode field in the security control field is set to 0 (implicit key derivation), obtain the key (LinkKey) associated with the source and destination addresses of the outgoing frame; See Table 2: if peer-to-peer key, KeySourceAddress=Destination address; if multicast/broadcast, KeySourceAddress=SourceAddress multicast/broadcast key & KeySeqNumber:=1-octet string identifying this version of the multicast/broadcast key.

- Otherwise, obtain the group link key (LinkKey) associated with the explicit key identification fields in the auxiliary frame header; See Table 2: KeySourceAddress and KeySeqNumber correspond ad verbatim to the fields in the Key Identification fields (Figure 3), after conversion of short address to the extended 64-bit address.

- If the LinkKey cannot be obtained, set SecurePayload equal to the empty set and fail with a status of NO_DATA_KEY.

- Obtain the extended source address to be used for security processing as follows:

- Obtain the 64-bit extended source address (SrcAddress) of the device originating the frame;

- If the SrcAddress cannot be obtained, set SecurePayload equal to the empty set and fail with a status of ADDRESS_ERROR.

- Execute the CCM* mode encryption and authentication checking operation, as specified in ANNEX B –, with the following instantiations:

- The parameter M shall have the integer value M obtained in step 1 above;

- The key Key shall be the bit string LinkKey obtained in step 3 above;

- The nonce N shall be the 13-octet string constructed using the SrcAddress obtained in step 4 above, the FrameCounter obtained in step 2 above, and the SecurityLevel obtained in step 1 above, as specified in 3.1.5;

- If encryption is enabled (ON), the octet string a shall be the string MACHeader and the octet string m shall be the string Payload. If encryption is disabled (OFF), the octet string a shall be the string MACHeader || Payload and the octet string m shall be the empty string.

- Return the results of the CCM* operation:

- If the CCM* mode invoked in step 5 outputs ‘invalid’, set SecurePayload equal to the empty set and fail with Status equal to SECURITY_ERROR;

- Let c be the output from step 5 above. Set the octet string SecurePayload to the string c if encryption is enabled (ON) and to the string Payload || c if encryption is disabled (OFF).

- Update the internal frame counter (FrameCounter) associated with the source address of the outgoing frame with (FrameCounter +1).

- Set Status equal to SUCCESS, and return.

### Incoming Frame Operations

The incoming frame operations vary depending on the security level. When security processing of an incoming frame is needed, the following operations shall be performed:

- Determine the security level identifier SecurityLevel from ProcessedMACHeader and use Table 6 to determine the octet length M of the authentication field. If the particular security level is not supported, set Payload equal to the empty set and fail with a status of UNSUPPORTED_SECURITY_LEVEL.

- Determine whether the security level applied to the incoming frame is adequate, as follows:

  - Determine the minimal level of security (SecurityMinimum) necessary for the given frame; ).

  - If SecurityLevel is less than SecurityMinimum, then set Payload equal to the empty set and fail with a status of SECURITY_ERROR.

- Obtain the received frame counter to be used for security processing as follows:

- Determine the received frame count (ReceivedFrameCounter) from the auxiliary frame header;

- If ReceivedFrameCounter is only a single octet (i.e., it is a reduced nonce – see 3.1.1), then recompute the full (uncompressed) ReceivedFrameCounter to the 4-octet representation of the least possible integer value that is not smaller than the FrameCounter associated with the source address of the incoming frame and has its least significant octet equal to ReceivedFrameCounter. In this computation, set FrameCounter to the default value (i.e., representing the integer 0) if the source address of the incoming frame is not included in the device list. If FrameCounter cannot be obtained or if this integer does not exist, set Payload equal to the empty set and fail with a status of COUNTER_ERROR;

- If ReceivedFrameCounter has as value the 4-octet representation of the integer 232-1, then set Payload equal to the empty set and fail with a status of COUNTER_ERROR;

- If the security level indicates "no security" and freshness is not to be checked (i.e., the CheckFreshness field corresponding to the source address of the incoming frame is not set), set Payload equal to SecurePayload and proceed to step 9 below.

- If freshness is to be checked (i.e., the CheckFreshness field corresponding to the source address of the incoming frame is set), execute the following steps:

- Obtain the frame counter (FrameCounter) associated with the source address of the incoming frame. If this entry is not contained in the device list, set FrameCounter to the default value (i.e., representing the integer 0);

- If FrameCounter cannot be obtained, set Payload equal to the empty set and fail with a status of COUNTER_ERROR;

- Ensure sequential freshness by verifying that ReceivedFrameCounter represents an integer that is greater than or equal to the one represented by FrameCounter. If the freshness verification fails, set Payload equal to the empty set and fail with Status equal to OLD_FRAME_COUNT;

- If security is disabled or if the key identification mode field in the security control field is set to 0 (implicit key derivation), obtain the BlackListStatus field of the source address of the incoming frame with respect to the group associated with the source and destination addresses of the incoming frame;

- Otherwise, obtain the BlackListStatus field of the source address of the incoming frame with respect to the group associated with the explicit key identification fields in the auxiliary frame header;

- If BlackListStatus cannot be obtained or if BlackListStatus has the value TRUE, set Payload equal to the empty set and fail with a status of COUNTER_ERROR;

- If the security level indicates "no security", set Payload equal to SecurePayload and proceed to step 9 below.

- Obtain the keying material to be used for security processing as follows:

- If the key identification mode field in the security control field is set to 0 (implicit key derivation), obtain the key (LinkKey) associated with the source and destination addresses of the incoming frame;

- Otherwise, obtain the group link key (LinkKey) associated with the explicit key identification fields in the auxiliary frame header;

- If the LinkKey cannot be obtained, set Payload equal to the empty set and fail with a status of NO_DATA_KEY.

- Obtain the extended source address to be used for security processing as follows:

- Obtain the 64-bit extended source address (SrcAddress) of the device originating the frame;

- If the SrcAddress cannot be obtained, set Payload equal to the empty set and fail with a status of ADDRESS_ERROR.

- Execute the CCM* mode decryption and authentication checking operation, as specified in 3.1.3 and ANNEX B –, with the following instantiations:

  - The parameter M shall have the integer value M obtained in step 1 above;

  - The key Key shall be the string LinkKey obtained in step 5 above;

  - The nonce N shall be the 13-octet string constructed using the SrcAddress obtained in step 6 above, the FrameCounter obtained in step 3 above, and the SecurityLevel obtained in step 1 above, as specified in 3.1.5;

  - Parse the octet string Payload as Payload1 || Payload2, where the right-most string Payload2 is an M-octet string. If this operation fails, output ‘invalid’;

  - If encryption is enabled (ON), the octet string a shall be the string ProcessedMACHeader and the octet string m shall be the string Payload. If encryption is disabled (OFF), the octet string a shall be the string ProcessedMACHeader || Payload1 and the octet string c shall be the string Payload2.

- Return the results of the CCM* operation:

- If the CCM* mode invoked in step c outputs ‘invalid’, set Payload equal to the empty set and fail with Status equal to SECURITY_ERROR;

- Let m be the output of step c above. Set the octet string Payload to the string m if encryption is enabled (ON) and to the string Payload1 || m if encryption is disabled (OFF).

- If the CheckFreshness field corresponding to the source address of the incoming frame is set, execute the following steps in order:

  - Check whether the source address of the incoming frame is contained in the device list. If not, add this source address to the device list and set the parameters of this entry to their default settings (this includes setting FrameCounter to the string representing the integer 0);

  - Update the frame counter FrameCounter associated with this source address with (ReceivedFrameCounter + 1);

  - Check whether the source address of the incoming frame is contained in the membership set corresponding to the group determined in step 4 above. If not, add this source address to the membership set and set the parameters hereof to their default settings (i.e., set the BlackListStatus field to FALSE);

  - If this updated value of FrameCounter has as value the 4-octet representation of the integer 232-1, set the BlackListStatus field determined in step 4 above to TRUE.

- Set Status equal to SUCCESS, and return.

# Security Status Codes

## Status Values

Table 8 describes the security status codes that may result from security processing.

Table 8 Error status enumerations for MAC security processing

| Security status code enumeration | Value | Security status description |
| --- | --- | --- |
| SEC_SUCCESS | 0x00 | No errors detected. |
| SEC_ERR_INVALID_PARAMETER | 0x01 | An invalid parameter was in the input. |
| SEC_ERR_INVALID_FORMAT | 0x02 | The frame format is invalid. |
| SEC_ERR_NO_LINK_KEY | 0x06 | No link key is available. |
| SEC_ERR_OLD_FRAME_COUNT | 0x07 | The frame count in the received frame is less than or equal to the stored frame count corresponding to the given source address. |
| SEC_ERR_COUNTER_ERROR | 0x08 | The frame count is not present or has exceeded its maximum allowable value. |
| SEC_ERR_UNSUPPORTED_SECURITY_LEVEL | 0x09 | The indicated security level is not supported. |
| SEC_ERR_SECURITY_ERROR | 0x0A | The security processing (e.g., decryption or authentication) failed. |
| SEC_ERR_ADDRESS_ERROR | 0x10 | Extended (source) address is missing. |

- Basic notions

  - Notation and representation

    - Strings and string operations

A string is a sequence of symbols over a specific set (e.g., the binary alphabet {0,1} or the set of all octets). The length of a string is the number of symbols it contains (over the same alphabet). The right-concatenation of two strings x and y of length m and n respectively (notation: x || y), is the string z of length m+n that coincides with x on its leftmost m symbols and with y on its rightmost n symbols. An octet is a symbol string of length 8. In our context, all octets are strings over the binary alphabet.

    - Integers, octets, and their representation

Throughout this specification, the representation of integers as octet strings and of octet strings as binary strings shall be fixed. All integers shall be represented as octet strings in most-significant-octet first order. This representation conforms to the convention in Section 4.3 of ANSI X9.63-2001 [3]. All octets shall be represented as binary strings in most-significant-bit first order.

    - Entities

Throughout this specification, each entity shall be a DEV and shall be uniquely identified by its 64-bit IEEE device address [1]. The parameter entlen shall have the integer value 64.

- In the specification of each transformation, equivalent computations that result in identical output are allowed, provided all checks and verifications are carried out completely.

- Each of the schemes in this standard may have certain prerequisites. These are conditions that shall be satisfied by implementations of the scheme. The actual specification of mechanisms that provide these prerequisites is beyond the scope of the scheme itself. There are no secrecy requirements for the choices that may have to be made in the prerequisites (e.g., choice of domain parameters, hash function, data formats, etc.).

- CCM* mode of operation

- See IEEE document [2].

- Security Building Blocks

This annex specifies the cryptographic primitives and mechanisms that are used to implement the security protocols in this standard.

  - Symmetric-key cryptographic building blocks

The following symmetric-key cryptographic primitives and data elements are defined for use with all security-processing operations specified in this standard.

    - Block-cipher

The block-cipher used in this specification shall be the Advanced Encryption Standard AES-128, as specified in FIPS Pub 197 [4]. This block-cipher shall be used with symmetric keys with the same size as that of the block-cipher: 128 bits. These symmetric keys shall be generated uniformly at random.

    - Mode of operation

The block-cipher mode of operation used in this specification shall be the CCM* mode of operation, as specified in ANNEX B –, with the following instantiations:

- Each entity shall use the block-cipher E as specified in section C.1.1;

- All octets shall be represented as specified in section A.1.2; (i.e., all octets shall be represented as binary strings in most-significant-bit first order).

- The parameter L shall have the integer value 2;

- The parameter M shall have one of the following integer values: 0, 4, 8, or 16.

- References

The following standards contain provisions, which, through reference in this document, constitute provisions of this standard. Normative references are given in D.1 and D.2 and informative references are given in D.3. At the time of publication, the editions indicated were valid. All standards are subject to revision, and parties to agreements based on this standard are encouraged to investigate the possibility of applying the most recent editions of the standards indicated below.

  - 802.15.4 References

- Institute of Electrical and Electronics Engineers, Inc., IEEE Std. 802.15.4-2003, IEEE Standard for Information Technology — Telecommunications and Information Exchange between Systems — Local and Metropolitan Area Networks — Specific Requirements — Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low Rate Wireless Personal Area Networks (WPANs). New York: IEEE Press. 2003.

- René Struik, Formal Specification of CCM* Mode of Operation, IEEE document 15-04-0537-00-004b, September 16, 2004.

  - Normative References

- ANSI X9.63-2001, Public Key Cryptography for the Financial Services Industry - Key Agreement and Key Transport Using Elliptic Curve Cryptography, American Bankers Association, November 20, 2001. Available from http://www.ansi.org.

- FIPS Pub 197, Advanced Encryption Standard (AES), Federal Information Processing Standards Publication 197, US Department of Commerce/N.I.S.T, Springfield, Virginia, November 26, 2001. Available from http://csrc.nist.gov/.

  - Informative References

- "IEEE Standards Style Manual", published and distributed in May 2000 and revised on September 20, 2001. Available from http://standards.ieee.org/guides/style/.

- FIPS Pub 140-2, Security requirements for Cryptographic Modules, US Department of Commerce/N.I.S.T., Springfield, Virginia, June 2001 (supersedes FIPS Pub 140-1). Available from http://csrc.nist.gov/.

- A.J. Menezes, P.C. van Oorschot, S.A. Vanstone, Handbook of Applied Cryptography, Boca Raton: CRC Press, 1997.

- NIST Pub 800-38C, Recommendation for Block Cipher Modes of Operation – The CCM Mode for Authentication and Confidentiality, NIST Special Publication 800-38C, US Department of Commerce/N.I.S.T., Springfield, Virginia, May 12, 2004. Available from http://csrc.nist.gov/.

- FIPS Pub 113, Computer Data Authentication, Federal Information Processing Standards Publication 113, US Department of Commerce/N.I.S.T., May 30, 1985. Available from http://csrc.nist.gov/.

- R. Housley, D. Whiting, N. Ferguson, Counter with CBC-MAC (CCM), submitted to N.I.S.T., June 3, 2002. Available from http://csrc.nist.gov/encryption/modes/proposedmodes/.

- J. Jonsson, On the Security of CTR + CBC-MAC, in Proceedings of Selected Areas in Cryptography – SAC 2002, K. Nyberg, H. Heys, Eds., Lecture Notes in Computer Science, Vol. 2595, pp. 76-93, Berlin: Springer, 2002.

- J. Jonsson, On the Security of CTR + CBC-MAC, NIST Mode of Operation – Additional CCM Documentation. Available from http://csrc.nist.gov/encryption/modes/proposedmodes/.

- PKIX, L. Bassham, R. Housley, W. Polk, Algorithms and Identifiers for the Internet X.509 Public Key Infrastructure Certificate and CRL Profile, Internet Draft, PKIX Working Group, October 2001.

- RFC 3280, Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile, Internet Request for Comments 3280, R. Housley, W. Polk, W. Ford, D. Solo, April 2002.

- P. Rogaway, D. Wagner, A Critique of CCM, IACR ePrint Archive 2003-070, April 13, 2003.

- F. Stajano, R. Anderson, "The Resurrecting Duckling: Security Issues in Ad-Hoc Wireless networks," in Proceedings of the 7th International Workshop on Security Protocols, B. Christianson, B. Crispo, J.A. Malcolm, and M. Roe, Eds., Lecture Notes in Computer Science, Vol. 1796, Berlin: Springer-Verlag, 1999.

- F. Stajano, "The Resurrecting Duckling: What Next?," in Proceedings of the 8th International Workshop on Security Protocols, B. Crispo, M. Roe, and B. Criso, Eds., Lecture Notes in Computer Science, Vol. 2133, Berlin: Springer-Verlag,  April 2000.

Processing is only required if the frame is intended for the layer that calls for security processing.