IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Next PNC Identification |  |
| Date Submitted | 14 March 2005 |  |
| Source | Bill Shvodian 8133 Leesburg Pike Suite 700 Vienna, VA 22182 | Voice: +1-703-269-3047 Fax: E-mail: bill dot shvodian at freescale dot com |
| Re: | LB 29 – TG3b draft |  |
| Abstract | This document describes a mechanism to enable smooth transition to a new PNC for the situation where the PNC turns off or moves out of range without performing a PNC handover. |  |
| Purpose | This document is provided in support of 802.15.3b activities in order to alleviate an identified deficiency in the 802.15.3 protocol. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Motivation

802.15.3 provides a mechanism for the PNC to perform handover of the PNC functionality.  The PNC is supposed to perform handover when a more capable device joins the piconet, or when it knows that it is going away.  Unfortunately, it is not always possible for the PNC to always know that it is going away.  The PNC may suddenly be turned off or lose power.  If mobile, the PNC could also move out of range of the other devices in the piconet.  When this happens the other devices are left unable to communicate because there is no more PNC.

In theory, one of the remaining devices could assume the role of PNC when it determines that the PNC is no longer active or in range, but there are drawbacks to this.  First, there is no standard way of saving the piconet.  A new piconet has to be formed.  Second, multiple devices in the piconet could simultaneously assume the role of PNC in the new piconet. Third, since it is a new piconet every device would have to join the new piconet and re-establish all communications.

Overview

The PNC always attempts to select another DEV in the piconet to become PNC if the PNC goes away without handing over.  If there is no other PNC capable device in the piconet, that is indicated.  If there is one other PNC capable device in the piconet, that devices is identified as the next PNC.  If there are multiple PNC capable devices the PNC chooses the best qualified device to be the next PNC and periodically advertises the next PNC via an IE in the Beacon.

In addition to the information in the PNC rating field, the PNC should also consider the information on which devices can hear which other devices in the piconet.  It is important that the new PNC is a device that as many as possible devices in the piconet can reliably communicate with.  In order to allow the PNC to determine which devices can hear other devices in the piconet, each DEV should listen to all MCTAs or the CAP and make note of which other devices in the piconet it can hear and how well.  A new command is introduced to provide this information to the PNC.  Since each DEV transmits a management frame to the PNC at least once per association timeout period (ATP), if the DEV has nothing else to send to the PNC it should send the new command with the information about all of the devices in the piconet.

Proposed Text

Add 7.4.18 Next PNC IE

The Next PNC IE shall be formatted as illustrated in Figure xx.  The Next PNC is the DEVID of the device that the PNC has chosen to be the Next PNC.  The Next PNID is the PNID that the next PNC shall use if it starts sending out beacons because the PNC disappeared without performing a handover.

| Octets: 2 | 1 | 1 | 1 |
| --- | --- | --- | --- |
| Next PNID | Next PNC | Length = 3 | Element ID |

Add 7.5.7.6 Piconet channel status

The Piconet Channel Status command shall be formatted as illustrated in Figure xx.  The Bitmap of Devices in Range is a 256 bit bitmap of all of the devices in the piconet that can be heard by the device sending the command.

| Octets: 32 | 2 | 2 |
| --- | --- | --- |
| Bitmap of devices in Range | Length = 32 | Command Type |

Add 8.2.x Next PNC

Although the PNC shall attempt to perform a handoff when it shuts down or leaves the piconet, this may not always be possible.  The PNC could be powered off or in such a way that handover is not performed.  It could also move out of range of the rest of the devices in the piconet, or vice versa.  In order to avoid the interruption in communication that would occur when the PNC disappears without handoff, the PNC shall choose a device in the piconet to be the Next PNC.

The PNC shall periodically evaluate the PNC capabilities of the members of the piconet and other factors to decide which device should be the next PNC if the current PNC were to somehow leave the piconet without performing a handover.  In addition to the PNC capabilities field, the PNC shall use information provided by the DEVs in the Piconet channel status command to determine which potential PNC could best be heard by the other devices in the piconet.

Listen always devices can continuously determine the devises they can hear.  Listen source or normal mode devices will only hear devices that have streams set up to talk to them.  However, if the CAP is use in the piconet, each DEV can also keep track of devices that are heard in the CAP.  If MCTAs are used in the piconet, DEVs that are not extremely power sensitive should listen to all of the uplink MCTAs to determine which ones it can hear.

DEVs are required to send a management frame to the PNC at least once per ATP.  If the DEV has nothing else to send, it should send the Next PNC command so that the PNC can track which DEVs can hear which other DEVs.   Based on the bitmaps in the Piconet channel status command, the PNC shall attempt to choose a DEV that can hear all of the other DEVs in the piconet to be the Next PNC.

The PNC shall periodically select a DEV to be the Next PNC. The Next PNC IE shall be inserted in the beacon at least once per the minimum ATP in the piconet.  The Next PNC IE shall also be inserted in all of the appropriate wake beacons for power save devices.  If no DEVs in the piconet are PNC capable, the PNC shall set the DEVID in the Next PNC IE to 0.  The PNC shall periodically send the handover information to the DEV deemed to be the next PNC so that it has the information that it needs to take over as PNC in case the PNC goes away.

When the DEV identified as the next PNC fails to detect five consecutive beacons, it shall scan the channel for any frames.  Since no frames can be transmitted in dynamic CTAs, the CAP or MCTAs if the beacon is not received, the Next PNC DEV shall use the presence of any of these frames as an indication that the beacon is still being transmitted but the Next PNC DEV has lost contact with the the PNC.  In this case the next PNC shall not take over the role of PNC for that piconet.  If nothing can be heard at all during the 2 superframe duration scan, the Next PNC DEV shall begin sending out beacons.  There is a possibility that the next PNC has left the range of all of the other devices in the piconet as opposed to the original PNC going out of range or powering off.  In order to eliminate the possility of two piconets in close range using the same PNIC, the new PNC shall use the Next PNID.  If the Next PNC can hear one or more other DEVs in the piconet continuing to transmit, it shall determine that the original PNC is still transmitting beacons and it shall not begins sending beacons in the same channel.

The Next PNC shall purposely start its beacon after the time it would have expected the beacon from the previous PNC so that if they are in range their beacons don't collide.