IEEE Standards

PROJECT AUTHORIZATION REQUEST (PAR)

# 1. Date of Request:

February 6, 1998

# 2. Assigned Project #:

------------------------------------------------------------------------

# 3. Does this PAR revise a previously approved PAR?

__ YES X_ NO

------------------------------------------------------------------------

# 4. Description of Standard

X_ New X_ Trial Use __

Proposed Document: Recommended Practice __ Revision __ of Std._______________________ Full Use X_

Guide __

------------------------------------------------------------------------

# 5. Project Title:

Standard for Wireless Medium Access Method (MAC) and Physical Layer (PHY) Specifications

------------------------------------------------------------------------

# 6. Scope of Proposed Standard:

(Use attachment sheet if necessary)

To develop a Medium Access Control (MAC) and Physical Layer (PHY) specification for wireless connectivity for with fixed, portable and moving device within a Personal Operating Space (POS).

A POS is the space about a person that typically extends up to 10M in all directions and follows the person as they move about.

Refer to the attachment for details

------------------------------------------------------------------------

7. Purpose of Proposed Standard: (Use attachment sheet if necessary)

To provide wireless connectivity and support interoperability of devices within the POS that are carried, worn, or placed about the body.

Refer to the attachment for details

------------------------------------------------------------------------

8. SPONSOR: Society: Computer Society

Committee: Technical Committee on Computer Communications (TCCC)

------------------------------------------------------------------------

9. Name of Group that will write the standard: ____________

10. Target Completion Date:    July 1999

------------------------------------------------------------------------

11. Proposed Coordination: (See instructions.) Method of Coordination:

SCC10 (IEEE Dictionary)

Refer to the attachment for details

------------------------------------------------------------------------

12. Are you aware of any patent, copyright, or trademark issues? _X_ YES __ NO

Are you aware of any standards or projects with a similar scope? __ YES _X_ NO

(If yes, attach a sheet with a complete description of the impact of the similarities.)

------------------------------------------------------------------------

13. Copyright Agreements for IEEE Standards

I hereby acknowledge my appointment as Official Reporter to the _______________Committee to write/revise a Standards Publication (entitled or to be entitled ______________________

_____________________________________________________________________________________

In consideration of my appointment and the publication of the Standards Publication identifying me, at my option, as an Official Reporter, I agree to avoid knowingly incorporating in the Standards Publication any copyrighted or proprietary material of another without such other's consent and acknowledge that the Standards Publication shall constitute a "work made for hire" as defined by the Copyright Act, and, that as to any work not so defined, I agree to and do hereby transfer any right or interest I may have in the copyright to said Standards Publication to IEEE.

Name_________________________________________________________________

(chair of working group)

Title________________________________________________

Date__________________________________________________________________________

------------------------------------------------------------------------

14. Person delegated to receive communications and conduct liaison with interested bodies:

(This is normally the chair of the working group. If not please indicate IEEE position.)

Name_________________________________________________Telephone________

Company_______________________________Fax_____________

Address__________________________________________ Telex__________________

City______________________________________________State_____________ Zip_________

E-Mail____________

------------------------------------------------------------------------

15. Submitted by:

(This is normally the sponsor's liaison to the Standards Board. If not please indicate IEEE position and relationship to the sponsor.)

Name______________________________________ Telephone_________

Company______________________________Fax__________

Address______________________ Telex_______________

City_________________________________________State_______ Zip______

# 6. Scope of proposed standard

To develop a Medium Access Control (MAC) and Physical Layer (PHY) specification for wireless connectivity for with fixed, portable and moving devices within a Personal Operating Space (POS).

## Type of medium

The goal is that the MAC shall support PHYs using electromagnetic waves through free space    (radio waves).

PHY layer suitable for use with the electromagnetic frequency spectrum as described in the following paragraph will be defined with this standard. If evidence of need and sufficient interest exists other PHY layers will be considered at a later time.

## Radio spectrum

Currently the only globally available unlicensed spectrum with enough band is in the 2.4GHz.

The initial effort will be for the 2.4GHz ISM band and to consider the use of additional frequencies other than ISM.

We will support the 802.11 committee’s efforts to make sure the devices in the applicable bands can co-exist.

To further enhance the standard our committee will undertake to document the benefits of, and make recommendations for international spectrum allocation and use, where possible.

## Supported Devices

The standard shall support stationary, portable, movable, and mobile devices moving at pedestrian and optionally vehicular speeds within the POS.    The standard shall support permanent and transient connections through identification and communication with devices that are within or pass through the POS.    This is to be implemented with one PHY if feasible.

## Environment

The PAN shall have limited range to alleviate band congestion and be designed to be tolerant of    in-band interference from other ISM devices, including overlapping PANs.    Support for connectivity with LANs and WANs will be through other specifically designed devices and will not be included in this standard.

The standard will include support of the following:

- Intelligent devices capable of application layer protocol (e.g. computer).

- Non-Intelligent devices only capable of input or output (e.g. scanner, printer).

- Intelligent transient devices (e.g. drop stations, print server).

## Possible target environments include:

•users with multiple devices such as, courier, ........

## Supported service

The Wireless MAC shall support both connectionless service as defined in the MAC Service definition at rates less than or equal to 1Mbps, as well as, a service supporting packetized voice.

## Compatibility requirements

The specification shall conform with relevant standards

# 7 Purpose of proposed standard.

To provide wireless connectivity and support interoperability of devices within the POS that are carried, worn, or placed about the body.

To offer a standard for use by regulatory bodies to standardize access to one or more radio frequency bands for the purpose of personal area communication.

Note: This PAR also authorizes IEEE 802 to petition or provide comments to regulatory bodies worldwide (e.g. the FCC in the USA, the Department of Communications in Canada, the RF agency of the Department of Trade and Industry in the UK and the Radio Frequency Commission of the CEPT of Europe)

# 10 Target completion

Architecture definition available July 1998

First draft standard ready for ballot in January 1999.

First draft conformance standard ready for ballot in July 1999

TCCC ballot of MAC & PHY standard _____________

TCCC ballot for conformance standard _____________

Submission to ISO of MAC & PHY standard and conformance standard _____________

# 11 Proposed Coordination??????????????????

SCC28 circulation of drafts

EMC circulation of drafts

CCIR Task Group 8/1 (formerly IWP 8/13) draft circulation

CEPT/RFC/FM draft circulation

ETSI RES corresp/membership overlap

ECMA TC32 corresp/participation

ISA SP-50 membership overlap

SCC10 (IEEE dictionary) draft circulation

ASC X3S3 draft circulation

ISO/IEC JTC1/SC6/WG1 and WG3 Through ASC X3S3

ASC T1P1 correspondence

# 12. Patent, Related Project

Patents potentially relevant to the work of this committee are known to exist.