IEEE P802.11
Wireless LANs

Comparison of MAC PICS versus WPAN requirements

Date:	September 16, 1998

Author:	Tim Blaney
	Commcepts
	10052 Alta Sierra Dr., Suite C, Grass Valley, CA, 95949, USA
	Phone: +1 530 273-3482
	Fax:          +1 530 273-3543
	e-Mail: tim@wireless-network.com

Greg Ennis
	Ennis Associates
	16331 Englewood Ave., Los Gatos, CA, 95032, USA
	Phone: +1 408 358-5544
	Fax:          +1 408 358-5541
	e-Mail: gennis@mindspring.com

Bob O'Hara
	Informed Technology, Inc.
	1750 Nantucket Circle, Suite 138, Santa Clara, CA, 95054-3822, USA
	Phone: +1 408 986-9586
	Fax:          +1 408 727-2654
	e-Mail: bob@informed-technology.com

Abstract

# In this submission we consider a comparison of the MAC portion of the existing IEEE 802.11 PICS as it relates to the requirements of a WPAN application. The intent of this submission is to assist the WPAN SG in understanding how the existing 802.11 MAC can be adapted to work as a MAC for a WPAN.

Discussion

In our investigation of the 802.11 MAC, as it applies to WPAN applications, we concluded that a small number of changes to the 802.11 MAC could be made that would tailor it more closely for meeting the WPAN requirements.    These changes include

- Providing stations with the ability to rotate the AP responsibility, including a mechanism that will recognize when an AP is lost

- Adding power management modes to the AP

- Removing the RTS/CTS handshake

- Significantly slimming the MIB, and

- Eliminating support for IBSS

These changes extend the current capabilities of 802.11 and simplify the implementation of 802.11 stations.

There are still two "classes" of 802.11 devices in a WPAN

- Devices capable of becoming a WPAN AP, and

- Devices that are independent stations (not capable of becoming a WPAN AP)

These device classes are likely to differ somewhat in complexity and cost.    Our idea of a WPAN consists of a "few"

devices capable of being WPAN APs and the rest being independent stations.

The MAC portion of the PICS in Annex A of the 802.11 standard is reproduced below, twice.    The first instance of the PICS is completed with the IUT being a hypothetical WPAN AP.    The second instance of the PICS is completed with the IUT being a WPAN station.    Each PICS shows where the WPAN devices differ from current 802.11 devices.    It is at these points of difference, where WPAN devices would be non-compliant with the current 802.11 standard and new conditional items would be used to create the new IUT classes for WPAN AP and WPAN station.    These new items would be CF6 for a WPAN AP and CF7 for a WPAN station.

Following is the dual listing of the Annex:

A.	Annex - Protocol Implementation Conformance Statement (PICS) Proforma (normative)

A.4	PICS Proforma—IEEE Std 802.11-1997††

A.4.3	IUT Configuration (AP capable)

| Item | IUT Configuration | Status | Support |
| --- | --- | --- | --- |
|  | What is the configuration of the IUT? |  |  |
| * CF1 | Access Point (AP) | O.1 | Yes þNo ¨ |
| * CF2 | Independent Station (NOT an AP) | O.1 | Yes ¨No þ |
| * CF3 | Frequency Hopping Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes þNo ¨ |
| * CF4 | Direct Sequence Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes ¨No þ |
| * CF5 | Infrared PHY Layer | O.2 | Yes ¨No þ |

A.4.4	MAC Protocol

A.4.4.1	MAC Protocol Capabilities

| Item | Protocol Capability | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC protocol capabilities supported? |  |  |
| PC1 | Authentication Service | M | Yes þNo ¨ |
| PC1.1 | Authentication State | M | Yes þNo ¨ |
| PC1.2 | Open System Authentication | M | Yes þNo ¨ |
| PC1.3 | Shared Key Authentication | PC2:M | Yes ¨No ¨N/A þ |
| * PC2 | WEP Privacy Algorithm | O | Yes ¨No þ |
| PC2.1 | WEP Encryption Procedure | PC2:M | Yes ¨No ¨N/A þ |
| PC2.2 | WEP Decryption Procedure | PC2:M | Yes ¨No ¨N/A þ |
| PC2.3 | Security Services Management | M | Yes ¨No þ |
| PC3 | Distributed Coordination Function | M | Yes þNo ¨ |
| PC3.1 | Net Allocation Vector (NAV) Function | M | Yes þNo ¨ |
| PC3.2 | Interframe Space Usage & Timing | M | Yes þNo ¨ |
| PC3.3 | Random Backoff Function | M | Yes þNo ¨ |
| PC3.4 | DCF Access Procedure | M | Yes þNo ¨ |
| PC3.5 | Random Backoff Procedure | M | Yes þNo ¨ |
| PC3.6 | Recovery Procedures and Retransmit Limits | M | Yes þNo ¨ |
| PC3.7 | RTS/CTS Procedure | M | Yes ¨No þ |
| PC3.8 | Directed MPDU Transfer | M | Yes þNo ¨ |
| PC3.9 | Broadcast & Multicast MPDU Transfer | M | Yes þNo ¨ |
| PC3.10 | MAC Level Acknowledgment | M | Yes þNo ¨ |
| PC3.11 | Duplicate Detection and Recovery | M | Yes þNo ¨ |
| * PC4 | Point Coordinator | CF1:O | Yes ¨No þN/A ¨ |
| PC4.1 | Maintenance of CFP Structure & Timing | PC4:M | Yes ¨No ¨N/A þ |
| PC4.2 | PCF MPDU Transfer from Point Coordinator | PC4:M | Yes ¨No ¨N/A þ |
| * PC4.3 | PCF MPDU Transfer to Point Coordinator | PC4:O | Yes ¨No ¨N/A þ |
| PC4.4 | Overlapping PC Provisions | PC4:M | Yes ¨No ¨N/A þ |
| PC4.5 | Polling List Maintenance | PC4.3:M | Yes ¨No ¨N/A þ |
| * PC5 | CF-Pollable | CF2:O | Yes ¨No þN/A ¨ |
| PC5.1 | Interpretation of CFP Structure & Timing | PC5:M | Yes ¨No ¨N/A þ |
| PC5.2 | PCF MPDU Transfer to/from CF-Pollable STA | PC5:M | Yes ¨No ¨N/A þ |
| PC5.3 | Polling List Update | PC5:M | Yes ¨No ¨N/A þ |
| PC6 | Fragmentation | M | Yes þNo ¨ |
| PC7 | Defragmentation | M | Yes þNo ¨ |
| PC8 | MAC Data Service | M | Yes þNo ¨ |
| PC8.1 | Reorderable-Multicast Service Class | M | Yes þNo ¨ |
| PC8.2 | Strictly-Ordered Service Class | O | Yes ¨No þ |
| PC9 | Multirate Support | M | Yes þNo ¨ |
| * PC10 | Multiple Outstanding MSDU Support | O | Yes ¨No þ |
| PC10.1 | Multiple Outstanding MSDU Transmission Restrictions | PC10:M | Yes ¨No ¨N/A þ |
| PC11 | Timing Synchronization | M | Yes þNo ¨ |
| PC11.1 | Timing in an Infrastructure Network | CF1:M | Yes þNo ¨N/A ¨ |
| PC11.2 | Timing in an Independent BSS | CF2:M | Yes ¨No ¨N/A þ |
| PC11.3 | Beacon Generation Function | M | Yes þNo ¨ |
| PC11.4 | TSF Synchronization & Accuracy | M | Yes þNo ¨ |
| PC11.5 | Infrastructure BSS Initialization | CF1:M | Yes þNo ¨N/A ¨ |
| PC11.6 | Independent BSS Initialization | CF2:M | Yes ¨No ¨N/A þ |
| PC11.7 | Passive Scanning | CF2:M | Yes ¨No ¨N/A þ |
| PC11.8 | Active Scanning | CF2:M | Yes ¨No ¨N/A þ |
| PC11.9 | Probe Response | M | Yes þNo ¨ |
| PC11.10 | Hop Synchronization Function | CF3:M | Yes þNo ¨N/A ¨ |
| PC12 | Infrastructure Power Management | M | Yes þNo ¨ |
| PC12.1 | Station Power Management Modes | CF2:M | Yes ¨No ¨N/A þ |
| PC12.2 | TIM Transmission | CF1:M | Yes þNo ¨N/A ¨ |
| PC12.3 | Access Point Function During Contention Period | CF1:M | Yes þNo ¨N/A ¨ |
| PC12.4 | Access Point Function During CFP | PC4:M | Yes ¨No ¨N/A þ |
| PC12.5 | Receive Function During Contention Period | CF2:M | Yes ¨No ¨N/A þ |
| PC12.6 | Receive Function During CFP | PC5:M | Yes ¨No ¨N/A þ |
| PC12.7 | Aging Function | CF1:M | Yes þNo ¨N/A ¨ |
| PC13 | Independent BSS Power Management | CF2:M | Yes ¨No ¨N/A þ |
| PC13.1 | Initialization of Power Management | CF2:M | Yes ¨No ¨N/A þ |
| PC13.2 | STA Power State Transitions | CF2:M | Yes ¨No ¨N/A þ |
| PC13.3 | ATIM and Frame Transmission | CF2:M | Yes ¨No ¨N/A þ |
| PC14 | Association & Reassociation | M | Yes þNo ¨ |
| PC14.1 | Association State | M | Yes þNo ¨ |
| PC14.2 | STA Association Procedure | CF2:M | Yes ¨No ¨N/A þ |
| PC14.3 | AP Association Procedure | CF1:M | Yes þNo ¨N/A ¨ |
| PC14.4 | STA Reassociation Procedure | CF2:M | Yes ¨No ¨N/A þ |
| PC14.5 | AP Reassociation Procedure | CF1:M | Yes þNo ¨N/A ¨ |
| PC15 | Management Information Base | M | Yes ¨No þ |
| PC15.1 | SMT Object Class | M | Yes ¨No þ |
| * PC15.2 | Privacy Package | PC2:M | Yes ¨No ¨N/A þ |
| PC15.3 | MAC Object Class | M | Yes ¨No þ |
| * PC15.4 | MAC Statistics Package | O | Yes ¨No þ |
| PC15.3 | Resource Type Object Class | M | Yes ¨No þ |

A.4.4.2	MAC Frames

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is transmission of the following MAC frames supported? |  |  |
| FT1 | Association Request | CF2:M | Yes ¨No ¨N/A þ |
| FT2 | Association Response | CF1:M | Yes þNo ¨N/A ¨ |
| FT3 | Reassociation Request | CF2:M | Yes ¨No ¨N/A þ |
| FT4 | Reassociation Response | CF1:M | Yes þNo ¨N/A ¨ |
| FT5 | Probe Request | CF2:M | Yes ¨No ¨N/A þ |
| FT6 | Probe Response | M | Yes þNo ¨ |
| FT7 | Beacon | M | Yes þNo ¨ |
| FT8 | ATIM | CF2:M | Yes ¨No ¨N/A þ |
| FT9 | Disassociation | M | Yes þNo ¨ |
| FT10 | Authentication | M | Yes þNo ¨ |
| FT11 | Deauthentication | M | Yes þNo ¨ |
| FT12 | PS-Poll | CF2:M | Yes ¨No ¨N/A þ |
| FT13 | RTS | M | Yes ¨No þ |
| FT14 | CTS | M | Yes ¨No þ |
| FT15 | ACK | M | Yes þNo ¨ |
| FT16 | CF-End | PC4:M | Yes ¨No ¨N/A þ |
| FT17 | CF End+CF-Ack | PC4:M | Yes ¨No ¨N/A þ |
| FT18 | Data | M | Yes þNo ¨ |
| FT19 | Data + CF-Ack | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FT20 | Data + CF-Poll | PC4.3:M | Yes ¨No ¨N/A þ |
| FT21 | Data + CF-Ack+CF-Poll | PC4.3:M | Yes ¨No ¨N/A þ |
| FT22 | Null | M | Yes þNo ¨ |
| FT23 | CF-Ack (no data) | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FT24 | CF-Poll (no data) | PC4.3:M | Yes ¨No ¨N/A þ |
| FT25 | CF-Ack+CF-Poll (no data) | PC4.3:M | Yes ¨No ¨N/A þ |

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is reception of the following MAC frames supported? |  |  |
| FR1 | Association Request | CF1:M | Yes þNo ¨N/A ¨ |
| FR2 | Association Response | CF2:M | Yes ¨No ¨N/A þ |
| FR3 | Reassociation Request | CF1:M | Yes þNo ¨N/A ¨ |
| FR4 | Reassociation Response | CF2:M | Yes ¨No ¨N/A þ |
| FR5 | Probe Request | M | Yes þNo ¨ |
| FR6 | Probe Response | M | Yes þNo ¨ |
| FR7 | Beacon | M | Yes þNo ¨ |
| FR8 | ATIM | CF2:M | Yes ¨No ¨N/A þ |
| FR9 | Disassociation | M | Yes þNo ¨ |
| FR10 | Authentication | M | Yes þNo ¨ |
| FR11 | Deauthentication | M | Yes þNo ¨ |
| FR12 | PS-Poll | CF1:M | Yes þNo ¨N/A ¨ |
| FR13 | RTS | M | Yes ¨No þ |
| FR14 | CTS | M | Yes ¨No þ |
| FR15 | ACK | M | Yes þNo ¨ |
| FR16 | CF-End | M | Yes ¨No þ |
| FR17 | CF End+CF-Ack | M | Yes ¨No þ |
| FR18 | Data | M | Yes þNo ¨ |
| FR19 | Data + CF-Ack | M | Yes ¨No þ |
| FR20 | Data + CF-Poll | PC5:M | Yes ¨No ¨N/A þ |
| FR21 | Data + CF-Ack+CF-Poll | PC5:M | Yes ¨No ¨N/A þ |
| FR22 | Null | M | Yes þNo ¨ |
| FR23 | CF-Ack (no data) | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FR24 | CF-Poll (no data) | PC5:M | Yes ¨No ¨N/A þ |
| FR25 | CF-Ack+CF-Poll (no data) | PC5:M | Yes ¨No ¨N/A þ |

A.4.4.3	Frame Exchange Sequences

| Item | Frame Exchange Sequence | Status | Support |
| --- | --- | --- | --- |
|  | Are the following Frame Sequences Supported? |  |  |
| FS1 | Basic Frame Sequences | M | Yes þNo ¨ |
| FS2 | CF-Frame Sequences | (PC4 or PC5):M | Yes ¨No ¨N/A þ |

A.4.4.4	MAC Addressing Functions

| Item | MAC Address Function | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC Addressing Functions Supported? |  |  |
| AD1 | STA Universal Individual IEEE802 Address | M | Yes þNo ¨ |
| AD2 | BSS Identifier Generation | M | Yes þNo ¨ |
| AD3 | Receive Address Matching | M | Yes þNo ¨ |

A.4.3	IUT Configuration (Station only)

| Item | IUT Configuration | Status | Support |
| --- | --- | --- | --- |
|  | What is the configuration of the IUT? |  |  |
| * CF1 | Access Point (AP) | O.1 | Yes ¨No þ |
| * CF2 | Independent Station (NOT an AP) | O.1 | Yes þNo ¨ |
| * CF3 | Frequency Hopping Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes þNo ¨ |
| * CF4 | Direct Sequence Spread Spectrum PHY Layer for the 2.4GHz Band | O.2 | Yes ¨No þ |
| * CF5 | Infrared PHY Layer | O.2 | Yes ¨No þ |

A.4.4	MAC Protocol

A.4.4.1	MAC Protocol Capabilities

| Item | Protocol Capability | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC protocol capabilities supported? |  |  |
| PC1 | Authentication Service | M | Yes þNo ¨ |
| PC1.1 | Authentication State | M | Yes þNo ¨ |
| PC1.2 | Open System Authentication | M | Yes þNo ¨ |
| PC1.3 | Shared Key Authentication | PC2:M | Yes ¨No ¨N/A þ |
| * PC2 | WEP Privacy Algorithm | O | Yes ¨No þ |
| PC2.1 | WEP Encryption Procedure | PC2:M | Yes ¨No ¨N/A þ |
| PC2.2 | WEP Decryption Procedure | PC2:M | Yes ¨No ¨N/A þ |
| PC2.3 | Security Services Management | M | Yes ¨No þ |
| PC3 | Distributed Coordination Function | M | Yes þNo ¨ |
| PC3.1 | Net Allocation Vector (NAV) Function | M | Yes þNo ¨ |
| PC3.2 | Interframe Space Usage & Timing | M | Yes þNo ¨ |
| PC3.3 | Random Backoff Function | M | Yes þNo ¨ |
| PC3.4 | DCF Access Procedure | M | Yes þNo ¨ |
| PC3.5 | Random Backoff Procedure | M | Yes þNo ¨ |
| PC3.6 | Recovery Procedures and Retransmit Limits | M | Yes þNo ¨ |
| PC3.7 | RTS/CTS Procedure | M | Yes ¨No þ |
| PC3.8 | Directed MPDU Transfer | M | Yes þNo ¨ |
| PC3.9 | Broadcast & Multicast MPDU Transfer | M | Yes þNo ¨ |
| PC3.10 | MAC Level Acknowledgment | M | Yes þNo ¨ |
| PC3.11 | Duplicate Detection and Recovery | M | Yes þNo ¨ |
| * PC4 | Point Coordinator | CF1:O | Yes ¨No þN/A ¨ |
| PC4.1 | Maintenance of CFP Structure & Timing | PC4:M | Yes ¨No ¨N/A þ |
| PC4.2 | PCF MPDU Transfer from Point Coordinator | PC4:M | Yes ¨No ¨N/A þ |
| * PC4.3 | PCF MPDU Transfer to Point Coordinator | PC4:O | Yes ¨No ¨N/A þ |
| PC4.4 | Overlapping PC Provisions | PC4:M | Yes ¨No ¨N/A þ |
| PC4.5 | Polling List Maintenance | PC4.3:M | Yes ¨No ¨N/A þ |
| * PC5 | CF-Pollable | CF2:O | Yes ¨No þN/A ¨ |
| PC5.1 | Interpretation of CFP Structure & Timing | PC5:M | Yes ¨No ¨N/A þ |
| PC5.2 | PCF MPDU Transfer to/from CF-Pollable STA | PC5:M | Yes ¨No ¨N/A þ |
| PC5.3 | Polling List Update | PC5:M | Yes ¨No ¨N/A þ |
| PC6 | Fragmentation | M | Yes þNo ¨ |
| PC7 | Defragmentation | M | Yes þNo ¨ |
| PC8 | MAC Data Service | M | Yes þNo ¨ |
| PC8.1 | Reorderable-Multicast Service Class | M | Yes þNo ¨ |
| PC8.2 | Strictly-Ordered Service Class | O | Yes ¨No þ |
| PC9 | Multirate Support | M | Yes þNo ¨ |
| * PC10 | Multiple Outstanding MSDU Support | O | Yes ¨No þ |
| PC10.1 | Multiple Outstanding MSDU Transmission Restrictions | PC10:M | Yes ¨No ¨N/A þ |
| PC11 | Timing Synchronization | M | Yes þNo ¨ |
| PC11.1 | Timing in an Infrastructure Network | CF1:M | Yes ¨No ¨N/A þ |
| PC11.2 | Timing in an Independent BSS | CF2:M | Yes ¨No þN/A ¨ |
| PC11.3 | Beacon Generation Function | M | Yes þNo ¨ |
| PC11.4 | TSF Synchronization & Accuracy | M | Yes þNo ¨ |
| PC11.5 | Infrastructure BSS Initialization | CF1:M | Yes ¨No ¨N/A þ |
| PC11.6 | Independent BSS Initialization | CF2:M | Yes ¨No ¨N/A þ |
| PC11.7 | Passive Scanning | CF2:M | Yes þNo ¨N/A ¨ |
| PC11.8 | Active Scanning | CF2:M | Yes ¨No þN/A ¨ |
| PC11.9 | Probe Response | M | Yes ¨No þ |
| PC11.10 | Hop Synchronization Function | CF3:M | Yes þNo ¨N/A ¨ |
| PC12 | Infrastructure Power Management | M | Yes þNo ¨ |
| PC12.1 | Station Power Management Modes | CF2:M | Yes þNo ¨N/A ¨ |
| PC12.2 | TIM Transmission | CF1:M | Yes ¨No ¨N/A þ |
| PC12.3 | Access Point Function During Contention Period | CF1:M | Yes ¨No ¨N/A þ |
| PC12.4 | Access Point Function During CFP | PC4:M | Yes ¨No ¨N/A þ |
| PC12.5 | Receive Function During Contention Period | CF2:M | Yes þNo ¨N/A ¨ |
| PC12.6 | Receive Function During CFP | PC5:M | Yes ¨No ¨N/A þ |
| PC12.7 | Aging Function | CF1:M | Yes ¨No ¨N/A þ |
| PC13 | Independent BSS Power Management | CF2:M | Yes ¨No þN/A ¨ |
| PC13.1 | Initialization of Power Management | CF2:M | Yes ¨No þN/A ¨ |
| PC13.2 | STA Power State Transitions | CF2:M | Yes ¨No þN/A ¨ |
| PC13.3 | ATIM and Frame Transmission | CF2:M | Yes ¨No þN/A ¨ |
| PC14 | Association & Reassociation | M | Yes þNo ¨ |
| PC14.1 | Association State | M | Yes þNo ¨ |
| PC14.2 | STA Association Procedure | CF2:M | Yes þNo ¨N/A ¨ |
| PC14.3 | AP Association Procedure | CF1:M | Yes ¨No ¨N/A þ |
| PC14.4 | STA Reassociation Procedure | CF2:M | Yes þNo ¨N/A ¨ |
| PC14.5 | AP Reassociation Procedure | CF1:M | Yes ¨No ¨N/A þ |
| PC15 | Management Information Base | M | Yes ¨No þ |
| PC15.1 | SMT Object Class | M | Yes ¨No þ |
| * PC15.2 | Privacy Package | PC2:M | Yes ¨No ¨N/A þ |
| PC15.3 | MAC Object Class | M | Yes ¨No þ |
| * PC15.4 | MAC Statistics Package | O | Yes ¨No þ |
| PC15.3 | Resource Type Object Class | M | Yes ¨No þ |

A.4.4.2	MAC Frames

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is transmission of the following MAC frames supported? |  |  |
| FT1 | Association Request | CF2:M | Yes þNo ¨N/A ¨ |
| FT2 | Association Response | CF1:M | Yes ¨No ¨N/A þ |
| FT3 | Reassociation Request | CF2:M | Yes þNo ¨N/A ¨ |
| FT4 | Reassociation Response | CF1:M | Yes ¨No ¨N/A þ |
| FT5 | Probe Request | CF2:M | Yes þNo ¨N/A ¨ |
| FT6 | Probe Response | M | Yes ¨No þ |
| FT7 | Beacon | M | Yes ¨No þ |
| FT8 | ATIM | CF2:M | Yes ¨No þN/A ¨ |
| FT9 | Disassociation | M | Yes þNo ¨ |
| FT10 | Authentication | M | Yes þNo ¨ |
| FT11 | Deauthentication | M | Yes þNo ¨ |
| FT12 | PS-Poll | CF2:M | Yes þNo ¨N/A ¨ |
| FT13 | RTS | M | Yes ¨No þ |
| FT14 | CTS | M | Yes ¨No þ |
| FT15 | ACK | M | Yes þNo ¨ |
| FT16 | CF-End | PC4:M | Yes ¨No ¨N/A þ |
| FT17 | CF End+CF-Ack | PC4:M | Yes ¨No ¨N/A þ |
| FT18 | Data | M | Yes þNo ¨ |
| FT19 | Data + CF-Ack | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FT20 | Data + CF-Poll | PC4.3:M | Yes ¨No ¨N/A þ |
| FT21 | Data + CF-Ack+CF-Poll | PC4.3:M | Yes ¨No ¨N/A þ |
| FT22 | Null | M | Yes þNo ¨ |
| FT23 | CF-Ack (no data) | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FT24 | CF-Poll (no data) | PC4.3:M | Yes ¨No ¨N/A þ |
| FT25 | CF-Ack+CF-Poll (no data) | PC4.3:M | Yes ¨No ¨N/A þ |

| Item | MAC Frame | Status | Support |
| --- | --- | --- | --- |
|  | Is reception of the following MAC frames supported? |  |  |
| FR1 | Association Request | CF1:M | Yes ¨No ¨N/A þ |
| FR2 | Association Response | CF2:M | Yes þNo ¨N/A ¨ |
| FR3 | Reassociation Request | CF1:M | Yes ¨No ¨N/A þ |
| FR4 | Reassociation Response | CF2:M | Yes þNo ¨N/A ¨ |
| FR5 | Probe Request | M | Yes þNo ¨ |
| FR6 | Probe Response | M | Yes þNo ¨ |
| FR7 | Beacon | M | Yes þNo ¨ |
| FR8 | ATIM | CF2:M | Yes ¨No þN/A ¨ |
| FR9 | Disassociation | M | Yes þNo ¨ |
| FR10 | Authentication | M | Yes þNo ¨ |
| FR11 | Deauthentication | M | Yes þNo ¨ |
| FR12 | PS-Poll | CF1:M | Yes ¨No ¨N/A þ |
| FR13 | RTS | M | Yes ¨No þ |
| FR14 | CTS | M | Yes ¨No þ |
| FR15 | ACK | M | Yes þNo ¨ |
| FR16 | CF-End | M | Yes ¨No þ |
| FR17 | CF End+CF-Ack | M | Yes ¨No þ |
| FR18 | Data | M | Yes þNo ¨ |
| FR19 | Data + CF-Ack | M | Yes ¨No þ |
| FR20 | Data + CF-Poll | PC5:M | Yes ¨No ¨N/A þ |
| FR21 | Data + CF-Ack+CF-Poll | PC5:M | Yes ¨No ¨N/A þ |
| FR22 | Null | M | Yes þNo ¨ |
| FR23 | CF-Ack (no data) | (PC4 or PC5):M | Yes ¨No ¨N/A þ |
| FR24 | CF-Poll (no data) | PC5:M | Yes ¨No ¨N/A þ |
| FR25 | CF-Ack+CF-Poll (no data) | PC5:M | Yes ¨No ¨N/A þ |

A.4.4.3	Frame Exchange Sequences

| Item | Frame Exchange Sequence | Status | Support |
| --- | --- | --- | --- |
|  | Are the following Frame Sequences Supported? |  |  |
| FS1 | Basic Frame Sequences | M | Yes þNo ¨ |
| FS2 | CF-Frame Sequences | (PC4 or PC5):M | Yes ¨No ¨N/A þ |

A.4.4.4	MAC Addressing Functions

| Item | MAC Address Function | Status | Support |
| --- | --- | --- | --- |
|  | Are the following MAC Addressing Functions Supported? |  |  |
| AD1 | STA Universal Individual IEEE802 Address | M | Yes þNo ¨ |
| AD2 | BSS Identifier Generation | M | Yes þNo ¨ |
| AD3 | Receive Address Matching | M | Yes þNo ¨ |

† Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.