IEEE P802.11
Wireless LANs

Wireless Initiatives Comparisons, a PHY Layer Review

Date:	October 26, 1998

Author:	Jim Zyren c/o Bruce Kraemer
	Harris Semiconductor
	2401 Palm Bay Road Palm Bay, FL 32905 USA
	Phone: +1 407-729-5683
	Fax: +1 407-724-7886
	e-Mail: jzyren@ or bkraemer@harris.com

Abstract

Note 2: This document was revised during the Atlanta, GA IEEE 802.11 Working Group Ad Hoc Meeting of the WPAN SG.

Note 1: This document was revised for the Westford, MA IEEE 802.11 Working Group Interim Meeting of the WPAN SG.

IEEE 802.11, PAN, HRFWG, and Bluetooth each represent initiatives to develop wireless products. Each initiative is seeking the creation of a relatively open specification either through authorized standards bodies or industry special interest groups.

The purpose of    each “standard” is to provide a common specification to a multitude of software and hardware vendors each of which will independently work to build compatible equipment while simultaneously building consumer confidence that    interoperability and multiple sources exist. Both of these components is required in order to promote market growth.

While 802 PAN is still soliciting proposals for MAC and PHY, each of the other initiatives endorses FHSS radio technology. In fact, other than transmit power, the technical specifications for physical layer (PHY) for all four applications are almost identical.

Each of the initiatives being considered here has marketing and technical information available on their respective website.

www.ieee.org

www.homerf.org

www.bluetooth.com

The technical PHY information displayed below is available on these websites.

# Technical Requirements

A summary of key technical requirements for the RF front ends for all three specifications appears below:

| Parameter | IEEE 802.11 FHSS | HRFWG SWAP CA | Bluetooth |
| --- | --- | --- | --- |
| Operating Frequency | 2.400 - 2.4835 GHz | 2.400 - 2.4835 GHz | 2.400 - 2.4835 GHz |
| Spread Spectrum Method | FHSS | FHSS | FHSS |
| Data Rate | 1 Mbps 2 Mbps | 1 Mbps 2 Mbps | 1 Mbps 2 Mbps (future) |
| Modulation Method | 2-FSK 4-FSK (optional) | 2-FSK 4-FSK (required) | 2-FSK TBD |
| Modulation Index (h) | 0.32 0.16 |  | 0.32 TBD |
| Effective Filter BW | Gaussian (BT = 0.5) |  | ????? |
| Hop Rate | 2.5 Hz | 50 Hz | 1600 Hz (max) |
| Channel Switch Time | 224 microsec | 300 microsec | 220 microsec |
| Rx/Tx Turnaround Time | 19 microsec | 25 microsec | 220 microsec |
| Antenna Diversity | Optional |  | Not Required |
| Tx RF Power | <1W (US) 100 mW (Europe & Japan) | 100 mW (North America) | 1 mW |
| Rx Sensitivity (@ BER?) | -80 dBm @ 1 Mbps -75 dBm @ 2 Mbps | -76 dBm @ 1 Mbps | -70 dBm @ 1 Mbps |
| Tx Channel Freq. Tol. | +/- 60 kHz |  | ? |
| Tx Spectrum Shape | ¦N-M¦ = 2 -20 dBm or -40 dBc ¦N-M¦ > 3 -40 dBm or -60 dBc |  | ? |
| Hop Sequence (# Ch.) |  |  |  |

Radio Requirements for IEEE802.11, HRFWG, and Bluetooth