IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG1-Status-Update |  |
| Date Submitted | [8 March 00] |  |
| Source | [Tom Siep] [Texas Instruments] [12500 TI Blvd, m/s 8723] | Voice: [214-480-6786] Fax: [972-761-5581] E-mail: [siep@ti.com] |
| Re: | [Original document.] |  |
| Abstract | [Follow-up letter to 25February00 meeting participants in Copenhagen between 802.15.1 editors and Bluetooth SIG Test and Verification Working Group representatives, documented in 00054r0P802-15_TG1-Bluetooth-f2f-Meeting-Minutes.doc] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TO:

Mårten Mattsson

Stefan Agnani

Thomas Müller

FROM:

Tom Siep

Mike McInnis

David Cypher

Allen Heberling

We have just completed our March meeting and we wanted to update you on our progress.  Almost all of our action items have been completed (at least the first pass, anyway).  As a review, the action item list is below.  Our status is indicated in the new column.  Subsequent paragraphs have the details/deliverables.

# ACTION ITEMS

| What | Who | When | Status |
| --- | --- | --- | --- |
| Minutes of meeting | Tom | Publish Monday | Done |
| Preliminary SAP definitions | Mike | 10Mar00 | Included |
| Map HCI primitives to IEEE request/response/indicate/confirm format | Allen | 10Mar00 | Included |
| Determine if ASN.1 from Bluetooth is feasible to directly import into TeleLogic tool | Allen | 1Mar00 | Done, Yes |
| ASN.1 source for L2CAP | Mårten | 6Mar00 | Not received |
| Plan for creation of IEEE Draft, based on version 1.1 of Bluetooth Specification | Tom | 10Mar00 | Included |
| Plan for PICS verification. Formulate proposition; initiate conversation between Ian and JimK; revise plan | Tom | 1Mar00 | In Progress |
| Co-simulation thoughts and evaluation | ALL, both IEEE and SIG | 16Mar00 | Need to schedule telecon |

# Preliminary SAP Definitions/HCI Primitives

Mike and Allen have taken the SDLs that Allen and David having been working on and have extracted the signals that pass across the SAP interface.  Figure 1 shows their refinement of the SAP illustration we all developed in Copenhagen.

The listing at the end of this document shows their preliminary SAP and HCI Primitives definitions.   The SDLs and SAPs we presented in Copenhagen have not changed.

Figure 1

# ASN.1 import into TeleLogic

Allen's discussion with the people from Telelogic determined that there should be no problem in importing your ASN.1.  We will be able to use it as soon as it is available.

# ASN.1 source for L2CAP

During our work planning session we have determined that we need both the ASN.1 and Signal Names (with structures) from you.  This will aid us in verifying that we will have a good match when we begin to get ready for cross-verification.  The signal names may become publicly known (buried in our project documentation), but will not be used directly if there is a problem with copyrights.

# Plan for creation of IEEE Draft, based on version 1.1

Figure 2 shows the timeline for the creation of the next two drafts.  Draft 0.7 will not be voted upon by the membership.  Its purpose is to gather all of the new sections together for further integration and smoothing.  It will be the text upon which we create the SDLs. The SDLs will be tested and verified initially though the TeleLogic toolset.

Figure 2.

One thing that is not indicated in the timeline is our decision on having two versions of the SDLs, one Normative (requirement of the Standard) and the other Informative (information only).  The Normative SDL limits the scope to the behavior of the protocol and does not implement any of the detailed algorithms.  For example, the HEC process block returns a Boolean and does not really check the contents of the header.  Keeping the Normative SDL at this level is in keeping with the IEEE philosophy of describing attributes of a system, not implementations.

The Normative SDL obviously is not testable in the SIG testbed.  The Informative (testable) SDL is directly mapped from the Normative and extends it to the point that it will function in the Testing and Verification suite.  It is our intent to include the source for both of the SDLs on the IEEE CD, but only print the Normative version in the document itself.

# Plan for PICS verification

Arto Palin and Fujio Watanabe will be the editors for the PICS portion of the Standard.  They will determine if the IEEE will formulate a PICS derived from the Draft text, or a version will be derived from the BSIG PICS.  In either case, the final product will be compared with the original BSIG version and the Draft for completeness, correctness, and consistency.

In addition to that activity, we have initiated a conversation between IanG and JimK about the legal aspects of adopting or adapting the BSIG PICS as well as all non-Specification information from the Verification and Testing group.  We will, of course, modify our plan based on their determination.

# Co-simulation thoughts and evaluation

We would like to discuss the topics presented here, as well as your thoughts in a teleconference on or near 16March.  Please let us know your availability.

Proposed Agenda for Teleconference on or near 16March

| Introductions | 5 min |
| --- | --- |
| SAP Definitions | 15 min |
| ASN.1 | 10 min |
| IEEE Draft Plans | 10 min |
| PICS | 10 min |
| Discussion/Next Steps | 10 min |

Preliminary SAP and HCI Primitives definitions

List of BT signals determined to be SAPs.

BT 1.0B page 261 paragraph 3.1.4 Upper-Layer to L2CAP events and page 264 paragraph 3.2.4 L2CAP to Upper Layer actions

# LLC/L2CAP SAPs

L2CA_ConnectReq (7.2)

L2CA_ConnectCfm (7.2)

L2CA_ConnectCfmNeg (7.2) (implied by result parameter of the response)

L2CA_ConnectInd (7.1)

L2CA_ConnectRsp (7.3)

L2CA_ConnectRspNeg (implied by response parameter of the response signal)

L2CA_ConnectPnd (7.3) (implied by response parameter of the response signal)

L2CA_ConfigReq (7.4)

L2CA_ConfigCfm (7.4)

L2CA_ConfigCfmNeg (7.4)

L2CA_ConfigInd (7.1)

L2CA_ConfigRsp (7.5)

L2CA_ConfigRspNeg  (implied by result parameter of the response)

L2CA_DisconnetReq (7.6) (text does not agree with MSC on page 271)

L2CA_DisconnectCfm (7.6) (text does not agree with MSC on page 271)

L2CA_DisconnectInd (7.1) (text does not agree with MSC on page 271)

L2CA_DisconnectRsp (text does not agree with MSC on page 271)

L2CA_DataRead (7.8)

L2CA_DataWrite (7.7)

L2CA_QosViolationInd (7.1)

L2CA_TimeOutInd (7.2, 7.4)

L2CA_GroupCreate (7.9 ?) (Should this command specified in this section be part of the HCI?)

L2CA_GroupClose (7.10 ?) (Should this command specified in this section be part of the HCI?)

L2CA_GroupAddMember (7.11 ?) (Should this command specified in this section be part of the HCI?)

L2CA_GroupRemoveMember (7.12 ?) (Should this command specified in this section be part of the HCI?)

L2CA_GroupMembership (7.13 ?) (Should this command specified in this section be part of the HCI?)

L2CA_Ping (7.14 ?) (Should this command specified in this section be part of the HCI?)

L2CA_GetInfo (7.15 ?) (Should this command specified in this section be part of the HCI?)

L2CA_DisableCLT (7.16 ?) (Should this command specified in this section be part of the HCI?)

L2CA_EnableCLT (7.17 ?) (Should this command specified in this section be part of the HCI?)

# HCI SAPs

From BT 1.0B page 540 paragraph 4.5 Link Control Commands, page 576 paragraph 4.6 Link Policy Commands, and page 594 paragraph 4.7 Host Controller & Baseband Commands.

(BB)=The destination is Baseband

(LM)=The destination is Link Manager

(MLME) The destination is MAC Layer Management Entity

(?)=We don’t know in which category to place this item.

Inquiry (BB)

Inquiry_Cancel (BB)

Periodic_Inquiry_Mode (BB)

Exit_Periodic_Inquiry_Mode (BB)

Create_Connection (LM)

Disconnect (LM)

Add_SCO_Connection (LM)

Accept_Connection_Request (LM)

Reject_Connection_Request (LM)

Link_Key_Request_Reply (?)

Link_Key_Request_Negative_Reply (?)

PIN_Code_Request_Reply (?)

PIN_Code_Request_Negative_Reply (?)

Change_Connection_Packet_Type (LM)

Authentication_Requested (?)
Set_Connection_Encryption (LM)

Change_Connection_Link_Key (LM)

Master_Link_Key (LM)

Remote_Name_Request (LM)

Read_Remote_Supported_Features (LM)

Read_Remote_Version_Information (LM)

Read_Clock_Offest (LM)

Hold_Mode (LM)

Sniff_Mode (LM)

Exit_Sniff_Mode (LM)

Park_Mode (LM)

Exit_Park_Mode (LM)

Qos_Setup (LM)

Role_Discovery (LM)

Switch_Role (LM)

Read_Link_Policy_Settings (LM)

Write_Link_Policy_Settings (LM)

Read_PIN_Type (LM)

Write_PIN_Type (LM)

Create_New_Unit_Key (LM)

Read_Stored_Link_Key (LM)

Write_Stored_Link_Key (LM)

Delete_Stored_Link_Key (LM)

Change_Local_Name (MLME)

Read_Local_Name (MLME)

Read_Connection_Accept_Timeout (LM)

Write_Connection_Accept_Timeout (LM)

Read_Page_Timeout (BB)

Write_Page_Timeout (BB)

Read_Scan_Enable (BB)

Write_Scan_Enable (BB)

Read_Page_Scan_Activity (BB)

Write_Page_Scan_Activity (BB)

Read_Inquiry_Scan_Activity (BB)

Write_Inquiry_Scan_Activity (BB)

Read_Authentication_Enable (MLME)

Write_Authentication_Enable (MLME)

Read_Encryption_Mode (LM)

Write_Encryption_Mode (LM)

Read_Class_of_Device (MLME)

Write_Class_of_Device (MLME)

Read_Voice_Setting (BB)

Write_Voice_Setting (BB)

Read_Automatic_Flush_Timeout (?)

Write_Automatic_Flush_Timeout (?)

Read_Num_Broadcast_Retransmissions (BB)

Write_Num_Broadcast_Retransmissions (BB)

Read_Hold_Mode_Activity (BB)

Write_Hold_Mode_Activity (BB)

Read_Transmit_Power_level (BB)

Read_SCO_Flow_Control_Enable (BB)

Write_SCO_Flow_Control_Enable (BB)

Read_Link_Supervision_Timeout (LM)

Write_Link_Supervision_Timeout (LM)

Read_Number_Of_Supported_IAC (BB)

Read_Current_IAC_LAP (BB)

Write_Current_IAC_LAP (BB)

Read_Page_Scan_Period_Mode (BB)

Write_Page_Scan_Period_Mode (BB)

Read_Page_Scan_Mode (BB)

Write_Page_Scan_Mode (BB)

## HC SAP

(HC SAP)= The destination is Hardware Host Controller

Set_Host_Controller_To_Host_flow_Control (HC SAP)

Host_Buffer_Size (HC SAP)

Host_Number_Of_Completed_Packets (HC SAP)

# SCO SAP

SCO Protocol Data Units Indications

SCO Protocol Data Units Requests