IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | High Rate Selection Critera for CFA/CFP |  |
| Date Submitted | 21 January, 2000 |  |
| Source | [Kevin Marquess] [Hyper Corporation] [315 Iron Horse Court, Alamo, CA 94507] | Voice: [(925) 280-7779] Fax: [(925) 280-7751] E-mail: [kevin.marquess@ieee.org] |
| Re: | Development of Selection Criteria from both a standards point of view and marketing activities. |  |
| Abstract | Draft version of developing requirements and selection processes for determination technology solution for the High Rate–WPAN – MAC & PHY |  |
| Purpose | Begin processing CFAs/CFPs and must be used in conjunction with those documents. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

P802.15 High Rate Study Group –Selection Criteria for CFA/CFP

The following document summarizes the selection criteria for the proposed High Rate activities within the WPAN (Wireless Personal Area Network) space.  These criteria will be used  (in conjunction with the PAR and 5 criteria documents) to communicate the scope and intention of this class of devices to potential users and manufacturers.  It will also be used to evaluate the PHY, Protocol and Systems proposals submitted in response to the WPAN High Rate Call for Applications and Call for Proposals (CFA and CFP).

Due to the rapid rate with which this technology changes, and the fact that 802.11 WPAN study group, HomeRF and other consortia have already done some work in this area, the High Rate WPAN CFA a nd CFP are being done in parallel.

The following document is broken-up into two sections:  market requirements of a “High-Rate” device, and , the technical criteria that would be necessary to fulfill those requirements.

| General consumer | Corporate consumer |
| --- | --- |

Who is the target user?

- General consumer

What are the primary target application environments?

- Personal area within

- Home

- Automobile

- School

What are typical applications?

- Video conferencing

- Consumer video

- Video games

- DVD recording/playback

- MP3 file transfers

- Security

- Digital still/motion camera image transfer

- Computer to peripheral connections

- Audio/visual entertainment

- Toys/games

- Video projection/delivery systems

- Kiosk services

- Pump data / Pump gas

- Cellphone to PDA connections

- Data sharing

What are the devices?

This class includes both portable and non-portable devices. Compact form factor and low power consumption are important requirements for portable devices.

- Video imaging devices

- Digital still/motion cameras

- DVD devices

- MP3 devices

- Audio/visual entertainment systems

- Kiosks

- Toys/games

- Video projectors

- Cellular phones

- Communication dongles

- PDA’s

- Digital Picture Frames

- Computers

- Appliances

- Printers

- Scanners

- Modems

- Set-top boxes

|  | Camera – Still & Motion | Entertainment Systems – A/V |  |
| --- | --- | --- | --- |

When is the device needed?

Now. There is an immediate need for low cost, low powered, short range, high-rate wireless solutions to address the requirements of consumer oriented applications. Technical feasibility and time to market will be important factors in the selection of a proposal.

Technical Criteria

| General | Guideline |
| --- | --- |
| Unit manufacturing cost | Comparable to 802.15.1 class devices in quantities of 1M units |
| Interoperability with 802.15.1 (Bluetooth TM) | May be dual mode |
| Coexistence | Coexist with other devices in the 2.4 GHz band |
| Topology | Point-to-point or point-to-multipoint |
| Frequency band of operation | ISM band 2.4 GHz |
| Delivered data throughput | > 16 Mbps |
| Number of high-rate channels or simultaneous users per band |  8 |
| Multi-path immunity |  |
| Jamming resistance |  |
| Half-duplex symmetry |  |
| Sensitivity |  |
| Open field range | > 10 meters |
| Radius and environment (i.e. home -walls/floors) | > 10 meters |
| Technical feasibility |  |
| Maturity of solution | Concept, Model, Prototype, ASIC, etc. |
| Scalability (power, frequency band, data rate, cost, etc.) |  |
| Power consumption |  |
| Power management |  |
| Ease of Use | Self organizing, provisioning, administration |
| Ability to add/remove devices |  |
| Registration |  |
| Data Requirements |  |
| Type |  |
| Asynchronous | Yes |
| Isochronous | Yes |
| Audio | Yes |
| Video – real time | Yes |
| Voice – real time | Yes |
| Data Rates |  |
| Minimum raw “high” data rate | 20 Mbs |
| Support adaptive date rates for best BER |  |
| Data reliability |  |
| Predicted BER |  |
| Measured BER |  |
| Predicted packet data rate |  |
| Measured packet data rate |  |
| Link Management Features |  |
| Network detection |  |
| Attach time | < 1 sec. |
| Power control |  |
| Standby |  |
| Rx/Tx |  |
| Security |  |
| Authentication |  |
| QoS (Quality of Service) |  |
| Latency |  |
| Priority |  |
| Packet failure rate |  |

Add Criteria you feel is important and explain why, or submit a supporting CFA.  In the final evaluation, new criteria will be evaluated for all of the submitted proposals.