IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3-Criteria-Definitions |  |
| Date Submitted | [5 November, 2000] |  |
| Source | [Mary DuVal/Tom Siep] [Texas Instruments] [12500 TI Blvd, MS 8723 Dallas, TX 75243] | Voice: [972-575-2330 / 214-480-6786] Fax: [ ] E-mail: [m-duval@ti.com siep@ti.com ] |
| Re: | [00062r0P802-15_HRSG-MAC-PHY-Criteria.xls, 00007r5P802-15_HRSG-Selection-Criteria.doc, 99165r6P802-15_HRSG-PAR.doc, 00141r0P802-15_Weight_ballot_tally.xls, 00282r0P802-15_TG3-Survey-Analysis.xls, 00245r10P802-15_TG3-Proposal Evaluations.xls, 00303r0_TG3-MAC-Criteria-Clarifications-Contribution-to-110r13.doc, 00354r2P802-15-TG3-MAC-Throughput-Scenarios-Comparison-WrkBk, 00371r0P802-15-TG3-MIN_MAX_Throughput_Charts] |  |
| Abstract | [Definitions for the proposal evaluation for Task Group 3] |  |
| Purpose | [This is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

TABLE OF CONTENTS

1.	Introduction	5

2.	General Solution Criteria	5

2.1.	Unit Manufacturing Cost (UMC)	5

2.1.1.	Definition	5

2.1.2.	Values	7

2.2.	Signal Robustness	8

2.2.1.	General Definitions	8

2.2.2.	Interference and Susceptibility	8

2.2.3.	Intermodulation Resistance	9

2.2.4.	Jamming Resistance	9

2.2.5.	Multiple Access	11

2.2.6.	Coexistence	12

2.3.	Interoperability	12

2.3.1.	Definition	12

2.3.2.	Values	13

2.4.	Technical Feasibility	13

2.4.1.	Manufactureability	13

2.4.2.	Time to Market	13

2.4.3.	Regulatory Impact	13

2.4.4.	Maturity of Solution	14

2.5.	Scalability	14

2.5.1.	Definition	14

2.5.2.	Values	14

2.6.	Location Awareness	15

2.6.1.	Definition	15

2.6.2.	Values	15

3.	MAC Protocol Criteria	15

3.1.	Transparent to Upper Layer Protocols	15

3.1.1.	Definition	15

3.1.2.	Values	15

3.2.	Ease of Use	15

3.2.1.	Unique 48 bit address	15

3.2.2.	Simple Network Join/Un-Join Procedures for RF enabled device	16

3.2.3.	Device Registration	16

3.3.	Delivered Data Throughput	16

3.3.1.	Definition	16

3.3.2.	Minimum delivered data throughput	17

3.3.3.	High end delivered data throughput	17

3.3.4.	Breakdown of Application Requirements	17

3.4.	Data Transfer Types	21

3.4.1.	Definition	21

3.4.2.	Values	21

3.5.	Topology	22

3.5.1.	General	22

3.5.2.	Maximum Number of Active Connections	22

3.5.3.	Ad Hoc Network	22

3.5.4.	Access to a Portal	22

3.6.	Reliability	23

3.6.1.	General Definition	23

3.6.2.	Master Redundancy	23

3.6.3.	Loss of connection	23

3.7.	Power Management Types	23

3.7.1.	Definition	23

3.7.2.	Values	24

3.8.	Power Consumption of MAC controller	24

3.8.1.	Definition	24

3.8.2.	Value	24

3.9.	Security	24

3.9.1.	Authentication	24

3.9.2.	Privacy	25

3.10.	Quality of Service	25

3.10.1.	Definition	25

3.10.2.	Values	25

4.	PHY Layer Criteria	25

4.1.	Size and Form Factor	25

4.1.1.	Definition	25

4.1.2.	Values	26

4.2.	MAC/PHY Throughput	26

4.2.1.	Minimum MAC/PHY Throughput	26

4.2.2.	High End MAC/PHY Throughput	26

4.3.	Frequency Band	26

4.3.1.	Definition	26

4.3.2.	Values	26

4.4.	Number of Simultaneously Operating Full Throughput PAN’s	27

4.4.1.	Definition	27

4.4.2.	Values	27

4.5.	Signal Acquisition Method	27

4.5.1.	Definition	27

4.5.2.	Values	27

4.6.	Range	27

4.6.1.	Definition	27

4.6.2.	Values	27

4.7.	Sensitivity	27

4.7.1.	Definition	27

4.7.2.	Values	28

4.8.	Multi-Path Immunity	28

4.8.1.	Environment model	28

4.8.2.	Delay Spread Tolerance	29

4.9.	Power Consumption	29

4.9.1.	Definition	29

4.9.2.	Values	29

5.	Evaluation Matrix	30

5.1.	General Solution Criteria	30

5.2.	MAC Protocol Criteria	32

5.3.	PHY Protocol Criteria	35

6.	1st Pass Pugh Matrix Comparison Value	36

6.1.	General Solution Criteria Comparison Values	36

6.2.	MAC Protocol Criteria	38

6.3.	Phy Protocol Criteria	40

# Introduction

Task Group 3 High Rate WPAN (TG3) of the IEEE 802.15 has defined the criteria for the eventual selection of a Draft Standard from a set of Draft Proposals.  In order to accurately and consistently judge the proposals submitted, a common set to terms with definitions is needed.

This paper is a working document that will become the repository for the terms and definitions to be used in the selection process for a Draft Standard for TG3.  It may also contain more general Marketing Requirements on which the proposals are asked to comment.

The document is divided into four sections: General Solution Criteria, MAC Protocol Criteria, PHY Protocol Criteria and Evaluation Matrix.  Since some proposals can be submitted as only a MAC or PHY, these proposals will be expected to also address the general solution criteria.  The evaluation matrix provides the summary of criteria assessments expected with each proposal.

# General Solution Criteria

This section defines the system level concerns of the solution, both technical and marketing related.  These criteria address issues that effect both the MAC and PHY protocol layers.  This section should allow us to reduce redundancy of issues.

## Unit Manufacturing Cost (UMC)

### Definition

It is important for cost to be as small as possible for this type of consumer oriented device.    The UMC will be dependent on the complexity of the PHY and MAC.  The systems cost should be optimized.  Since some proposals can be submitted as only a MAC or PHY, the proposals should estimate as much systems cost, typical MAC functions are shown in Figure 1.  Block Diagram of MAC and while typical PHY functions are shown in Figure 2.  Logical blocks in the transceiver PHY layer

Figure 1.  Block Diagram of MAC

- PduInd stands for Protocol Data Unit Indicate.

- PduReq stands for the Request.

- Physical Layer Convergence Protocol (PLCP) – Preambles, control headers, data whitening.

- Physical Media Dependent (PMD) – Where it actually writes to the hardware.

- Media Access Control (MAC) – Segmentation, fragmentation, creates data units and controls access to the medium based on its rules.

- Mac Layer Management Entity (MLME) – Control interface between the application and the MAC and PHY.

Not all blocks in Figure 2.  Logical blocks in the transceiver PHY layer are required to implement a communications system.  However, if the functionality is used (even optionally) in the specification, then the cost for implementing the functionality must be included in the cost estimate.  The blocks may occur in different orders in the chain, for example, the frequency spreading may be a part of the modulate/demodulate portion or the encryption may precede the source encoding and the decryption follow the source decoding.

Figure 2.  Logical blocks in the transceiver PHY layer

- Source Encode/Decode – packet formation including headers, data interleaving, error correction/detection (FEC, CRC, etc), compression/decompression. This function is optional, include if it applies to the proposed system.

- Encrypt/Decrypt – bit level operations to protect data. This function is optional, include if it applies to the proposed system.

- Channel encode/decode – bias suppression, symbol spreading/de-spreading (e.g. DSSS), data whitening/de-whitening (or scrambling).  This function is optional, include if it applies to the proposed system.

- Modulate/Demodulate – convert digital data to analog format, can include symbol filtering, frequency conversion, frequency filtering.

- Frequency Spreading/De-spreading – can include frequency hopping or other techniques to decrease or increase, respectively, the bits/Hz of the analog signal in the channel. This function is optional, include if it applies to the proposed system.

- Transmit/Receive – transition the signal to/from the channel.

### Values

Cost should be specified in US dollar amounts.  It is important to indicate cost as a function of volume or time.  Reasonable and conservative values are important to present, and will be challenged by competing proposals.  The cost estimates should reflect the proposed configuration, i.e. MAC only, PHY only or MAC/PHY combination.

## Signal Robustness

### General Definitions

The error rate criterion is either the maximum bit error ratio (BER) or the maximum packet error ratio (PER) for a specified packet length or a combination of the two. The proposal will be asked to indicate both the PER, and the corresponding BER used in the determination of this value when indicating the sensitivity of the proposed system in section 4.7.  Payload size for the PER test should be 512 bytes which is intended to be a value between the minimum and maximum packet size potentially chosen in the final specification.

The minimum required sensitivity is the power level of a signal, in dBm, present at the input of the receiver modulated by the proposed method with a pseudo-random data for which the error rate criterion is met.  The power level shall be specified at the antenna to receiver connection (i.e. it shall not include any antenna gain).  The error ratio shall be determined after any error correction methods required in the proposed system have been applied.   Systems may exceed the minimum required sensitivity, but the following measurements are taken relative to the minimum value specified in the proposal.

The net throughput of the system is the net amount of bi-directional data, measured in bits, that is transferred from the MAC to/from higher layers divided by the elapsed time.  The elapsed time shall be at least 1 second.  The connection shall already have been established and in progress prior to the 1 second interval.  The units of the net throughput are Mb/s.

Unless otherwise noted, the 802.15.1 and 802.11 transceivers are assumed to use ideal isotropic radiators (i.e. 0 dBi antennas).

### Interference and Susceptibility

#### Definition

System interference from other RF energy sources including both intentional and unintentional radiators.  This includes RF energy in band and out of band.  The performance shall be measured as follows: with the desired signal 3 dB above the minimum required sensitivity, the system shall meet the error rate criterion with the interferer at a level of x dBm, x to be specified.  These levels shall be specified for frequency ranges between 30 MHz and 13 GHz.  In band interferers shall be signals modulated by the proposed method with pseudo-random data that is uncorrelated in time to the desired signal.  Out of band signals shall be single tone (sine wave) interferers.

#### Values

Proposals shall provide the frequency ranges and the corresponding power level of the interferering signal for which the error ratio criterion is met.

### Intermodulation Resistance

#### Definition

The intermodulation resistance is the ability of the system to withstand multiple in-band, but off-channel, interferers whose frequency products may be converted into on-channel signals by non-linearities in the receiver.  One measurement will be with the desired signal 3 dB above the minimum specified sensitivity and the two interfering signals at a signal power x dBm, x to be specified, located in frequency at fc+foffset and fc+n*foffset where fc is the center frequency of the desired signal and foffset is specified by the proposers.  In general, the interfering signals shall be located in the desired frequency band of operation.  One signal shall be an 802.15.1 signal modulated with pseudo-random data and the other shall be a static sine wave.

Another measurement shall be an 802.15.1 signal 100% AM modulated at a rate of 2 kHz located at various positions in the operational frequency band.  The proposals shall specify a signal amplitude level for the 802.15.1 signal at each of the 1 MHz frequency steps in the band.

Both measurements shall be specified in the proposal.

#### Values

The result is the maximum value, in dBm, of the intermodulating signals that can be withstood while retaining the desired error ratio performance.

### Jamming Resistance

#### Definition

Jamming resistance is the ability of the system to maintain performance in the presence of other uncoordinated in-band systems or interferers.  A typical environment is shown in Figure 3, which shows a proposed piconet in the presence of a potential jamming system.  It is measured by the jamming power (Pj) that causes a factor of 2 reduction in the net throughput of the proposed system if its available, and otherwise a reduction of BER from 10^-9 without jamming, to 10^-3 with jamming, given the test geometry defined below. The jamming power may be computed from measurements scaled in range and/or power using the standard free-space link budget and antenna equations:

Pr=PtGtAe4πR2, and G=4πAeλ2	(1)

as long as far-field conditions are maintained.  Antenna pattern effects (such as front to back ratio) shall be accounted for such that the resulting jamming power reflects results as if the measurements were taken with isotropic antennas on the interfering systems.  The proposed system shall be rotated for maximum degradation and that result reported.  The proposer shall provide the front-to-back ratio and effective-aperture area of its antenna at the interfering frequency and as measured by a matched filter receiver for its own signal.

Figure 3.  A typical wireless network environment with interfering sources.

The physical test geometry for the proposed network and interfering network is shown in Figure 4.  It is intended to be simple to test derivative of the typical environment shown in Figure 3. As shown, the measurement geometry is along two parallel lines that are less than 0.5m apart, with a pair of proposed systems (A1 and A2) that are 6m apart on the first line, and an interleaved pair of interfering systems (B1 and B2) that are also 6m apart on the second line; (i.e. A1, 3m, B1, 3m, A2, 3m, B2).  The power of the interfering signals shall be scaled together, i.e. they shall be the same power, with the exception of the 802.11a and 802.11b cases where the power of B2 in the setup shall be 20 dB less than the power in B1 in order to account for the longer ranges typical in a WLAN environment.  When the test is performed, the interfering systems must be operating with the specified traffic before the network connection of the proposed network is started.  The testing environment should conform to that specified in ANSI c63.4-1992 or comparable environments.

Figure 4.  The physical layout of the desired network and the interfering sources used to model jamming resistance and coexistence.

#### Values

The value is the jamming power measured according to the definition, with the following interference sources, taken one at a time.

- A microwave oven at 3 m with a power and time profile specified in ??? (look in 802.11 doc).

- An 802.15.1 piconet transmitting at 1 mW, with one HV1 voice transmission active.

- An 802.15.1 piconet transmitting at 1 mW with bi-directional DH5 packets active.

- An 802.15.3 data connection (as proposed) operating in an uncoordinated manner transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).

- An 802.11a piconet transmitting at 100mw data connection transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).

- An 802.11b piconet transmitting at 100mw transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).

### Multiple Access

#### Definition

Multiple access is the ability of coordinated systems to simultaneously share the medium.  It is measured by the net throughput of one system in the presence of other coordinated systems.

#### Values

Multiple access is measured by the net throughput of one of the proposed systems with two other systems co-located (in space) and operating in a coordinated manner as compared to the net throughput of a single system with no other interferers or coordinated systems present.  All of the systems shall consist of two nodes and shall be operating under each of the following scenarios:

- All of the systems transmitting a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6)

- The desired system transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6), the other two transferring asynchronous data with a payload size of 512 bytes.

- The desired system transferring asynchronous data with a payload size of 512 bytes and one other system transferring asynchronous data with a payload size of 512 bytes and the third transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).

### Coexistence

#### Definition

Coexistence is the net throughput of an alternate system in the presence of the proposed system divided by the net throughput of the alternate system with no other interferers or systems present. The physical layout of the network is the same as specified in section Error: Reference source not found.

#### Values

The value reported shall be the ratios of the net throughput of the following alternate systems in the presence of the proposed system.  The reference node of the proposed system is communicating with a desired node that is located at a distance of 3 m.  Both nodes of the proposed system shall be operating at the nominal transmitting power required for the proposal.

- IC1 - An 802.15.1 piconet with one HV1 voice transmission active.  Both devices in the piconet shall be transmitting at 1 mW.  One device participating in the piconet shall be at a distance of 3 m, the other at a distance of 13 m.

- IC2 - An 802.15.1 transferring data with DH5 packets bi-directionally.  Both devices in the piconet shall be transmitting at 1 mW.  One participant of the piconet shall be at a distance of 3 m, the other at a distance of 13 m.

- IC3 - An 802.11b network transferring data with 500 byte packets bi-directionally.  Both devices shall be transmitting at 100 mW.  One participant shall be at a distance of 3 m, the other shall be at a distance of 100 m.

- IC4 - An 802.11a data connection transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).  Both 802.11a devices shall be transmitting at 100 mW.  One device shall be located at a distance of 3 m, the other at a distance of 50 m.

- IC5 - An 802.11b data connection transferring a DVD video stream compressed with MPEG2 (as described in section 3.3.4.6).  Both 802.11b devices shall be transmitting at 100 mW.  One device shall be located at a distance of 3 m, the other at a distance of 50 m.

## Interoperability

### Definition

Can this system talk with and/or control an 802.15.1 system.    Some systems will have MACs or PHYs which are not compatible with 802.15.1.  In these cases, dual mode (e.g. dual radio) designs are allowed.  The ultimate measure becomes final UMC in order to get interoperability.  Reuse of system components may be important to keep cost down.  Proposals are asked to describe their approach.  Actual measurements are preferred over models.

### Values

TRUE - The proposed system is able to communicate with a 802.15.1 system.

FALSE - The proposed system is not able to communicate with a 802.15.1 system.

## Technical Feasibility

This is intended to determine if the proposal is real or academic.  Any proposal may be submitted, but demonstrated feasibility, and manufactureability should receive favor over equal but untested proposals.  Proposals will be asked to comment on criteria listed in the following sections.

### Manufactureability

#### Definition

Is the proposal manufactureable with proven technologies and IP?  Issues of UMC and the impact of yield on cost are listed in section 2.1.

#### Values

The proposals are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations.  Globally accepted concepts that will be quick to market, with little risk will be favored.

### Time to Market

#### Definition

When will the proposed system be is ready for deployment.

#### Values

The proposal shall indicate when it is ready for deployment.

### Regulatory Impact

#### Definition

Is this proposal in compliance with the current international intentional radiator regulatory standards?  If not, are actions in place to changed the regulations and what is the current status?

#### Values

TRUE – The proposed system is in compliance with the current international intentional radiator regulatory standards.

FALSE – The proposed system is not in compliance with the current international intentional radiator regulatory standards.

If false, the proposed should include indication of plans or actions to address this issue.

### Maturity of Solution

#### Definition

How do we know the design will work?  Is it modeled, tested, similar to some other existing technology?  Is invention required to create this proposal?

#### Values

The proposals are asked to submit proof of the claims by way of expert opinion, models, experiments, pre-existence examples, or demonstrations.  Globally accepted concepts that will be quick to market, with little risk will be favored.

## Scalability

### Definition

When one parameter of a standard changes, such as it’s interface, data rate, frequency band of operation, cost, and function, it may be necessary to write a new standard.  Scalability refers to the ability to adjust important parameters such as those mentioned below (if they are required by the applications) without rewriting the standard.  Examples of scalability are listed in the following sections.

#### Power consumption

This could be controlled by variable transmit power, data rate, and similar parameters.

#### Data Rate

There may be a trade off for number of channels, immunity, cost, power, or range.

#### Frequency Band of Operation

For example, if this device can be used at 2.4 GHz and 5 GHz, there may be value in volumes.

#### Cost

Is there an opportunity to change a parameter, keep interoperability, but achieve a less expensive solution (i.e. range)?

#### Function

If the device can be implemented with or without certain functions such as interoperability, or certain complexity of protocol, it might result in an optimized solution.  Note, however, that this may result in an interoperability problem and needs to be carefully considered.

### Values

The proposals should identify areas of scalability, which could be used by the applications.

## Location Awareness

### Definition

Location awareness is the ability to determine information about the relative location of one transceiver with respect to another.  The purpose is to improve usability of portable devices.  This data can be used to locate, identify and discriminate amongst users in crowded environments and to simplify device registration in constantly changing network topology.  Provisions must be made to propagate location information to higher layers of the stack.

### Values

Resolution in centimeters of the proposed location method.

# MAC Protocol Criteria

## Transparent to Upper Layer Protocols

### Definition

The function of the proposed MAC has sufficient functionality to allow direct interface to the higher level stacks such as, IEEE 802.2 Logical Link Layer (LLC), in such a way as to enable incorporation into the higher level TCP/IP stack.

### Values

TRUE – Allows interface to higher level stacks such as TCP/IP

FALSE – Proposal insufficient or unable to interface to higher level stacks such as TCP/IP

## Ease of Use

Ease of use refers to the level of user intervention necessary to perform common networking tasks, such as identifying, joining and leaving networks. The proposed system should have the capability to automatically perform these common tasks.  The goal is for the user to turn on the device and have it work.

### Unique 48 bit address

#### Definition

The MAC shall have a unique address to identify each node.

#### Values

TRUE – Has Address storage

FALSE – Does not have Address storage

### Simple Network Join/Un-Join Procedures for RF enabled device

#### Definition

For high speed communications, the ability to quickly establish and remove ad hoc connections is important.  If the detect/link/negotiate/communicate cycle is too long, it could exceed the duration of the message or otherwise affect the total average throughput.   In addition, the process has to be simple for the User.  Given the mobile nature of WPAN devices entering or leaving piconet environment rapid joining or unjoining of the piconet.

#### Values

Identify network join/unjoin procedures proposed by this system.  Note:  Fast synchronization/ detect/link/communicate cycles with respect to a packet period are preferred.  Therefore, the proposal should indicate the min/max/average time frame.

### Device Registration

#### Definition

Ease of use by typical customers requires that the devices register with each other without requiring the help of a system administrator, or special procedure by the user.  Authentication for the purpose of registration is covered in section 3.9.1.  The system should allow the user to configure which class of devices that can be registered without user intervention.

#### Values

Identify device registration process proposed by this system.  Simpler registration processes are preferred.

## Delivered Data Throughput

### Definition

Delivered data throughput is the rate at which the user’s data is passed through the system.  In a simple case, it is the data rate after the protocol overhead is subtracted.    The values presented here assume that a microwave oven or other channel impairment will not be in operation at the same time as the desired signals are transmitted.  If there is an operating microwave oven in the Personal Operating Space (POS) of this device, it is assume that the user has enough control of the POS environment to turn it off when desiring to transmit.

In order to enable several various levels of functionality without setting the requirements too high or too low, it is best to bound the data throughput by a minimum value necessary to add value and determine a goal for achieving the desired high demand applications.  This does not preclude implementations that can achieve values beyond the guideline layout in this criteria document.

These throughput values are based on the needs of desired applications that have been considered in the criteria development.  The applications have been outlined below to allow clarifications of bandwidth needs based the desired functionality.  There are situations where more than one application would be desired on the same channel.  The mixed media transfer section outlines some potential situations to allow better understanding of the overall data throughput desired at any one time.

### Minimum delivered data throughput

#### Definition

The minimum delivered throughput should be 20 Mbps. The 20 Mbps refers to the aggregate data transfer in both directions.  The partition between the two directions should be adaptive.

#### Values

TRUE – Proposed system can deliver a data throughput of at least 20 Mbps.

FALSE – Proposed system can not deliver a data throughput of at least 20 Mbps.

### High end delivered data throughput

#### Definition

Some proposed systems might be able to deliver a data throughput greater than the specified minimum.  Therefore, it would be beneficial to the selection process to consider the high end rate possible by the proposed system.

#### Value

Specify the maximum delivered data throughput possible reliably by the proposed system.

### Breakdown of Application Requirements

This section lays out the data throughput required by different applications.  While this section may not contain all applications that can be handled with this standard, it does document the applications considered in determining the throughput requirement values.  This section is intended to allow CFAs to provide information.

#### File Transfers

File transfers include applications such as Music files (average 3MB) and Image files (average 500KB).   The user expects transfer for as many files as it wants, to be completed within 7 to 15 seconds.  This is driving fast data rates even for asynchronous systems.

#### Email

#### Internet Packet Data

#### Voice

Low quality voice requires 16 Kbps, while high quality voice requires 64 Kbps.

#### Audio

The table below characterizes the data rate for the explored audio formats.

| Application | Format | Average Data Rate (Kbps) | Maximum Data Rate (Kbps) |
| --- | --- | --- | --- |
| Streaming Audio | MP3 |  | 128 |
| Home Entertainment | Dolby Digital (5.1 channels) | 384 | 448 |
|  | 2 Dolby Digital (2x5.1 channels) | 768 | 896 |
|  | SDDS |  | 1280 |
|  | DTS |  | 1536 |
| Standard Audio CD | RedBook Digital Audio |  | 1450 |

Table 1

#### Video

The table below characterizes the data rate for the explored video formats.

| Application | Format | Average Data Rate (Mbps) | Maximum Data Rate (Mbps) |
| --- | --- | --- | --- |
| H.323 Video Conferencing | H.261 |  |  |
|  | H.263 |  |  |
| Internet Streaming | MPEG1 | 1.5 | 2.5 |
|  | Others? |  |  |
| Streaming Video | MPEG4 |  |  |
| DVD | MPEG2 | 4.5 | 9.8 |
| Digital Broadcast Satellite – SDTV | MPEG2 | 3.55 | 7.3 |
|  | MPEG4 |  |  |
| Terrestrial Broadcast – SDTV | MPEG2, 480I* | 3-8 | [TBD] |
|  | MPEG2, 480P* | 6-10 | [TBD] |
| Terrestrial Broadcast – HDTV | MPEG2, 720P | 14-16 | [TBD] |
|  | MPEG2, 1080I | 18 | [TBD] |

*  Note:  480I indicates 480 lines interlaced, while 480P indicates 480 lines progressive scan

Table 2

#### Computer Graphics

Traditional computer graphics systems consist of a computing platform, display and interface devices.  There are opportunities to display the information and use interface devices remotely. Table 3 outlines applications that might drive different bandwidth needs.

The typical computer display information option listed refers to computer usage that do not requires real-time display updates, such as documentation, presentations and other traditional personal computing applications.  Interactive network gaming involves the sharing of control and position information of various players with the game that is running locally on the device that is displaying the results.  Games can also be run on a remote computer while displaying the information in real-time on a large display to promote group participation.  This real-time update requirement drives the need for refresh rates approaching 60 Hz.  The standard and high resolution categorization allows selection based on quality for the implemented data rate determined by each resulting product.

| Application | Format | Average Data Rate (Mbps) | Maximum Data Rate (Mbps) |
| --- | --- | --- | --- |
| Typical Computer Display Information |  | [TBD] | [TBD] |
| Interactive Network Games |  |  |  |
| Standard Quality Game | 640x480 Progressive Scan @ 60 Hz, 24bpp | N/A | 15 |
| High Quality Game | 1024x768 Progressive Scan @ 60 Hz, 24 bpp | N/A | 38 |

Table 3

#### Typical Mixed Media Usage Scenarios

It is likely that some usage scenarios will involve data applications used simultaneously.   Table 4 lays out several scenarios that have been considered in the development of the criteria.  The data throughput required to handle these mixed usage scenarios can be determined by referencing the previous sections outlined in this breakdown.

| Usage Scenarios | Applications |
| --- | --- |
| Home Entertainment Systems (both SDTV and HDTV) | Video (DVD or Satellite or Terrestrial) Sound (Dolby Digital) |
| Internet Gaming | Voice Interactive Network Game MP3 |

Table 4

## Data Transfer Types

### Definition

#### Asynchronous Data

| Data Characteristics | Application |
| --- | --- |
| Bursty Data | Email |
|  | Internet Packet Data |
|  | File Transfer |
|  | Interactive Network Games |
| Bulk Data | High Speed Transfer of Image Data |
|  | Typical Computer Display Information |

#### Isochronous Data

| Data Characteristics | Application |
| --- | --- |
| Real-time | Voice |
|  | Audio |
|  | Video |
|  | Standard Quality Game |
|  | High Quality Game |

#### Mixed Traffic Load Management

Provides mixed traffic load management capabilities for isochronous data (such as video) and asynchronous data.

### Values

Indicated data transfer types handled by the proposed system and any mixed traffic load management capabilities.

## Topology

### General

#### Definition

The topology of the network specifies the type of connections that are supported.  Examples of this are point-to-point, point-to-multipoint, ad-hoc, peer-to-peer, etc.

#### Values

The proposal shall include information about the network topology supported by the proposed system.

### Maximum Number of Active Connections

#### Definition

The number of active connections is defined as the number of low duty cycle nodes in a network that can be concurrently interleaved.

#### Values

TRUE – The network shall be capable of supporting a maximum of at least 7 active connections.

FALSE – The network does not capable of supporting a maximum of at least 7 active connections.

### Ad Hoc Network

#### Definition

An ad-hoc network is one where any two (or more) compliant devices can form a network for data exchange.

#### Values

TRUE – The proposed system supports Formation of Ad Hoc Network (2 or more active nodes).

FALSE – The proposed system does not support Formation of Ad Hoc Network (2 or more active nodes).

### Access to a Portal

#### Definition

A portal is a node in the network that supports the bridging of data from the WPAN to another network, either wired or wireless.

#### Values

TRUE – The proposed network supports access to a portal.

FALSE – The proposed network does not supports access to a portal.

## Reliability

### General Definition

Reliability is the ability of the network to recover from either damage or interference to the network.

### Master Redundancy

#### Definition

If a master/slave configuration is required in the proposed systems, there shall be a method for recovering from the loss of a master.

#### Values

TRUE – Proposed system can recover from the loss of a master.

FALSE – Proposed system can not recover from the loss of a master.

N/A – The proposed system does not support a master/slave mode.

### Loss of connection

#### Definition

In a dynamic environment it is possible for a link to be dropped.  The proposed system shall provide a method for detecting and recovering (when possible) from the loss of a link.

#### Values

TRUE – The proposed system does provide a method for detection and recovering from the loss of a link.

FALSE - The proposed system does not provide a method for detection and recovering from the loss of a link.

## Power Management Types

### Definition

It is important to be able to reduce power consumption for consumer electronic devices.    One method is to use power management and to include protocols that allow methods for sleeping, wakeup, polling, etc.

### Values

The proposals should indicate what power management approaches they support and what the potential power savings are for that approach.

## Power Consumption of MAC controller

### Definition

The MAC controller can be an important contributor to the overall power consumption of the system.  The power consumption is defined as the DC power in mW required by the blocks that implement the MAC functionality in each of the power management states in the protocol.

#### Transmit

The MAC is actively sending data to a remote unit within a packet.

#### Receive

The MAC is actively receiving data from a remote unit within a packet.

#### Sleep

The sleep mode is a low power mode in which data is not being actively exchanged but the network connection is being maintained.  As such it may include periods of transmission and reception as well as low power standby states.

### Value

The proposals shall estimate the power requirements of the MAC implementation.  Because this may be a DSP or a separate ASIC, a range may be given.  Those submitting combination MAC/PHY proposals should provide two sets of power estimates: a MAC layer only and MAC/PHY combined power estimate.  As a minimum the power estimates will include the peak and average power consumption for each of the following three states: transmit, receive and sleep.  Values shall for the MAC (or MAC/PHY) used to calculate the unit manufacturing cost figures in section 2.1.

## Security

It is the desire of P802.15.3 to have security to the same level as Bluetooth requirement or better.

### Authentication

#### Definition

The service used to establish the identity of one station as a member of the set of stations authorized to associate with another station.

#### Values

The proposal should indicate methods used for authentication.

### Privacy

#### Definition

The service used to prevent the content of messages from being read by other than the intended recipients.

#### Values

The proposal should indicate methods used for privacy.

## Quality of Service

### Definition

Many of the types of data to be supported by the network, as specified in section 3.3.4, require guarantees of Quality of Service (QoS).  Potential measures of the QoS are:  jitter, delay, packet loss rates and traffic partitioning.  For example, good quality voice transmission requires less than 20 ms delay and less than 10% packet loss rates.  The specific criteria will be developed in cooperation with the efforts of 802.11e.

### Values

The proposals shall indicate how the QoS requirements for the applications in section 3.3.4 will be met in the proposed system.

# PHY Layer Criteria

## Size and Form Factor

### Definition

Size is important for consumer electronic systems such as PDAs, and cameras.   The smaller the package, the easier it is to embed.   It is important that the final radio system be compatible with accessory formats as well.  Compact flash, type I is the current example of packing requirement.   (It also indirectly sets a power and voltage limit).  Antennas are not considered in the size requirements.  The ability to create Radio modules will be an implementation requirement for regulatory approval and integration reasons.

### Values

The proposal shall indicate the size (LxWxH in mm) of the preferred implementation of the PHY.  The preference is that the size of the PHY should not exceed the size of a CompactFlash Type I card.

## MAC/PHY Throughput

### Minimum MAC/PHY Throughput

#### Definition

Task Group 3 High Rate WPAN (TG3) has determined the minimum throughput between the MAC and the PHY should be 20 Mbps plus the overhead of the MAC. Overhead should be based on needs determined by the MAC layer and is TBD (MAC subgroup shall provide this information).  The 20 Mbps refers to the aggregate data transfer in both directions.  The partition between the two directions should be adaptive.

#### Values

TRUE – Proposed system can deliver at least 20 Mbps throughput at the MAC/PHY interface plus overhead.

FALSE – Proposed system can not deliver at least 20 Mbps throughput at the MAC/PHY interface plus overhead.

### High End MAC/PHY Throughput

#### Definition

Some proposed systems might be able to deliver a MAC/PHY throughput greater than the specified minimum.  Therefore, it would be beneficial to the selection process to consider the high end MAC/PHY throughput possible by the proposed system.

#### Values

Specify the maximum MAC/PHY throughput possible reliably by the proposed system.

## Frequency Band

### Definition

The frequency band is defined as the range of frequencies for which the proposed system can operate.

### Values

Indicate the range of operating frequencies to be used by the proposed system.

## Number of Simultaneously Operating Full Throughput PAN’s

### Definition

The proposed system shall provide for the capability for multiple independent, co-located networks to operate simultaneously.  Each of these networks shall be operating at the minimum required MAC/PHY throughput defined in 4.2.1.

### Values

The proposal will indicate the number of simultaneously operating full throughput WPAN’s in their proposal.

## Signal Acquisition Method

### Definition

The signal acquisition methods are the techniques by which the proposed receiver acquires and tracks the incoming signal in order to correctly receive the transmitted data.

### Values

The proposal should indicate how the physical layer will acquire and synchronize to the incoming packet.  Information may include AGC, AFC, timing, etc.

## Range

### Definition

Based on the 802.15.3 PAR, the proposed system shall be able to initiate a WPAN connection within a 10 meter radius 99.9% of the time.

### Values

Proposals should indicate the range possible with the proposed system.  Provide publicly references available in the open literature that provide the bases for the environment models.

## Sensitivity

### Definition

Sensitivity was defined in 2.2.1 as part of the Signal Robustness description.  It is important for the proposal to specify the sensitivity level used in the determination of the signal robustness criteria.

### Values

The proposal should indicate the power level at which the error criterion is met.  The proposal should also indicate both the PER, and the corresponding BER used in the determination of this value.

## Multi-Path Immunity

### Environment model

The exponentially decaying Rayleigh fading channel model will be used for the comparison of proposed methods.  The model was originally proposed by Naftali Chayat in IEEE P802.11-97/96.  The channel is assumed static throughout the packet and generated independently for each packet.

Figure 5.  Channel impulse response; black illustrates average magnitudes, gray illustrates magnitudes of a specific random realization of the channel; the time positions of black and gray samples are staggered for clarity only.

The impulse response of the channel, hi, is composed of complex samples with random uniformly distributed phase and Rayleigh distributed magnitude with average power decaying exponentially as shown in Figure 5.

hi=N(0,12σk2)+jN(0,12σk2)σk2=σ02e−kTsTRMSσ02=1−e−TsTRMS

where N(0,12sk2) is a zero mean Gaussian random variable with variance 12sk2, and s02=1−e−kTsTRMSis chosen so that the conditionsk2=1is satisfied to ensure same average received power.

It is assumed that the sampling time Ts in the simulation is shorter than a symbol time (or chip time) by at least a factor of four (typically in simulations it is a sub-multiple of the symbol duration). The number of samples to be taken in the impulse response should ensure sufficient decay of the impulse response tail, e.g. kmax=10TRMS/Ts.

### Delay Spread Tolerance

#### Definition

The delay spread tolerance is the value of TRMS for which the error rate criterion is met with the input signal 3 dB above the minimum required sensitivity using the channel model defined in section 4.8.1.  The system shall have a delay spread tolerance of at least 25 ns.

#### Values

TRUE – The proposed system meets the minimum delay spread tolerance

FALSE – The proposed system does not meet the minimum delay spread tolerance

## Power Consumption

### Definition

The power consumption is defined as the total amount of DC power required by the proposed system to operate in either transmit or receive mode.  The power consumption includes all blocks that may be required for the operation of the radio (e.g. voltage regulators, reference oscillators, digital control logic and traditional analog blocks).

### Values

Proposals should indicate the peak and average power in mW necessary to provide the minimum required MAC/PHY throughput.  Values shall be given for both transmit and receive modes for the transceiver used to calculate the unit manufacturing cost figures in section 2.1.

# Evaluation Matrix

These matrices are the summarization of the criteria defined in the previous sections.  As proposals are submitted for consideration, these matrices should be completed based on the proposed system parameters.  All proposals should include the general solution criteria matrix.  If the proposal is a MAC or PHY only submission use only the appropriate MAC or PHY matrix.  Comments can be added by the submitter for specified explains and clarity.

Weighting values are included to inform the proposer of the group think of the IEEE 802.15 Working Group on priority of the indicated criteria.

Note:  Weights have been temporarily removed in July meeting due to an addition of a new criteria.  New weighting will be established through an email ballot before the September meeting.

## General Solution Criteria

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Unit Manufacturing Cost ($) | 2.1 | 8.0 |  |  |
| Interference and Susceptibility | Error: Reference source not found | 6.4 |  |  |
| Intermodulation Resistance |  | 4.8 |  |  |
| Jamming Resistance | Error: Reference source not found | 5.7 | Source 1: Source 2: Source 3: Source 4: Source 5: Source 6: |  |
| Multiple Access | Error: Reference source not found | 7.5 | Scenario 1: Scenario 2: Scenario 3: |  |
| Coexistence | Error: Reference source not found | 7.5 | Source 1: Source 2: Source 3: Source 4: Source 5: |  |
| Interoperability | 2.3 | 7.2 | TRUE FALSE |  |
| Manufactureability | 2.4.1 | 7.0 |  |  |
| Time to Market | 2.4.2 | 5.7 |  |  |
| Regulatory Impact | 2.4.3 | 5.9 | TRUE FALSE |  |
| Maturity of Solution | 2.4.4 | 5.2 |  |  |
| Scalability | 2.5 | 4.9 | Power consumption: Data rate: Frequency Band: Cost: Function: |  |
| Location Awareness | 2.6 | .1 | Resolution: |  |

## MAC Protocol Criteria

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Transparent to Upper Layer Protocols (TCP/IP) | 3.1 | 7.2 | TRUE FALSE |  |
| Unique 48-bit Address | Error: Reference source not found | 5.4 | TRUE FALSE |  |
| Simple Network Join/UnJoin Procedures for RF enabled devices | 3.2.2 | 7.3 |  |  |
| Device Registration | 3.2.3 | 5.8 |  |  |
| Minimum delivered data throughput | 3.3.2 | 7.6 | TRUE FALSE |  |
| High end delivered data throughput (Mbps) | 3.3.3 | 6.4 |  |  |
| Data Transfer Types | 3.4 | 5.5 | Asynchronous Data Isochronous Data Mixed Traffic Load Management |  |
| Topology | 3.5.1 | 4.7 |  |  |
| Min. # of active connections | 3.5.2 | 5.6 | TRUE FALSE |  |
| Ad-Hoc Network | 3.5.3 | 6.4 | TRUE FALSE |  |
| Access to a Portal | 3.5.4 | 5.3 | TRUE FALSE |  |
| Master Redundancy | 3.6.2 | 3.0 | TRUE FALSE NOT APPLICABLE |  |
| Loss of Connection | 3.6.3 | 5.7 | TRUE FALSE |  |
| Power Management Types | 3.7 | 5.4 |  |  |
| Power Consumption of MAC controller | 3.8 | 6.7 | TX: RX: Sleep: |  |
| Authentication | 3.9.1 | 6.7 |  |  |
| Privacy | 3.9.2 | 6.8 |  |  |
| Quality of Service | 3.10 | 6.2 |  |  |

## PHY Protocol Criteria

| CRITERIA | REF. | WEIGHT | VALUE |  |
| --- | --- | --- | --- | --- |
| Size and Form Factor | 4.1 | 6.5 |  |  |
| Minimum MAC/PHY Throughput | 4.2.1 | 7.4 | TRUE FALSE |  |
| High End MAC/PHY Throughput (Mbps) | Error: Reference source not found | 6.2 |  |  |
| Frequency Band | 4.3 | 6.0 |  |  |
| Number of Simultaneously Operating Full-Throughput PANs | Error: Reference source not found | 5.4 |  |  |
| Signal Acquisition Method | Error: Reference source not found | 2.7 |  |  |
| Range | 4.6 | 6.4 |  |  |
| Sensitivity | 4.7 | 3.8 | Power level: PER: BER: |  |
| Delay Spread Tolerance | 4.8.2 | 4.8 | TRUE FALSE |  |
| Power Consumption | 4.9 | 8.4 |  |  |

# 1st Pass Pugh Matrix Comparison Value

To manage the evaluation process of proposals, each proposal will be compared against the following values.  These values were determined through a combination of information from this document and discussions on the conference calls between the May and July meetings.  Consensus has been reached with the people involved on these calls that these values are fair, meet the outlined criteria priories and adhere to the 802.15.3 PAR.

Each proposal will be compared against the following Pugh Matrix comparison values.  To get a numerical ranking of all proposals, values can be assigned to the different value options.  A “-“ can be given a –1 value, “same” can be given a 0 value and “+” can be given a +1 value.

## General Solution Criteria Comparison Values

| CRITERIA | REF. |  | Comparison Values |  |
| --- | --- | --- | --- | --- |
|  |  | - | Same | + |
| Unit Manufacturing Cost ($) as a function of time (when product delivers) and volume | 2.1 | > 2 x equivalent Bluetooth 1 | 1.5-2 x equivalent Bluetooth 1 value as indicated in Note #1 Notes: 1. Bluetooth 1 value is assumed to be $20 in 2H2000. 2. PHY and MAC only proposals use ratios based on this comparison | < 1.5 x equivalent Bluetooth 1 |
| Interference and Susceptibility | Error: Reference source not found | Out of the proposed band: Worse performance than same criteria In band: -: Interference protection is less than 25 dB (excluding co-channel and adjacent channel) | Out of the proposed band: based on Bluetooth 1.0b (section A.4.3) In band: Interference protection is less than 30 dB (excluding co-channel and adjacent and first channel) | Out of the proposed band: Better performance than same criteria In band: Interference protection is less greater than 35 dB (excluding co-channel and adjacent channel) |
| Intermodulation Resistance |  | < -45 dBm | -35 dBm to –45 dBm | > -35 dBm |
| Jamming Resistance | Error: Reference source not found | Any 3 or more sources listed jam | 2 sources jam | No more than 1 sources jams |
| Multiple Access | Error: Reference source not found | No Scenarios work | Handles Scenario 2 | One or more of the other 2 scenarios work |
| Coexistence (Evaluation for each of the 5 sources and the create a total value using the formula shown in note #3) | Error: Reference source not found | Individual Sources: less than 40% (IC = -1) Total: < 3 | Individual Sources: 40% - 60% (IC = 0) Total: 3 | Individual Sources: greater than 60% (IC = 1) Total: > 3 |
| Interoperability | 2.3 | False | True | N/A |
| Manufactureability | 2.4.1 | Expert opinion, models | Experiments | Pre-existence examples, demo |
| Time to Market | 2.4.2 | Available after 1Q2002 | Available in 1Q2002 | Available earlier than 1Q2002 |
| Regulatory Impact | 2.4.3 | False | True | N/A |
| Maturity of Solution | 2.4.4 | Expert opinion, models | Experiments | Pre-existence examples, demo |
| Scalability | 2.5 | Scalability in 1 or less than of the 5 areas listed | Scalability in 2 areas of the 5 listed | Scalability in 3 or more of the 5 areas listed |
| Location Awareness | 2.6 | N/A | FALSE | TRUE |

Note 3:  Total equation for coexistence value calculation.  Individual comparison values (-, same, +) are represented by the following numbers:  - equals –1, same equals 0, + equals +1.  The individual comparison values will be represented as IC in the equation below, with the subscript representing the source number referenced.

Total = 2 * IC1 + 2 * IC2 + IC3 + IC4 + IC5

## MAC Protocol Criteria

| CRITERIA | REF. |  | Comparison Values |  |
| --- | --- | --- | --- | --- |
|  |  | - | Same | + |
| Transparent to Upper Layer Protocols (TCP/IP) | 3.1 | FALSE | TRUE | N/A |
| Unique 48-bit Address | Error: Reference source not found | Not Qualified (required by 802) | Essential | N/A |
| Simple Network Join/UnJoin Procedures for RF enabled devices | 3.2.2 | Extended procedure for joining network | 802.15.1 style join as specified in sections 8.10.6, 9.3.23 and 11.6.5.5 | Enhanced self-configuration of network |
| Device Registration | 3.2.3 | Requires manual configuration | 802.15.1 style registration as specified in sections 8.10.7 and 11.6.5.1-4. | Auto registration based on profile |
| Minimum delivered data throughput | 3.3.2 | < 20 Mbps | 20 Mbps | > 20 Mbps |
| High end delivered data throughput (Mbps) | 3.3.3 | N/A | 40 Mbps | > 40 Mbps |
| Data Transfer Types | 3.4 | Asynchronous only | Asynchronous or Isochronous | Mixed Mode (Asynchronous & Isochronous simultaneously) |
| Topology | 3.5.1 | Point-to-Multipoint only | Point-to-Multipoint & Point-to-Point (with no Peer-to-Peer) | Point-to-Multipoint, Point-to-Point & Peer-to-Peer |
| Max. # of active connections | 3.5.2 | < 7 | 7 | > 7 |
| Ad-Hoc Network | 3.5.3 | FALSE | TRUE | N/A |
| Access to a Portal | 3.5.4 | FALSE | TRUE | N/A |
| Master Redundancy | 3.6.2 | FALSE | TRUE | N/A |
| Loss of Connection | 3.6.3 | FALSE | TRUE | N/A |
| Power Management Types | 3.7 | Does not support power savings modes | Supports 802.15.1 power savings modes as specified in sections 8.10.8.2-4 and 11.6.6.1-5 | Enhanced power savings modes |
| Power Consumption of MAC controller (the peak power of the MAC combined with an appropriate PHY) | 3.8 | > 1.5 watts | Between 0.5 watt and 1.5 watts | < 0.5 watt |
| Authentication | 3.9.1 | No authentication | 802.15.1 style authentication as specified in sections 8.14.4 and 9.3.2 | Enhanced authentication at MAC layer |
| Privacy | 3.9.2 | No encryption | Encryption as specified in 802.15.1 section 8.14.3 and 9.3.6 | Packet encryption |
| Quality of Service | 3.10 | No provisions for QoS | Equivalent to QoS specified in 802.15.1 section 9.3.20 , 10.6.3 and 11.6.6.6 | 802.11e level of QoS |

## Phy Protocol Criteria

| CRITERIA | REF. |  | Comparison Values |  |
| --- | --- | --- | --- | --- |
|  |  | - | Same | + |
| Size and Form Factor | 4.1 | Larger | Compact Flash Type 1 card | Smaller |
| Minimum MAC/PHY Throughput | 4.2.1 | < 20 Mbps + MAC overhead | 20 Mbps + MAC overhead | > 20 Mbps + MAC overhead |
| High End MAC/PHY Throughput (Mbps) | 4.2.2 | N/A | 40 Mbps + MAC overhead | > 40 Mbps + MAC overhead |
| Frequency Band | 4.3 | N/A (not supported by PAR) | Unlicensed | N/A (not supported by PAR) |
| Number of Simultaneously Operating Full-Throughput PANs |  | < 4 | 4 | > 4 |
| Signal Acquisition Method | Error: Reference source not found | N/A | N/A | N/A |
| Range | 4.6 | < 10 meters | > 10 meters | N/A |
| Sensitivity | 4.7 | N/A | N/A | N/A |
| Delay Spread Tolerance | 4.8.2 | < 25 ns | 25 ns - 40 ns | > 40 ns |
| Power Consumption (the peak power of the PHY combined with an appropriate MAC) | 4.9 | > 1.5 watts | Between 0.5 watt and 1.5 watts | < 0.5 watt |

# Annex:  Criteria Definition Clarifications based on Committee Work

As the defined criteria were used by the PHY and MAC sub-committees, several criteria warranted clarification and/or redefinition.  This section is intended to capture the efforts of these committees and the resolution chosen to evaluate the proposals.

## PHY Sub-Committee Contributions

### Delay spread

(reference section 4.8.2)

After reviewing the delay spread tolerance definition in the criteria document, the PHY subcommittee determined that it was not a unique definition that allowed comparisons.  A subcommittee was formed that developed the following definition and criteria:

#### Delay spread tolerance definition:

The delay spread tolerance is the value of T_RMS for which a maximum FER of 1% is met for 95% of the channels generated using the channel model defined in 4.8.1.  The power level at the transmitter is set 14 dB above the level required for a 1% FER in an AWGN channel.  At least 1000 channels should be generated. 512 byte data frames are assumed.  The delay spread tolerance shall be simulated without the use of advanced channel selection and/or diversity (space, frequency, etc.) techniques.  Detailed simulation requirements are defined in section 7.1.1.3.  Note that the channel model in 4.8.1 will generate channels with fading parameters, so at the receiver the signal level will vary from one channel realization to the next channel realization.  The system shall have a delay spread tolerance of at least 25 ns.

To aid in evaluating the various proposals, the PHY subcommittee requested additional specific information for evaluating delay spread tolerance:

FER of the proposed system versus the RF signal level at the receiver input (in dB relative to the RF signal level required for a 1% FER in an AWGN channel) shall be provided.  The delay spread, T_RMS, should be set to 25 ns. The FER must be met for at least 95% of the random channels. The data should be presented in two forms: 1) a graph where the FER is plotted against the RF signal level at the receiver input (in dB relative to the RF signal level required for a 1% FER in an AWGN channel) and 2) the minimum RF signal level (in dB relative to the RF signal level required for a 1% FER in an AWGN channel) required to achieve a 1% FER in a 25 ns delay spread multipath channel.

#### Value:

TRUE - the T_RMS is greater than 25 ns

FALSE - the T_RMS is less than 25 ns

#### Delay spread tolerance simulation requirements:

- Generate 1000 random channels according to 4.8.1, for the desired delay spread T_RMS.

- Apply a signal gain, such that the average RF signal level of the 1000 random channels is 14 dB higher than the RF signal level required to achieve a 1% FER in an AWGN channel. Because the channel model is Rayleigh faded, each instance of the channel may be either above or below the average power.

- Simulate the FER for each of the 1000 random channels. The simulation should include AWGN arising from the RF circuit (thermal noise + noise figure). Perfect symbol and carrier synchronization may be assumed. Equalizers must perform frame-based coefficient adaptation. The simulation should not include advanced channel selection and/or diversity techniques that alter the distribution of the channel model. Use one of the two simulation methods defined below:

  - Direct measurement of FER: Simulate at least 1000 frames per random channel. Each frame should consist of the proposed preamble, header, tail, and 512 bytes of data. Directly calculate the FER for each random channel.

or

  - Measurement of FER by BER: Simulate the BER for each random channel. The measurement may be performed on continuously transmitted data, however equalizer adaptation must be performed on the proposed preamble/header. Convert the BER to a FER, assuming 512 bytes of data, preamble, header, and tail bits.

- Discard the results of the 50 channels (5%) with the highest FER. Find the maximum FER of the remaining channels.

- Repeat steps 1 to 4, for different values of T_RMS. T_RMS values of 10, 25, and 40 ns must be simulated. Additional values of T_RMS may also be simulated to demonstrate the robustness of the system: 50, 75, 100 ns, etc. The delay spread tolerance is the maximum value of T_RMS in which the maximum FER over 95% of the channels is at most 1%. All lower values of T_RMS must also achieve 1% FER.

#### Responses:

The responses by the proposers to this  definition were as follows.  Two cases were considered, one with 90% of the channels and the power level at 10 dB above and the other with 95% of the channels and the power level at 14 dB above.

| Name | Company | T_RMS 90%, 10 dB | T_RMS 95%, 14 dB |
| --- | --- | --- | --- |
| Davis | Motorola | No response | No response |
| de Courville/Skellern | Motorola/Radiata | 50 ns | 50 ns |
| Allen/Carlson | Kodak | No response | 33 ns |
| O'Farrell | Supergold | 30 ns | 33 ns |
| Dabak | TI | 25 ns | 25 ns |
| McCorkle/Rofheart | XtremeSpectrum | 40 ns | No response |
| Karagouz | Broadcom | 150 ns | 105 ns |
| Rios | Lincom | 50 ns | 25 ns |

The subcommittee reviewed the submissions and had no objections to the values provided by the proposers.

### Size and Form Factor

(reference section 4.1)

To aid in evaluating the various proposals, the PHY subcommittee requested additional specific information for evaluating size and form factor:

- Radio functionality/size:

  - How backward compatibility with 802.15.1 spec is achieved, (RF blocks repeated, shared, etc.?)

  - Transmit power, power amplifier back-off, and efficiency at the transmit power

  - Chip area, process technology

- Baseband functionality/size (PHY baseband only):

  - A/D and D/A converter precision, speed

  - Digital filter lengths for pulse shaping

  - Equalizer length (i.e., number of coefficients)

  - Decoder complexity (e.g. type of decoder like convolutional or block)

  - CMOS chip area, gate count and process technology

- Total number of chips and external components for the overall PHY solution

#### Responses

The responses by the proposers to this new definition were as follows:

| Name | Company | IEEE document number |
| --- | --- | --- |
| Davis | Motorola | No response |
| de Courville/Skellern | Motorola/Radiata | P802.15-00/196r3 |
| Allen/Carlson | Kodak | No response |
| O’Farrell | Supergold | P802.15-00/210r2 |
| Dabak | TI | P802.15-00/200r3 |
| McCorkle/Rofheart | XtremeSpectrum | P802.15-00/195r6 |
| Karagouz | Broadcom | P802.15-00/211r4 |
| Rios | Lincom | P802.15-00/198r2 |

The subcommittee reviewed the additional information presented by the proposers and evaluated the size and form factor of the combined MAC and PHY radio.  The antenna is not included in the size estimate.  The ratings used were

| Size and Form Factor | Larger | Compact Flash Type 1 card | Smaller |
| --- | --- | --- | --- |
| Score | - | Same (0) | + |

The results of this evaluation are shown below:

| Name | Company | Score |
| --- | --- | --- |
| Davis | Motorola | -1 |
| de Courville/Skellern | Motorola/Radiata | 1 |
| Allen/Carlson | Kodak | 1 |
| O’Farrell | Supergold | 1 |
| Dabak | TI | 1 |
| McCorkle/Rofheart | XtremeSpectrum | 1 |
| Karagouz | Broadcom | 1 |
| Rios | Lincom | 1 |

### Interference and Susceptibility

(reference section Error: Reference source not found)

The PHY subcommittee clarified that for the purposes of the same rating, the in-band interference protection is less than 30 dB (excluding co-channel and adjacent channel), i.e. the first channel is not excluded.

### Intermodulation Resistance, additional information

(reference section Error: Reference source not found)

The PHY subcommittee also requested that the proposers provide additional information regarding intermodulation resistance.  The request was that the proposers evaluate their systems with intermodulating signals that whose power levels were set relative to the receiver sensitivity.  The test parameters are the same as in section Error: Reference source not found where the term x is calculated as:

x = (minimum specified sensitivity in dBm)+3 dB + y

where y is the relative power of the interferer given in the Table 5.

| Criteria | - | Same (0) | + |
| --- | --- | --- | --- |
| Intermodulation above (sensitivity +3 dB) for minimum required data rate | < 25 dB | 25-35 dB | > 35 dB |

Table 5:  Relative power of intermodulating signal for various scores.

#### Responses

| Name | Company | Score |
| --- | --- | --- |
| Davis | Motorola | -1 |
| de Courville/Skellern | Motorola/Radiata | +1 |
| Allen/Carlson | Kodak | +1 |
| O'Farrell | Supergold | +1 |
| Dabak | TI | +1 |
| McCorkle/Rofheart | XtremeSpectrum | +1 |
| Karagouz | Broadcom | +1 |
| Rios | Lincom | +1 |

### Jamming Resistance

(reference section Error: Reference source not found)

The PHY subcommittee convened a sub-subcommittee to provide a model for the microwave oven to be used for evaluating the criteria for section Error: Reference source not found.  The response was due by 8 October, 2000, after which the proposers will use this to update their self-ratings.  In addition, the subcommittee clarified that the proposers should use the average value, found in section 3.3.4.6, of the MPEG2 streaming of a DVD.

#### Model for microwave oven interference

The microwave oven is transmitting at a power of 100 mW with an active period of 8 ms, followed by a dormant period of 8 ms. That is, during the active period the transmit  power is 100 mW and during the dormant period the transmit power is 0 mW. During the active period, the microwave oven output can be modeled as a continuous wave interferer with a frequency that moves over a few MHz.  At the beginning of the active period, the frequency is 2452 MHz, and a the end of the active period, the frequency is 2458 MHz.  There is a continuous sweep in frequency as the active period progresses in time.

### Coexistence

(reference section Error: Reference source not found)

The PHY subcommittee clarified that the distances referenced in IC1 through IC5 are measured relative to A1 in Figure 4 and that the separation of A1 and A2 is 6 m.

The PHY subcommittee also requested that the proposals perform the same evaluation of the coexistence criteria with the power levels of the 802.15.1 systems in IC1 and IC2 at 100 mW rather than 1 mW.  This information should be submitted to the PHY subcommittee by October 8, 2000 for inclusion in this annex.

#### Responses

| Name | Company | Score |
| --- | --- | --- |
| Davis | Motorola | 0 |
| de Courville/Skellern | Motorola/Radiata | +1 |
| Allen/Carlson | Kodak | +1 |
| O'Farrell | Supergold | +1 |
| Dabak | TI | +1 |
| McCorkle/Rofheart | XtremeSpectrum | +1 |
| Karagouz | Broadcom | +1 |
| Rios | Lincom | +1 |

### Extensions to Proposals

Some of the proposers added extensions to their proposals to improve performance.  Although the PHY sub-committee did not have sufficient time to fully evaluate these enhancements, the sub-committee did provide the following wording to reflect these changes.

#### Karaoguz Proposal

Karaoguz has presented enhancements to the proposal (8 Mbaud, non hopping) which he believes would improve the overall score.  The self ratings would be

- 4.2.1 Minimum MAC/PHY throughput (24, 32 and 40 Mb/s raw) +1

- 4.4 Number of simultaneously operating full throughput PAN's (8) +1

Without adversely affecting any of the other ratings

The PHY subcommittee believes that the proposed enhancements appear to be reasonable, but did not have sufficient time to review them in depth.

#### Allen/Carlson Proposal

Allen/Carlson has presented enhancements to the proposal which they believe would improve the overall scores.  The self-ratings would be

- 4.2.1 Minimum MAC/PHY throughput (33, 44 Mb/s raw) +1

- 4.2.2 High end MAC/PHY throughput (33, 44 Mb/s raw) +1

Without adversely affecting any of the other ratings

The proposed enhancements (coding from O'Farrell and 16 QAM from Dabak) have individually been evaluated by the PHY subcommittee for other proposals.

PHY subcommittee did not have sufficient time to review the impact of the combination of the modes to the system performance, although each of the proposed enhancements have been individually evaluated.

#### O’Farrell Proposal

O'Farrell has presented enhancements to the proposal which he believes would improve the overall score.  The self ratings would be

- 4.2.1 Minimum MAC/PHY throughput +1

Without adversely affecting any of the other ratings

The proposed enhancement (33 Mb/s, MBCK 16-QAM) appears to be reasonable.  However, the PHY subcommittee did not have sufficient time to review the proposed enhancement in depth.

## MAC Sub-Committee Contributions

The TG3 MAC subcommittee found these MAC criteria to be either ill defined or incompletely defined:

1.  Ill defined

- Multiple Access

2.  Incompletely defined:

- Simple Network Join/Unjoin procedures for RF enabled devices

- Power Management Types

- Authentication

- Privacy

- Quality of Service

The TG3 MAC subcommittee found these MAC criteria to be dependent upon the particular MAC/PHY pairings:

- Multiple Access

- Location Awareness

- Minimum Delivered Data Throughput

- Maximum Delivered Data Throughput

Resolution of the identified criteria through the MAC sub-committee work will be identified in the sections below.

### Multiple Access

(reference section 2.2.5)

#### Definition

Multiple Access is the ability of a MAC to efficiently manage each node’s access to a common RF channel allocation without performance degrading delays caused by simultaneous transmissions, when used in a pico-net composed of multiple active nodes.

#### Values

During our evaluation at the interim meeting in Scottsdale the MAC subcommittee agreed to assign a “0” value to the each of the MAC proposals until the MAC subcommittee could agree on  appropriate test cases for the comparison values included in clause 6.2.

### Simple Network Join/Unjoin procedures for RF enabled devices

(reference section 3.2.2)

The TG3 MAC subcommittee determined that the comparison values for this criterion are binary.  Consequently, these comparison values were agreed to:

“-“  Requires an extended procedure for joining the piconet.

“0”  Supports 802.15.1 style join/unjoin procedures as specified in sections 8.10.6, 9.3.23 and 11.6.5.5.

### Power Management Types

(reference section 3.7)

The TG3 MAC subcommittee determined that the comparison values associated with this criterion in section 6.2 required clarification.

#### Values

“-“:  does not support power savings modes

“0”:  supports a centralized power management scheme similar to 802.15.1 as specified in sections 8.10.8.2-4 and 11.6.6.1-5.

“+”:  supports a decentralized power management scheme and provides buffered storage for packet forwarding.

### Authentication

(reference section 3.9.1)

The TG3 MAC subcommittee determined that the values indicated in the comparison values table in clause 6.2 needed clarification.

#### Values

“-“:  no authentication

“0”:  supports Authentication on a per link basis.

“+”:  this value was dropped.  Consequently, this criterion becomes a binary criterion.

### Privacy

(reference section 3.9.2)

TG3 MAC subcommittee determined that the current definition for privacy is sufficient.  However, we determined that the Values indicated in the comparison values table in clause 6.2  needed clarification.

#### Values

“-“:  no encryption

“0”:  supports  Packet Encryption on a per link basis

“+”:  this value has been dropped.  Consequently, this criterion becomes a binary criterion.

### Quality of Service

(reference section 3.10)

TG3 MAC subcommittee determined that the current definition for QoS is sufficient.  However, we determined that the Values indicated in the comparison values table in clause 6.2 needed clarification.

#### Values

“-“:  provides no support for QoS

“0”:  provides support for negotiating this QoS parameters:

      - Latency

      - Peak Bandwidth

      - Jitter

      - Service Types

- No traffic

- Best Effort

- Guaranteed

“+”:  in addition to the basic negotiated QoS parameters, a proposal must provide evidence of support for:

- dynamic bandwidth allocation

- isochronous stream prioritization

- multiple similar isochronous data streams with different QoS requirements

### Location Awareness

(reference section 2.6)

The TG3 subcommittee recognized that each of the currently submitted MACs do not provide this capability. However, we did agree that this capability could be incorporated into the proposed MACs if a PHY were available that could provide the necessary indications.

### Minimum and Maximum Delivered Data Throughput

(reference sections 3.3.2 and 3.3.3)

During our evaluation at the interim meeting in Scottsdale the MAC subcommittee agreed to assign a “0” value to the each of the MAC proposals until the MAC subcommittee could agree on  appropriate definitions and  test cases for the comparison values included in clause 6.2.

Detailed analysis is found in 00354r2P802-15-TG3-MAC-Throughput-Scenarios-Comparison-WrkBk.   Charts with the results are found in 00371r0P802-15-TG3-MIN-MAX-Throughput-Charts.