# Member Name

Sean Middleton

Philips Semiconductors

Target Applications (e.g. sensor, appliances):

HID devices such as keyboards, mice, PC to PC-peripherals, and joysticks

Remote controls

PC enhanced toys (PET)

Security systems + Smoke and heat detectors

Meter reading for electricity, gas, and water

Sensors

Lighting and other on/off, open/close devices such blinds, and fireplaces

Usage scenario (e.g., stick-on sensor in car door jamb):

- HID scenario – a multimedia PC where two or more people want to play a game with a wireless gaming control.

- Remote control scenario – programming a VCR out of line-of-sight or changing the channel on a set-top-box (STB) in another room.

- PET scenario – toy which downloads a song or story of the day when in range of a PC or STB

- Security, smoke and heat detectors scenario – 32 windows, 6 doors, 3 control pads, 5 smoke, 2 heat detectors, and 1 carbon monoxide detectors with a total of 49 nodes.

- Meter reading – meter readers for electricity, gas, and water. Send data to set-top-box for external distribution.

- Sensor scenario – motion detector triggered by a person, vehicle or animal. Send data to set-top-box for external distribution or computer for viewing.

- Lighting scenario – 15 rooms with 3 lights and/or wall outlets per room, 15 blinds, and 2 fireplace gas controls with a total of 62 nodes.

Summary of Characteristics:

| Characteristic | Value or Range |
| --- | --- |
| Data rate (effective, not raw) | 100 kbits/sec |
| Range | 30m |
| Battery life (with normal product usage) | 6 months to shelf-life of battery |
| Latency (time between data ready for sending in appl. to data received by other appl.) | 15 msec (maximum) |
| Nodes per network | 256 |
| Topology (master-slave/peer-to-peer, routing, repeaters) | Master-slave with repeaters |
| Types of traffic (small/big packets, steady/bursty, asynch/isoc) | Small packets, both steady and bursty |
| Security (yes/no/optional) | Optional |
| RF output power (units such as) dBm, subjective such as controllable) | -10 dBm and 0 dBm |
| Duty Cycle (what percentage of time the device has its RX/TX active) | 1% to 30% |
| Temperature ranges (industrial and indoor) | industrial (-40 oC to +85 oC) and indoor (0 oC to +70 oC) |