IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | <title> TG3 Merged MAC Proposal Executive Summary |  |
| Date Submitted | [The date the document is contributed, in the format “21 May, 1999”] |  |
| Source | [Allen Heberling] [ Eastman Kodak Company] [4545 East River Road Rochester, NY 14650-0898] | Voice: [ 716-781-9328 ] Fax: [ 716-781-9733 ] E-mail: [ adheberling@ieee.org ] |
| Re: | [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [This document is an executive summary of the TG3 Merged MAC protocol proposal.] |  |
| Purpose | [The purpose of this document is to: - Provide a summary of what was agreed to during the Tampa TG3 meeting - Summarize the key features from each of the MAC proposals. - Provide a target date by which the TG3 MAC committee intends to complete its integration effort and first draft.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

During the November, 2000 Session of the 802.15.3, TG3 voting members unanimously approved the high rate wireless personal area network (HR WPAN) media access control (MAC) proposal based upon the merger of the Kinney, Parks and Heberling MAC proposals.

The key components of the Kinney  MAC protocol proposal (documents 205r2, 218r1) are that it:

- provides proven support for mobile wireless personal area networks (WPAN).

- enables WPANs to interact with wired infrastructure access points.

- enables a WPAN to temporarily separate into two subnets and then recombine via an efficient reauthentication and reassociation procedure.

- provides proven protocol support for efficient power management policies

The key components of the Parks MAC protocol proposal (documents 208r3, 209r3) are that it supports:

- redundant master coordinators.

- repeater service.

- virtual data streams(Asynchronous and Isochronous).

- negotiating QoS paramenters in a WPAN environment.

- dynamic channel selection and/or migration.

- adaptive FEC and selective retransmission ARQs.

The key components of the Heberling MAC protocol  proposal (document 212r1) are that it provides:

- an efficient Slot Cycle TDMA algorithm for accessing the RF medium.

- support for Isochronous and Asynchronous data streams.

- bounded latencies.

- support for negotiating channel bandwidth allocations.

The goal of the TG3 MAC committee is to integrate by July, 2001 these key MAC components into a unified, draft MAC standard which will address the requirements of a high rate MAC protocol which will support both Isochronous and Asynchronous data exchanges in  HR-WPANs.