IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | QPSK spectral re-growth |  |
| Date Submitted | [4 January, 2001] |  |
| Source | [JPKG] [Mobilian] [11031 Via Frontera, Suite C] | Voice: [858-451-3438x201] Fax: [858-451-3546] E-mail: [gilb@ieee.org ] |
| Re: | [] |  |
| Abstract | [The spectral re-growth of filtered QPSK signals in a non-linear amplifier.] |  |
| Purpose | [To present some measured results to be used to determine the power spectral density for the 802.15.3 draft standard.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Spectral re-growth of QPSK signal

Results for spectral re-growth of QPSK signal measured on a spectrum analyzer.  The signal source is an Agilent E4432B ESG-D signal generator with the following settings:

| Modulation type |  | QPSK |
| --- | --- | --- |
| Modulation rate |  | 11 Ms/s |
| Bits/symbol |  | 2 |
| Filter: |  | Root Nyquist, alpha =0.3, optimized for EVM |
| Data |  | PN23 |
| I/Q scaling |  | 70% |
| Differential encoding |  | Off |

The spectrum analyzer is set to a RBW of 100 kHz, VBW of 1 kHz.  The test setup is as shown below:

The input P1dB was measured using a static sine wave from the ESG-D and varying the input power until the gain fell off by 1 dB.  The results for the spectrum were measured at the center frequency and the following offsets: 5, 8, 9, 10, 11, 12, 13, 14, 15, 20, 22, and 25 MHz.  The graph for various values of P1dB backoff are given below:

A curve fit of the maxim power in the first side lobe as a function of input power is given in the next figure (the equation is y = 1.9*(Pin-P1dB)+21 dB).