ISO/IEC JTC1/SC17		2000-03-10

Cards and personal identification

ISO/IEC JTC1/SC17 N  1695

DOCUMENT TYPE : TEXT FOR FCD BALLOT

TITLE:  - BALLOT ISO/IEC FCD 10373-6 - Identification cards - Test methods - Part 6: Proximity cards

BACKWARD POINTER: N 1556

SOURCE:   SECRETARIAT ISO/IEC JTC1/SC17

STATUS:  This text is submitted for FCD ballot

ACTION ID:  ACT

WORK ITEM:    1.17.2.6

DUE DATE:  2000-07-08

DISTRIBUTION P and L-Members of ISO/IEC JTC1/SC17

JTC1 Secretariat

ISO/IEC ITTF

MEDIUM:  SERVER

NO. OF PAGES:  23

Secretariat ISO/IEC JTC1/SC17, APACS, Mercury House, Triton Court, 14 Finsbury Square, London EC2A 1LQ, England; Telephone +44 (0) 20 7711 6292; Fax: +44 (0) 20 7711 6299; e-mail: freda.bennett@apacs.org.uk

ISO/IEC Form 8 - Electronic

|  | ISO/IEC FCD 10373-6 |  |
| --- | --- | --- |
|  | Date 2000-03-10 | Reference number ISO/JTC 1/SC 17 N 1695 |
|  | Supersedes document SC 17 N 1556 |  |

|  | THIS DOCUMENT IS STILL UNDER STUDY AND SUBJECT TO CHANGE. IT SHOULD NOT BE USED FOR REFERENCE PURPOSES. |
| --- | --- |

| ISO/JTC 1/SC 17 Identification cards and related devices Secretariat: APACS for BSI | Circulated to P- and O-members, and to technical committees and organizations in liaison for: - discussion at - comment by - voting by (P-members only) 2000-07-08 Please return all votes and comments in electronic form directly to the 17 Secretariat at APACS by the due date indicated. |
| --- | --- |

ISO/IEC FCD 10373-6 Title: Identification cards - Test methods - Proximity cards Project: 1.17.2.6

Introductory note: This standard has been developed by SC17/WG8 with oversight for consistency with other SC17 standards on Test Methods being provided by WG1. This part of ISO/IEC 10373 deals with test methods which are specific to contactless integrated circuit(s) card technology (Proximity cards). Part 1 of the standard, General characteristics, deals with test methods which are common to one or more ICC technologies and other parts deal with other technology specific tests. Unless otherwise specified, the tests in this part of IOS/IEC 10373 shall be applied exclusively to Proximity cards defined in ISO/IEC 14443-1 and ISO/IEC 14443-2. The CD ballot was issued as N 1556, the Ballot Result as N 1656 (there were no negative votes) and the Disposition of CD comments as N 1694. Medium: Server No. of pages:

Secretariat ISO/IEC JTC1/SC17, APACS, Mercury House, Triton Court, 14 Finsbury Square, London EC2A 1LQ, England; Telephone +44 (0) 20 7711 6292; Fax: +44(0) 20 7711 6299; e-mail: freda.bennett@apacs.org.uk

ISO/IEC Form 9 - Electronic

|  | Vote on Committee Draft ISO/IEC FCD 10373-6 |  |
| --- | --- | --- |
|  | Date of circulation 2000-03-10 Closing date 2000-07-08 | Reference number ISO/JTC 1/SC 17 N 1695 |

| ISO/JTC 1/SC 17 Identification cards and related devices Secretariat: APACS for BSI | Circulated to P-members of the committee for voting Please return all votes and comments in electronic form directly to the SC 17 Secretariat here, at APACS, by the due date indicated. |
| --- | --- |

ISO/IEC FCD 10373-6 Title: Identification cards - Test methods - Part 6: Proximity cards Project: 1.17.2.6

Vote:

| __ | APPROVAL OF THE DRAFT AS PRESENTED |
| --- | --- |
| __ | APPROVAL OF THE DRAFT WITH COMMENTS AS GIVEN BELOW (use separate page as annex, if necessary) |
| __ | general: |
| __ | technical: |
| __ | editorial: |
| __ | DISAPPROVAL OF THE DRAFT FOR REASONS BELOW (use separate page as annex, if necessary) |
| __ | Acceptance of these reasons and appropriate changes in the text will change our vote to approval |
| __ | ABSTENTION (for reasons below): |

______________________________________

P-member voting (National Body/Acronym):

Date:

Submitted by (Name/Signature):

Secretariat ISO/IEC JTC1/SC17, APACS, Mercury House, Triton Court, 14 Finsbury Square, London EC2A 1LQ, England; Telephone +44 (0) 20 7711 6292; Fax: +44 (0) 7711 6299; e-mail: freda.bennett@apacs.org.uk