IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Non-Collaborative AFH Mechanism |  |
| Date Submitted | 15 June, 2001 |  |
| Source | [Bijan Treister, Hong Bing Gan] [Bandspeed Inc.] [Level 9, 500 Collins St., Melbourne 3000 Australia] | Voice: [+61396146299] Fax: [+61396146699] E-mail: [b.treister@bandspeed.com.au h.gan@bandspeed.com.au] |
| Re: | A mechanism of adaptive frequency hopping for an 802.15.1 network. This revised document is now using the correct IEEE802 template and filename. |  |
| Abstract | [A Non-Collaborative AFH system for implementation in a 802.15.1 / Bluetooth Network. This document is based upon the merged proposal by IPC, TI and Bandspeed (01252r0P802-15_TG2-Merged-Adaptive-Frequency-Hopping.ppt) ] |  |
| Purpose | [To draft a Non-Collaborative AFH clause for the 802.15.2 recommended practice. This document will provide the basis for an Adaptive Frequency Hopping system to be utilized in Bluetooth or 802.15.1 systems. ] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Introduction	0

Mechanism outline	0

Device Identification and mechanism setup	0

LMP Commands	0

LMP_Support_AFH	0

Channel Classification	0

Methods of Classification	0

Packet Loss Ratio	0

RSSI measurements	0

CRC, HEC, Access code loss	0

Carrier Sensing	0

Combination	0

Integrating Slave’s classification data	0

Time at which Classification should take place	0

Classification during Connection State	0

Offline Classification	0

Speed of classification	0

Distribution of Classification data and commands	0

Distribution methods	0

LMP Commands	0

LMP_Available_Channel_Request	0

LMP_Slave_Available_Channel	0

LMP_AFH_Request	0

LMP_Regular_Hopping	0

Mechanism of Adaptive Hopping	0

Mode L	0

Method of support for Broadcast packets	0

Mode H	0

# Introduction

# Mechanism outline

# Device Identification and mechanism setup

## LMP Commands

### LMP_Support_AFH

# Channel Classification

## Methods of Classification

### Packet Loss Ratio

### RSSI measurements

### CRC, HEC, Access code loss

### Carrier Sensing

### Combination

## Integrating Slave’s classification data

## Time at which Classification should take place

### Classification during Connection State

#### Classification by Master

#### Classification by Slave

### Offline Classification

## Speed of classification

# Distribution of Classification data and commands

## Distribution methods

## LMP Commands

### LMP_Available_Channel_Request

### LMP_Slave_Available_Channel

### LMP_AFH_Request

### LMP_Regular_Hopping

# Mechanism of Adaptive Hopping

## Mode L

### Method of support for Broadcast packets

## Mode H