IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG2 Mobilian Draft Text |  |
| Date Submitted | [5 July, 2001] |  |
| Source | [Jim Lansford, Adrian P Stephens] [Mobilian Corporation] [7431 NW Evergreen Pkwy, Hillsboro, OR 97124, USA] | Voice: [ +1 (503) 681-8600 ] Fax: [ ] E-mail: [ jim.lansford@mobilian.com adrian.stephens@mobilian.com ] |
| Re: | 00308r0P802-15_TG2-Stage0_PHY_Model.ppt 00360r0P802-15_TG2-Mobilian-coexistence-proposal.ppt 01164r0P802-15_TG2-Mobilian-Symbol-802.11-Bluetooth.ppt 01172r0P802-15_TG2-Mobilian-Symbol-summary.ppt |  |
| Abstract | <<<missing phy bits>>> This document defines the Mobilian parts of the mechanism for MAC-level collaborative coexistence between 802.11 and 802.15.1. A top-level structure is defined showing the relationship of the TDMA and MEHTA parts of the coexistence mechanism. The MEHTA technique is described in detail. Each packet transmit request is allowed or denied by MEHTA according to the known state of the other protocol stack. The decision is based on defining priorities for packet types and activities within the protocol stacks. |  |
| Purpose | This document contains draft text for those clauses of the 802.15.2 recommended practice that are to be provided by Mobilian Corporation. |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Note, comments by the author that are not intended to be part of the final document are introduced by: “[Mobilian – “.

[ed – this  version contains changes made to r0 during review at the July 2001 TG2 meetings].

# Physical Layer Models

[ed –

Open issues:

should we present this as a quasi-analytic and brute-force simulation? AI:Cypher

should source code be available for both/either of the model? (AI: Mobilian)

]

## IEEE 802.11b (Wi-Fi) in the presence of IEEE 802.15.1 (Bluetooth)

[Mobilian – the outline shows separate sections for 6.2 and 6.4. However, the presentation naturally follows the calculation of mutual interference as presented in this section.  It could be re-organized (with some necessary duplication) to fit the original outline if that is deemed necessary by whatever powers that be!).]

### Introduction

This section describes a model that allows the PER to be calculated for 802.11b and 802.15.1 packets in the presence of mutual interference.

[Perhaps re-structure so that the POS is at a higher layer.  Two subordinate models: analytic and simulation.  New intro to say calculating BER.]

This is the  analytic model – using signal, noise and interference power levels at the receiver to calculate BER.

### Model Concepts

[move up to “structure” section]

The most powerful simplifying concept in this model is the period of stationarity (POS).  This is the period over which the transmissions of the devices being modeled do not change.

By definition, during the POS the transmit power and modulation type do not change, and it is also assumed that the position of the devices (and hence link loss) is constant. So receiving nodes experience constant signal, noise and interference powers from which a BER value can be calculated.

Consider the example shown in Figure 1.  Here an 802.15.1 device transmits two packets. An 802.11b device transmits a single PPDU using 11Mbps modulation type for the PSDU.  The start of the PLCP header overlaps the end of the first 802.15.1 packet.  The end of the PSDU overlaps the start of the second 802.15.1 packet.  There are six periods of stability.  Note that a new POS starts at the end of the PLCP header because the modulation type changes at this point.

Figure 1 - Example showing Periods of Stability

The BER calculated by the model is constant during the POS.

### Model Interface

The model is supplied with device positions and transmission parameters for each POS.  The model calculates the (possibly corrupted) bits received by each receiver during that POS.

The parameters described in Table 1 are supplied to the PHY model for each transmission that is active during a POS.

Table 1 - Transmission Parameters

| Field | Description |
| --- | --- |
| Source Position | Device position specified using Cartesian Coordinates |
| Destination Position |  |
| Modulation Type | Type of modulation used by the transmitter. One of: - 802.15.1 - 802.11b 11Mbps - 802.11b 5.5 Mbps - 802.11b 1Mbps - 802.11b 2Mbps |
| Transmit Power | Transmit power |
| Frequency | Center frequency of transmission |
|  | T |

The output of the PHY model is a BER value at the receiver of each transmission during the POS.

### SNIR Computation

[Ed -  add picture showing where SNIR is measured – i.e. after receive mask].

[Ed – add spectrum factor attenuation section]

The signal to noise and interference (SNIR) is calculated at each receiver of a transmission based on all transmissions active during that POS.

The interference power between an interferer and a receiver is calculated using the transmit power of the interferer, center frequencies of interferer and receiver, transmit and receive masks, path loss model, and an allowance for coding gain dependent on the modulation type of the interferer and receiver.

The signal power between a transmitter and its intended receiver is calculated using the transmit power, transmit and receive masks, and the path loss model.

The transmit and receive masks used are defined in Table 2 and shown in Figure 2.

Table 2 - Transmit and Receive masks

|  | Transmit | Receive |
| --- | --- | --- |
| 802.15.1 | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1; -20. 2; -40. 3; -60. 4 and greater; full attenuation. | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1; -11. 2; -41. 3 and greater; -51. |
| 802.11b | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1 to 11; -30. 12 and greater; -50. | Frequency Offset (MHz); Attenuation (dB). 0; 0. 1 to 12; -12. 13 to 36; -36. 37 and greater; -56. |

|  | Transmit |  | Receive |
| --- | --- | --- | --- |
| 802.15.1 |  |  |  |
| 802.11b |  |  |  |

Figure 2 - Transmit and Receive masks for 802.15.1 and 802.11b

When calculating the effects of an 802.15.1 packet on an 802.11b 5.5Mbps or 11Mbps packet, the CCK coding gain of 8dB is added.

### Path Loss Model

The path loss is given by Equation 1 and shown in Figure 3.

[Cite Kammerman’s submission to .11]. 11/00-162, July 2000.

[add comment that below 0.5m – model is no longer appropriate because of near-field and other effects].

Equation 1- Path Loss

| Path Loss =40.2+20log(d), | d < 8m |
| --- | --- |
| Path Loss =58.5+33log(d8), | d > 8m |

Figure 3- Path Loss (dB) vs distance (m)

### BER calculation based on SNIR

The BER is calculated for each modulation type based on the SNIR at the receiver.  The calculations are shown in Table 3.

Table 3 - BER Calculations

| Modulation Type | BER |
| --- | --- |
| 802.15.1 | 0.5×exp(−SNIR2) [Ed – this might be for orthogonal FSK. Ask Josie] |
| 802.11b 1Mbps | Q11×SNIR [Ed – need to validate references for this, ask Josie] |
| 802.11b 2Mbps | Q5.5×SNIR |
| 802.11b 5.5 and 11 Mbps | See below |

The Q function is defined in section 6.1.6.3.

#### 802.11b 5.5 Mbps BER Calculation

[Ed – change PEW to SER (symbol error rate). Change codewords to symbols]

[Ed – check that there is no normalization.  Is this a bound?]

For 802.11b 5.5 Mbps, the symbol error rate, SER5.5, is given by:

PEW5.5=14×Q8×SNIR+Q16×SNIR

[Ed – do we have a reference for this?  Look at .11b standard to align terminology]

As each symbol encodes 4 bits, the effective BER is

BER=1−(1−PEW5.5)14

#### 802.11b 11 Mbps BER Calculation

For 802.11b 11 Mbps, the symbol error rate, SER11 is given by:

PEW11=24×Q4×SNIR+16×Q6×SNIR+174×Q8×SNIR+16×Q10×SNIR+24×Q12×SNIR+Q16×SNIR

As each symbol encodes 8 bits, the effective BER is

BER=1−(1−PEW11)18

#### Q Function Definition

The Q function is defined as the area under the tail of the Gaussian probability density function with zero mean and unit variance.

Q(x)=12πx∞e−t22dt

#### SNIR Limits

The simulation is simplified by assuming that above a certain SNIR the BER is effectively zero and below a certain SNIR the BER is effectively 0.5.  These limits are defined in Table 4.

Table 4 - Assumed Limits on SNIR

| Receiver | Upper limit on SNIR | Lower limit on SNIR |
| --- | --- | --- |
| 802.11b | 10dB | -3dB |
| 802.15.1 | 20dB | 1dB |

#### BER versus SNIR Results

Figure 4 shows the results of calculating BER for SNIR values in the range –15 to 20dB for each modulation type .

[Ed – need to define where SNIR is referenced.  Needs to be clear where it is measured – after the receive filter.]

Figure 4 - BER results

# Collaborative Mechanism – IEEE 802.11 and 802.15.1 (Bluetooth)

[Mobilian  -  The ordering and numbering of these sections is a draft proposal by Mobilian subject to confirmation by Symbol.]

## Collaborative Mechanism – IEEE 802.11 and 802.15.1

### Introduction (Informative)

The collaborative mechanism provides coexistence of IEEE 802.11 and 802.15.1  by sharing information between collocated 802.11 and 802.15.1 stacks and locally controlling transmissions to avoid interference. No new on-air signaling is required. This mechanism is interoperable with devices that do not include it.

Two techniques are defined:  TDMA and MEHTA .   (fix footnote)

In the TDMA technique, the 802.11 AP and 802.15.1 master are collocated. This device allows the 802.11 and 802.15.1 networks to transmit alternately, defining the time that each has access to the medium. The TDMA technique can support multiple 802.15.1 piconets.  It cannot support 802.15.1 SCO links.

In the MEHTA technique, the 802.11 STA and 802.15.1 node are collocated.  There is no need to be either the 802.11 AP or the 802.15.1 master.  Each attempt to transmit by either the 802.11 or the 802.15.2 stack is submitted to MEHTA for approval.   MEHTA can deny a transmit request that would result in collision. The MEHTA technique can support 802.15.1 SCO links.

### Overall Structure

Figure 5 shows the overall structure of the mechanism.

The 802.11 MAC and 802.15.1 LM + LC entities provide status information to the TDMA control and MEHTA control entities.

The TDMA control entity provides a transmit enable (Tx Enable) signal to each stack.  This is a continuous signal that gates whether each stack can start a new packet transmission.

The MEHTA control entity receives a per-transmission transmit request (Tx Request) and issues a per-transmission transmit confirm (Tx Confirm) to each stack to indicate whether the transmission can proceed. The Tx Confirm carries a status value that is one of: allowed or denied.  The Tx Request and Tx Confirm are discreet signals exchanged for every packet transmission attempt.

Figure 5 – Overall Structure of 802.11 / 802.15.1 Coexistence Mechanism

Support for either technique is optional.   [ed – need to discuss what is optional or not]

### TDMA Technique

[Mobilian – to be provided by Symbol]

### MEHTA Technique

#### Introduction to MEHTA (Informative)

The MEHTA control entity provides per-packet  authorization of all transmissions.

MEHTA uses its knowledge of the duration of 802.11 activity and future 802.15.1 activity a number of slots into the future to predict collisions. When a collision would occur, MEHTA prioritizes transmissions based on simple rules that depend on packet types.

Note, this model is a logical model that executes in zero time.  An implementation needs to be cleverer than that. (reword)

#### Known Physical-Layer Characteristics

[make sure we use .11b, not .11]

The 802.11 PHY operates on a known static channel.  The 802.15.1 PHY hops following a known hopping pattern.   At any time, the 802.15.1 signal can be inside or outside the pass-band

[add explanation of pass-band – reference David’s definitions]

of the 802.11 PHY.  These are the in-band and out-of-band cases, and they affect the probability of a collision .

The different collision cases are summarized in Table 5.

Table 5 – Collision Cases as a Function of Local Activities

| Local 802.11 Activity | Local 802.15.1 Activity |  |  |  |
| --- | --- | --- | --- | --- |
|  | Transmit |  | Receive |  |
|  | In-band | Out-of-band | In-band | Out-of-band |
| Transmit | Transmit | None | Transmit-Receive or None | Transmit-Receive or None |
| Receive | Transmit-Receive or None | Transmit-Receive or None | Receive | None |

The different collision types are defined in Table 6.

Table 6 - Definition of Collision Types

| Collision Type | Definition |
| --- | --- |
| Transmit | Both stacks are transmitting in-band. One or both of the packets will be received with errors. |
| Receive | Both stacks are receiving in-band. One or both of the packets will be received with errors. |
| Transmit-Receive | One stack is transmitting and the other is simultaneously receiving. The received packet is received with errors. |
| None | Any co-incidence of activity in the two stacks does not increase the error rate. |

In the case of “Transmit-Receive or None” collisions,  whether there is a collision or not depends on a number of PHY-related parameters that can include:  transmit power,  received signal strength and the difference between 802.11 and 802.15.1 center frequencies.

An implementation predicts the difference between these collision outcomes based on its knowledge of the operating parameters of its PHY.  This section does not attempt to standardize that decision.  So, based on PHY-layer parameters, an implementation predicts whether a collision occurs.

Implementation constraints can also introduce addition types of “collision” based on simultaneous conflicting demands for hardware resource. For example, a single-antenna system is unlikely to be able to transmit and receive simultaneously.

#### MEHTA Structure

Figure 6 shows the structure of the MEHTA Control Entity.  Each stack has a corresponding control entity to which it submits its transmit requests.  This control entity allows or denies the request based on the known state of both stacks.

Figure 6 – Structure of the MEHTA Entity

#### Known 802.11 State

The MEHTA Control assumes that the following state defined in Table 7 is available from the 802.11 MAC.

Table 7 - Known 802.11 State

| 802.11 State Item | Description |
| --- | --- |
| State [find better name for state] current802.11State | Describes the current activity of the 802.11 MAC in terms of current or expected receive and transmit activity. The decision logic described in 6.2.4.7 requires that the state variable indicate if 802.11 stack is idle, transmitting or receiving. [add note describing effect of actually receiving and expecting to receive] Additional states can be exposed through this interface to support local priority policy as described in section 6.2.4.8. |
| Channel | Channel number |
| End Time | Time of the end of the current activity based on the last duration value received in an MPDU header |

When a transmit request is made from the 802.11 MAC, the following information described in Table 8 is known.

Table 8 - 802.11 Tx Request State

| 802.11 Tx Request Parameter | Description |
| --- | --- |
| Packet Type | Type of the MPDU |
| Duration | On-air duration of the MPDU |

#### Known 802.15.1 State

The MEHTA Control assumes that the state described in Table 9 is available from the 802.15.1 MAC.

Table 9 - Known 802.15.1 State

| 802.15.1 State Item | Description |
| --- | --- |
| State current802.15.2State | Describes the current activity of the 802.15.1 MAC in terms of current or expected receive and transmit activity. The decision logic described in 6.2.4.6 requires that the state variable indicate if 802.15.1 stack is idle, transmitting or receiving. |
| Channel List | List of channels for the current and future slots . |
| Packet Type | Indicates the type of packet predicted for the current and future slots. |
| Duration | On-air duration of the current packet |
| Time remaining | Time remaining in the current slot |

#### 802.11 Control

The purpose of the 802.11 Control entity is to allow or deny transmit requests from the 802.11 MAC.  The Tx Request signal is sent when the 802.11 MAC has determined that it can transmit according to its own protocol – i.e. after any required backoff has completed.

On receipt of a Tx Request signal, the 802.11 Control immediately generates a Tx Confirm signal containing a status value that is either allowed or denied.  Figure 7 defines how the status value is selected.

The effect of a denied result on the 802.11 MAC protocol depends on the access mechanism currently in use.  This is defined in Table 10

[expand first use of acronymns]

Table 10 - Effect of Denied status on the 802.11 MAC

| Access Mechanism | Effect of Tx Confirm (status=denied) |
| --- | --- |
| DCF | The denied result appears to be a transient carrier-sense condition that requires a DIFS time to expire before a subsequent transmit request can be made. The denied result has no effect on the contention window (CW) or retry variables because no transmission has occurred. |
| PCF (as CF-pollable STA) | No transmission from the STA occurs, and the AP can resume transmission after a PIFS. |
| PCF as PC | No transmission from the AP occurs, and the AP can resume transmission after a PIFS. |

Figure 7 - Decision Logic for 802.11 Tx Request

Table 11 defines the conditions examined by the decision logic.

Table 11 - Conditions Examined by 802.11 Tx Request Decision Logic

| Condition | Definition |
| --- | --- |
| Current collision | There is a transmit or transmit-receive collision between the current 802.15.1 activity and the 802.11b transmit request |
| Future collision | There is a transmit or transmit-receive collision between the 802.15.1 activity scheduled for a future slot and the current 802.11 Tx Request. For a collision to occur in a slot, the requested 802.11 transmit activity must continue until at least the start of that slot. |
| 802.15.1 current slot priority > 802.11 packet priority | The priority of the current 802.15.1 activity has greater priority than the 802.11 packet. See section 6.2.4.8. |
| 802.15.1 future slot priority > 802.11 packet priority | The priority of the colliding future 802.15.1 activity has greater priority than the 802.11 packet. 6.2.4.8. |
| Is 802.15.1 currently transmitting? | The 802.15.1 known state is in a transmitting state. |

#### 802.15.1 Control

In response to a Tx Request signal,  the 802.15.1 control immediately generates a Tx Confirm signal containing a status value that is either allowed or denied. Figure 8 defines how the status value is selected.

The effect of the denied result on the 802.15.1 stack is to prevent 802.15.1 transmission during the whole slot (or slot half in the case of scan sequences).

Figure 8 - Decision Logic for 802.15.1 Tx Request

Table 12 defines the conditions examined in the execution of this decision logic.

Table 12- Conditions Examined by 802.15.1 Tx Request Decision Logic

| Condition | Definition |
| --- | --- |
| Response or SCO? | True if the Tx Request packet type is Slave ACL, ID, FHS or SCO |
| Collision? | True if a transmit or transmit-receive collision between the 802.15.1 transmit request and the current state of the 802.11 stack |
| Slave Slot Collision? | True if a transmit-receive collision between the slave response to the 802.15.1 transmit request and the current state of the 802.11 stack |
| 802.11 current state priority > 802.15.1 packet priority ? | The priority of the 802.11 current state is greater than the 802.15.1 Tx Request packet priority. See section 6.2.4.8. |

#### Priority Comparisons

The decision logic that allows or denies a packet transmit request uses a priority comparison between the state of the requested transmit packet and the known state of the other protocol stack.

An implementation defines priority values for each separate state value exposed by its protocol stack,  and for each transmit packet type.

##### Recommended Priority Comparisons

An 802.15.1 SCO packet should have a higher priority than 802.11 DATA MPDUs.

An 802.11 ACK MPDU should have a higher priority than all 802.15.1 packets.

Other priority comparisons are a implementation-specific.

##### Maintaining QoS

A device can optionally monitor QoS by defining metrics (such as PER and delay) per protocol stack.  It can use these metrics to bias its priorities in order to meet locally-defined fairness criteria.

In the case of an 802.11e stack,  the device can record these metrics per traffic class and bias its priorities in order to meet known per traffic class QoS commitments.

Note, an implementation may need additional communication not shown here to decide whether to admit a connection-setup with particular QoS requirements,  given knowledge of QoS commitments in the other protocol stack.

##### Maintaining SCO QoS

An implementation can optionally attempt to maintain SCO QoS so as not to exceed some level of SCO packet loss.  It does this by monitoring the SCO PER and comparing with a threshold.  The priority of the SCO packet is increased when the SCO PER is above the threshold.

Modeled as non-coherent FSK

The results for 802.11b 11Mbps do not show calculated values for SNIR < -2dB due to limitations of the tools used.

Although this document consistently references 802.15.1, not Bluetooth ™, the mechanism is equally applicable to both 802.15.1 and Bluetooth.

MEHTA is the Hebrew word for “Conductor”.

A device normally supports just one of the two techniques.

The word packet is used here to mean an 802.11 MPDU or an 802.15.1 baseband packet.

A collision occurs when packets from the 802.11 and 802.15.1 are transmitted simultaneously resulting in the loss of one or both packets.

The Number of slots into the future is a local matter.