IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3 MAC Frame Exchange Clause |  |
| Date Submitted | [14 July, 2001] |  |
| Source | [Bill Shvodian] [XtremeSpectrum] [Vienna, VA 22182] | Voice: [ 703-269-3047 ] Fax: [ 703-749-0248 ] E-mail: [ bshvodian@xtremespectrum.com ] |
| Re: |  |  |
| Abstract | [Submission for the Frame Exchange Clause for the MAC layer of the 802.15.3 standard ] |  |
| Purpose | [This is the proposed text for the Frame Exchange clause] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

8.1.3 Frame Exchange Sequences

Only Association frames and TBD Authentication frames shall be transmitted during the CAP.  Additionally, Channel Time Request commands can be transmitted in the CAP if a DEV does not have any available GTS slots to transmit them in.  All other frames shall always be transmitted in assigned slots during the CFP.  Except for immediate ACKs, the PNC shall never transmit during the CAP since it can always assign itself a GTS slot.

| Sequence | Frames in Sequence | Usage |
| --- | --- | --- |
| Associate-ACK | 2 | Station associates with PNC |
| TBD Authentication - ACK | 2 | TBD Authentication process |
| Channel Time Request - ACK | 2 | Station Requests initial GTS slot(s) |

Figure 1 CAP Frame Sequences

| Sequence | Frames in Sequence | Usage |
| --- | --- | --- |
| Beacon(bc) | 1 | Beacon at start of superframe |
| Frame(bc/mc) | 1 | Broadcast or Multicast frame |
| Frame(dir) | 1 | Directed Frame with no ACK or delayed ACK |
| Frame(dir)-ACK | 2 | Directed Frame with Immediate ACK policy |

Figure 2 CFP Frame Sequences

During an assigned GTS slot, a station can transmit as many frames as will fit in the GTS slot, allowing enough time for an ACK if the frame is using immediate ACK.