IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Empirical Study For Coexistence of Bluetooth & IEEE802.11b |  |
| Date Submitted | 1st,July’2001 |  |
| Source | [Vinay Mitter, Dr.Ivan Howitt, Jose Gutierrez] [University Of Wisconsin-Milwaukee] | Voice: [ 1-858-845-7617 ] E-mail:[ vmitter@qualcomm.com ] |
| Re: | [ Submitted as Contribution for Recommended Practices IEEE802.15 TG2 based on guideline document IEEP802.15-01/239r0] |  |
| Abstract | A Empirical Study for Co-channel & Adjacent Channel Interference for Bluetooth (BT) & IEEE802.11b based WLAN (WLAN) |  |
| Purpose | [ Submitted for Discussion to be included in Recommended Practices] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Chapter –A Empirical Study for Co-channel & Adjacent Channel Interference for Bluetooth (BT)  & IEEE802.11b based WLAN (WLAN)

A.1 Introduction

A.2 Co-Channel & Adjacent Channel Effect of BT on WLAN

A.2.1 Introduction

A.2.2 Test Configuration

A.2.3 Measures of Performances

A.2.4 Empirical Results

A.2.5 Analytical Model for Interference Immunity of WLAN for BT Interference

A.2.6 Comparison between Analytical & Empirical Results

A.2.7 Conclusion

A.3 Co-Channel & Adjacent Channel Effect of WLAN on BT

A.3.1 Introduction

A.3.2 Test Configuration

A.3.3 Measures of Performances

A.3.4 Empirical Results

A.3.5 Analytical Model for Interference Immunity of BT for WLAN Interference

A.3.6 Comparison between Analytical & Empirical Results

A.3.6 Performance of Various BT Packets in Presence of WLAN Interference

A.3.7 Comparison between Analytical & Empirical Results

A.3.8 Conclusion

A.4 Summary

Chapter –B Empirical Results to Evaluate Co-existence for Specific Scenarios as Defined by IEEE802.15 TG2

B.1 Introductions

B.2 Test Procedure

B.3 Effect of WLAN on BT

B.3.1 Test Configuration

B.3.2 Measures of Performances

B.3.3 Experimental Results

B.3.4 Mapping Empirical Results into Topologies

B.3.4.1 Scenario

B.3.4.2 Propagation Model

B.3.4.3 Coexistence Analysis

B.3.5 Conclusions

B.4 Effect of BT on WLAN

B.4.1 Test Configuration

B.4.2 Measures of Performances

B.4.3 Experimental Results

B.4.4 Mapping Empirical Results into Topologies

B.4.4.1 Scenarios

B.4.4.2 Propagation Model

B.4.4.3 Coexistence Analysis

B.4.5 Conclusion

B.5 Summary