IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Clause 14.3 Adaptive Frequency Hopping |  |
| Date Submitted | 13 – Jul – 2001 |  |
| Source | [B. Treister et. al., KC Chen et. al., A. Batra et. al.] [IPC, TI, Bandspeed] [refer to whitepages] | Voice: [ ] Fax: [ ] E-mail: [ ] |
| Re: | [If this is a proposed revision, cite the original document.] [If this is a response to a Call for Contributions, cite the name and date of the Call for Contributions to which this document responds, as well as the relevant item number in the Call for Contributions.] [Note: Contributions that are not responsive to this section of the template, and contributions which do not address the topic under which they are submitted, may be refused or consigned to the “General Contributions” area.] |  |
| Abstract | [Adaptive Frequency hopping Clause 14.3.] |  |
| Purpose | [Description of what the author wants P802.15 to do with the information in the document.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

14.3.1	Introduction	3

14.3.2	Device Identification and Mechanism Setup	3

14.3.3	Channel Classification	4

14.3.3.1	Methods of classification	4

14.3.3.1.1	Packet loss ratio	4

14.3.3.1.2	RSSI measurements	5

14.3.3.1.3	Transmission sensing	5

14.3.4	Procedures of Classification	5

14.3.4.1	Block Channel Classification	6

14.3.4.2	Integrating Slave’s Classification Data	6

14.3.4.3	Classification during Connection State	7

14.3.4.4	Offline Classification	7

14.3.4.5	Slave’s classification data	7

14.3.4.6	Master’s classification	8

14.3.4.7	Distribution Methods	8

14.3.4.7.1	Uni-cast	8

14.3.4.7.2	Broadcasting	8

14.3.5	Mechanism of adaptive frequency hopping	9

14.3.5.1	Mode H	9

14.3.5.1.1	Partition sequence generation	10

14.3.5.1.2	Partition mapping	13

14.3.5.2	Mode L	15

14.3.5.2.1	AFH Box	15

14.3.5.2.2	Re-map Function	16

14.3.5.2.3	Method of producing RANDW	17

14.3.5.2.4	Method of Support for Broadcast Packets	17

14.3.6	LMP Commands	18

14.3.6.1	AFH mode request	18

14.3.6.2	Request of Slave’s Classification List	19

14.3.6.3	Slave’s Classification List	20

14.3.6.4	AFH mode start/terminate [Mode H]	20

14.3.6.5	Adaptive Hopping Request [Mode L]	23

14.3.6.6	Return to Regular Hopping Request [Mode L]	24

14.3.6.7	AFH mode check [Mode H]	25

Device Identification

Channel Classification

Classification Information exchange

Adaptive Hopping

1

2

3

4

Figure 1: Adaptive hopping outline

Introduction

Generic Implementation diagram to be put here

Adaptive frequency hopping (AFH) is a non-collaborative mechanism to enable the coexistence of IEEE 802.15.1 (Bluetooth) devices with frequency static devices in the 2.4 GHz ISM band, such as IEEE 802.11b (WLAN). This mechanism dynamically changes the frequency hopping sequence in order to avoid or minimize the interference seen by the 802.15.1 device.

The mechanism should be placed between the original hop selection kernel and the frequency synthesizer, as shown in the  figure 14.3.x.  The new hopping sequence is generated via a mapping from the original hopping sequence.  Before the hopping sequence can be changed, the quality of transmission for each channel needs to be classified.  Furthermore, the negotiation and information exchange are done by LMP messages.

The mechanism has been divided into four major sections;

- device identification,

- channel classification,

- mutual exchange of channel classification and

- mechanism of adaptively hopping.

### Device Identification and Mechanism Setup

This step allows the Master to know if the connecting device supports AFH, and other parameters of the AFH mechanism.

Refer to the LMP commands at the end of the section.

### Channel Classification

Ed note(HK): This clause is not clause 1.  This clause is split from  clause 14.3 AFH.

Channel classification is required in both of the two non-collaborative mechanisms.  Adaptive packet selection and scheduling adapts the packet types and transmission timing to the channel condition of the current hopping channel. Adaptive frequency hopping generates the new hopping sequence based on the result of channel classification.

The goal of channel classification is to determine the quality of each channel. The major concern of the quality should be interference.  The channel classification mechanism is left for implementation, and several alternatives are listed below.  The exchange of channel classification information should follow the LMP format and procedure defined in 14.3.6.

#### Methods of classification

The sections below describe methods for channel classification. There are a variety of suggested methods described here, which may be used separately or together. Once the channels have been classified, the classification list will be used to compile a final list of ‘good’ and ‘bad’ channels. The devices may then (after suitable agreement) adaptively hop based on this classification list.

These methods should use time based averaging to avoid incorrect classification due to instantaneous disturbances (e.g. other frequency hoppers).

##### Packet loss ratio

The quality of transmission may be determined by the packet loss ratio.  A packet may be considered lost due to failure to synchronize the access code, CRC error, or HEC error.

A packet loss is declared if either: the access code correlator fails, the HEC fails or, for a payload bearing packet the CRC fails. By measuring the ratio of erroneous packets to received packets, it is possible to compile a list of PLRs for each of the channels.

Ed Note: Requirement for detailed explanation? Maybe superfluous. Query about use of Access code for PLR measurements.

Ed Note: Is it Packet Loss Rate, or Ratio?

At the expiration of the classification quantum, a channel is declared ‘bad’ if the PLR exceeds the system defined threshold. The threshold is vendor specific.

Similarly, the Slave may also compute some classification on the received packets. Each time that a packet is received by a Slave (requiring that both the access code and header be received correctly) the CRC on the payload may be checked. If the CRC is correct, the packet has been received correctly, otherwise the packet is declared as lost. In the same way, the Slave may compute the packet loss ratio and apply a threshold to compile the classification list.

Ed note: Various systems should share the same threshold values, which should be decided and agreed upon first.

##### RSSI measurements

The reason for transmission failure may be determined by RSSI.  If RSSI is high and an error is detected or a packet is lost, it is likely to suffer from interference.

In time slots where no response is expected, the Master can monitor the Received Signal Strength. The averaged RSSI for each channel is recorded and at the end of the classification time a threshold is applied. The threshold is vendor specific. This then allows for the classification list to be compiled.

##### Transmission sensing

Transmission sensing spans a wide range of signal detection schemes. Energy detection is simple and useful regardless of the interference types.  Carrier sensing is more robust and helps to classify the type of the interference. Signal analysis and parameter extraction give more reliable interference identification.

### Procedures of Classification

This section describes the time at which classification should take place and suggests practices to adhere to during the classification period. Classification is a period of time in which some ‘bad’ channels should be used, either to ensure that they are still ‘bad’ or check whether the interferers at that channel have disappeared. In any case, the throughput at the time of classification will be degraded because of the use of these ‘bad’ channels.

Ed note: Use ‘good’ and ‘bad’ or something else

#### Block Channel Classification

Ed Note: This scheme is under further investigation

To reduce the time that classification will take, it is possible to reduce the number of measurements required at each channel. The procedure is to group channels into blocks and classify the blocks instead of the channels. This will, however, compromise the accuracy of the measurements at each channel.

Using the PLR classification method as an example, we may suggest that the requirements be as follows:

NC	= number of channels (79 or 23), depends on mode

NBLK 		= new channel block size where NBLK∈0,...,NC−1

PLRNC		= packet loss ratio on each of the NC channels where PLRNC∈ℜ[0,1]

PLRNCBLK	= packet loss ratio on each of the NCBblocks where PLRNCBLK∈ℜ[0,1]

thus:

PLRNCBLK[k]=n=0NBLK−1PLRNCk⋅NBLK+nNBLK0≤k<NCNBLKn=0NCmodNBLKPLRNCk⋅NBLK+nNCmodNBLKk=NCNBLK,if(NCmodNBLK0)

the resolution of the packet loss ratio is less accurate per channel, however the time required to complete the classification might be reduced by a factor of NBLK.

#### Integrating Slave’s Classification Data

The Slave may classify channels based on of the methods described in Section ??. This section discusses how the Master may use the classification information from multiple Slaves to compile a list of ‘good’ and ‘bad’ channels. The method of distributing this data is described later.

There may be up to seven active Slaves in a piconet, and each may support the function to produce a classification list. Once these classification lists have been received by the Master, they should be integrated into the final classification list which will be used during adaptive hopping.

Si,j 	= Slave i's assessment of channel j, either ‘good’ (binary ‘1’) or ‘bad’ (binary ‘0’)

Mj	= Master’s assessment of channel j, either ‘good’ (binary ‘1’) or ‘bad’ (binary ‘0’)

NC	= number of channels (79 or 23), depends on mode

NS	= number of Slaves which have sent back their classification data

where the quality of channel j is given by:

Qj=Mj+i=1NSSi,jNS+11≤NS70≤j<NC

To determine if indeed a channel is bad, a threshold should be applied to Qj to determine if the quality of channel j is high enough.

The Master then compiles the final list of ‘good’ and ‘bad’ channels to be distributed to every supporting device in the piconet.

#### Classification during Connection State

During the classification period it is advantageous to use single slot packets (such as DM1 or DH1 packets). This will increase the number of packets that can be used for the channel classification measurements and decrease the likelihood of an incorrect classification. Using such packets will allow for the device to dedicate a much shorter period of time to classification.

#### Offline Classification

Offline classification takes place at a time in which there is no connection with other devices. This classification may involve background RSSI measurements. These measurements should be completed quickly as to allow for the reduction of the classification interval.

To implement this kind of classification, the Master would typically put the network on hold and start scanning the channels as described above. Once the channels have been scanned for a long enough amount of time a threshold may be applied to the measurements, and those channels which exceed the threshold will be deemed to be ‘bad’ channels.

Ed Note: Put classification outside AFH for use by Packet scheduling etc.

#### Slave’s classification data

A Slave may perform channel classification and send the classification data to the Master when it is requested by the Master.   Each channel is classified as one of the two types: “good” or “bad”.  The transmission of slave’s classification data should follow the LMP format and procedure defined in 14.3.6.

#### Master’s classification

Master should perform channel classification.  Master may collect slaves’ classification data.  Master should make the finial decision for the channel classification of the piconet.  The final decision should be transmitted to the slaves and should follow the LMP format and procedure defined in 14.3.6.

In Mode H, each channel is classified as one of three types: good, bad or unused.  Thus we have:

- A list of good channels, where NG is the number of good channels,

- A list of bad channels, where NB is the number of bad channels,

- A list of unused channels, where NU is the number of unused channels,

where NG + NB + NU = N = 79(23).

Ed note: need some more explanation on unused channels. What is the advantage of having three types of channels? Or the advantage of the different operation mode with some simulation data?

In Mode L, each channel is classified as one of the two types: good and bad.  Thus we have:

- A list of good channels, where NG is the number of good channels,

- A list of bad channels, where NB is the number of bad channels,

where NG + NB = N = 79(23).

#### Distribution Methods

This section relates to the method of sending the LMP commands, which command the Slaves to start or stop adaptively hopping. These LMP commands are described further in Section 14.3.6.

Ed note (all): The decision to use either method is implementation dependent (non critical) - input from task group on mechanism requirements

##### Uni-cast

Each supporting Slave is sent requests to start and stop adaptive hopping one by one. This allows for the Master to ensure that the communicating Slave has indeed received the request and has started the appropriate action. In the case where a Slave does not support the AFH mechanism, the Master will not send to it these requests.

This is the most robust mechanism of distributing the AFH commands to Slaves since the Master may verify that no errors were encountered in the transaction.

##### Broadcasting

Another method of distributing these commands to the Slaves is by broadcasting the requests for adaptive hopping. Broadcast packets sent by the Master use the AM_ADDR of ‘000’. The Slaves that support the AFH mechanism will start adaptively hopping immediately using the received channel classification list. The Slaves that do not support the AFH mechanism will continue hopping as normal and ignore the broadcast packet. The Master knows ahead of time which Slaves support adaptively hopping.

This is a less robust method of distributing the AFH commands to the Slaves, since the Master cannot verify that no errors were encountered in the transaction. It is however a faster method of distributing the commands.

### Mechanism of adaptive frequency hopping

Ed note: Other modes of operation (apart from Connection state) need to be investigated for interoperability with the AFH mechanisms)

The mechanisms of adaptively hopping relate to the hopping system itself and how it must be adapted to support the AFH system. There are two modes of operation, Mode L and Mode H. These two modes exist to support restrictions on frequency hopping systems in the United States of America (at the time of writing).

The goal of adaptive frequency hopping is to avoid using bad channels if possible.  However, a regulatory body always sets a lower bound on the minimal number of channels in the hopping sequence. System robustness also requires a minimal number of hopping channels.  Depending on the number of available good channels, some bad channels may have to be placed in the hopping sequence.  Mode H is thus designed to optimally arrange good channels and bad channels in the hopping sequence for supporting different types of traffic.

Ed note: Clarification on FCC restrictions, and a discussion here?

Mode L is a mode in which the existing Bluetooth selection kernel is used in the usual manner. When the output of the selection kernel is a ‘bad’ channel, the ‘bad’ channel is replaced with a ‘good’ channel. This allows for enhanced and legacy units to operate in the same piconet with support for all modes of traffic.

#### Mode H

Frequency synthesizer

Partition mapping

Original hopping sequence generator

RF input signal

Partition sequence

Ed note: An explanation paragraph for the above diagram Mode H AFH mechanism should be placed here.

The principle of mode H is to optimally support the traffic requirement considering the possibility of having some bad channels in the hopping sequence.  Furthermore, mode H can support the use of only good channels in the hopping sequence by setting the number of bad channels to be used to zero.  For an ACL link, throughput is the main consideration.  Good channels are grouped together as a large window of channels.  For the SCO link, good channels are arranged to match the periodic reserved SCO slots.  A level of QoS is thus guaranteed in interference environment.

##### Partition sequence generation

###### Structure of partition sequence

The elements in the partition sequence consist of a binary set {0, 1}, where 1 represents the partition of good channels, and 0 represents the partition of bad channels.  The partition sequence is periodic with superframe.  The duration of each superframe is Ns slots.  In order to prevent clock wrap-around problems and align with SCO slot reservation, an initialization flag in the LMP setup message indicates whether initialization procedure 1 or 2 is being used.  The slave should apply the initialization method as indicated by the initialization flag. The master uses initialization 1 when the MSB of the current master clock (CLK27) is 0; it uses initialization 2 when the MSB of the current master clock (CLK 27) is 1.  The start of each superframe is at the slot where master clock CLK satisfies the following equation:

CLK27-1 mod Ns = 0			for initialization 1

(CLK27,CLK26−1) mod Ns = 0	for initialization 2

###### Traffic parameters

For traffic parameters, four fields are defined and transmitted through LMP message.  The definitions are listed in the next table.

| Field Name | Size | Value | Meaning | Conditions |
| --- | --- | --- | --- | --- |
| Link type (LT) | 1 bit | 0 | ACL only |  |
|  |  | 1 | SCO and ACL |  |
| K | 8bit | 1-255 | ACL window factor | LT=0 |
| Frame type (FT) | 1 bit | 0 | HV2 | LT=1 |
|  |  | 1 | HV3 |  |
| Reservation indication (RI) | 3 bit | XX1 | Slots with Dsco=0 and 1 are reserved | LT=1 and FT=0, or LT=1 and FT=1. |
|  |  | X1X | Slots with Dsco=2 and 3 are reserved |  |
|  |  | 1XX | Slots with Dsco=4 and 5 are reserved | LT=1 and FT=1. |

###### Partition sequence for ACL link

The sequence is shown in the next figure.

| Good channels | Bad channels | Good channels | Bad channels |
| --- | --- | --- | --- |
| Superframe Superframe RG | RB | RG | RB |

The value of the partition sequence at the l-th slot in a superframe is defined by

s(l)= 10 if 0≤l<RGotherwise

where

l= CLK27−1modNs(CLK27,CLK26−1)modNs for initialization 1for initialization 2

Ns= RG +RB is the length of the superframe.

RG=2*K* NG is the length of the good window.

RB=2*K* NB is the length of the bad window.

NG is the number of channels in the good partition.

NB is the number of channels in the bad partition.

K is the ACL window factor.

###### Partition sequence for SCO link

The SCO link is a full-duplex link, where a slave slot always follows a master slot.  Thus a master slot and the corresponding slave slot are considered as a minimal allocation unit (MAU).  The SCO reservation slots have the period of Tsco slots, and each period of Tsco slots is considered as a frame.  As shown in the figure below, a superframe is composed of Ls frames, while a frame is composed of Lf MAUs.

Superframe

Superframe

Superframe

0

2

1

Lf -1

Frame

0

2

1

Ls -1

MAU

The value of the partition sequence at the k-th MAU of the j-th frame in a superframe is defined as s(j,k) and calculated as shown in the next figure.

Distribution Unit

Arrangement Unit

j

k

MG

mG(j)

RI

s(j,k)

MG is the number of good MAUs in a superframe,

where

MG=Lf×Ls×NGNG+NB.

Lf= 23 ,FT=0,FT=1

Ls=NG+NB

The distribution unit is to distribute the MG good MAUs into the LS frames.  The j-th frame should have mG(j) good MAUs, where

mG(j)=MGLS+d(j)

d(j)= 10 ifjmodC==0orjmodC==1andjC<Eotherwise

R=MGmodLs is the number of residual good MAUs.

C=LsR is the minimized max spacing between the residual good MAUs.

E=R−LsC

The operation of the arrangement unit is shown in the next table, which follows the rules that

- If RIm=1 and RIn=0, then the m-th MAU has higher priority than the n-th MAU.

- If RIm=RIn and m<n, then the m-th MAU has higher priority than the n-th MAU

FT=0

| MG(j) | RI1-0 | k=1 | k=0 |
| --- | --- | --- | --- |
| 0 | xx | 0 | 0 |
| 2 | xx | 1 | 1 |
| 1 | 00 or 01 or 11 | 0 | 1 |
|  | 10 | 1 | 0 |

FT=1

| MG(j) | RI2-0 | k=2 | k=1 | k=0 |
| --- | --- | --- | --- | --- |
| 0 | xxx | 0 | 0 | 0 |
| 3 | xxx | 1 | 1 | 1 |
| 1 | 000 or 001 or 011 or 101 or 111 | 0 | 0 | 1 |
|  | 010 or 110 | 0 | 1 | 0 |
|  | 100 | 1 | 0 | 0 |
| 2 | 000 or 001 or 011 or 010 or 111 | 0 | 1 | 1 |
|  | 101 or 100 | 1 | 0 | 1 |
|  | 110 | 1 | 1 | 0 |

##### Partition mapping

Partition mapping is a method to select one channel from the desired partition.  This selection should be uniform among all channels in this partition.  That is, each channel should have the same probability of selection on average.

###### Pseudo-random mapping

The pseudo-random mapping device maps the selected channel from the original hopping sequence into one channel of the partition specified by the partition sequence.  The block diagram is shown in the accompanying figure.  The content of each channel table is a list of the channels of this partition.  The channels are listed with their corresponding channel number in ascending order.

Mod

Size of partition

Good

Nj

Selected channel number of original hopping sequence (0~N-1)

Current partition = j

(from partition sequence)

shifter signal

Mapping table of this partition

Bad

The averaging shifter signal equalizes the channel utilization after partition mapping.  This shifter signal is a set of counters, one counter for each partition.  The counter of the j-th partition counts periodically within 0,1,2,...(Nj−1), where Nj is the number of channels of this j-th partition.  Only the counter of the selected partition counts to the next value and its content is output as the value of the shifter signal.

###### Bypass for sequence similarity

To keep the similarity between the original sequence and the mapped sequence, we want the following characteristic: if the channel of the original hopping sequence is already in the desired partition, then it should remain unchanged.  This rule is summarized in the table below.

| Channel in the original hopping sequence | Desired partition specified by the partition sequence | Action |
| --- | --- | --- |
| Good | Good | Bypass |
| Good\Unused | Bad | Mapping |
| Bad \Unused | Good | Mapping |
| Bad | Bad | Bypass |

###### Setting contents of the counters

The values of the counters for the shifter signal should be easy to synchronize between the master and the slave.  For the j-th good slot in a superframe, the counter for the good partition should be j mod NG; for the j-th bad slot in a superframe, the counter for the bad partition should be j mod NB. The beginning slot is treated as j = 0.

Ed note: There should be unified typing of the master/slave and the Master/Slave throughout the text.

#### Mode L

Selection

Box

28

27

UAP/LAP

CLOCK

fgen

23/79 Mode

AFH

Box

fadp

AM_ADDR

CLKE12-1 / CLKN12-1

12

Figure 6: Mode L system overview.

In this mode of operation, re-mapping occurs at the output of the Bluetooth selection kernel. The output of the selection kernel (termed fgen ) is fed into the AFH unit. The output of the AFH unit is the frequency which will be used for transmission or receiving at the current slot.

Legacy units (which do not include the AFH Box) will map the output frequency fadp directly to fgen. These devices are fully supported by a Master implementing AFH.

##### AFH Box

There are two control inputs to the AFH Box; the input AM_ADDR is a standard identifier that relates to the device that the unit is addressing. This control input has used only in the Master device. The third input is twelve clock bits which are CLKE12-1 (for the Slave) and CLKN12-1 (for the Master). This input determines the addressing of the good channels, which will be described later.

The AFH Box re-maps the input fgen as follows:

for the Slave:

fadp=remap(fgen)if  fgen∈ 'bad' Channelsfgen             otherwise

and for the Master:

fadp=remap(fgen)if  fgen∈ 'bad' Channels; if  AM_ADDR∈AFH Supportfgen             otherwise

That is, the output from the Selection Box is remapped if it is one of the ‘bad’ channels, and the communicating device supports and is using the AFH mechanism. The Master must check to see if the communicating Slave supports AFH, if the communicating Slave does not support the AFH mechanism then fadp is not remapped. The information about each Slave should be stored from the device identification section.

Ed note: Remapping functions for both modes, even the modes themselves,  should investigate the possibility of merging them together. Also the remapping function needs to be renamed.

##### Re-map Function

The re-map function is used when the device being communicated to is using AFH and the current output channel (from the selection kernel) is a ‘bad’ channel.

A ‘bad’ channel will be remapped to a ‘good’ channel from the good channel bank. This re-mapping is non-static, ‘bad’ channels are not remapped to the same ‘good’ channel every time. This ensures that all of the ‘good’ channels are still used evenly.

fgen

remap(fgen)

RANDW

12

Mod NG

.

.

GNG

GNG-1

GNG-2

GNG-3

GNG-4

GNG-5

GNG-6

GNG-7

.

G1

G2

G3

G4

G6

G5

G7

Good channel bank

Figure 7: Mode L channel re-mapping system

7

Figure 7 shows the method of re-mapping ‘bad’ channels to ‘good’ channels. The input channel (fgen) is added with RANDW (this is a modulo 212 addition). By finding the mod of the addition (using the number of good channels as a basis for the mod function) the ‘good channel bank’ is addressed with this number and the output of the re-mapping function is fed out as required. This ensures that all channels in the good channel bank are used evenly when addressed.

##### Method of producing RANDW

RANDW is a pseudo-random number based on the bits of the clock.

To be decided

The output sequence is ‘pseudo-random’ according to four NIST statistical tests: The mono-bit test, the poker test, the runs test and the long runs test. This is the randomness test recommended in the NIST Federal Information Processing Standards 140-1 (FIPS PUB 140-1) publication dated March 1995.

##### Method of Support for Broadcast Packets

The Mode L adaptive frequency hopping system supports the ability to broadcast packets on the ‘good’ channels. Since the hopping sequence is unchanged for all devices at these ‘good’ channels, broadcast packets may be sent at these times.  This requires that when the AM_ADDR is equal to ‘000’ that the Master only transmit on the ‘good’ channels.

Thus full support for legacy devices is maintained.

### LMP Commands

#### AFH mode request

If the master supports AFH and wishes to enter AFH mode for some slaves, the master sends LMP_AFH_mode_req to ask the slave to prepare to operate in AFH mode. If the slave agrees to the master’s parameters, LMP_accepted will be returned. If the slave does not support AFH, the reason for LMP_not_accepted  shall be Unsupported LMP Feature. If the slave does not support the master’s AFH mode, the reason shall be Unsupported parameter value. If the Slave supports AFH, but the AFH mechanism is locally disabled then the Slave will respond with LMP_not_accepted with reason code PDU not allowed.

Master

LM

Slave

LM

LMP_AFH_mode_req

LMP_accepted or LMP_not_accepted

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_Support_AFH | 2 | ?? | DM1 | m  s | AFH_mode | 2 |

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFH mode | 1 | u_int8 |  | 0: mode H 1: mode L others: reserved |

This step allows the Master to know if the connecting device supports AFH, and other parameters of the AFH mechanism.

#### Request of Slave’s Classification List

The Master issues a command to the Slave to request its classification list. This function may or may not be supported by the Slave.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_Available_Channel_Request | 1 | ?? | DM1 | m  s | - | - |

Table 3: PDUs used for Master to request Slave’s classification information

A Slave that supports this function will respond with the LMP_Slave_available_channel PDU.

Master

LM

Slave

LM

LMP_available_channel_request

LMP_slave_available_channel

Sequence 2: Master requests and receives slaves’ classification information.

A Slave that does not support AFH will respond with LMP_not_accepted with the reasons Unknown LMP PDU. A Slave that supports AFH, but does not itself support the function of classifying the channels will respond with LMP_not_accepted with the reason code PDU not allowed.

Master

LM

Slave

LM

LMP_available_channel_request

LMP_not_accepted

Sequence 3: Master request of slaves’ classification information fails

#### Slave’s Classification List

In response to the LMP_available_channel_request, a supporting Slave will reply with a packet containing its classification of all the channels. For a system supporting 79 channels, 79 bits will be used, each bit representing each of the 79 channels. A binary ‘0’ will represent a ‘bad’ channel and a binary ‘1’ will represent a ‘good’ channel. The LSB of the payload will relate to the quality of channel ‘0’, the MSB of the payload will relate to the quality of the last channel.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_Slave_Available_Channel | 13 | 81 | DM1 | s  m | channel list | 2 – 12:7 |

Table 4: PDUs used for Slave to send classification information to Master

Since the Master initiated the transaction, the Master should respond with a LMP_accepted.

Master

LM

Slave

LM

LMP_slave_available_channel

LMP_accepted

Sequence 4: Slave sends classification list to Master.

#### AFH mode start/terminate [Mode H]

After the master has classified channel information, the master uses broadcast packets to broadcast AFH start/terminate and AFH operation information. To make sure that all slaves in the piconet can receive the broadcast message, there are two ways for the master to control encryption mode for broadcast operation. One is not to use encryption in broadcast packets, that is, slaves shall operate in either “no encryption” or “point-to-point encryption” mode. The other is to use master key for all slaves, that is, all slaves must operate in “point-to-point and broadcast encryption” mode with master key. (Please refer to Bluetooth / IEEE 802.15.1 Specification Part C: Link Manager Protocol 3.6.1 Encryption Mode.)

There are two broadcast packets LMP_AFH_start0 and LMP_AFH_start1 to provide necessary AFH operation information. These broadcast packets will interrupt SCO packets since they have higher priority.

There are two frequency hopping cases for the master to broadcast AFH packets. The first case is broadcasting in Bluetooth original frequency hopping sequence. It could happen when first time master wants the piconet to operate in AFH mode or for some slaves to join AFH mode if other slaves are in AFH mode already. The second case is broadcasting in existing AFH mode to notify AFH slaves to switch to new AFH sequence or to terminate existing AFH mode.

The AFHid in the AFH messages is a sequence number to identify different AFH operations. It is used to make sure that AFH mode start/terminate and AFH mode check operate on the same AFH operation parameters. The slave needs to keep track of the identification until the next time LMP_AFH_start0 with a different AFHid received. If the slave receives LMP_AFH_start1 with a different AFHid, it just ignores the received PDU.

Master

LM

Slave

LM

LMP_AFH_start0

LMP_AFH_start0

LMP_AFH_start0

LMP_AFH_start1

LMP_AFH_start1

LMP_AFH_start1

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_start0 | 15 |  | DM1 | m->s | AFHid AFH slave bitmap AFH instant AFH traffic type AFH ACL window factor timing control flags AFH channel info | 2 3 4-7 8 9 10 11-15 |
| LMP_AFH_start1 | 17 |  |  | m->s | AFH handle AFH channel info | 2 3-17 |

AFH slave bitmap and AFH instant are used to indicate which slave shall start or terminate AFH mode after the AFH instant. With corresponding AFH slave bitmap setting to 0, the slave addressed by AM_ADDR shall terminate AFH mode and operate at Bluetooth’s original hopping sequence after the AFH instant. If the slave receives LMP_AFH_start0 and is informed to terminate AFH mode, it will ignore the received LMP_AFH_start1 PDU. With corresponding AFH slave bitmap setting to 1, the slave addressed by AM_ADDR shall start to operate in new AFH mode after the AFH instant if LMP_AFH_start1 is received successfully.

| Name | Length (bytes) | Type | Unit | Detailed |
| --- | --- | --- | --- | --- |
| AFHid | 1 | u_int8 |  |  |
| AFH slave bitmap | 1 | u_int8 |  | Bit0: reserved Bit1 = 0: AM_ADDR1 slave terminates AFH Bit1 = 1: AM_ADDR1 slave starts AFH Bit2 = 0: AM_ADDR2 slave terminates AFH Bit2 = 1: AM_ADDR2 slave starts AFH Bit3 = 0: AM_ADDR3 slave terminates AFH Bit3 = 1: AM_ADDR3 slave starts AFH Bit4 = 0: AM_ADDR4 slave terminates AFH Bit4 = 1: AM_ADDR4 slave starts AFH Bit5 = 0: AM_ADDR5 slave terminates AFH Bit5 = 1: AM_ADDR5 slave starts AFH Bit6 = 0: AM_ADDR6 slave terminates AFH Bit6 = 1: AM_ADDR6 slave starts AFH Bit7 = 0: AM_ADDR7 slave terminates AFH Bit7 = 1: AM_ADDR7 slave starts AFH |
| AFH traffic type | 1 | u_int8 |  | Bit0: Link type (LT) Bit1: Frame type (FT) Bit2-4: Reservation indication (RI) |
| AFH ACL window factor | 1 | u_int8 |  | ACL window factor |
| AFH instant | 4 | u_int32 |  | Bits 27:1 of the master Bluetooth clock value |
| AFH channel info k-th byte (k = 0~19): (4k~4k+3)-th channel info | 1 | u_int8 |  | Bit0-1 = 0: channel 4k is good channel Bit0-1 = 1: channel 4k is bad channel Bit0-1 = 2: channel 4k is unused channel Bit0-1 = 3: reserved Bit2-3 = 0: channel 4k+1 is good channel Bit2-3 = 1: channel 4k+1 is bad channel Bit2-3 = 2: channel 4k+1 is unused channel Bit2-3 = 3: reserved Bit4-5 = 0: channel 4k+2 is good channel Bit4-5 = 1: channel 4k+2 is bad channel Bit4-5 = 2: channel 4k+2 is unused channel Bit4-5 = 3: reserved Bit6-7 = 0: channel 4k+3 is good channel Bit6-7 = 1: channel 4k+3 is bad channel Bit6-7 = 2: channel 4k+3 is unused channel Bit6-7 = 3: reserved |

#### Adaptive Hopping Request [Mode L]

Finally, after negotiation as to which of the channels are ‘good’ or ‘bad’, the Master compiles the final classification list and it is distributed to the Slaves as required.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_Request | 13 | 82 | DM1 | m  s | channel list | 2 – 12:7 |

Table 5: PDUs used for Master to request Slave to adaptively hop using enclosed classification sequence.

The payload of this packet will contain the full list of ‘bad’ and ‘good’ channels. A binary ‘0’ will represent a ‘bad’ channel and a binary ‘1’ will represent a ‘good’ channel. The LSB of the payload will map directly to the channel quality of channel 0.

Master

LM

Slave

LM

LMP_AFH_Request

LMP_accepted

Sequence 5: Master sends new classification list to slave and AFH is started.

If the Slave accepts then it will respond with LMP_accepted and start adaptively hopping

If the Slave does not accept the AFH request because it does not support AFH, then it will respond with LMP_not_accepted with reason code Unknown LMP PDU. If the Slave does not accept the AFH request because it is locally disabled then it will respond with the reason code PDU not allowed.

Master

LM

Slave

LM

LMP_AFH_Request

LMP_not_accepted

Sequence 6: Slave reject AFH request.

#### Return to Regular Hopping Request [Mode L]

After the piconet has been adaptively hopping for a time, the Master may force all the Slaves back to the normal 802.15.1 / Bluetooth hopping sequence. This command is issued by the Master and contains no payload.

| LMP PDU | Length (bytes) | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_Regular_Hopping | 1 | 83 | DM1 | m  s | - | - |

Table 6: PDUs used for Master to request Slave return to regular hopping mode.

The Master will not send such a command to an unsupporting Slave, so the default response after receiving this command is an LMP_accepted message.

Master

LM

Slave

LM

LMP_regular_hopping_request

LMP_accepted

Sequence 5: Master requests slave to return to regular 802.15.1 hopping mode.

#### AFH mode check [Mode H]

Once the slave’s AFH mode is changed, the master needs to use the slave’s AFH mode to check whether the slave is in the correct AFH mode by LMP_AFH_check_req. That is, if the master starts the slave’s AFH mode, the master needs to check this slave in AFH sequence; if the master terminates the slave’s AFH mode, the master needs to check this slave in the original hop sequence. Note that the LMP_AFH_check_req will also interrupt SCO packets for their higher priority.

If the slave receives LMP_AFH_check_req with correct AFHid, it shall respond by LMP_AFH_check_res with AFHid to inform the master that it operates in the correct AFH mode. If the slave receives LMP_AFH_check_req with a different AFHid, it just ignores the received LMP PDU.

In order for AFH to converge quickly, LM layer of the master shall start a new time-out event (AFH_time_out) after the AFH instant. If slave has SCO channels, the time-out value is chosen by the master and should be at least 6*Tsco. If slave uses ACL only, the time-out value is chosen by the master and should be at least (6*Tpoll + RBinit). RBinit is the value of the first bad window if AFH instant is in bad window (see the figure below). If AFH instant is in good window, RBinit is set to zero. If the master does not receive LMP_AFH_check_res within the time-out period, the master will return to the slave’s old hop sequence to perform “AFH mode start/terminate” procedure again.

Superframe

AFH instant

|  | RBinit |  |
| --- | --- | --- |
| Good channels | Bad channels | Good channels |
| RG | RB |  |

Master

LM

Slave

LM

LMP_AFH_check_req

LMP_AFH_check_res

| LMP PDU | Length | op code | Packet type | Possible direction | Contents | Position in payload |
| --- | --- | --- | --- | --- | --- | --- |
| LMP_AFH_check_req | 2 |  |  | m->s | AFHid | 2 |
| LMP_AFH_check_res | 2 |  |  | s->m | AFHid | 2 |