IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | TG3 PHY Ammendments Portland |  |
| Date Submitted | [14 May, 2001] |  |
| Source | [James P. K. Gilb] [Mobilian] [11031 Via Frontera, Suite C] | Voice: [858-451-2201] Fax: [858-451-3201] E-mail: [gilb@ieee.org ] |
| Re: | [] |  |
| Abstract | [Amendments to the PHY layer of 00000d04P802.15.3__Draft-Standard.pdf] |  |
| Purpose | [To record the amendments to the PHY layer for the purpose of modifying the baseline draft standard.] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Approve clause 11 in 00000d05P802-15-3 and the following modifications to clause `` and 6.

- Sub-clause 11.6.6, Delete “to the SME and MLME” from the last sentence.  Ad the sentence “The RSSI power shall be the average power measured in the last CAZAC sequence of the PHY preamble, 11.4.6.” as the second to last sentence.

- Sub-clause 11.6.7, Delete “to the SME and MLME” from the last sentence.  Ad the sentence “The LQI SNR shall be measured in the last CAZAC sequence of the PHY preamble, 11.4.6.” as the second to last sentence.

- Add PLME-TESTMODE.confirm following 6.4.2 to be the response to PLME-TESTMODE.request, the only parameter is Status which can be either SUCCESS, FAILED, UNSUPPORTED_TESTMODE

- In 11.6.3, change “for uncoded QPSK modulation.  The receiver maximum input level is not defined for the other modulation formats.” to be “for each of the modulation formats that the device supports.”

- Remove references to OperationalRateSet and PiconetBasicRateSet in 6.3.3.1, Table 7, Table 23, 6.3.9.1

- Table 54 (50 in d05): Change RSSI to have values from 0 to PHYPIB_RSSI_max and define PHYPIB_RSSI_max for the 2.4 GHz PHY to be 7. Include the LQI, and give it a range of values from 0 to PHYPIB_LQI_max.  Define PHYPIB_LQI_max in the 2.4 GHz PHY PIB to be 31.

- Change PHY-CCA commands to be: PHY-CCA.start, PHY-CCA.end, PHY-CCA.request and PHY-CCA.confirm, sub-clauses 6.4.8.8, 6.4.8.9 and 6.4.8.10

- Remove PHYPIB_CCAModesSupported and PHYPIB_CurrentCCAMode from PHY PIB, 6.6.8, table 44 and 36.

- Add PHY-PWRMGT.request and PHY-PWRMGT.confirm sub-clause 6.8.4

- Add text to 11.6.7 to clarify that the SNR includes more than just thermal noise, for example: “The SNR includes all distortion, uncorrected interference and other signal impairments”

- Change “accuracy for each step” to be “step-size accuracy” in 11.6.6.