IEEE P802.15

Wireless Personal Area Networks

NOTE: To change <title> and other required fields, select FileProperties and update the appropriate fields in the Summary tab.  DO NOT replace field codes with text.  After updates are entered, delete this paragraph and update all fields (ctl-A then F9) Note: dates will not be updated until document is saved.  After fields are updated, delete this paragraph..

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Coexistence Modeling Overview |  |
| Date Submitted | 28 June 2001 |  |
| Source | Nada Golmie NIST 100 Bureau Dr. Stop 8920 Gaithersburg, MD 20899 | Voice: (301) 975-4190 Fax: (301) 590-0932 E-mail: nada@nist.gov |
|  | R.E. Van Dyck NIST 100 Bureau Dr. Stop 8920 Gaithersburg, MD 20899 | Voice: (301) 975-2923 Fax: (301) 590-0932 Email: vandyck@nist.gov |
|  | The is proposed text for clauses 5.2 of the Recommended Practice Document |  |
| Abstract | [] |  |
| Purpose | [] |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

# Clause 5.2 Coexistence Modeling Overview

The coexistence modeling approach used is based on detailed simulation models for the RF channel, the MAC and PHY layers developed in OPNET  and C respectively.

The PHY layer models for the Bluetooth and IEEE 802.11 transceivers are based on models developed in C. The MAC layer models that we develop are interfaced to these PHY models, and the integrated MAC and PHY simulation models constitute an evaluation framework that is critical to studying the various intricate effects between the MAC and PHY layers. Although interference is typically associated with the RF channel modeling and measured at the PHY layer, it can significantly impact the performance of higher layer applications including the MAC layer. Similarly, changes in the behavior of the MAC layer protocol and the associated data traffic distribution could play an important factor in the interference scenario and affect the overall system performance.

OPNET is a Trademark of OPNET Technologies Inc.