IEEE 802.15

Wireless Personal Area Networks™

| Project | IEEE 802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | Draft WPAN Article for IEEE Microwave & Spectrum Magazines |  |
| Date Submitted | [March 5, 2001] |  |
| Source | [Ian Gifford] [M/A-COM, Inc.] [1011 Pawtucket Boulevard Lowell, MA USA 01853-3295] | Voice: [+1 978 442 4650] Fax: [+1 978 442 5442] E-mail: [giffordi@ieee.org] |
| Re: | [Verbal 12Jan01 from Roger Marks & -00/126r1] |  |
| Abstract | [Edits applied to first merge, An article that provides an update on the IEEE 802.15 – as of 25Feb01.] |  |
| Purpose | [The purpose of this Magazine Article is to promote the emerging wireless standards-making WGs inside of IEEE 802 and the IEEE Computer Society. There is reason to believe we can place this article into IEEE Spectrum and/or other IEEE Magazines NLT this Summer.] |  |
| Notice | This document has been prepared to assist the IEEE 802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by 802.15. |  |

Monday, March 05, 2001

EDITORS NOTE TO BE REMOVED – The PDF sent by Roger Marks was converted to ASCII and

Then I imported to MS Word to apply Tracking Changes.  The PDF to Text translator barfed a little on the following but all in all it should provide my edits.

From pdf2txt@sun.trace.wisc.edu Mon Mar  5 05:40:58 2001

Date: Mon, 5 Mar 2001 04:37:13 -0600 (CST)

From: PDF to Text translator <pdf2txt@sun.trace.wisc.edu>

To: "Gifford, Ian C." <giffordi@world.std.com>

[ The following text is in the "unknown-8bit" character set. ]

[ Your display is set for the "US-ASCII" character set.  ]

[ Some characters may be displayed incorrectly. ]

Result of converting file IEEE_Microwave.pdf

*Pages 1--18 from  Standards in IEEE 802 Unleash the Wireless Internet*

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 1

Standards in IEEE 802 Unleash the

Wireless Internet

Roger B. Marks

National Institute of Standards and Technology

325 Broadway

Boulder, CO 80305 USA

marks@ boulder. nist. gov

Ian C. Gifford

M/ A-COM, Inc.

1011 Pawtucket Blvd.

Lowell, MA 01853-3295 USA

giffordi@ieee.org

Bob O'Hara

Informed Technology, Inc.

1750 Nantucket Circle, Suite 138

Santa Clara, CA 95054-3822 USA

bob@ informed-technology. com

Draft for submission to IEEE Microwave Magazine

Portions are U. S. Government work and not subject to U. S. Copyright

1.0 Abstract

Within the Institute of Electrical and Electronics Engineers, Inc. ( IEEE) , the IEEE 802 LAN

MAN Standards Committee is developing common air interface standards for Wireless Local Area Net-

work ( LAN) , Wireless Metropolitan Area Network ( MAN) , and Wireless Personal Area Network

technology. These standards are enabling the development of an infrastructure for the Wireless

Internet.

2.0 Introduction

The Internet and wireless communications networks revolutionized communications in the 1990s.

The convergence of these two technologies leads naturally to the Wireless Internet. Just as stan-

dards determined the development of the Internet and of wireless cellular telephony, so will stan-

dards in   uence the evolution of the Wireless Internet as a social phenomenon. Both the cellular

telephony industry and the Internet communications industry bring their own models for standard-

1

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 2

ization, and the two offer competing visions. The Internet-based standardization model is cur-

rently centered around the The Institute of Electrical and Electronics Engineers, Inc. ( IEEE) and

its IEEE 802 LAN MAN Standards Committee, which is the world leader in Local Area Network

( LAN) and Metropolitan Area Network ( MAN) standards. IEEE 802 is currently developing and

enhancing standards for wireless Local Area Networks ( Wireless LAN) , Wireless Personal Area

Networks ( Wireless PAN) , and Wireless Metropolitan Area Networks ( WirelessMAN   ) .

[EDITORS NOTE TO BE REMOVED – WHAT ABOUT IETF?  I SUGGEST READING RFC 2026 “The Internet Standards Process”]

The model of the Wireless Internet based on cellular telephony is embodied in the standards of the

International Telecommunication Union Radiocommunication Bureau (ITU-R) <ITU-R http://www.itu.int/ITU-R/index.html> third-generation International Mobile Telecommunications (IMT-2000) wireless communications family of standards. This set

of standards foresees an evolution from circuit-switched mobile voice services to packet-switched

mobile voice and data. The infrastructure is to be based on cellular base stations serving both

highly-mobile users ( at automotive speeds) at 144 kbit/ s, with the theoretical capability to also

serve   xed users at rates up to 2 Mbit/ s. The standards were developed at a technical level by

national or regional bodies so that, when worldwide standards were forged by the ITU-R, the vot-

ing members, which are national governments, were centrally involved in the negotiations. As a

result, the standards debates turned into highly politicized events often publicized as   wars.   The

world now eagerly anticipates the economic success of this technology. For example, license auc-

tions in Europe have brought in thousands of euros per resident. By the time these costly systems

are engineered, constructed, and marketed, investors will have paid dearly and will be expecting a

heavy return.

In a contrast of broad proportions, IEEE 802 is developing an alternative series of Wireless Inter-

net standards. IEEE   s global standardization effort involves no national bodies and therefore no

national politics; the work proceeds on a technical and business basis. To a large degree, the intent

of the work is to succeed by bringing to market low-cost products that serve customer needs.

Much of the work involves license-exempt spectrum. This removes the spectrum acquisition costs

from the economic picture. Furthermore, it weakens the concept of a monolithic   operator   with

strong control over the provided services. Instead, it opens up the market to entrepreneurs and

innovations. Some of the IEEE 802 Wireless Internet technologies are geared around data rates much

higher than those provided by even IMT-2000   s projected   xed-user rates; for example, the cur-

rently-popular IEEE 802.11b standard supports 11 Mbit/ s. The IEEE standards do not offer the

mobility of IMT-2000 in the sense of providing services to moving vehicles, and they are not

aimed at providing blanket coverage to users at arbitrary locations within a city. Instead:

° 802.16 WirelessMAN   standards will support high--rate Broadband Wireless Access services

to buildings, mostly through rooftop antennas, from central base stations;

° 802.11 Wireless LAN standards support users roaming through homes, of   ce buildings, cam-

puses, hotels, airports, restaurants, cafes, etc. ;

° 802.15 Wireless PAN standards will support short-range links among computer, cell phones, and other consumer

electronics applications that are worn or carried.  One of the primary goals for WPANs is replacing wires between objects that are close to each other and then hook to the larger world when/if convenient. This wire replacement technology is intended as an embedded connection between a large variety of devices, many with limited capabilities. While the WLAN & WMAN technologies are specifically designed for devices in and around the office or home, a WPAN device will travel from country to country, be used in cars, airplanes, and boats, and is truly designed for international use.

[EDITORS NOTE TO BE REMOVED – THE ABOVE “This set of standards foresees an evolution from circuit-switched mobile voice services to packet-switched mobile voice and data.” &…IEEE 802 Wireless Internet technologies are geared around data rates much higher than…” DOES NOT HOLD TRUE FOR 802.15.1 and 802.15.4.  Please add and/or discuss with me how to add:]

Note: The Bluetooth WPAN baseband protocol is a combination of circuit and packet switching. Slots can be reserved for synchronous packets. Each packet is transmitted in a different hop frequency. A packet nominally covers a single slot, but can be extended to cover up to five slots. Bluetooth can support an asynchronous data channel, up to three simultaneous synchronous voice channels, or a channel which simultaneously supports asynchronous data and synchronous voice. Each voice channel supports 64 kb/s synchronous (voice) link. The asynchronous channel can support an asymmetric link of maximally 721 kb/s in either direction while permitting 57.6 kb/s in the return direction, or a 432.6 kb/s symmetric link.

2

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 3

3.0 Standardization in IEEE 802

3.1 The IEEE Standards Association

The IEEE is a nonpro   t transnational technical professional organization with  366,135 mem-

Bers, as of Dec01. IEEE supports many technical activities, including conferences, publications, and local

activities. In addition, IEEE carries out an active program in standardization through the IEEE

Standards Association ( IEEE-SA < http: / / standards. ieee. org> ) . While many IEEE-SA activities

are global in scope, its efforts are accredited by the American National Standards Institute

( ANSI) . ANSI oversight ensures that its guiding principles of consensus, due process, and open-

ness are followed.

IEEE-SA standards are openly developed with consensus in mind. Participation in their develop-

ment, and use of them, is entirely voluntary. However, history has shown that standards developed

in an open forum can produce high-quality, broadly accepted results that can focus companies and

forge industries.

The IEEE-SA carries out oversight of the standardization process through the IEEE-SA Standards

Board. Project development is delegated to individual standard sponsors; these are generally units

of the IEEE   s technical societies. One of the most important of the IEEE-SA sponsor groups is the

IEEE 802 LAN MAN Standards Committee.

3.2 The IEEE 802 LAN MAN Standards Committee

The IEEE 802 LAN MAN Standards Committee < http: / / ieee802. org> is sponsored by the IEEE

Computer Society. It   rst met in 1980 to develop a Local Area Network ( LAN) standard that

evolved into separate technologies. It develops and maintains standards for physical layers ( PHY)

and medium access control sublayers ( MAC) , each of which   ts under a common Logical Link Con-

trol ( LLC) sublayer. 1 Together, these make up the two lowest layers of the OSI seven-layer model for

data networks.

IEEE 802 holds week-long plenary meetings three times a year under the leadership of a Sponsor

Executive Committee ( SEC) , Chaired since 1996 by Jim Carlo. In between these plenaries, most

of its active Working Groups hold interim meetings.

Historically, 802 has been best known for the IEEE 802.3 standard, informally known as Ethernet,

which is a tremendous worldwide success. Ethernet is the foundation of so many of the world   s

LANs that, for most practical purposes,   LAN   simply means a connection of Ethernet devices. .

Like all successful 802 standards, however, IEEE 802.3 continuously evolves, moving from shared

coaxial media to twisted pair with the 10BaseT standards and raising the supported data rates with

100BaseT and 1000BaseT. Optical media are supported, and 802.3 is currently developing a 10

Gbit/ s standard. 802   s portfolio of active projects in the cabled realm grew in late 2000 with the

approval of the 802.17 Working Group on Resilient Packet Rings.

1. Overview and Guide to the IEEE 802 LMSC < http: / / ieee802. org/ overview2000. pdf> .

3

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 4

While Ethernet has been its greatest success, 802 is now the home of a number of wireless net-

work standardization projects. These projects take advantage of the highly-successful system of

standards development pioneered by 802. Before continuing with detailed discussion of the 802

Wireless Standards Program, it will be useful to overview this process.

3.3 The 802 Standardization Process

IEEE 802 process is designed for quick development of standards with broad consensus. The

demand for consensus helps to ensure that standards are technically superior and meet market

needs.

The development process in IEEE 802 follows the chronological steps outlined below. The pro-

cess is overseen by 802   s SEC and de   ned by a set of rules and procedures 1 . In brief:

° A Study Group is chartered to study the prospects of a standard in a   field and potentially to

develop a Project Authorization Request ( PAR) requesting IEEE-SA approval of a new project.

The SEC also requires that each PAR be accompanied by a statement addressing 802   s   Five

Criteria for Standards Development,   demonstrating that the intended standard has broad mar-

ket potential, compatibility with other 802 standards, distinct identity within 802, technical fea-

sibility, and economic feasibility.

° The SEC assigns each approved project to an existing or new Working Group. Technical deci-

sions are made by the Working Group by vote of at least 75% of its members. Working Group

membership belongs to individuals, not to companies or other entities, and it is awarded on the

basis of participation at meetings. Nonmembers participate actively as well.

° The initial draft development method varies among groups, but the typical process is to dele-

gate the problem to a subordinate Task Group and issue a public Call for Contributions for doc-

umented input. Eventually a baseline draft is selected and then developed.

° Before the SEC will advance the draft, it must be approved in a Working Group Letter Ballot in

which the Members are asked to approve the document. Any vote against the document must

be accompanied by speci   c comments on what changes are required in order that the voter will

approve it. This process forces constructive change suggestions and helps drive the process to

quick improvement after a few cycles. Members voting to approve, and nonmembers as well,

are also solicited for suggestions. A 75% approval rate is required to pass. However, changes

made in response to comments, and negative comments that have not been accepted by the edi-

torial team, must be   re-circulated   for approval by the voters. . In effect, the ballot cannot close

until those voting negative have had their say and failed to attract other voters to their argu-

ment. In the end, the approval margin is typically much higher than 75% at closure.

° The Working Groups   nal task is to see the draft standard through   Sponsor Ballot,   in which it

is put before a broad group of interested individuals. This is similar to a rerun of the Working

Group Letter Ballot except that the Ballot Group is not restricted to members of the Working

Group. IEEE requires a balanced Ballot Group, which ensures that is it not dominated by pro-

ducer or use interests. In addition to the vote, of course, critical comments are vital to the suc-

cess of the process.

1. Operating Rules of IEEE Project 802 LAN MAN Standards Committee < http: / / ieee802. org/ rules. pdf>

4

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 5

Experience has shown that the IEEE 802 process is extremely effective at engaging a wide array

of interested parties, bringing forth comments, and making constructive changes. As a result, 802

drafts are in a continual state of improvement. When a standard   nally makes it through the sys-

tem, users have solid con   dence in it. With careful attention and the will of the developers, it is

possible to drive the draft through the system in a reasonable time.

4.0 The IEEE 802 Wireless Standards Program

The IEEE 802 wireless standards program comprises three Working Groups:

° The IEEE 802.11 Working Group develops the IEEE 802.11 standards for Wireless Local Area

Networks ( Wireless LAN)

° The IEEE 802.15 Working Group develops the IEEE 802.15 standards for Wireless Personal

Area Networks ( Wireless PAN or WPAN)

° The IEEE 802.16 Working Group on Broadband Wireless Access develops the IEEE 802.16

standards for Wireless Metropolitan Area Networks ( WirelessMAN   ) .

These groups work in a loose association to coordinate their activities. 802.11 and 802.15 have

worked particularly closely since they both address unlicensed bands. 802.16 has historically dealt

with licensed bands and been more independent. However, a new license-exempt project in

802.16 now requires it to coordinate more closely. Some of this coordination takes place through

the IEEE 802 Regulatory Ombudsman, who oversees interactions with regulatory activities that

affect any or all of the Working Groups.

Below, we summarize the status and technology of the projects in the IEEE 802 wireless stan-

dards program.

4.1 The IEEE 802.11 Working Group for Wireless Local Area Networks

The IEEE 802.11 Working Group for Wireless Local Area Network ( Wireless LAN or WLAN)

Standards < http: / / ieee802. org/ 11> was the   rst wireless effort in IEEE 802. As with other stan-

dards in the 802 family, 802.11 describes a Medium Access Control ( MAC) sublayer and multiple

physical ( PHY) layers. For the   rst time in an 802 standard, 802.11 also describes MAC Manage-

ment functionality.

The initial base standard, published in 1997, describes the requirements for a LAN implementa-

tion using both infrared and spread spectrum radio frequency communications designed in accor-

dance with rules for unlicensed operation. Since then, the base standard has been revised ( as

802.11-1999) , and the Working Group has published two additional PHY layer amendments

( 802.11a and 802.11b) . The existing standard and its amendments describe several WLAN PHYs:

° infrared at 1 and 2 Mbit/ s

° frequency hopping spread spectrum radio at 1 and 2 Mbit/ s in the 2.4 GHz band

° direct sequence spread spectrum radios with data rates up to 11 Mbit/ s in the 2.4 GHz band

° orthogonal frequency division multiplexing radios in the 5-6 GHz band

5

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 6

Current work includes extending the MAC and MAC Management functionality to provide

expanded international operation and roaming, improved support for quality of service, enhanced

security, dynamic channel selection, transmit power control, and standardized communication

between 802.11 access points. Work is also proceeding to increase the data rate of one of the

existing PHYs.

The 802.11 standard 1 describes two types of WLANs, an ad hoc network ( an independent Basic

Service Set ( BSS) , in the language of the standard) and an infrastructure network ( comprised of

infrastructure BSSs of one access point and the associated mobile stations) . An ad hoc WLAN

consists of only mobile stations. This type of WLAN is often set up for a very speci   c purpose,

such as exchanging   les during a single meeting, and its lifetime is usually limited. The infra-

structure BSS, on the other hand, is typically a long-lived WLAN that integrates mobile stations

into a large network infrastructure through the use of access points ( APs) that perform a bridging

function between wired and wireless LANs. The MAC and MAC Management functions allow

the mobile stations to   nd other mobile stations and APs, register with the WLAN, request

encryption and power management services from the WLAN, and exchange data with other

mobile stations and APs. The MAC and MAC management functions operate over any and all of

the PHYs.

The 802.11 MAC incorporates mechanisms to increase the reliability of exchanging information

in the wireless medium. The basic access mechanism of 802.11 ( distributed coordination function

( DCF) , in the language of the standard) is carrier sense multiple access with collision avoidance

( CSMA/ CA) . The DCF may be used in either the ad hoc or infrastructure WLANs. The DCF is

quite similar to the carrier sense multiple access with collision detection ( CSMA/ CD) used in

IEEE 802.3 Ethernet. CSMA/ CA works by sensing the medium for activity before every trans-

mission and deferring the transmission if the medium is active with another transmission. As in

802.3, 802.11 uses a binary exponential backoff mechanism to spread transmission opportunities

in time and minimize the likelihood of subsequent collisions.

Because it is more dif   cult to have a radio simultaneously transmit and receive on the same chan-

nel than would be the case for a wired medium, 802.11 uses a collision avoidance mechanism

rather than physical collision detection. Every frame transmitted by 802.11 is part of a frame

exchange sequence that includes an acknowledgement from the destination. Each frame includes

information about the duration remaining in the sequence of frames being exchanged. Every sta-

tion processes this duration information and maintains a network allocation vector ( NAV) indicat-

ing how much longer the medium will be occupied by the frame exchange. The NAV is used to

reduce the problem caused by   hidden nodes,   in which a station receives only one side of the

frame exchange; the potential for this problem makes physical carrier sense an unreliable means

of detecting activity on the medium. Thus, a station senses the medium using both its physical car-

rier sense and a virtual carrier sense derived from the NAV. If either of these indicate that the

medium is in use, a station   s transmission will be deferred.

In an infrastructure WLAN, the 802.11 standard provides an optional access mechanism, called

the point coordination function ( PCF) , where the AP acts as a central coordinator for the WLAN,

1. Bob O Hara and Al Petrick, IEEE 802.11 Handbook: A Designers Guide , IEEE Standards Press, 1999.

6

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 7

scheduling nearly all of the transmissions. The PCF offers a signi   cant boost in ef   ciency since

almost all collisions are eliminated. Of course, this ef   ciency comes at an increased cost due to

the higher complexity of the AP and its required scheduling algorithms. It is upon this PCF that

the current work in the 802.11e task group is proceeding to build improved support for quality of

service. This work is in the very early stages of development. The resulting 802.11e amendment

may be available by early 2002.

The 2.4 GHz PHYs of the current standard describe the requirements for operating in a limited

number of areas of the world: the United States, Canada, Europe ( within the domain of the Euro-

pean Telecommunications Standards Institute) , France, Spain, and Japan. In no other locations

can a device that implements the 802.11 standard using these PHYs be called compliant with

802.11. The 802.11d task group has addressed this shortcoming of the standard by describing a

protocol that will allow an 802.11 device to receive the regulatory information required to con   g-

ure itself properly to operate anywhere on planet Earth. It is still incumbent upon the manufac-

turer to obtain any required certi   cations before allowing its equipment to operate in locations

requiring certi   cation. In the winter of 2001, 802.11d was in Sponsor Ballot, with expectations of

approval as an amendment to the 802.11 standard in the spring of 2001.

Because a wireless medium offers even less protection from eavesdropping than a wired medium,

802.11 incorporates encryption into the MAC. Called the Wired Equivalent Privacy ( WEP) mech-

anism, this encryption function is intended to prevent   casual eavesdropping   . It is not a guaran-

teed privacy mechanism but is intended to provide basic protection to the information transmitted

over the air. The 802.11e task group is working to improve this capability signi   cantly. The capa-

bility envisioned by 802.11e will provide a simple upgrade path, based on the current WEP, to bet-

ter security for current implementations. It will also offer a new encryption function based on the

recently selected Advanced Encryption System ( AES) , the result of a years-long selection process

by the U. S. Department of Commerce. The work of 802.11e is in the very early stages of develop-

ment and could be completed late in 2001.

Of the three initial PHYs in the 802.11 standard, the IR has seen the least use. This PHY uses

baseband pulse position to transmit data at 1 and 2 Mbit/ s. It provides the greatest physical secu-

rity of the 802.11 PHYs since most walls and windows block IR radiation. For the same reason,

the number of APs required to provide WLAN coverage for a given area is often signi   cantly

greater than that required for the radio PHYs. This is the likely reason that the IR PHY has been

used so infrequently.

The frequency hopping ( FH) PHY also provides data rates of 1 and 2 Mbit/ s. This PHY was suit-

able for operation under the U. S. FH spread spectrum rules for the 2.4 GHz Industry, Scienti   c,

and Medical ( ISM) band at the time of development; it remains usable under the rules as liberal-

ized in 2000 to allow   wideband   FH spread spectrum systems. The 802.11 FH PHY provides 79

channels with a channel bandwidth of 1 MHz. For 1 Mbit/ s, the modulation used is two-level

Gaussian frequency shift keying ( GFSK) with a nominal bandwidth bit period of 0.5. Minimum

transmit power is 10 mW, though transmit power may be as high as 1 W. The receiver sensitivity

is   80 dBm. For 2 Mbit/ s, the modulation used is four-level GFSK.

The Direct Sequence ( DS) PHY, with the extension in the 802.11b amendment, provides data

rates of 1, 2, 5.5, and 11 Mbit/ s. In the U. S. , this PHY operates under the DS spread spectrum

7

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 8

rules for the 2.4 GHz ISM band. The standard provides for 14 overlapping channels of 22 MHz

between 2.4 GHz and 2.5 GHz. Not all channels are usable in all regulatory areas; e. g. , only chan-

nels 1 through 11 may be used in the United States. Channel centers are spaced 5 MHz apart. The

1 and 2 Mbit/ s data rates use a   xed 11-chip spreading sequence, based on a Walsh code, to meet

the minimum spreading requirements of the U. S. regulations. The modulation used for these rates

is DBPSK and DQPSK, respectively. The 5.5 and 11 Mbit/ s data rates use complementary code

keying ( CCK) as the spreading mechanism. With CCK, a data symbol is created from 8 data bits

that select one quaternary ( 4-level) code from a universe of 2 16 possible codes. This provides

some coding gain, though it is arguable whether it meets the letter of the DS spread spectrum rules

of the regulations. However, it does pass the test in the regulations to determine the minimum cod-

ing gain. The work currently proceeding in the 802.11g task group is to increase the data rates for

the DS PHY beyond 20 Mbit/ s. This task group is working cooperatively with the Of   ce of Engi-

neering Technology ( OET) at the U. S. Federal Communications Commission ( FCC) to identify

potential rule changes for this band that can increase its utility.

The orthogonal frequency division multiplexing ( OFDM) PHY, described in 802.11a, is de   ned to

operate only in the 5-6 GHz Unlicensed National Information Infrastructure ( U-NII) bands in the

United States. This band offers three subbands of 100 MHz each, at 5.15-5.25, 5.25-5.35, and

5.725-5.825 GHz. Some of these bands are available in Europe, with similar regulatory require-

ments, though the standard does not de   ne operation outside of the U. S. The OFDM PHY pro-

vides eight data rates: 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/ s. It uses BPSK, QPSK, 16-QAM, and

64-QAM modulation schemes coupled with forward error correction coding of rates 1/ 2, 2/ 3, and

3/ 4. The OFDM symbol has a period of 4 ms. It uses 48 data subcarriers and 4 pilot subcarriers.

The data subcarriers are all modulated using the same modulation scheme; the pilot subcarriers

are always modulated using BPSK. Products based on the 802.11a standard are not yet commer-

cially available but are expected in 2001.

An 802.11 study group is currently investigating an extension of 802.11a in order to achieve com-

pliance with ETSI regulations for the corresponding frequency bands in Europe. There is signi   -

cant industry support to   harmonize   802.11a with a similar ETSI standard ( ( HIPERLAN/ 2) so

that the market for high speed WLANs is not fragmented in those bands. The study group is inves-

tigating means to allow the two standards to coexist in the same band as well as means to merge

the best of both 802.11 and HIPERLAN/ 2 into a single, new standard. This work is in its early

stages and its outcome is uncertain.

4.2 The IEEE 802.15 Working Group for Wireless Personal Area Networks

The IEEE 802.15 Working Group for Wireless Personal Area Networks ( WPANs   ) < http: / /

ieee802. org/ 15> develops standards for WPANs, which link pervasive computing devices that

may be portable or mobile and could be worn or carried by individuals. Communication with

nearby static devices is also included. The work is exclusively in unlicensed bands ( primarily at

2.4 GHz) , with ranges up to 10 meters and data rates from the kbit/ s range to beyond 20 Mbit/ s.

Low power consumption, small size ( less than 10 ml) and low cost relative to target devices are

primary considerations. The goal is to develop interoperability standards that have broad market

applicability that can coexist with Wireless LANs.

8

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 9

The Group has four authorized projects underway:

° Task Group 1 is developing a standard derived from the Bluetooth   Speci   cation Version 1.1.

° Task Group 2 is developing a recommended practice for Coexistence of WLAN and WPAN

devices.

° Task Group 3 is developing a High Rate WPAN standard supporting at least 20 Mbit/ s for

applications such as digital imaging and multimedia.

° Task Group 4 is developing a Low Rate WPAN standard supporting rates of 2-200 kbit/ s with

extremely low power consumption and complexity for sensors, toys, etc.

4.2.1 History of IEEE 802.15

The chain of events leading to the formation of IEEE 802.15 began in June 1997 when the IEEE

Ad Hoc   Wearables   Standards Committee was initiated during the IEEE New Opportunities in

Standards Committee meeting in June 1997. The purpose of the Committee was to   encourage

development of standards for wearable computing and solicit IEEE support to develop standards.

The Wearables committee met three more times, agreed to focus on Wireless PAN standards, and

decided to approach IEEE 802. IEEE 802.11 welcomed the initiative and launched the Wireless

Personal Area Network ( WPAN) Study Group within 802.11 in March 1998. At the time, no other

WPAN initiatives had been publicized. However, by March 1999, when the Study Group and

802.11 submitted a PAR to 802, the Bluetooth   Special Interest Group ( ( SIG) had over 600

adopter companies and HomeRF   had over 60. . The PAR was approved and placed in the hands

of a new Working Group 802.15. Bob Heile was named Chair and continues in that position. The

Working Group has 74 Members.

The personal area network is a new area that is being addressed by the IEEE.  During the development of the project authorization for the initial Working Group the Sponsor agreed to break from the previous “Single MAC Working Group” reference model and allow for more than one MAC Sublayer to be developed in a single Working Group.  The current projects indicate that there will be at least three (3) MACs and (3) PHYs: one (1)  unique pair for each project with the likely common Service Access Point (SAP) being the IEEE 802.2 LLC.

4.2.2 Task Group 1: WPAN   Based on Bluetooth

IEEE 802.15   s Task Group 1 ( TG1) is deriving a draft standard based on the Bluetooth   Speci   -

cation Version 1.1 under IEEE PAR 802.15.1.

The Bluetooth  technology is a specification for small form factor, low-cost, wireless communication and networking between PCs, mobile phones and other portable devices.

The Bluetooth technology is expected to re-define the personal communications market by expanding the capabilities of mobile devices, making these devices work better together. According to market research firm Cahners In-Stat Group, there will be over 670 million Bluetooth enabled devices worldwide by 2005. Bluetooth technology will provide an easier way for a variety of mobile computing, communications and other devices to communicate with one another and connect to the Internet without the need for wires or cables. Bluetooth technology will also make it easier for users of mobile computers, mobile phones and handheld devices to keep their data wirelessly synchronized.

[EDITORS NOTE TO BE REMOVED – add footnotes as depicted above and/or below.]

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 10

Figure 1 shows the protocol stacks in the Open Systems Interconnect ( OSI) seven-layer network

model ( a standard of the International Standards Organization ( ISO) ) and their relationship to the

Bluetooth   reference model as it pertains to the 802.15.1 standard. .

FIGURE 1. Mapping of ISO OSI model to scope of WPAN   standard

4.2.3 Task Group 2: WPAN Coexistence

Task Group 2 ( TG2) is developing, under IEEE PAR 802.15.2, a draft recommended practice for

coexistence of WLAN and WPAN devices.

.

In the context of this project, multiple wireless devices are said to   coexist   if they can be collo--

cated without signi   cantly impacting their performance. Since the 802.15.1 standard addresses

the same license-exempt 2.4 GHz band as 802.11   s DS and FH PHYs, mutual interference is a

concern when they operate nearby. TG2 is developing a coexistence model that quanti   es the

effect of the mutual interference, with coexistence mechanisms to follow. The   nal model will

consist of four elements: physical layer ( PHY) , medium access control layer ( MAC) , RF propaga-

tion, and data traf   c. The coexistence model is intend to predict the effects of a nearby 802.11 net-

work on the performance of an 802.15.1 network, and vice versa. TG2 also plans to study the

High Rate WPAN being developed in Task Group 3.

10

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 11

4.2.4 Task Group 3: High Rate WPAN

Task Group 3 ( TG3) is developing a standard for a high-rate WPAN under IEEE PAR 802.15.3,

approved in March 2000. The goal is support data rates of at least 20 Mbit/ s for applications such

as digital imaging and multimedia.

In November 2000, six (6) candidate PHYs and four (4) candidate MAC proposals were reduced to a single

working version of the standard. In January 2001, an eight-state trellis-code modulated 16/ 32/ 64-

QAM PHY operating in the 2.4 GHz band was selected. The proposed system provides adaptive

data rates from 22-55 Mbit/ s. The group remains interested in additional

PHYs e.g., operating in the unlicensed 5 GHz band.

4.2.5 Task Group 4: Low Rate WPAN

Following the December 2000 approval of PAR 802.15.4, Task Group 4 held its   first meeting and

began by assessing eight contributions in response to a Call For Applications.

Potential applications include sensor and automation needs,

interactive toys, and location tracking for smart tags and badges.

4.3 IEEE 802.16 Working Group on Broadband Wireless Access Standards

The IEEE Working Group 802.16 on Broadband Wireless Access ( BWA) Standards < http: / /

WirelessMAN. org> develops standards and recommended practices to support the development

and deployment of   xed broadband wireless access systems. It refers to its products as the IEEE

802.16 Family of WirelessMAN   Standards for Wireless Metropolitan Area Networks. . The

Working Group   s projects are co-sponsored by the IEEE Microwave Theory and Techniques

( MTT) Society as well as the IEEE Computer Society.

11

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 12

802.16 is addressing applications of wireless technology to link commercial and residential build-

ing to high-rate core networks and thereby provide access to those networks; this link is some-

times known colloquially as the   last mile,   though the term     rst mile   is more appropriate for

data   owing out of the customer site. 802.16   s work has aimed at a point-to-multipoint topology

with a cellular deployment of base stations, each tied into core networks and in contact with   xed

wireless subscriber stations. The subscriber stations typically include rooftop-mounted antenna/

radio units connected to indoor network interface units, though in some cases both units could be

indoors or both outdoors. Initial work has aimed at businesses, with much of the market focus on

small-to-medium-sized enterprises. Attention has increasingly turned toward residential opportu-

nities, particularly at the lower frequencies.

802.16 has three active projects to develop air interface standards. Task Group 1 ( TG1) is com-

pleting the IEEE 802.16 Standard Air Interface for Fixed Broadband Wireless Access Systems .

This project addresses a PHY to support licensed bands from 10-66 GHz. The document will

include an accompanying MAC. The standard is not yet   nal, but the draft is stable and in Work-

ing Group Letter Ballot.

Task Group 3 ( TG3) is developing a PHY for licensed bands from 2-11 GHz and supporting MAC

extensions. This work is planned as an amendment to the baseline 802.16 standard. The TG1 and

TG3 projects are 802   s only work targeted at licensed bands. 802.16   s newest project, led by Task

Group 4, looks at license-exempt applications in the 5-6 GHz region, sometimes known as U-NII

bands due to their U. S. designation. Again, the plan is for an amendment to the base 802.16 stan-

dard, with a PHY and with MAC extensions.

Finally, Task Group 2 is   nalizing the IEEE 802.16.2 Recommended Practice on Coexistence of

Fixed Broadband Wireless Access Systems . The emphasis is on supporting deployment of systems

built according to the TG1 standard, primarily in the range 23.5 to 43.5 GHz. This is the   rst doc-

ument to have completed 802.16 Working Group Letter Ballot, and publication is anticipated for

the summer of 2001.

Historically, 802.16 was initiated by activity of the National Wireless Electronics Systems Testbed

( N-WEST < http: / / nwest. nist. gov> ) at the U. S. National Institute of Standards and Technology. N-

WEST organized a kickoff meeting at the 1998 IEEE Radio and Wireless Conference ( RAW-

CON) . The group of 45 accepted an invitation to meet along with IEEE 802 in November, and 802

then approved the formation of a Study Group under Chair Roger Marks. That group met twice

and wrote the TG1 PAR. The Working Group   s Session # 1 took place in July 1999. At that meet-

ing, the TG2 PAR was approved by 802; Leland Langston chaired the project. In November 1999,

802.16 created the Study Group which, under the leadership of Brian Kiernan, developed the TG3

PAR that was approved in March 2000. At that time, a Study Group for the license-exempt bands

was set up under Chair Durga Satapathy, who developed the acronym WirelessHUMAN   ( ( Wire-

less High-Speed Unlicensed Metropolitan Area Network) to describe the standard effort. The TG3

PAR was approved in December 2000. The original TG1, TG3, and TG4 chairs remain in place; in

addition, Carl Eklund and Jay Klein serve as MAC and PHY chairs, respectively, in TG1. The

TG2 project is chaired by Phil Whitehead, following a brief stint by Andy McGregor. At the time

of this report, 802.16 has 124 members, 76 Potential Members, and 91 of   cial Observers. Its work

has been closely followed; for example, the IEEE 802.16 web site received over 2.8 million   le

requests in the year 2000.

12

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 13

IEEE 802.16 maintain close working relationship with standards bodies in the International Tele-

communications Union and the European Telecommunications Standards Institute ( ETSI) , partic-

ularly with the HIPERACCESS program of ETSI   s Broadband Radio Access Networks ( BRAN)

project and with ETSI Working Group TM4.

The 802.16 technology is summarized below. Several publications 1 2 provide additional detail.

4.3.1 802.16 MAC

The 802.16 Working Group follows the traditional 802 approach of developing multiple PHY

options supported by a common MAC. The MAC was developed by TG1 along with the original

10-66 GHz PHY. Although the service requirements of the other air interface projects differ, the

original MAC design is   exible enough to, with extensions, support all three projects.

The 802.16 MAC ( Figure 2) draws from the Data-Over-Cable ( DOCSIS 3 ) standard that has been

successfully deployed in hybrid-  ber coaxial ( HFC) cable systems, which have a similar point-to-

multipoint architecture. However, the MAC protocol engine is a new design. It is a connection-

oriented MAC able to tunnel any protocol across the air interface with full Quality of Service

( QoS) support. ATM and packet-based convergence layers provide the interface to higher proto-

cols. While extensive bandwidth allocation and QoS mechanisms are provided, the details of

scheduling and reservation management are left unstandardized and provide an important mecha-

nism for vendors to differentiate their equipment.

An important MAC feature is the option of granting bandwidth to a subscriber station rather than

to the individual connections it supports. This provides the option of allowing a smart subscriber

1. Jeff Foerster and Glen Sater, LMDS Standards Architectural Issues, 2000 IEEE Wireless Communications and

Networking Conference.

2. Brian Petry, Broadband Wireless Access: High Rate, Point to Multipoint, Fixed Antenna Systems, in The RF and

Microwave Handbook , CRC Press, 2001, pp. 2-41 to 2-51.

3. Cable Television Laboratories, Inc. , Radio Frequency Interface Specification ( version 1.1) , Data-Over-Cable Ser-

vice Interface Specifications.

13

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 14

station to manage its bandwidth allocation among its users. This can make for more ef   cient allo-

cation.

The 802.16 MAC is versatile and   exible. For example, it supports several multiplexing and

duplexing schemes; some possibilities are described below. In general, the point-to-multipoint

architecture is implemented with a controlling base station interacting with many subscriber sta-

tions. The downlink from the base station may be channelized and sectorized but, within a chan-

nel and sector, all subscriber stations receive the same signal and retain only messages addressed

to them. The uplink from the subscriber stations is shared, with access assigned by the base sta-

tion.

4.3.2 10-66 GHz Physical Layer ( PHY)

The 10-66 GHz PHY assumes line of sight propagation with little multipath. Either of two basic

modes may be used. The Continuous Mode uses frequency division duplexing ( FDD) , with the

uplink and downlink on separate frequencies simultaneously. A continuous time division multi-

plexed downstream allows a powerful concatenated coding scheme with interleaving. The Burst

Mode allows time division duplexing ( TDD) , with the uplink and downlink sharing a channel but

not transmitting simultaneously. This allows dynamic reassignment of the uplink and downlink

capacity. This mode also allows   Burst FDD,   which supports half--duplex FDD subscriber sta-

tions that do not simultaneously transmit and receive ( see Figure 3) and may therefore be less

expensive. Both TDD and Burst FDD support adaptive burst pro   les in which modulation ( QPSK,

16-QAM, or 64-QAM) and coding may be dynamically assigned on a burst-by-burst basis. This

MAC Sublayer -Common Part

Security Sublayer

PHY Layer

Data/ Control Plane Management Plane

PHY

MAC Management Entity

Network

Management

System

Service Specifics

Management Entity

MAC Sublayer -Common Part

PHY Layer

Management Entity

MAC SAP

PHY SAP

SSCS SAP

Security Sublayer

Convergence Sublayers

Scope of standard

ATM and Packet Convergence Sublayers

FIGURE 2. IEEE 802.16 Reference Model and Protocol Stack

14

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 15

real-time trade-off of capacity versus robustness again offers vendors opportunities to implement

sophisticated algorithms to differentiate their approach while retaining interoperability.

The choice of Continuous or Burst Mode may depend on the available channel allocations and

other regulatory issues. Because the standard is intended for worldwide use, the channelization is

left   exible. Recommendations are included, however. These suggest symbols rates as high as

43.4 MBaud in a 50 MHz channel, which, assuming 64-QAM, translates to data rates as high as

260 Mbit/ s in that channel.

4.3.3 Licensed Bands, 2-11 GHz

802.16   s Task Group 3 has been developing a standard for 2-11 GHz BWA. In the U. S. , the pri-

mary targeted frequencies are in the Multichannel Multipoint Distribution Service ( MMDS)

bands, mostly from 2.5-2.7 GHz. Worldwide, 3.5 GHz and 10.5 GHz are likely applications.

Because non-line-of-sight operation is practical and because of the lower component costs, these

bands are seen as good prospects for residential and small business services. The spectrum avail-

ability is suitable to these uses. The Task Group 1 has considered a number of physical layer

approaches and was scheduled to select a baseline draft in the spring of 2001. MAC enhance-

ments are also under development.

4.3.4 Unlicensed Bands, 5-6 GHz

In order to provide for rapid development, Task Group 4 is working under a narrow charter. It is

tasked to develop a physical layer based on the 802.11a OFDM and/ or HIPERLAN/ 2 PHYs. It is

working closely with TG3 to ensure harmony, and it is developing MAC enhancements. Coordi-

nation of base stations under independent operators in unlicensed spectrum is an important issue

for this group to face. One proposal is to consider an optional mesh architecture in addition to a

point-to-multipoint topology. Some participants have proposed MAC enhancement to support a

mesh; this is testimony to the   exibility of the 802.16 MAC.

frame

Broadcast

Full Duplex Capable User

Half Duplex Terminal # 1

Half Duplex Terminal # 2

FIGURE 3. Example of Burst FDD Bandwidth Allocation

UPLINK

DOWNLINK

15

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 16

5.0 Coexistence and Regulatory Issues Across 802

As 2.4 GHz 802.11 products proliferate, the prospect of a large number of 802.15 products oper-

ating in the same spaces and in the same unlicensed bands prompts signi   cant concern about

coexistence. In the 5-6 GHz bands, products based on the 802.11a OFDM standard are expected

soon. 802.15   s TG3 has had discussions about a PHY in the same bands, and 802.16   s TG4 is

rmly engaged in developing a   xed wireless access standard at the same frequencies. Mean-

while, 802.11 is working with European and Japanese standards groups with the goal of harmo-

nizing the world   s 5 GHz WLAN standards or, failing that, to ensure that the systems can coexist.

These overlaps have brought about discussions of a coexistence coordinating group that would

interact with all of 802   s development projects addressing those unlicensed bands.

The IEEE 802 wireless projects have also increasingly focussed on Radio Regulations. Again, the

unlicensed bands cause the most concern, for here the operation rules strongly impact the allowed

technology and vary from country to country. In March 2000, the SEC assigned regulatory mat-

ters to a Regulatory Ombudsman < http: / / ieee802. org/ Regulatory> and elected Vic Hayes to   ll

this position.

6.0 Applications

The IEEE 802 wireless standards program builds on the success of IEEE 802.3 ( Ethernet) , since

much of the success of the Internet is based on the availability of low-cost Ethernet equipment. In

2000, the Dell   Oro Group projected revenue from Ethernet switch equipment to grow at 20% per

year to over $ 23 billion in 2004. Dell   Oro also projected 10 Gbit/ s Ethernet, the most advanced

Ethernet technology, to reach revenue of $ 1 billion by 2004 and said   One of the most important

trends in networking is the extension of the economics ( i. e. price/ performance) of the LAN to the

metro and wide area ( MAN/ WAN) .

The IEEE 802.11 Wireless LAN standards have begun to emulate Ethernet   s success, showing

that standardization opens new markets. As a recent publication noted,   The curve of wireless

interest over time shows three key turning points. The   rst was the   nalization of the IEEE

802.11b standard for 11 Mbit/ s direct-sequence radios. The rate itself wasn   t the key, though.

Standardization was. This allowed interoperability among vendors and helped bring the cost down

to where it is today, with a bill of materials of less than $ 40 for the radio.   1 This has dramatically

shifted Wireless LAN applications. While the original target was primarily businesses who

wanted to save on LAN installation costs, 802.11 products are now signi   cantly penetrating home

networking markets. Even more signi   cant is surge in public deployments, a trend strong enough

to receive recent treatment in the mainstream press. The New York Times , in a major article on the

topic, wrote   Wireless high-speed Internet access, a longtime dream of the technophile and busi-

ness traveler, is   nally arriving at hundreds of access points in public and private places across the

United States. With a laptop computer equipped with a wireless card, anyone within a few hun-

dred feet or so of one of these access points, or hot spots, can tap into a wireless network that is in

turn connected to the Internet via a broadband connection. The user can then send e-mail or surf

1. P. Mannion, Ad Hoc Networks Ascend to the Airwaves, EE Times , February 26, 2001.

16

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 17

the Web at speeds in the megabit range. . . By late this year, industry experts say, the hundreds of

hot spots will become thousands as service providers and entrepreneurs install the necessary

equipment   generally, , a small transceiver and a broadband connection   in all major airport

terminals, sports arenas and other business and consumer sites. By sometime next year, one com-

pany expects to have access points in 5,000 Starbucks stores. Some of these services may be free,

run by volunteers intrigued by the community-building prospects of wireless networking. . . But

most access points are and will be commercial, run by companies that will charge for the ser-

vices.   1 An accompanying article on the standard 2 said   The protocol called IEEE 802.11b has

been put in place by so many companies offering wireless short-range networks that it is emerging

as the standard for the   eld.   The success of 802.11 products is based on only on the price drops

but also on the increasing ubiquity of the service, a result that is also driven by standardization.

The Times expressed this by quoting one industry manager as saying that this new wireless access

is about   giving you the ability to roam from one network to another and be blissfully ignorant   of

the technical intricacies and quoted a user as saying   It   s the kind of thing that   s such a fundamen-

tal capability that it starts feeding on itself.   With the user of a portable computer now able to

access the Internet at work, home, restaurants, cafes, hotels, and airports, all with the same equip-

ment and all at blazing speed, the Wireless Internet is arriving, and it is based on IEEE standards.

While 802.15-based products are not yet available, they promise to further extend the Wireless

Internet to a wider range of devices.  Specifically, this can be seen in the already released Bluetooth specification v1.1 a.k.a. IEEE Std 802.15.1-2001 which achieves its goal by embedding tiny, inexpensive, short-range transceivers into the mobile devices that are available today, either directly or through an adapter device such as a PC Card. The radio operates on the globally-available unlicensed radio band, 2.4 GHz, and supports data speeds of up to 721 Kbps, as well as three voice channels. Bluetooth devices are available today. Specification compliant radios will cost around $20 initially, eventually falling to around $5. The Bluetooth specification targets power consumption of the device from a "hold" mode consuming 30 micro amps to the active transmitting range of 8-30 milliamps (or less than 1/10th of a watt).

Or the recently announced USB-dongle device targeted initially at PCs, “…a Bluetooth PC card from one manufacturer has connected to other devices as far as 150 feet away. The Bluetooth standard calls for connections to be made between devices located no more than 30 feet away.  The company says it obtains the extra distance by using its ceramic antenna technology. It claims the design gives Bluetooth wireless technology the same range as a wireless 802.11b local area network but with far lower power consumption.”

Or on 1Mar01 a product was announced called Bluetooth Local Infotainment Point, BLIP. “…With BLIP, the Internet moves onto the street and becomes local. BLIP is a stand-alone local platform, which communicates through Bluetooth technology, and enables users to gain access to information through their mobile phone, PDA or other mobile device.  BLIP is a Bluetooth platform for local communication. Information is transferred to and from BLIP through wireless Bluetooth technology, which can support transfer speeds of up to 720 kbps. BLIP communicates with mobile telephones and handheld devices through WAP or Web interfaces. It is equipped with the Linux operating system. The first version of BLIP will have a wide, generalized functionality. Future versions may vary in terms of capacity and design.  Anyone within a range of 10 metres having some kind of Bluetooth product can interact with it. They can download maps, timetables, even music - or show an interest in contact at a later time. The advantage of BLIP is that it adapts itself to any situation.”

Or also on 1Mar01 “…version 1.1 of the Bluetooth specification came out and yet the next version is already in process. This next version is expected to provide transfer speed capabilities of up to 10 Mbit/s and may be available by the end of the year”.   Additionally, "The 2 Mbit/s spec "will be really important for 3G cellular as well as some applications that require audio/video. The 10 Mbit/s spec opens up another set of applications that are more limited in number."

Again, while 802.15-based products are not yet available, it is clear that IEEE is developing standards for the true consumer electronics industry e.g., from mobile telephones and handheld devices to sensors and toys.  The number of units deployed based on the 802.15 standards will be staggering.

If IEEE 802 maintains its record of success in ushering tech-

nology into the economy and into society, then IEEE 802.15 will be the peer-to-peer (P2P) glue or enabling tool to provide connectivity for the proliferating Internet appliances e.g., PDAs, Notebooks, Cell Phones, etc. and 802.11 will provide the wireless LAN connectivity for fixed, portable, and moving stations within a local area.  Finally, 802.16 will be the tool to make   fixed broad-

band wireless MAN access as a mainstream application in the metropolitan area. IEEE 802 will have unleashed the Wireless

Internet.

EDITORS NOTE TO BE REMOVED – I HAVE MODIFIED THE CONCLUSION TO BALANCE THE ARTICLE.  SURELY THE ADOPTION OF 802 WIRLESS STDS (BASED ON END UNITS) WILL BE 802.15>802.11>802.16 or it assumed by this Author that the 802.15.1 will have a SIGNIFICANT share (read measured in end units not dollars) of the adopters of 802 Wireless technology either directly through 802.15.1 or indirectly via the Bluetooth Specifications; ergo my modification to the conclusion.

7.0 About the Authors

Roger B. Marks initiated an effort in 1998 to standardize   xed broadband wireless access within

the IEEE 802 LAN/ MAN Standards Committee. This effort led to creation, in March 1999, of the

IEEE 802.16 Working Group on Broadband Wireless Access, which Marks has chaired since

inception. Marks received his A. B. in Physics in 1980 from Princeton University and his Ph. D. in

Applied Physics in 1988 from Yale University. Following a postdoctoral appointment at the Delft

University of Technology ( The Netherlands) , his professional career has been with U. S. Com-

merce Department   s National Institute of Standards and Technology ( NIST) in Boulder, Colorado,

USA. A Fellow of the IEEE and the author of over 70 journal and conference publications, Marks

has received many awards, including the 1995 Morris E. Leeds Award ( an IEEE Technical Field

Award) . He currently serves as an IEEE Distinguished Lecturer. He participates in many profes-

1. Glenn Fleishman, The Web, Without Wires, Wherever, The New York Times, February 22, 2001.

2. Glenn Fleishman, Protocol Wars: A Standard Emerges, but Watch Out for Those Microwave Ovens, The New

York Times, February 22, 2001.

17

Standards in IEEE 802 Unleash the Wireless Internet March 5, 2001 18

sional activities; for example, developed the IEEE Radio and Wireless Conference ( RAWCON)

and chaired it from 1996 through 1999.

Ian Gifford was born in Framingham, Massachusetts, USA. He received a Bachelors in Econom-

ics from Boston University, Boston Massachusetts, USA, in 1981. He is Director of Standards at

M/ A-COM, Inc. , a Tyco Electronics Company, in Lowell, Massachusetts, USA. For the past 15

years he has worked on wireless solutions for wide area, local area and personal area networks.

Additionally, he is an active member in Standards Development Organizations such ITU-R, TIA,

and Committee T1 as well as a few Industry Special Interest Groups and Alliances including the

Bluetooth   Special Interest Group. . He serves as Vice Chair of the IEEE 802.15 Working Group

for WPANs.

Bob O   Hara is the president of Informed Technology, Inc. , a company that specializes in strate-

gic, technology, and network consulting. He is actively involved in the development of network-

ing, telecommunications and computing standards and products. His areas of expertise are:

network and communication protocols and their implementation, operating systems, system spec-

i   cation and integration, standards development, cryptography and its application, strategy devel-

opment, and product de   nition. Bob has been involved with the development of the IEEE 802.11

WLAN standard since 1992. He is the technical editor that standard and chairman of the revisions

( 802.11rev) and regulatory extensions ( 802.11d) task groups. Prior to starting Informed Technol-

ogy, Bob worked for Advanced Micro Devices in both senior engineering and management posi-

tions for the I/ O and Network Products Division and in the Advanced Development Lab, as well

as engineering positions at Fairchild Space and Communications and TRW Defense and Space

Systems Group. Bob Graduated with a BSEE from the University of Maryland in 1978.

18

Bluetooth is a trademark owned by Telefonaktiebolaget LM Ericsson, Sweden and licensed to promoters and adopters of the Bluetooth SIG.

Tom Siep, “An IEEE Guide: How to Find What You Need In The Bluetooth Spec”, IEEE Standards Information Network, IEEE Press, ISBN 0-7381-2635-7.

TDK Systems http://www.tdksys.com/bluetooth/index.html

Ericsson http://www.ericsson.com/press/20010301-0030.html

Ericsson Components http://www.ericsson.com/bluetooth/