IEEE P802.15

Wireless Personal Area Networks

| Project | IEEE P802.15 Working Group for Wireless Personal Area Networks (WPANs) |  |
| --- | --- | --- |
| Title | IEEE802.15 TG3 MAC Sub-Group Conference Call Minutes |  |
| Date Submitted | 12 March, 2001 |  |
| Source | [Pat Kinney] [Invensys Metering Systems] [Forrest Hills, PA] | Voice: [] Fax: [] E-mail: [pat.kinney@ieee.org] |
| Re: | IEEE 802.15.3 MAC Subgroup Conference Calls |  |
| Abstract | Conference Call Minutes between the Monterey and Hilton Head Meetings |  |
| Purpose | Official minutes of the MAC Subgroup Conference Calls |  |
| Notice | This document has been prepared to assist the IEEE P802.15. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor acknowledges and accepts that this contribution becomes the property of IEEE and may be made publicly available by P802.15. |  |

Tuesday, 30 January, 2001

11:00	Conference call initiated.  9 Attendees:

Allen, Jim		Kodak

Gubbi, Raju		Broadcom

Heberling, Al		Kodak

Kinney, Pat		Intermec

Kubler, Joe		Intermec

Parks, Greg		Sharewave

Schmidl, Tim		TI

Schrader, Mark	Kodak

Shvodian, Bill		XtremeSpectrum

Tan, Teik-Kheong	3Com

11:03CST	Subgroup chair, J Gilb reviewed the agenda for this meeting:

Phone:    1-888-385-5669

Pass Code:     81939

Today's Agenda is:

1) Roll Call

2) Progress Report

a) Project Plan - Greg Parks

b) Clauses

3) Spreadsheet with issues and priorities( spreadsheet will be sent later

this morning)

4) Agenda Input for Ad-hoc meeting in Denver/Boulder(see attached

spreadsheet for time allotments)

5) New Business

Next Con-Call Scheduled for 6 February 2001 at 11:00am CST (9:00amPST,

12:00pmEST)

11:05	Meeting started.

Progress Report by G Parks.  G Parks will need  J Allen's power point document to proceed.

A Heberling reviewed the assignments from Monterey.

| Tasks | Editor | Due Date |
| --- | --- | --- |
| MLME SAP | A Heberling | 5 March, 2001 |
| PLME SAP | R Roberts | 5 March, 2001 |
| PHY SAP | R Roberts | 5 March, 2001 |
| MAC Functional Descriptions |  | 5 March, 2001 |
| Architecture |  | 5 March, 2001 |
| Synchronization | Parks/Gubbi/Kubler | 5 March, 2001 |
| Contention Access Period | P Kinney | 5 March, 2001 |
| Contention Free Period | R Gubbi | 5 March, 2001 |
| Fragmentation | M Schrader | 5 March, 2001 |
| Defragmentation | M Schrader | 5 March, 2001 |
| Power Management | P Kinney | 5 March, 2001 |
| Authentication |  | 5 March, 2001 |
| Privacy |  | 5 March, 2001 |

Discussion ensued as to the duration of Friday's meeting.  Since the airline flights ranged from  5-11 PM should we stop the meeting at 3:00 PM?  All attendees were coming in Wednesday night.  Al suggested that on Thursday we would start at 8:00 AM with a break at 10, lunch at 12  with another break at 3 and then go till 5:30.  Before the end of week we need an agenda since we need to post this agenda to website.

A Heberling reviewed the list of open items from Monterey.

| Voters Initials | Sequence # | Clause Title | Type of Issue (E/e/T/t) |  |  | Issue and reason for considering it an issue |  |  |  |  | Recommended change (What change(s) it would take to resolve this issue?) |  | Disposition /Rebuttal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RR |  |  | T |  | SC-TDMA concern regarding what impact the hidden node problem would have on this access method |  |  |  |  |  |  |  |
| RR |  |  | t |  | PLCP concern regarding embedding addresses in the PLCP |  |  |  |  |  |  |  |
| RG |  |  | T |  | FEC- If FEC is needed can it be moved into the PHY(PMD) |  |  |  |  |  |  |  |  |
| RG |  |  | T |  | SEC Field: Should 15.3 support WEP? |  |  |  |  |  |  |  |
| RG |  |  | t |  | Duration Field: Is duration required? |  |  |  |  |  |  |  |
| RG |  |  | T |  | Addressing: Should PAN addresses be 1 byte, 2 bytes or 6 bytes? |  |  |  |  |  |  |  |  |
| RG |  |  | T |  | SC-TDMA: how would RTS/CTS work with such a scheme? |  |  |  |  |  |  |  |  |
| RG |  |  | t |  | PRNG: Network ID requires a randomization algorithm. |  |  |  |  |  |  |  |  |
| RG |  |  | t |  | MPDU: Maximum length? |  |  |  |  |  |  |  |
| RG |  |  | T |  | Beacon: Does the beacon need to contain slot/cycle information? |  |  |  |  |  |  |  |  |
| RG |  |  | t |  | Beacon: Does there need to be bandwidth allocation information in the beacon body? |  |  |  |  |  |  |  |  |
| RG |  |  | T |  | CAP: how are CAP transmissions kept from extending into the CFP |  |  |  |  |  |  |  |  |
| JG |  |  |  |  | TSF: how is it measured? |  |  |  |  |  |  |  |
| JG |  |  |  |  | TU: Why is it 1024us? |  |  |  |  |  |  |  |
| JG |  |  |  |  | Addressing: How about 128 bit addressing? |  |  |  |  |  |  |  |
| JG |  |  |  |  | Beacon: Does beacon provide a field for indicating channel status? If it does, how often? |  |  |  |  |  |  |  |  |
| Voters Initials | Sequence # | Clause Title | Type of Issue (E/e/T/t) |  | Issue and reason for considering it an issue | Recommended change (What change(s) it would take to resolve this issue?) |  |  |  |  |  | Disposition /Rebuttal |
| JG |  |  |  |  | Beacon: Does beacon provide fields to indicate power saving mode or sleep mode? |  |  |  |  |  |  |  |  |
| JG |  |  |  |  | Beacon: Does the beacon have unused ID fields? |  |  |  |  |  |  |  |  |
| PK |  |  |  |  | Transfer of Coordination: Need more detail. |  |  |  |  |  |  |  |
| PK |  |  |  |  | Authentication: Does every station in a PSS need to authenticate with each PSS member station? Or do the stations just need to authenticate with the coordinator? |  |  |  |  |  |  |  |  |  |
| PK |  |  |  |  | Authentication Key: Is there a single shared private key per PSS? |  |  |  |  |  |  |  |  |
| PK |  |  |  |  | Multiple Data Rates: Shall a PSS support multiple data rates? |  |  |  |  |  |  |  |  |
| RR |  |  |  |  | Hidden Nodes: How do stations hidden from each other communicate with each other? |  |  |  |  |  |  |  |  |
| BS |  |  |  |  | SC-TDMA: Are ACKs part of the packet slot? |  |  |  |  |  |  |  |
| BS |  |  |  |  | RTS/CTS: is it used for both isochronous and asynchronous data streams? |  |  |  |  |  |  |  |  |
| BS |  |  |  |  | % Efficiency: how does SC-TDMA compare with ShareWave's adaptive TDMA? |  |  |  |  |  |  |  |  |
| JB |  |  |  |  | Mini-slot: Need proof that it works with isochronous data. |  |  |  |  |  |  |  |  |
| ADH |  |  |  |  | MAC-SAP: In addition to the 802 defined SAP there is a need for a non-802 dependent SAP. This is for those PAN applications where supporting TCP-IP is not required. |  |  |  |  |  |  |  |  |  |
| ADH |  |  |  |  | MAC-SAP: Need to decide whether the non-802 based SAP will be compliant with an interface into the upper BT layer protocol stack or some other protocol stack. |  |  |  |  |  |  |  |  |  |
| ADH |  |  |  |  | MLME-SAP: Does 15.3 reuse the 15.1 HCI SAP signals and parameters? |  |  |  |  |  |  |  |  |
| JG |  |  |  |  | CCA: Is there a defined requirement for CCA? |  |  |  |  |  |  |  |
| JG |  |  |  |  | Beacons: are they at fixed intervals? Or are they at soft intervals? |  |  |  |  |  |  |  |  |
| Voters Initials | Sequence # | Clause Title | Type of Issue (E/e/T/t) |  | Issue and reason for considering it an issue |  | Recommended change (What change(s) it would take to resolve this issue?) |  |  |  |  | Disposition /Rebuttal |
| JG |  |  |  |  | Time Quantization(TQ) : How accurate must these be? |  |  |  |  |  |  |  |  |
| RG |  |  |  |  | TSF: how accurate must the TSF be? |  |  |  |  |  |  |  |
| RG |  |  |  |  | Beacons: Can the beacon interval be dynamically changed? |  |  |  |  |  |  |  |  |
| AD |  |  |  |  | Inquiry/ Page Modes: Will these be similar to BT or different? |  |  |  |  |  |  |  |  |
| TS |  |  |  |  | Joining/Un-joining: Will this process be similar to BT or different? |  |  |  |  |  |  |  |  |
| AD |  |  |  |  | Multi-Rate support is requested. |  |  |  |  |  |  |  |
| JK |  |  |  |  | Multi-Rate support is requested. |  |  |  |  |  |  |  |

What signal names do we want to use for the MAC SAP? J Barr questioned as to defining the interface compliant to upper layers of Bluetooth.

Action Item: Need time during Boulder for discussion about the reuse of HCI commands as the MLME SAP for 15.3.

The priorities for Boulder were addressed as a way to generate the agenda.  R Gubbi commented that the slot cycle TDMA was the most important.  P Kinney agreed with Raju but furthered that the CAP and synchronization would follow in priorities with fragmentation and defragmentation as the lowest priorities.

Action Item: A Heberling to send out the current issues list according to categorized topics.  A Heberling will start filling agenda with time slot for specific presentations.

New Business:

B Shvodian expressed concern about complexity of the 802.15.3 MAC.  B Shvodian suggested presenting some ideas at Boulder.

Action Item:  B Shvodian to send out an email detailing these ideas this week .

11:45 CST	Meeting adjourned.

Thursday, 8 February, 2001

8:55A	Al Heberling, TG3 MAC chair called the ad-hoc conference to order at the Broker Inn, Boulder, CO.  7 Attendees:

Gubbi, Raju		Broadcom

Heberling, Al		Kodak

Kinney, Pat		Intermec

Kubler, Joe		Intermec

Rypinski, Chan

Schrader, Mark	Kodak

Shvodian, Bill		XtremeSpectrum

9:05	M Schrader lead a discussion on Slot Cycle TDMA (SC-TDMA) with his presentation IEEE802.15-01/061r1.

Issue arose on what the PAN network can be called, is Piconet available?  A Heberling to check.  Are the time slots of a fixed duration or are they dynamic in duration?  M Schrader believed that it would simplify the algorithm if they were fixed. C Rypinski pointed out that:

- we're allocating bandwidth in response to availability rather than channel time in response to service need

- all use of channel should be in continuous block leaving idle time unfragmented

- imperative that the frequency of beacons have an integer relationship to the network isochronous rate, but not phase locked

Question on definition of minislot size Schrader stated it to be 3-6% but was noted that it should be based on PHY parameters such as Rx-Tx, etc.

Note:  the station assigned to the slot may transmit in that slot as may the recipient but the recipient is limited to CTS or ACK/NACK response.

Note: It was noted that the minislot concept causes a problem that a station cannot reserve time in which no transmission is made, i.e. quiet time.  This point will be discussed later.

Question on the necessity of RTS/CTS for the case of  hidden nodes.  P Kinney noted that the probability of hidden nodes would be large.  P Kinney also noted that adding RTS/CTS added overhead but more importantly required the coordinator to be always on, which for a PAN was often an unacceptable assumption.  Question on the proposed quantization of time.  Confusion on the existence of a duration field and to which duration that field stipulates.  The duration of the packet should be in the PLCP header.  The duration required for the remainder of the total exchange is needed in the MAC header.

Discussion on the issue of discrimination between a minislot versus a bad PLCP header.  R Gubbi commented that assuming minislot when an invalid PLCP header was received was a bad policy. J Kubler commented that since the minislot seems to be a CCA, so if bad PLCP header treat it as a minislot.  This point needs to be raised before the committee.  B Schvodian commented that slot*cycles is only a number and the coordinator only need would be better served to assign the stations slots in a linear fashion rather than the matrix fashion implied by slot-cycles. It was agreed that the slot-cycle illustrations in this presentation were confusing , from here on the representation will be a two dimensional matrix with slots, cycles.  M Schrader described a case where slot-cycle TDMA could be devolved into TDMA.

12:30	break for lunch

1:40 	resume meeting

M Schrader continued his presentation.  Significant discussion on allowable jitter for isochronous transmissions.

B Schvodian presented Suggestions for 802.15.3 MAC (IEEE802.15-01/01xxr0)

C Rypinski suggested a dynamic TDMA where changes to occupation time were only tolerated from superframe to superframe

A Heberling initiated a matrix comparing these three concepts.

| Criteria | SC-TDMA | Hybrid | Adaptive TDMA |
| --- | --- | --- | --- |
| Battery Drain |  |  |  |
| Coordinator |  |  |  |
| Station |  |  |  |
| Latency Variance |  |  |  |
| Hidden Nodes |  |  |  |
| BW efficiency |  |  |  |
| Complexity |  |  |  |
| Schedule algorithm |  |  |  |
| Time to Market |  |  |  |
| Channel robustness |  |  |  |

R Gubbi presented his fifteen issues with SC-TDMA.  Significant discussion ensued on these issues.

A Heberling asked for a method to resolve the issues with the CFP. M Schrader proposed a dual mode MAC which either was pure SC-TDMA or TDMA.  The pure SC-TDMA would not have RTS/CTS for the contention period, instead if the hidden node raises its ugly head then the coordinator would switch to TDMA.

Thursday, 8 February, 2001

8:30	Chair called the meeting to order.

Discussion on agenda for Hilton Head: Monday morning review status (meeting minutes, etc.), re-present MAC overview.  Tuesday morning: present MAC draft, working session on gaps.  Tuesday afternoon: vote on MAC draft.

9:30	Discussion on the Contention Access Period. (IEEE802.15-01/1xxr0)

R Gubbi argued that the CAP should be at the end of the end of the superframe to make any  unused bandwidth available to CAP.  J Kubler and P Kinney argued against moving the CAP to the end since it added complexity and could cost current drain in some stations.  They argued that moving the CAP to the end did not necessarily add bandwidth to CAP users since it would require these stations to stay awake for the whole CFP which is not acceptable to many low powered portable stations.

C Rypinski commented that any available time resulting from non-used CFP time should be made available is for foreign systems as well.  C Rypinski was against fragmenting the unused/available time.  He believed that it was improper to have unused channel time not available for use and that this time should not be fragmented.  I.e. argues for it to be at the end.

Action Item:  MAC group needs to address Dynamic Frequency Selection (DFS)

Discussion ensued as to requiring the awake window.  Should this be put in the data frame or should it be put in management frames?  How often is this changed?  J Kubler believed to put into contract between stations(at authentication?).

Point raised what functionality should the coordinator have to advised participants of other co-located PANs? This would assist in coexistence.  Consensus was none, this suggestion would add too much complexity.

There was a significant opinion that the beacon should contain durations for the CAP, superframe, and the CFP.  By allocating CAP and CFP durations that are less than the superframe, silence will be assured for coexistence.  R Gubbi did not agree with this but did agree that the coordinator should be able to specify a time allocation where no station in the PAN will transmit.

Upon reviewing the schedule it was noted that authentication and encryption had not been addressed.  It was clear that there were no participants in the MAC committee who believed they were capable of drafting this standard.  J Kubler suggested that this group pick up the 802.11e security sub-group proposals for 802.1X authentication and AES security, not the WEP proposal.  R Gubbi commented that 802.11e would have a draft proposal by the next meeting.  It was commented that adopting the 802.11 authentication and security methods would allow this group to continue on the MAC design since the 802.11e methods need to interface to the existing 802.11 MAC, i.e. we know the 802.11 interface.  Consensus was to recommend the above mentioned 802.11e proposal to the task group. R Gubbi suggested that if the TG doesn't agree then we need to put out a CFP for authentication and encryption.

Discussion on SAPs.  It was noted that we need to support a multimedia SAP but it wasn't necessary to define an HCI SAP.  The point of a location awareness SAP was addressed, it was decided that this effort would delay the standard unnecessarily.

12:45	Meeting broke for lunch.

1:30	Meeting reconvened.  Discussion resumed on the CFP protocol.  The proposals for the protocol on the table are:

- Hybrid D-TDMA, SC-TDMA

- D-TDMA

- SC-TDMA

- Switched SC-TDMA/D-TDMA

A straw poll was taken to show support for these protocols the vote results were: 4 votes for 1, 2 votes for 2, 4 votes for 3, and 1 vote for 4.  It was commented by a number of individuals that they could not make a decision today, rather they needed to analyze it further.

J Kubler proposed that this group proceed with the hybrid and  if the SC-TDMA could be made to overcome the objections pointed out then the Fixed Assignment period could be stripped off.  Moved by J Kubler, seconded by P Kinney.  Vote was 5/1/1.  A Heberling noted that we would need a more complete presentation on this protocol. B Schvodian will be responsible for this presentation.   It must be ready in time for review by the MAC subcommittee on the last call before Hilton Head.  B Schvodian will send it out in two weeks.

3:00	Discussion of Hilton Head Agenda.  Major changes were to change Monday morning to a MAC overview,  Tuesday morning changes to a review of the MAC draft, and the vote moves from Monday evening to Tuesday evening.

Tuesday, 13 February, 2001

11:05 CST

Attendees:

Gubbi, Raju		Broadcom

Heberling, Al		Kodak

Kinney, Pat		Intermec

Siep, Tom		TI

Schrader, Mark	Kodak

Welborn, Matt		XtremeSpectrum

Tan, Teik-Kheong	3Com

11:07 CST	Meeting commenced

Agenda:

- Roll Call

- Old Business

- Review Hilton Head agenda ... to be sent via a later e-mail as an attachment.

- Results of MAC ad-hoc meeting in Denver/Boulder, CO.

- Progress Reports

- MAC Editing Project Plan - G. Parks

- MAC Management SAP - Roberts

- PHY Management SAP - Roberts

- Sub-clauses of MAC function description

- Synchronization:		Parks/Gubbi/Kubler

- Contention Access Period:	Kinney

- Contention Free Period:	Gubbi/Shvodian/Schrader

- Power Management:	Kinney

- Authentication/Privacy:	?

- New business

Results of MAC ad-hoc meeting in Boulder, CO: Three presentations: M Schrader on SC-TDMA, B Shvodian on a Hybrid using SC-TDMA and D-TDMA, and P Kinney on the Contention Access Period.  Group adopted the hybrid proposal as a means to move forward.  The group suggested that the 802.11e effort on authentication and encryption be leveraged into the 802.15.3 standard.  Finally discussed the SAPs, support for the 802.2 interface, and maintain the current MLME SAP as defined in 802.11.  On the subject of SAPs, T Siep mentioned that requests for approval from the BT SIG need to be specific, not general.  T Siep believed that the BT SIG wants to work with the IEEE on the issue of interfaces.  Suggested that we draft an interface definition and submit that draft to the SIG for approval.

Hilton Head Agenda: Monday will be MAC overview and review of issues identified .  Tuesday morning to review the MAC draft.  Wednesday will be resolution of any additional issues and then a vote.

Progress Reports:  Main issues from TG3 perspective are the target dates that the standard needs to meet.  Action Item: A Heberling to work with G Parks on project plan.  Synchronization: Gubbi is working with Kubler
CAP: Kinney is working on this document
CFP: editors are Shvodian, Gubbi, Schrader.  
Power Management: Kinney is working on this document.  M Schrader raised concerns about the criteria used to assess the proposals put forward.
Authentication/Privacy:  suggested that we attend the 802.11e presentation on this subject

New Business:

Issue of association and reassociation:  T Siep suggested use of the 802.11 terminology.  Needs further discussion on the point of reassociation.

MIBs:  does PAN have to support the data structure of MIBs.  T Siep commented that a PAN should not support MIBs.  Consensus seems to lean away from formal management implied by MIBs.

TG1:  T Siep requested the attendees to vote as early as possible.

12:05 CST 	Conference adjourned.

Tuesday, 20 February, 2001

The agenda for the con-call is:

- Roll Call

- Progress Reports

- MAC Editing Project Plan - G. Parks

- MAC Management SAP - Roberts

- PHY Management SAP - Roberts

- Sub-clauses of MAC function description Clause

- Synchronization ... Parks/Gubbi/Kubler

- Contention Access Period ... Kinney

- Contention Free Period ... Gubbi/Shvodian/Schrader

- Power Management ... Kinney

- Authentication/Privacy ... ?

- New business

- Adjourn

11:05 CST

Attendees:

Alfvin, Rick		Kodak

Bain, Jay		Time Domain

Gubbi, Raju		Broadcom

Heberling, Al		Kodak

Kinney, Pat		Intermec

*Siep, Tom		TI

Schrader, Mark	Kodak

*Welborn, Matt	XtremeSpectrum

*Tan, Teik-Kheong	3Com

11:07 CST

MAC editing project plan.  G Parks not present to report

Action Item: A Heberling to call G Parks about MAC editing project plan.

MAC Management SAP MAC management SAP from R Roberts is underway, but he has questions on additional signals.

Action Item: A Heberling to send R Roberts a copy of 802.11 TGe group's new signals needed for MLME needed for multimedia streams.

Sub-clauses of MAC function description Clause (use draft template on Web, needs correct copyright release on the front page.)

- Synchronization : not done Raju to prioritize the CFP period clause which should be available prior to the 2/27 call, Raju will try to have the synchronization clause ready for the March 5 timeline.

- Contention Access Period: not done yet, P Kinney to send out preliminary draft by 2/26/01.

- Power Management:  not done yet, P Kinney to send out preliminary draft by 5 March.

- Clause 9: Raju working on some aspects of this clause

- Authentication/Privacy: Jesse Walker will present 802.11e's efforts on 01/018r2 at Hilton Head, time is TBD.

Action Item: A Heberling to send out a table of contents with titles and document numbers.

Authentication: R Gubbi commented that there could be a problem in transferring the coordinator with the 802.11e scheme, this would be a good point to ask Jesse.

11:31 CST	call adjourned.

Tuesday, 27 February, 2001

Secretary: James Gilb

11:12 CST Meeting  called to order by Alan Heberling, 10 attendees

Roll Call: Tom Seip, Greg Parks, James Gilb, Bill Schvodian, Jay Bain, Darrel Diem, Alan Heberling, Mark Schrader, Rick Roberts, Raju Gubbi (joined late)

Agenda:

1) Roll Call

2) Progress Reports

a) MAC Management SAP - Roberts

b) PHY Management SAP - Roberts

c) Sub-clauses of MAC function description Clause

- Synchronization ... Parks/Gubbi/Kubler

- Contention Access Period ... Kinney

- Contention Free Period ... Gubbi/Shvodian/Schrader

- Power Management ... Kinney

- Authentication/Privacy ... ?

3) New business

a) Table of Contents per J. Gilb e-mail

4) Adjourn

Heberling added the Sydney venue to the new business.

Roberts reported on the PHY SAP.  The PLME is in good shape because he knows about it.  It is based on the 802.11 text capturing the spirit of .15.3.  The MLME is in not as good of shape, perhaps many of the things have not been discussed.  Roberts suggested submitting the document as-is for baseline.  Seip suggested that the WPAN management should be different from the WLAN management.  Schrader commented that is an open issue, Seip agreed.

Seip wanted to clarify that things in the baseline should not have controversial items.  Heberling asked Seip to identify what things in the MLME were controversial.  Seip felt that the MIB oriented MLME (ala .11) was inappropriate for a WPAN device.  Heberling said that .11`s MIB had many items that were only for initialization.  He suggested that there were specific PHY attributes that needed to be made available but that would not necessarily be managed from a remote station.

Roberts said that .11 used Primitives (like function calls) with parameters associated with them.  He felt that the parameters were the most controversial.  He also felt the terminology of the parameters are not appropriate for the WPAN.  Roberts felt that the primitives were high level and could be agreed to, but that perhaps the parameters might be removed.

Seip preferred that more information would be in so that it could be removed in the meeting.  Heberling said that some items would be set by the characteristics of the piconet.

Gilb suggested that Roberts put in his best pass, removing any obvious non-WPAN items.  Seip suggested a "flower-box" that indicated that this section was intended to start discussion and heavy editing was to be expected.

Heberling began discussion of clause 7 and 9 submitted by Gubbi (not present on the call) 01034r1 (clause 7, Frame Formats) 01121r0 (clause 9, MAC Functional Description, MACFD).  Schrader had comments on MACFD, the contention period precedes the beacon (Tampa agreement was that it followed the beacon),  Seip indicated that following the beacon was important for power saving.  Schrader said that the beacon can set the duration of this time.  Gilb asked if FF had been approved by MAC committee consensus vote, Heberling said it had not been approved.  Heberling indicated that they would not be able to have MACFD reviewed by Feb 28.

Gilb said that he would take FF and MACFD as is, the MAC committee will take a consensus vote on the next meeting March 6.  The result of the evaluation will be passed on the voters via the MAC committee report.  Action Item Heberling to send out announcement of vote.

Heberling discussed the Jesse Walker tutorial on Monday evening.  Bain made the comment that he thought that the discussion should be 90 minutes long.  Action Item:  Jim Allen to verify that Jesse Walker had been invited.  Action Item:  Alan H. should tell Allen to ask Walker how long for the presentation and if 6:30 pm is OK and pass an email to John Barr requesting an change to 6:30 pm to 8:00 pm.

New Business

Gilb sent an outline of the TOC for the draft.  Heberling asked for suggestions.  Clause 7 will be Layer Management with MLME, PLME and MIB.

Gilb said that 10. QoS policies had been requested to be moved to 9, it is still open for discussion.  Schrader felt that the issue of how it was managed vs. how it was used might have it put into two different.

Seip had to leave, said that it was unlikely that TG1 would attend.  Ian Gifford was willing to host it.

Heberling took a straw vote on how many are in favor of Sidney, none said yes.  He took another straw vote on how  many would be in favor of a US site.  Bain suggested that we should pick a low cost site in the US.  Raju wanted to clarify if there was a change in site, would it imply that .11 would meet in a different site.  Gilb said that Seip indicated that both groups would try to move the meeting.

How many would be opposed to an inexpensive venue in the US, non in favor of the proposal.

Action Item:  Heberling will send an email to Heile with the results of the straw vote and request change of venue, indicate that Ian Gifford volunteered.

Gilb asked who was responsible for 6) MAC Service, answer is Heberling.

Gubbi asked if there was any comments on clause 9.  Heberling said that there were too many comments to go over in this call.  Heberling reviewed the process that FF and MACFD.

Gilb reviewed how the document would go together.

Gilb said that the draft clauses should only contain items that have consensus by the committee.  Items that are TBD should be indicated as such and added to the committee's issues list.  Items that are contentious should be noted as such, ideally with all competing proposals included.

Gilb said that for issues that are nonresolveable in the committee, the committee should take competing proposals, schedule time for presentation to the TG and have TG vote on the proposals.  This process is similar to the PHY coding resolution.

Heberling asked Parks for the status of the project plan.  Parks said that he hadn't done 
"a damn thing" on it.  He hoped to have an update out by Friday, March 2.

Gubbi asked for the process for document submission.

Gilb:  The process is that for all major contributions should submissions to the IEEE, which means copyright release.  They should also be submitted to the editor or appropriate sub-editor.  He clarified that small contributions do not require separate submissions.

Seip asked if the Tuesday conference call was appropriate for issue resolution.  Heberling said that it was the appropriate time.  Gubbi indicated that email was OK for comments as well.

Heberling asked for more new business

12:16 CST	Meeting adjourned.

Tuesday, 6 March, 2001

The MAC con-call scheduled for Tuesday, 06-March-2001 will use these

numbers:

Toll Free Dial in Number:		1-800-403-2022

International Access/Caller Paid Number:	865-673-6715

Participant Code:	908659

The con-call will start at 11:00 CST ( 9PST, 12 EST).

The agenda is:

Roll Call

Progress Status

Overview Clause  	Pat Kinney

MAC SAP Clause	Allen Heberling

MLME SAP Clause	Rick Roberts

MAC Frame Formats	Raju Gubbi

MAC Functional Description	Raju Gubbi/Mark Schrader

MAC Committee Approvals for Clauses

Overview Clause

MAC SAP Clause

MLME SAP Clause

MAC Frame Formats

MAC Functional Description

New Business

Issues arising out of Approvals

Implications for Hilton Head meeting

Adjourn

Attendees: T Siep, Al Heberling, P Kinney, B Schvodian, R Roberts, J Bain, J Gilb, R Gubbi

Al started off the call by mentioning that we may need another conference call this week to determine what clauses are done and accepted and what clauses are still contentious.

Progress:

Overview: not done yet. P Kinney will send a copy of the draft to J Gilb so that James can work his magic on it.

MAC SAP:  J Gilb didn't get a copy of this clause from A Heberling, Al will forward this document to James.  Data interface to LLC

MAC MLME: James to combine this with ASP into one clause.

MAC Frame Formats:  Raju sent it to James.

MAC Functional Description:  Raju

J Gilb stated that the overview clause is basically empty, P Kinney needs to send to MAC committee for review.

MAC SAP clause that was presented at Monterey, is it sufficient to go into the baseline?  J Gilb asked that any inconsistencies and issues be resolved before submission.

MLME clause from R Roberts looks more like 802.11 than .15.  It is roughed in but needs to be refined.  It has been submitted to James as clause 6.

It was agreed that we will strike all references to MIBs since it isn't appropriate for PANs, instead we'll use PIBs and state the differences.

MAC frame format: Al asked if there were any changes before recommendation to baseline?  J Gilb had issues that have been noted in the draft as editorial notes.

MAC functional description: J Gilb noted that the coordinator selection process needs additional work,  e.g. change all times to variables.

Al Heberling suggested that by Friday afternoon we need comments on which clauses need to be included in baseline.

PHY issues for the MAC committee:  Channel assessment slot for sending: what are the problems with peer to peer selecting data rate?  How to negotiate data rates?

Holes that need to be filled by MAC committee: Authentication and privacy hope to use 802.11e, messaging is not complete, etc.

A Heberling requested that the committee quickly go through documents and respond by email to Al if they will support the recommendation to the TG.  Al asked the question of do we go with the compromise or do we present to TG3 that we need to make a decision between TDMA?  Raju commented that he believed that we should move to the baseline with the compromise and then have an ad-hoc meeting before Orlando to resolve between the two, or do a letter ballot.  T Siep suggested that we try to separate the agreed upon clauses and get those approved and then get on the contentious issues in meeting sessions at Hilton Head.

Al noted  that the schedule for Hilton Head is still valid.  On Monday morning we will advise TG3 which clauses we are comfortable with and which are still in the air.  We'll use the rest of the week to resolve the contentious issues.

New Business:  we need CCA according to the current definition of the MAC but do we need RSSI?  This will be the final call until Hilton Head.  All further communications will be email.

12:15 CST call adjourned